3GPP TSG-RAN WG4 Meeting # 95-e 												R4-2009027

Electronic Meeting, 20 – 30 Apr., 2020

Agenda item:			6.8.1, 6.8.2.1.1, 6.8.2.1.3, 6.2.1.5

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [95e][215] NR_pos_RRM_1

Document for:	Information

Introduction

The scope of this email discussion includes the following agenda items:

- AI 6.8.1: General

- AI 6.8.2.1.1: PRS-RSTD measurements

- AI 6.8.2.1.3: UE Rx-Tx timing difference measurements

- AI 6.8.2.1.5: Link-level evaluations for PRS-RSTD and PRS-RSRP measurements

In providing comments, companies are encouraged to:

- Ensure that the comments are inserted in the latest version of the document by checking the folder before uploading

- Use “Track changes” to help identify added comments/changes

- Append the company name and round number before uploading

Topic #1: General Aspect

The topic is for the possible down-scoping of the WI, as well as CRs for the structure of the positioning measurement requirements in 38.133.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006562 | Intel Corporation | Proposal 1: In order to meet the target timeline, the aspects of Cat 2 below can be deferred to FFS beyond Rel16. - Scheduling restriction in FR1: FFS - Impacts of NR positioning on RRM requirements - New gap pattern (e.g. MGL>6ms) : FFS - Gap sharing between Rel15 RRM and positioning measurements: FFS - BWP switching impacts on PRS measurement: FFS - Impact of concurrent RRM/PRS processing/measurement: FFS. |
| R4-2006560 | Intel Corporation | We made the following modifications on the spec, - Add new sections for measurements for NR positioning in 9.8.x |
| R4-2007955 | Ericsson | Introducing requirements structure for positioning measurements in section 10 |

Open issues summary

Sub-topic 1-1 Possible down-scoping of the WI

- Option 1: In order to meet the target timeline, the aspects of Cat 2 below can be deferred to FFS beyond Rel16. (Intel)

  - Scheduling restriction in FR1: FFS

  - Impacts of NR positioning on RRM requirements

    - New gap pattern (e.g. MGL>6ms) : FFS

    - Gap sharing between Rel15 RRM and positioning measurements: FFS

    - BWP switching impacts on PRS measurement: FFS

    - Impact of concurrent RRM/PRS processing/measurement: FFS

- Option 2: others, including no down-scoping

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 1-2 Structure of the positioning measurement requirements in 38.133

Please provide your comments to the CRs directly in section 1.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub topic 1-1: These topics are being discussed. We prefer to continue discussing them under their respective topics/subtopics. |
| Qualcomm | Support option 2. We don’t share the same view as Intel that the issues in the list are not urgent. It is also common understanding that R16 WIs will get at least another quarter of extension. |
| Intel | For the items to be down scoped, we can defer it after 1st round discussion in the corresponding topics. Then we can justify and identify more or less issues which are possible or impossible to be completed before June RAN plenary meeting. To Qualcomm’s comments: so far we have no any confirmation from RAN to extend Rel16 WI. Before that, we still target to complete this WI by June. |
| Huawei | Maybe we can check this proposal latter to see the progress the on topics listed. |
| MTK | Agree with Ericsson’ view |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006560 (Intel) | Company A |
|  | Company B |
|  | Qualcomm: instead of DL RSTD, we suggest to use PRS-RSTD because in R17, RSTD based on signals other than PRS will be defined so we need to future proof it. Also, the word DL is not necessary since it is already known that PRS is a DL reference signal. Same for DL PRS-RSRP. Suggest to change it to PRS-RSRP. |
|  | Intel: the terminology (including DL RSTD and others) has been discussed in previous meeting. The agreements of this are based on RAN1’s definition of these physical measurement metric[38.215],e .g. - “5.1.29 DL reference signal time difference (DL RSTD)” |
|  | Huawei: is this structure already added in the spec with Ericsson CR R4-2004677 in last meeting? We understand this CR is not needed. |
| R4-2007955 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: Same comments as for 6560 |
|  | Huawei: in this meeting, there are CRs from Ericsson and CATT introducing the report mapping for UE measurement, which also includes the structure for section 10. This CR is not needed. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic#1 | Sub-topic 1-1 Possible down-scoping of the WI Tentative agreements: None Candidate options: - Option 1: In order to meet the target timeline, the aspects of Cat 2 below can be deferred to FFS beyond Rel16. (Intel) - Scheduling restriction in FR1: FFS - Impacts of NR positioning on RRM requirements - New gap pattern (e.g. MGL>6ms) : FFS - Gap sharing between Rel15 RRM and positioning measurements: FFS - BWP switching impacts on PRS measurement: FFS - Impact of concurrent RRM/PRS processing/measurement: FFS - Option 2: continue the discussions on the issues and decide later (Ericsson, QC, HW, MTK) Recommendations for 2nd round: Not to be discussed further. Continue the discussion on these issues and make decision whether each individual issue will be still pursued in Rel-16 or deferred to Rel-17 under the respective topics. |
| --- | --- |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #2: Intra/inter-frequency definition

The topic is for intra/inter-frequency definition for RSTD and Rx-Tx time difference measurements. Also, the applicable scenarios for which requirements are to be defined is also addressed.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006018 | ZTE Corporation | Proposal 1: RAN4 defines intra-f and inter-f PRS-RSTD measurements while the requirements are made mainly based on if the measurement requires a MG or not. Proposal 2: Intra-frequency RSTD measurement: the center frequency of PRS BW is the center frequency of an serving cell SSB and has the same SCS as that of the serving cell SSB, otherwise it is inter-frequency. |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Proposal 1. Intra-frequency RSTD measurement is defined as when the neighbor DL PRS resource and the reference DL PRS resource belong to the same positioning frequency layer. Otherwise, the RSTD measurement is inter-frequency. - Define intra-frequency RSTD requirements for the scenarios - SCS and CP of the positioning frequency layer is the same as the active BWP of UE - BW of positioning frequency layer is within the active BWP of the UE Proposal 1. UE Rx-Tx timing difference measurement is defined as intra-frequency if PRS resources of all the Rx-Tx time difference measurements on the same TRP belong to the same positioning frequency layer. Otherwise, the UE Rx-Tx timing difference measurement is inter-frequency. - Define intra-frequency Rx-Tx time difference measurement requirements for the scenarios - SCS and CP of the positioning frequency layer is the same as the active BWP of UE - BW of positioning frequency layer is within the active BWP of the UE - Positioning SRS resource is on the same band as positioning frequency layer |
| R4-2006232 R4-2006234 | CATT | Proposal 3: Intra-frequency RSTD measurement is defined when the DL PRS resources to be measured, including reference cell and neighbor cell, have the same center frequency, SCS and CP type as serving cell and the BW of these PRS are all within the active BWP. Otherwise, the RSTD measurement is inter-frequency measurement. - Note: If UE support mixed SCS, the same SCS is not requested. Proposal 9：The definition of intra/inter RSTD measurement can also be used for UE Rx-Tx time difference measurement. |
| R4-2006304 | MediaTek inc. | Proposal 2: Intra-frequency RSTD measurement is defined when neighbor DL PRS resource and reference DL PRS resource belong to the same positioning frequency layer and the BW of the positioning frequency layer is within the BW of UE’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. |
| R4-2006556 | Intel Corporation | Proposal 2: Regarding to the consistent definitions for NR measurement, the intra/inter frequency PRS measurement can be defined as: Definition; Need gap or Not. intra-frequency; DL PRS resources of all measured reference/neighbor/serving cells/TRPs have - same SCS and CP type - same centre frequency - same point-A - same configured PRS BW; NO: If BW of PRS to be measured is contained in the active BWP.. Yes: If BW of PRS to be measured is not contained in the active BWP.. inter-frequency; DL PRS resources of the measured neighbor/reference/serving cells/TRPs is different in any of - SCS and CP type - centre frequency - point-A - configured PRS BW; Yes. |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 1: Intra-frequency RSTD measurement is defined as when the neighbor DL PRS resource and the reference DL PRS resource belong to the same positioning frequency layer. Otherwise, the RSTD measurement is inter-frequency. The intra/inter definition for RSTD, which is addressed in our companion paper [2], can also apply for Rx-Tx time difference. |
| R4-2007944 R4-2007943 | Ericsson | Proposal 2: Intra/inter-frequency RSTD measurement definitions are as in Table 1. Definition; Need for measurement gaps Note 1. Intra-frequency; - The center frequency of PRS BW is the center frequency of a serving cell SSB - The SCS PRS is the same as that of a serving cell SSB NOTE: for RSTD, the above conditions are met for both reference and the other DL link; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Inter-frequency; if at least one of the two conditions above is not met NOTE: for RSTD, this applies for at least one of the reference and the other DL link; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. NOTE 1: For RSTD configured for a reference link and the other DL link, measurement gaps may be needed for both DL links, for one of the two DL links, or for none of the two DL links, depending for which of the two DL links the conditions are met.. Proposal 3: The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not: - not needed: Measured PRS bandwidth is fully within the active BWP of the UE - needed: Measured PRS bandwidth is not fully within the active BWP of the UE For RSTD configured for a reference link and the other DL link, measurement gaps may be needed for both DL links, for one of the two DL links, or for none of the two DL links, depending for which of the two DL links this condition is met. Proposal 4: Measurement gaps applicability for PRS RSTD: - At least all Rel-15 measurement gap configurations for NR are also applicable for OTDOA measurements (this has been already agreed for NR E-CID measurements) - FFS: Additionally, new measurement gap patterns may need to be specified - Measurement gaps applicability is clarified for OTDOA measurements in Section 9.1.2. Proposal 14: Intra- and inter-frequency UE Rx-Tx measurements are defined according to the table below. Definition; Need for measurement gaps. Intra-frequency; - The center frequency of PRS BW is the center frequency of a serving cell SSB - The SCS PRS is the same as that of a serving cell SSB; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Inter-frequency; if at least one of the two conditions above is not met; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Proposal 15: The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not: - Not needed: Measured PRS bandwidth is fully within the active BWP of the UE - Needed: Measured PRS bandwidth is not fully within the active BWP of the UE Proposal 16: Measurement gaps applicability for UE Rx-Tx: - At least all Rel-15 measurement gap configurations for NR are also applicable for UE Rx-Tx measurements (this has been already agreed for NR E-CID measurements) - FFS: Additionally, new measurement gap patterns may need to be specified - Measurement gaps applicability is clarified for UE Rx-Tx measurements in Section 9.1.2. |

Open issues summary

Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement

Option 1/2/2a/3/4 are from WF R4-2005378 in RAN4#94-e-bis. Option 2b is from MTK paper R4-2006304.

To MTK: In the discussion there is a statement to support option 2, but the proposal is different from option 2. Could you please check which one of option 2 and 2b is your preference?

- Option 1. Intra-frequency RSTD measurement is defined as when the neighbor DL PRS resource and the reference DL PRS resource belong to the same positioning frequency layer. Otherwise, the RSTD measurement is inter-frequency. (QC, Intel, HW)

- Option 2. Intra-frequency RSTD measurement is defined when neighbour DL PRS resource and reference DL PRS resource belong to the same positioning frequency layer, and the BW of the positioning frequency layer is within the BW of UE’s active DL BWP, and SCS and CP of the positioning frequency layer are the same as those of  UE’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. (MTK)

- Option 2a. Intra-frequency RSTD measurement is defined when the DL PRS resources to be measured, including reference cell and neighbor cell, have the same center frequency, SCS and CP type as serving cell and the BW of these PRS are all within the active BWP. Otherwise, the RSTD measurement is inter-frequency measurement. (CATT)

- Option 2b. Intra-frequency RSTD measurement is defined when neighbor DL PRS resource and reference DL PRS resource belong to the same positioning frequency layer and the BW of the positioning frequency layer is within the BW of UE’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. (MTK)

- Option 3. Intra-frequency RSTD measurement: the center frequency of PRS BW is the center frequency of a serving cell SSB and has the same SCS as that of the serving cell SSB, otherwise it is inter-frequency. MG may be needed for intra- or inter-frequency, depending on whether or not the PRS BW is within the active BWP (NOTE: for RSTD, the above conditions are met for both reference and the other DL links) (ZTE, Ericsson)

- Option 4. Intra-frequency PRS measurement is defined when the BW of the PRS layer is confined within the UE active BWP. Otherwise, the PRS measurement is inter-frequency. (HW)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements

- Option 1. Define intra-frequency RSTD requirements for the scenarios (QC)

  - SCS and CP of the positioning frequency layer is the same as the active BWP of UE

  - BW of positioning frequency layer is within the active BWP of the UE.

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 2-3 Intra/inter-frequency definition for UE Rx-Tx time difference measurement

- Option 1. UE Rx-Tx timing difference measurement is defined as intra-frequency if PRS resources of all the Rx-Tx time difference measurements on the same TRP belong to the same positioning frequency layer.  Otherwise, the UE Rx-Tx timing difference measurement is inter-frequency. (QC)

- Option 2. Same definition as for RSTD measurement. (CATT, HW)

- Option 3: similar to PRS-RSRP (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 2-4 Applicable scenarios for UE Rx-Tx time difference measurement requirements

- Option 1. Define intra-frequency Rx-Tx time difference measurement requirements for the scenarios (QC)

  - SCS and CP of the positioning frequency layer is the same as the active BWP of UE

  - BW of positioning frequency layer is within the active BWP of the UE

  - Positioning SRS resource is on the same band as positioning frequency layer

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement We prefer the approach similar to what was agreed in RAN4#94-e-Bis for intra-frequency definition of CSI-RS measurements. We proposed Option 3 but it can be compromised to: - The intra-frequency RSTD definition is based on the relation between SCS, CP, and center frequency of the serving cell and the PRS of each of the reference cell and measured cell: for intra-frequency RSTD they are the same for the reference cell and they are the same for the measured (“neighbor”) cell. - FFS: SCS of the serving cell Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements RAN4 will define requirements with and without measurement gaps, for intra- and inter-frequency measurement requirements. No need to define different scenarios proposed in Option 1 in the requirements, but this can be further discussed for test case configurations. Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Similar to PRS-RSRP (since RSTD is based on two DL links, while UE Rx-Tx is based on one DL link like PRS-RSRP, in addition to UL link): - The intra-frequency UE Rx-Tx definition is based on the relation between SCS, CP, and center frequency of the serving cell and the PRS of the measured cell: for intra-frequency UE Rx-Tx they are the same. - FFS: SCS of the serving cell Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements RAN4 will define requirements with and without measurement gaps, for intra- and inter-frequency measurement requirements. No need to define different scenarios proposed in Option 1 in the requirements, but this can be further discussed for test case configurations. |
| ZTE | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement We still prefer Option 3. We can accept the assumption that intra-f measurements might require MG. Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Prefer Option 2. |
| Qualcomm | Sub-topic 2-1: We can compromise to option 1 but define intra-frequency requirements under certain conditions (sub topic 2-2) which is a similar approach that was agreed for CSI-RS based L3 measurement. Sub-topic 2-2: This is our condition for agreeing in option 1 for sub-topic 2-1. This is not any different from CSI-RS based L3 measurement definition of intra-/inter-frequency so we think this is a reasonable compromise. Sub-topic 2-3: We cannot agree to option 2 because RSTD is defined based on a reference and measured TPR whereas UE Rx-Tx is not. So we proposed option 1, which considering how we have proposed the intra-frequency definition for PRS-RSRP, is the same as option 3. However, our definition of intra-frequency is different from Ericsson’s. Sub-topic 2-4: Same comment as for sub topic 2-2. In particular for positioning SRS, we believe the simplest case (SRS and PRS to be in the same band) should be applicable for intra-frequency. |
| Intel | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement The most important concerns to define such intra/inter measurement is the consistence among 3GPP RAN4 specs. At least in the last meeting, the similar definition for CSI-RS L3 measurement was agreed to decouple with the active BWP. That is the definitions of Option 2,3,4 will violate such principle indeed. And Ericsson’ proposal above is most likely for the discussion of gap applicability for intra-frequency measurement, which is other separated issue we will addressed. Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements In our proposal, the intra-frequency measurement with and w/o gap ane inter-frequency measurement are also given. That is at least the intra RSTD measurement requirements can be differentiated regarding to the gap needed or not. So we are fine with Option 1. But more other requirements shall be needed. E.g. - Intra-frequency measurement without gap - Intra-frequency measurement with gap - Inter-frequency measurement with gap Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement We support option 2. That is the definition of UE Rx-Tx time difference intra/inter measurement can be same as that of RSTD. Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements Similarly, the following requirements for UE Rx-Tx time difference measurement are necessary: - Intra-frequency measurement without gap - Intra-frequency measurement with gap - Inter-frequency measurement with gap |
| CATT | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement Support option 2a with an update as below: Option 2a: Intra-frequency RSTD measurement is defined when the DL PRS resources to be measured, including reference cell and neighbor cell, have the same center frequency, SCS and CP type as active BWP and the BW of these PRS are all within the active BWP. Otherwise, the RSTD measurement is inter-frequency measurement Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements This issue is based on the option 1 in sub-topic 2-1 and shall be discussed after the conclusion of sub-topic 2-1 Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Support option 2, and I don’t understand why all the resource on the same TRP is required in the same positioning frequency layer in option 1 while the measurement is performed on individual resource. Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements Similar view as sub-topic 2-2, shall be discussed after the conclusion of sub-topic 2-3 |
| Huawei | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement Option 1 as a compromise to move forward. For option 2/2a/4, from the need for gap discussion, it seems difficult to have intra-frequency = measurement without gap, or inter-frequency = measurement with gap. For option 3 and new proposal from Ericsson above, we fail to see the relevance of serving cell for intra/inter-frequency definition for PRS. For CSI-RS, intra-frequency is defined based on serving cell CSI-RS, but for PRS serving cell is not relevant in the assistance data. Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements We do not see the need for option 1. Regardless of the intra/inter-frequency definition, it is possible that PRS layer is not fully confined within serving cell BWP, and MG is needed. For CSI-RS, both BWP and CSI-RS are configured by the serving cell, but this is not the case for PRS. We are also wondering what is the difficult for UE to measure the PRS layer with reference resource outside serving cell BWP with MG. Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Option 2. Even PRS-RSRP and Rx-Tx are single cell measurement, reference resource is still needed for UE to get the timing reference, and we think it is desirable to have a common definition for all PRS measurements. For option 1, we do not understand why it should be defined per TRP basis instead of per PRS layer basis. For example, if TRP1 has resources on 2 PRS layers f1 and f2, and TRP2 has resources on a single PRS layer f2, which PRS layer is considered as intra-frequency? Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements Similar comments as for 2-2. But for Rx-Tx we can consider a general principle that requirements are only defined for the case where PRS and SRS are in the same band. |
| MTK | Sub-topic 2-1: - We support option 2. To move forward, we suggest we can discuss the following points: - Whether reference and neighbor PRS should be in the same frequency layer - Whether active BWP should be involved in the intra-frequency definition - Whether intra-frequency measurement can be performed without measurement gap - Whether UE may need to adjust its center measurement frequency for intra-frequnecy measurement - Whether the definition involves SSB Our view is Yes for 1, neutral for 2, neutral for 3, NO for 4, and NO for 5. Sub-topic 2-2: Support option 1, we also think this is a baseline scenario to define requirements Sub-topic 2-3: Support option 2 Sub-topic 2-4: Support option 1, we also think this is a baseline scenario to define requirements |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement Tentative agreements: None Candidate options (based on comments, option 2b and option 4 are removed, option 2a is updated, and option 5 is added): - Option 1. Intra-frequency RSTD measurement is defined as when the neighbor DL PRS resource and the reference DL PRS resource belong to the same positioning frequency layer. Otherwise, the RSTD measurement is inter-frequency. (QC, Intel, HW) - Conditional on sub-topic 2-2 (QC) - Option 2. Intra-frequency RSTD measurement is defined when neighbour DL PRS resource and reference DL PRS resource belong to the same positioning frequency layer, and the BW of the positioning frequency layer is within the BW of UE’s active DL BWP, and SCS and CP of the positioning frequency layer are the same as those of UE’s active DL BWP. Otherwise, the RSTD measurement is inter-frequency. (MTK) - Option 2a. Intra-frequency RSTD measurement is defined when the DL PRS resources to be measured, including reference cell and neighbor cell, have the same center frequency, SCS and CP type as active BWP and the BW of these PRS are all within the active BWP. Otherwise, the RSTD measurement is inter-frequency measurement. (CATT) - Option 3. Intra-frequency RSTD measurement: the center frequency of PRS BW is the center frequency of a serving cell SSB and has the same SCS as that of the serving cell SSB, otherwise it is inter-frequency. MG may be needed for intra- or inter-frequency, depending on whether or not the PRS BW is within the active BWP (NOTE: for RSTD, the above conditions are met for both reference and the other DL links) (ZTE, Ericsson) - Option 5. The intra-frequency RSTD definition is based on the relation between SCS, CP, and center frequency of the serving cell and the PRS of each of the reference cell and measured cell: for intra-frequency RSTD they are the same for the reference cell and they are the same for the measured (“neighbor”) cell. (Ericsson) - FFS: SCS of the serving cell Recommendations for 2nd round: Further discuss in 2nd round. As companies’ views are still very diverse, based on suggestion from MTK, companies are encouraged to provide views on the following principles: - should the definition be based on PRS BW being within active BWP? - should the definition be based on PRS center frequency being same as serving cell BWP? - should the definition be based on PRS center frequency being same as serving cell SSB? - should the definition be based on PRS SCS/CP being same as serving cell BWP? - should the definition allow existence of both intra and inter-frequency when more than one PRS frequency layer are configured? - In this case some neighbor PRS resources are in the same frequency layer as the reference resource, and some in the different frequency layer than the reference resource. |
| Sub-topic#2 | Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements Tentative agreements: None Candidate options: - Option 1. Define intra-frequency RSTD requirements for the scenarios (QC, MTK) - SCS and CP of the positioning frequency layer is the same as the active BWP of UE - BW of positioning frequency layer is within the active BWP of the UE. - Option 2. Define requirements with and without measurement gaps, for intra- and inter-frequency measurement requirements. (Ericsson, HW) - Option 3. Define requirements for following scenarios (Intel) - Intra-frequency measurement without gap - Intra-frequency measurement with gap - Inter-frequency measurement with gap - Option 4. FFS (CATT) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#3 | Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Tentative agreements: None Candidate options: - Option 1. UE Rx-Tx timing difference measurement is defined as intra-frequency if PRS resources of all the Rx-Tx time difference measurements on the same TRP belong to the same positioning frequency layer. Otherwise, the UE Rx-Tx timing difference measurement is inter-frequency. (QC) - Option 2. Same definition as for RSTD measurement. (CATT, HW, ZTE, Intel, MTK) - Option 3: The intra-frequency UE Rx-Tx definition is based on the relation between SCS, CP, and center frequency of the serving cell and the PRS of the measured cell: for intra-frequency UE Rx-Tx they are the same. (Ericsson) - FFS: SCS of the serving cell Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#4 | Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements Tentative agreements: None Candidate options: - Option 1. Define intra-frequency Rx-Tx time difference measurement requirements for the scenarios (QC, MTK) - SCS and CP of the positioning frequency layer is the same as the active BWP of UE - BW of positioning frequency layer is within the active BWP of the UE - Positioning SRS resource is on the same band as positioning frequency layer - Option 2. Define requirements with and without measurement gaps, for intra- and inter-frequency measurement requirements. (Ericsson, HW) - Option 3. Define requirements for following scenarios (Intel) - Intra-frequency measurement without gap - Intra-frequency measurement with gap - Inter-frequency measurement with gap - Option 4. Requirements are only defined for the case where PRS and SRS are in the same band (HW) - Option 5. FFS (CATT) Recommendations for 2nd round: Further discuss in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

For sub-topic 2-1, besides the options, companies are encouraged to provide views on the following principles:

- should the definition be based on PRS BW being within active BWP?

- should the definition be based on PRS center frequency being same as serving cell BWP?

- should the definition be based on PRS center frequency being same as serving cell SSB?

- should the definition be based on PRS SCS/CP being same as serving cell BWP?

- should the definition be based on reference PRS resource and neighbor PRS are in the same positioning frequency layer

- should the definition allow existence of both intra and inter-frequency when more than one PRS frequency layer are configured?

  - In this case some neighbor PRS resources are in the same frequency layer as the reference resource, and some in the different frequency layer than the reference resource.

For sub-topic 2-2 and 2-4, moderator would ask companies to consider the latest RAN1 agreements:

Agreement: - UE is not expected to process DL PRS without configuration of measurement gap in Rel-16 - RAN1 assumes that no RAN4 requirements are to be defined for the case w/o configured measurement gap in Release 16 - Inform RAN4 about this agreement

| Company | Comments |
| --- | --- |
|  | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements |
| Qualcomm | Sub-topic 2-1: we support option 1. Given the RAN1 agreement that UE is not required to measure PRS without MG, we think the best definition is the simplest one that is outlined in option 1 and is consistent with RAN1 definition of intra-frequency vs. inter-frequency. Sub-topic 2-2: given the RAN1 agreement, only requirements with MG should be defined for positioning. Our understanding is that UE capability signaling for PRS measurement without MG is not even defined in RAN1. Suggest an alternative option: “Define requirements with measurement gaps, for intra- and inter-frequency measurement requirements” Sub-topic 2-3: we can support option 2 Sub-topic 2-4: this is somewhat similar to sub-topic 2-2. However, our view is that RAN4 should define requirements for scenario when SRS is in the same band as PRS as the baseline requirements. SRS in a different band than PRS should be an extra UE capability. |
| CATT | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement Support option 2a. for option 1, it will cause misalign with other measurement, since in any measurement, the definition of intra/inter is related to serving cell, if only PRS measurement definition is decoupled with serving cell, it may cause confusion. Actually according to RAN1 agreement, there is only one set of requirement with gap, no need to define intra/inter-frequency. Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements Follow RAN1’s agreements, only define the requirement with gap in R16. Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Support option 2 Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirementsSame as sub-topic 2-2, follow RAN1’s agreements, only define the requirement with gap in R16. |
| MTK | Sub-topic 2-1: Our preference is - should the definition be based on reference PRS resource and neighbor PRS are in the same positioning frequency layer? Yes - should the definition allow existence of both intra and inter-frequency when more than one PRS frequency layer are configured? No - In other words, if there are more than one POS frequency layers, then all the measurement are considered as inter-frequency measurements - For the other discussion bullets, our preference is NO. - We note it is not true that UE is not required to measure PRS without MG. However, the PRS processing capability assumes that MG is configured. So it is expected that UE measures PRS within MG. Therefore, the definition of intra-frequency measure doesn’t need to be bounded by SSB, CP, SCS. Sub-topic 2-2: Agree with QC’s view. Define requirements with measurement gaps, for intra- and inter-frequency measurement requirements Sub-topic 2-3: Option 2 Sub-topic 2-4: Agree with QC’s view. Define requirements with measurement gaps, for intra- and inter-frequency measurement requirements, with the assumption that SRS is in the same band as PRS |
| Huawei | Sub-topic 2-1 Intra/inter-frequency definition for RSTD measurement We support option 1, and have similar comments as MTK. It is not clear what the confusion mentioned by CATT is, but we are also ok to not define intra/inter-frequency for PRS measurement if RAN4 cannot reach consensus, as the definition will not impact the core requirements. Sub-topic 2-2 Applicable scenarios for RSTD measurement requirements Based on latest RAN1 agreement, we also agree with QC and MTK to define requirements with measurement gaps, for intra- and inter-frequency measurement. Sub-topic 2-3 Intra/inter-frequency definition for Rx-Tx time difference measurement Option 2. Sub-topic 2-4 Applicable scenarios for Rx-Tx time difference measurement requirements Similar as 2-2, RAN4 should define requirements with measurement gaps, for intra- and inter-frequency measurement. In addition, this should be combined with option 4, i.e. requirements are only defined for the case where PRS and SRS are in the same band. |
| Intel | As agreed in GTW, no further discussion needed for this topic |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #3: Measurement period for RSTD

The topic is for measurement period requirements for RSTD measurements.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007145 | ZTE Corporation | Proposal 1: Take SSB-based RRM measurement in R15 as baseline. Proposal 2: Further study the exact value of the beam sweeping factor (should be a fixed value). |
| R4-2006168 | Qualcomm Incorporated | Proposal 7. RAN4 to define RSTD measurement period based on Type 2 PRS duration calculation. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. Proposal 8. If the time span of a DL PRS resource instance (i.e., the time duration spanned after repetition by DL-PRS-ResourceRepetitionFactor) ± its corresponding DL-PRS-expectedRSTD-uncertainty is greater N, where N is the duration of DL PRS symbols in ms that UE is capable of processing, measurement requirements do not apply. Proposal 9. The RSTD measurement period is defined as: TRSTD,base= { max LPRSN,NPRSslotN' -1}. maxT, TPRS,MGP+T Where - N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE - N' is number of DL PRS resources that UE can process in a slot, which is reported per SCS per band - LPRS represents the span of a PRS occasion defined as the time from the first slot of the earliest PRS resource to the last slot of latest PRS resource based on Type 2 duration calculation - NPRSslot is the number of configured PRS resources in a slot - TPRS is the periodicity of PRS - MGP is the measurement gap period Proposal 10. Measurement period to be based on maximum periodicity value; TPRS,max, among all configured DL PRS resources. Proposal 11. UE sweeps its Rx beams within a PRS instance (occasion) if the number of repetitions, DL-PRS-ResourceRepetitionFactor, is larger than what is required to meet the accuracy requirements. If not, UE sweeps its Rx beams across PRS instances (occasions). Proposal 12. Measurement periods adds up for each frequency layer that is configured to be measured, i.e., if measurement period for frequency layer i is TRSTD,i, then total measurement period is TRSTD,total= iTRSTD,i Observation 2. Overlap of PRS and SSB symbols is not possible per RAN1 specification. Extension of measurement period to account for dropped PRS occasions due to overlap with SSB is unnecessary. |
| R4-2006232 | CATT | Proposal 4: The extension due to overlap with SSB symbols is not needed. Proposal 5: The similar way in SSB-based RRM for Rx beam sweeping factor is preferred. Proposal 6: The measurement period due to HO is expanded according to the expression below: TRSTD FDD, NR,HO=TRSTD FDD, NR+k*TPRS+THO ms Proposal 7: The measurement period for more than one frequency layer is expanded according to the expression below: TRSTD FDD, NR,Multi-layer=Nfre_layer *TRSTD FDD, NR ms Where Nfre_layer is the number of frequency layers. |
| R4-2006304 | MediaTek inc. | Proposal 3: PRS measurement period is defined as Tmeas_PRS = NRxBeam * Nfreq * [Ceil(LPRS/N) * max(Tres, Tproc) ] where - NRxBeam is the scaling factor for Rx beam sweeping, - Nfreq is the number of PRS frequency layers, - Tres is the maximum resource periodicity among all PRS resources on the PRS frequency layer, - Tproc is the PRS processing time as indicated in UE capability reporting, - LPRS is the actual PRS duration per occasion, and N is reported capability on the maximum duration of PRS symbols UE can buffer and process every Tproc ms |
| R4-2006556 | Intel Corporation | Proposal 3: The RSTD measurement delay can be defined as: Measurement scenario; Measurement delay. Intra-frequency measurement w/o gap; TRSTD_intra_nonMG, NR=KPRS_RXBeam* TRSTD_perlayer, NR. Intra-frequency measurement with gap; TRSTD_intra_MG, NR=KPRS_RXBeam*CCSFintraRSTD_noMG *TRSTD_perlayer, NR. Inter-frequency measurement; TRSTD_inter, NR=KPRS_RXBeam*i=1NPRS_layer{ CCSFinterRSTD,i *TRSTD_perlayer, NRi}. Where in , - i is the index of PRS positioning frequency measurement layer - TRSTD_perlayer, NR=TPRSi ⋅(Msi-1)+NPRSi ms - NPRS_layer is the number of PRS positioning frequency layers - TPRSi is the PRS resource set periodicity for ith PRS positioning frequency layer - Msi is the number of PRS periods need for all PRS resource (LPRStotal,i) within a PRS resource occasion in the i th PRS positioning frequency layer , which is Msi=ceilLPRStotal,iTPRSi∙TiNi - NPRS,i=NiTiTPRSi if BWPRS<Bi - UE DL PRS processing capability is indicated by {Ni, Ti, Bi} for the i th RSTD positioning frequency layer, defined in TS38.214 - KPRS_RXBeam is the scaling factor for FR2 RX beam sweeping, which is [8] for UE supporting FR2 power class 1 and [4.8] for UE supporting FR2 power class 2/3/4 - CSSFintraRSTD_noMG = CSSFoutside_gap,i is the scaling factor for the SSB-based measurements on the ith carrier frequency as defined in clause 9.1.5.1.2 of TS38.133 [9] for the equal splitting gap sharing scheme only - CSSFinterRSTD = CSSFwithin_gap,i is the scaling factor for the RSTD measurements on the “i”th positioning frequency layer as defined in clause 9.1.5.2.2 of TS38.133 [9] for the equal splitting gap sharing scheme only Proposal 4: The total measurement delay when serving cell changed (e.g. HO) can defined as: TRSTD_HO, NR=TRSTD_withoutHO, NR+ TPRS ⋅k+THO ms Where k is the number of times the intra/inter-frequency handover occurs during TRSTD_HO, NR. THO is the time during which the intra/inter-frequency RSTD measurement may not be possible due to intra/inter-frequency handover. |
| R4-2006561 | Intel Corporation | Change #1: NR DL RSTD measurement reporting criteria is introduced Change #2: NR DL RSTD measurement requirements are introduced in Section 9.8. |
| R4-2007841 | Huawei, HiSilicon | Proposal 2: SSB collision is not accounted in PRS measurement period. RAN4 can consider to add a statement that PRS measurement period will be extended if some PRS occasions collides with SSB symbols of serving or neighbour cells. Proposal 3: Rx beam sweeping in FR2 is accounted similar to SSB-based RRM measurement in R15. Proposal 4: PRS measurement period is scaled by ⌈M/S⌉, where - M is the maximum number of PRS resources per slot, and S is the reported capability on the maximum of PRS resources UE can buffer and process per slot, - The requirements apply provided that the configured PRS duration per occasion K is no larger than the reported capability on the maximum duration of PRS symbols N UE can buffer and process every Tms. Proposal 5: PRS measurement period should be scaled based on max{Tres, T}, where - Tres is the maximum resource periodicity among all PRS resources on the PRS frequency layer, - T is the PRS processing time as indicated in UE capability reporting. Proposal 6: the basic number of PRS occasions for PRS measurement period is 4. Proposal 7: Baseline PRS measurement period is defined as Tmeas_PRS = NRxBeam * [⌈M/S⌉ * max{Tres, T} * [4] + T] where - NRxBeam = 8 is the scaling factor for Rx beam sweeping, - Tres is the maximum resource periodicity among all PRS resources on the PRS frequency layer, - T is the PRS processing time as indicated in UE capability reporting, - M is the maximum number of PRS resources per slot, and S is the reported capability on the maximum of PRS resources UE can buffer and process per slot, - The requirements apply provided that the configured PRS duration per occasion K is no larger than the reported capability on the maximum duration of PRS symbols N UE can buffer and process every Tms. |
| R4-2007944 | Ericsson | - Proposal 7: When no measurement gaps are used, the RSTD measurement period can be defined as: T = ceil(NPRS,req / KPRS) TPRS + T, where - KPRS= LPRS/CombSizeN ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, - LPRS is the number of PRS symbols pe slot, - T is the extension due to dropped signals, - NPRS,req comb pattern realizations are required for RSTD measurement, where NPRS,req can depend on FR1/FR2, bandwidth, Es/Iot, etc. - TPRS is the maximum PRS periodicity among the PRS resources in neighbor TRPs measured during T. - Proposal 8: When measurement gaps are used, K´PRS (K´PRS≤KPRS) is the number of comb realizations within the effective measurement time of a measurement gap, and the measurement period becomes: T = ceil(NPRS,req / K´PRS) max(TPRS, MGRP) CSSF + T. - Proposal 10: RSTD measurement period is extended by T to compensate for the number of PRS occasions not available at the UE over a certain time period due to their overlap with SSB symbols, at least when the number of the non-available PRS occasions is large and the SSB symbols location is known to the UE. The allowed extension is limited by an upper bound. - Proposal 11: When the RSTD measurement period is extended, the increase in the measurement period depends at least on the periodicity of the PRS resource which has non-available occasions and the number of such non-available PRS occasions, but may further depend on the measurement gap configuration, etc. - Proposal 12: The extension T is proportional to max(TPRSref*XPRSref,TPRS*XPRS), where TPRSref and TPRSref are the PRS periodicity for the reference and neighbor TRPs, respectively, and XPRSref and XPRS are the numbers of periodic occasions with dropped PRS in the reference and neighbor TRPs, respectively. - Proposal 13: If the number of PRS occasions not available at the UE exceeds an acceptable limit, the measurement can be dropped, i.e., no further extension of the measurement period is allowed. - Proposal 14: For FR1, no impact of UE rx beam sweeping on RSTD measurement period shall be considered. - Proposal 15: For FR2, at least for the case when the QCL information is available to the UE, there is no impact of UE rx beam sweeping on RSTD measurement period. |

Open issues summary

There are many issues in the measurement period requirements, and the views from companies are diverse. To facilitate the discussion, moderator suggests to

- first discuss the requirements for the baseline scenarios with sub-topic 3-1 to 3-4, where the baseline scenario is FR1, single PRS periodicity for all resources, single PRS frequency layer, no PRS occasion dropping due to SSB collision, no HO occurred during the measurement period, no gap sharing with RRM measurement, and

- then discuss the scaling or extension of the baseline requirements due to

  - Rx beam sweeping in FR2 (sub-topic 3-6)

  - multiple PRS periodicities (sub-topic 3-7)

  - multiple PRS frequency layers (sub-topic 3-8)

  - PRS occasion dropping due to SSB collision (sub-topic 3-9)

  - HO occurred during the measurement period (sub-topic 3-10)

Sub-topic 3-1 Basic number of PRS occasions

- Option 1. The basic number of PRS occasions is 4, as margin to account for different PRS configurations including BW, comb, repetition and muting, different propagation conditions and AGC. (HW)

- Option 2 (Ericsson): LTE-like approach used for FeMTC and NB-IoT. NPRS,req is the number of required comb realizations which is derived from simulations (e.g., see table below); the number of occasions is then ceil(NPRS,req / KPRS) where KPRS= LPRS/CombSizeN  ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, LPRS is the number of PRS symbols per slot.

| FRx, SCS, number of PRBs | Minimum number of comb realizations, NPRS,req |
| --- | --- |
| FR1, 15 kHz, ≤52 PRBs (10 MHz) | 4 |
| FR1, 15 kHz, ≤104 PRBs (20 MHz) | 2 |
| FR1, 15 kHz, >104 PRBs | 1 |
| FR1, 30 kHz, ≤48 PRBs (20 MHz) | 4 |
| FR1, 30 kHz, ≤132PRBs (30 MHz) | 2 |
| FR1, 30 kHz, ≤272 PRBs (100 MHz) | 1 |
| FR2, 120 kHz, ≤32 PRBs (50 MHz) | 4 |
| FR2, 120 kHz, >32 PRBs | 1 |

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-2 Periodicity of PRS occasions

The options are listed for MG based measurement. If the measurement is without MG, then MGRP is not needed in any of the options.

- Option 1. maxT, TPRS,MGRP (QC, MTK, HW)

  - T is the PRS processing time as indicated in UE capability reporting for a given maximum bandwidth (B) in MHz supported by UE, TPRS is the periodicity of PRS, MGRP is the periodicity of MG

- Option 2. maxTPRS,MGRP (Intel, Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability

As some companies relate the measurement period requirements to the UE processing capability, the relevant conclusion from RAN1 is copied here for information.

Agreement: - For UE DL PRS processing capability, - UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE - Additionally, UE reports new parameter, number of DL PRS resources that UE can process in a slot, which is reported per SCS per band. - Values: {1, 2, 4, 8, 12, 16, 32, 64} - The following sets of values for N, T and B are supported - Values for N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms - Values for T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - Values for maximum BW reported by UE = {5, 10, 20, 40, 50, 80, 100, 200, 400} MHz Agreement: The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Agreement: UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) Agreement: UE capability for DL PRS processing is defined assuming the case with configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than X% - FFS: X

- Option 1. { max LPRSN,NPRSslotN'  -1}   (QC)

  - N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE

  - N' is number of DL PRS resources that UE can process in a slot, which is reported per SCS per band

  - LPRS represents the span of a PRS occasion defined as the time from the first slot of the earliest PRS resource to the last slot of latest PRS resource based on Type 2 duration calculation

  - NPRSslot is the number of configured PRS resources in a slot

- Option 2. LPRSN (MTK)

- Option 3. NPRSslotN' , and the requirements apply provided that the configured PRS duration per occasion K is no larger than the reported capability on the maximum duration of PRS symbols N UE can buffer and process every Tms (HW)

- Option 4. (Msi-1) (Intel)

  - Ms=ceilLPRSTPRS∙TN, TPRS is the periodicity of PRS

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-4 Sampling and processing time for baseline requirements

This sampling and processing time corresponds to below component in LTE RSTD requirements e.g. in 8.1.2.5.1 of 36.133.

= ms is the measurement time for a single PRS positioning occasion which includes the sampling time and the processing time.

- Option 1. T (QC, HW)

  - T is the PRS processing time as indicated in UE capability reporting,

- Option 2. NPRS (Intel)

  - NPRS,i=NTTPRS if BWPRS<B

  - N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-5 Rx beam sweeping in FR2

- Option 1. UE sweeps its Rx beams within a PRS instance (occasion) if the number of repetitions, DL-PRS-ResourceRepetitionFactor, is larger than what is required to meet the accuracy requirements. If not, UE sweeps its Rx beams across PRS instances (occasions) (QC)

- Option 2. The similar way in SSB-based RRM for Rx beam sweeping factor is preferred, i.e. no intra-PRS-occasion Rx beam sweeping (CATT, MTK, Intel, ZTE, HW)

- Option 3.  For FR1, no impact of UE rx beam sweeping on RSTD measurement period shall be considered. For FR2, at least for the case when the QCL information is available to the UE, there is no impact of UE rx beam sweeping on RSTD measurement period. (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-6 Measurement period with multiple PRS periodicities

- Option 1. Use the maximum PRS resource periodicity among all PRS resources (QC, MTK, HW, Ericsson)

- Option 2. Other

Recommended WF: Check if option 1 is agreeable.

Sub-topic 3-7 Measurement period with multiple PRS frequency layers

- Option 1. Measurement periods adds up for each frequency layer that is configured to be measured (QC, Intel)

- Option 2. Measurement period is scaled by number of frequency layers (CATT, MTK)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period

- Option 1. No (QC, CATT, HW)

- Option 2. Yes (Ericsson)

  - When the RSTD measurement period is extended, the increase in the measurement period depends at least on the periodicity of the PRS resource which has non-available occasions and the number of such non-available PRS occasions, but may further depend on the measurement gap configuration, etc.

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-9 Measurement period extension due to HO

- Option 1. TRSTD FDD, NR,HO=TRSTD FDD, NR+k*TPRS+THO  ms (CATT, Intel)

  - k is the number of times the handover occurs during TRSTD FDD, NR,HO

  - THO is the time during which the RSTD measurement may not be possible due to handover.

- Option 2. Extension of positioning measurement period due to HO shall not exceed responseTime. UE behaviour in such case is FFS. (QC)

- Option 3. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 3-10 Calculation of PRS occasion duration

This sub-topic is related to sub-topic 3-3. RAN1 defined two ways how the duration of PRS occasion is calculated. The issue in this sub-topic is which one should be assumed to derive LPRS for options in sub-topic 3-3.

Agreement: - For the purpose of DL PRS processing capability, the duration of DL PRS symbols (K) in ms within any P msec window, is calculated by - Type 1 duration calculation with Ks=Tsend-Tsstart - Type 2 duration calculation with K=12μSwhere - Type 1 or Type 2 is reported as UE capability, - S is the set of slots of a serving cell within the P msec window in the positioning frequency layer that contains potential DL PRS resources considering the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference), - for Type 1, [Tsstart, Tsend]Tsstart, Tsend is the smallest interval in ms within slot s corresponding to an integer number of OFDM symbols of a serving cell that covers the union of the potential PRS symbols and determines the PRS symbol occupancy within slot s. - Interval[Tsstart, Tsend] considers the actual nr-DL-PRS-ExpectedRSTD, nr-DL-PRS-ExpectedRSTD-Uncertainty provided for each pair of DL PRS Resource Sets (target and reference).

- Option 1. RAN4 to define RSTD measurement period based on Type 2 PRS duration calculation. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. (QC)

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | We do not agree on that everything else will be based on scaling or extension with respect to “baseline” scenarios. Better to discuss issue by issue. Sub-topic 3-1 Basic number of PRS occasions We prefer option 2, the exact numbers in the table can be further discussed. “Sample” is ambiguous, so we prefer the term “comb realization” instead. Sub-topic 3-2 Periodicity of PRS occasions Further discussion is needed. We prefer to discuss the processing time and how to account for it after the measurement period is settled. So, it is Option 2 for the time being. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability Needs further discussion. Are the proposals only for scenarios without measurements gaps? How can the LMF use this capability, if it is not aware of whether and which gaps are configured? Sub-topic 3-4 Sampling and processing time for baseline requirements Further discussion is needed. We prefer to discuss the sampling/processing time after the measurement period is settled. Sub-topic 3-5 Rx beam sweeping in FR2 We support option 3. Sub-topic 3-6 Measurement period with multiple PRS periodicities We prefer to discuss this after we agree on a measurement period for the same PRS periodicity. Sub-topic 3-7 Measurement period with multiple PRS frequency layers We prefer the approach similar to Rel-15 RRM, i.e., based on CSSF. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period We think Option 2 is more reasonable. Option 1 is actually more demanding, because it is a fact that the transmissions PRS can be dropped (agree by RAN1) and Option 1 suggests the same measurement period but with a fewer PRS. Sub-topic 3-9 Measurement period extension due to HO Option 1 and option 2 are not alternatives rather different issues. We support option 1. But option 2 does not require any work in RAN4. This is a time parameter configured via LPP, same as in Rel-9. Sub-topic 3-10 Calculation of PRS occasion duration Needs further discussion. The measurement period does not need to depend on this, this capability is for the network to consider. |
| ZTE | Sub-topic 3-5 Rx beam sweeping in FR2 We prefer Option 2 which is to use a similar way as SSB-based RRM. |
| Qualcomm | Subtopic 3-1: Support option 1. This is similar approach as in LTE where 4 unmuted occasions were present for every cell in the assistance data. Option 2 is quite confusing and is inventing new terminology which is not necessary. “sample” does not even appear in option 1. Moreover, formulation in option 2 necessitates cross-occasion combining which creates many performance issues and cannot be agreed. Subtopic 3-2: Support option 1. Option 2 completely disregards the defined UE capability signaling in RAN1 which is a key factor in determining the measurement period. Subtopic 3-3: We support option 1. Option 2 does not take {N,T} capability signaling into consideration. Option 4 does not take the number of PRS resources in a slot that UE can process (the other UE capability) into consideration. Option 3 makes a restrictive assumption. Huawei argues that the DL PRS resource in yellow straddling two other DL resources cannot be measured sequentially in two occasions which we agree but in our view such configuration is not common because the level of interference changes for resource 1 and 3. Our view is that time-aligned PRS configuration are more typical, i.e., resource#2 completely aligned with resource#1 or resource#3. We don’t think we should impose the limitation in option 3 for a corner PRS configuration depicted below. Subtopic 3-4: Support option 1. In our view, option 2 is wrong formulation. Subtopic 3-5: Support option 1. In our view, the positioning latency requirements will be very hard, if not impossible, to meet if UE sweeps its beam once every occasion. We note again that the presence of large repetition slots (up to 32) in PRS is to enable this type of operation. SSB did not have these repetition slots so one beam per SMTC period made sense. Option 3 discusses a very limited case when the QCL information is available AND QCL source (SSB or PRS) has already been measured which is not guaranteed at all. SSB side condition is much higher than PRS and RAN4 made an agreement in last meeting that UE is not required to make extra SSB measurements for the purpose of positioning so option 3 lacks coverage. We can also compromise to define UE capability signaling for this operation to accommodate both types of UE implementation. This should not impact gNB operation. The support or lack of support of this feature means smaller or larger measurement period which can be considered by LMF when setting the responseTime in QoS IE. Subtopic 3-6: Support option 1. Subtopic 3-7: Support option 1. The PRS configuration, UE processing capability can be very different between frequency layers (e.g., one layer FR1 and one layer FR2). So scaling the measurement period by the worst period of the two layers is very conservation and we prefer not to do it. In response to Ericsson’s comment, RAN1 has already made this agreement: Agreement: UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) Subtopic 3-8: We support option 1 and would support Huawei’s proposal that the measurement period will be defined for cases when PRS occasions are not dropped for any reason. Subtopic 3-9: Options 1 and 2 are complementary not alternatives. We believe unlimited extension of measurement period has no meaning or use case. Subtopic 3-10: We support option 1 as it allows formulation of measurement period to proceed in a more straightforward manner. To Ericsson’s comment, all the subtopics in topic 3 which Ericsson wants to defer until “measurement period is settled” are actually key ingredients of defining measurement period. |
| Intel | Sub-topic 3-1 Basic number of PRS occasions for baseline requirements In our understanding, the PRS occasions number within a measurement period is up to NW configuration (e.g. the proposed comb size which depends further simulation results). A little bit margin left to UE implementation is reasonable. But “4” here is too large. Is there any practical justification for this? Sub-topic 3-2 Periodicity of PRS occasions for baseline requirements Firstly from general principle in RAN used, when LMF configure the proper PRS configuration including Tprs and PRS symbols the network entity (LMF) will follow UE’s capability(N,T) which was signalled when PRS measurement preparing phase(e.g. capability request and report). That is in the realistic deployment Tprs configured by LMF can guarantee UE’s capability. As a result, if UE follow the measurement interval of Tprs, we can assume that UE can fulfil the processing for the PRS resource within a Tprs. Secondly, it is obviously the network no need to schedule the denser PRS resource which is out of UE’s capability. This decrease the network resource efficiency significantly. Thirdly, from RAN4 requirements perspective, the measurement latency can depend on the minimum interval when the network reference signal is available (e.g. SMTC ) instead of the duration UE needs. So Tprs shall be taken as the basic timing interval for PRS measurement latency requirements. To Qualcomm’s comments on topic 3-2: we can’t agree “Option 2 completely disregards the defined UE capability signaling in RAN1 which is a key factor in determining the measurement period”. On the other hand, NW will respect UE’s capability more. Sub-topic 3-3 Scaling of PRS occasions for baseline requirements due to UE processing capability If Tprs is larger than T and Lprs is not larger than N, the option 1, 2, 4 can be quite similar. Otherwise, the PRS resource which UE can process need to be scaled regarding to UE capability. So the essential point which shall be agreed is same as that of Sub-topic 3-2. Sub-topic 3-4 Sampling and processing time for baseline requirements NPRS here is for the case in which UE can finish PRS processing shorter than 1 PRS periodicity. For an example, if Tprs is quite longer than the necessary processing time, given UE real processing time is proportional with its capability the less measurement duration is possible. We are fine also if majority companies prefer at least the whole PRS periodicity is minimum requirements. Sub-topic 3-5 Rx beam sweeping in FR2 Similar scaling factor as SSB measurement with Rx beam sweeping. Sub-topic 3-6 Measurement period with multiple PRS periodicities In our view, based on the single UE capability on PRS processing, LMF needs not to provide multiple PRS periodicity to UE with a single layer at least. Sub-topic 3-7 Measurement period with multiple PRS frequency layers For the measurement on each layer can be quite independent because of RAN1’ agreements. So the total delay can be accumulated all of them in a simple way. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period We support Option 2 also according to our proposal below [R4-2006555] “Observation 4: When PRS resource boundary is close to other RRM reference signal (e.g. SSB and CSI-RS) , up to UE processing capability the impact of PRS processing (e.g. PRS measurement gap request) on other RRM measurements is possible. Sub-topic 3-9 Measurement period extension due to HO We can support both Option 1 and Option 2 since Option 2 can be taken as other additional condition when Option 1 applied. Sub-topic 3-11 Calculation of PRS occasion duration We may not need such restriction. If the parameter defined in the requirements in slot, we can use type 2 duration. But seems type 2 is simpler way . |
| CATT | Sub-topic 3-1 Basic number of PRS occasions Support option 1, and we cannot agree the option 2, since from the simulation results for RSTD measurement, what matters is repetition not combsize. Sub-topic 3-2 Periodicity of PRS occasions Support option 1. The UE capability should also be considered. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability Support option 3. Sub-topic 3-4 Sampling and processing time for baseline requirements Support option 1. Sub-topic 3-5 Rx beam sweeping in FR2 Support option 2, for option 1, UE need to sweep its Rx beam within the PRS occasion, but this is not a general implementation and its feasibility has not been proved. Sub-topic 3-6 Measurement period with multiple PRS periodicities Support option 1. Sub-topic 3-7 Measurement period with multiple PRS frequency layers Support option 2. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Support option 1, since RAN1 has agreed the PRS is not configured on the same symbols of SSB and other DL signals. And I think this issue is considered for without gap case while the gap sharing is considered for with gap case. Sub-topic 3-9 Measurement period extension due to HO Support option 1. And I think the option 2 and option 1 are not conflicted. Since option 1 is the calculation way of measurement period when HO and option 2 is the limitation for the calculation result. Sub-topic 3-10 Calculation of PRS occasion duration Support option 1. |
| Huawei | Sub-topic 3-1 Basic number of PRS occasions Option 1, which is same as in LTE. On option 2, we understand the assumption is that UE can do same coherent combining for repetitions within a PRS occasion and across PRS occasions, but we understand this is not the case. Our suggestion is to account for the number of required repetitions in the side condition for accuracy requirements. Sub-topic 3-2 Periodicity of PRS occasions Option 1. T is a reported UE capability on processing time, so it needs to be accounted. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability We are compromise to max LPRSN,NPRSslotN' , if the specification captures that there is no requirement for resource 2 in the figure as in Qualcomm comment. We understand there should be no “-1” in the scaling factor. Sub-topic 3-4 Sampling and processing time for baseline requirements Option 1. We do not think the processing time can be taken as proportional to N/T. Sub-topic 3-5 Rx beam sweeping in FR2 Option 2. On option 1, we think the actual time offset between different PRS resources depends not only on expected RSTD uncertainty but also expected RSTD and the slot offsets. Moreover, there can be different resource repetition patterns and muting patterns, so UE complexity in scheduling the Rx beams will be quite high. If we define requirement with this option, the muting pattern also needs to be considered. On option 3, we agree with the FR1 part, but not for the FR2 part. UE will measure PRS on the same OFMD symbols from multiple TRPs in multiple directions. Even the Rx beam for each TRP is known, UE still needs to sweep Rx beam to receive multiple TRPs. Sub-topic 3-6 Measurement period with multiple PRS periodicities Option 1. Sub-topic 3-7 Measurement period with multiple PRS frequency layers We are in principle fine with option 1, but would like to make decision when we have requirements for the single PRS layer. To Ericsson: we understand CSSF will be applied for the single layer requirements for gap sharing between PRS and RRM. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Option 1, and we agree with Qualcomm that requirements are defined only for the case without PRS dropping. To Intel, we understand it is a separate issue addressed by concurrent RRM and PRS measurement in email 217. Sub-topic 3-9 Measurement period extension due to HO We are fine with option 1. For option 2, we think RAN4 requirements on measurement period should be independent of responseTime, and if the configured responseTime is smaller than the measurement period, UE should still report but does not need to meet the accuracy requirements, or the report can be only for a subset of measurable resources. Sub-topic 3-10 Calculation of PRS occasion duration We have some concerns with option 1 because the reported N can be different depending on type 1 or type 2. For example, - a certain PRS configuration gives K=2ms with type 1 and K=5ms for type 2 - UE1 reports with type 1 N=2ms - UE2 reports with type 2 N=3ms Following option 1, the scaling factor will be 3 for UE1 and 2 for UE2, and it is unnecessarily relaxed for UE1. |
| MTK | Sub-topic 3-1: - In our view, basic number of PRS occasions is related to accuracy requirements but not related to measurement period. We can move this discussion to the catalog of accuracy requirements. - The number of PRS occasions for defining accuracy requirements need further discussion. The value 4 can be a current working assumption. - For defining accuracy requirements, we cannot agree to Ericsson’s proposal where PRS comb pattern is related to BW and SCS. Sub-topic 3-2: Support option 1. Regarding option 2, it is not reasonable to ignore the PRS processing capability reported by UE Sub-topic 3-3: We understand that N' and NPRSslot should be taken into consideration. We update our view correspondingly. Assuming TPRS=T, we propose that the scaling factor should be LPRSN×NPRSslotN' For example, if LPRS=2N and NPRSslot=2N', then the required PRS occasion should be scaled by 4 since the PRS resources for UE to process is 4 times larger than UE’s PRS processing capability. If TPRS<T, then the proposal still applies with the understanding that UE supporting (N, T) = (8ms, 160ms) also supports (N, T) = (8ms, 160ms)*k, where 0<k≤1 is an scaling factor. If TPRS>T, considering UE is given more time to process PRS, the scaling factor should be LPRSN×NPRSslotN'×TTprs Note that the UE buffering capability is the same. Sub-topic 3-4: Support option 1 Sub-topic 3-5: Support option 2 Sub-topic 3-6: Support option 1 Sub-topic 3-7: Support option 2. We cannot accept option 1. Note that in UE’s PRS processing capability, UE is assumed to process 1 layer at a time. Sub-topic 3-8: Support option 1 Sub-topic 3-9: Support option 1 Sub-topic 3-10: Support option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006561 (Intel) | Intel: we can update and resolve the issues after1st round discussion |
|  | Company B |
|  | Qualcomm: many many issues with this CR. Cannot be agreed. |
|  | Company A |
|  | Company B |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 3-1 Basic number of PRS occasions Tentative agreements: None Candidate options: - Option 1. A certain number (X) of PRS occasions, X=TBD (QC, Intel, CATT, HW, MTK) - X=4 (Qualcomm, CATT, Huawei, MTK) - X < 4 (Intel) - Option 2. ceil(NPRS,req / KPRS) (Ericsson) - NPRS,req is the comb pattern realizations that are required for RSTD measurement, derived from simulation results - KPRS= LPRS/CombSizeN ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, - LPRS is the number of PRS symbols per slot Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#2 | Sub-topic 3-2 Periodicity of PRS occasions Tentative agreements: None Candidate options: - Option 1. maxT, TPRS,MGRP (QC, CATT, HW, MTK) - Option 2. maxTPRS,MGRP (Intel, Ericsson) - In the realistic deployment Tprs configured by LMF can guarantee UE’s capability (Intel) Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#3 | Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability Tentative agreements: None Candidate options: - Option 1. { max LPRSN,NPRSslotN' -1} (QC) - Option 1a. { max LPRSN,NPRSslotN' } (HW) - requirements do not apply for resources spanning over two sampling periods of N - Option 2. Depends on T and Tprs (MTK) - LPRSN×NPRSslotN', if Tprs <= T - LPRSN×NPRSslotN'×TTprs , of Tprs > T - Option 3. NPRSslotN' (CATT, HW) - requirements apply provided that the configured PRS duration K is no larger than N - Option 4. FFS (Ericsson) - Also need to consider the case where PRS is measured with MG Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#4 | Sub-topic 3-4 Sampling and processing time for baseline requirements Tentative agreements: None Candidate options: - Option 1: T (QC, Intel, CATT, HW, MTK) - Option 2. NPRS (Intel) - NPRS=NTTPRS - Option 3: FFS (Ericsson) - discuss the sampling/processing time after the measurement period is settled Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#5 | Sub-topic 3-5 Rx beam sweeping in FR2 Tentative agreements: None Candidate options: - Option 1. UE sweeps its Rx beams within a PRS instance (occasion) if the number of repetitions, DL-PRS-ResourceRepetitionFactor, is larger than what is required to meet the accuracy requirements. If not, UE sweeps its Rx beams across PRS instances (occasions) (QC) - Define UE capability to differ UE that supports Rx beam sweeping within a PRS occasion and UE that uses one Rx beam per PRS occasion - Option 2. The similar way in SSB-based RRM for Rx beam sweeping factor is preferred, i.e. no intra-PRS-occasion Rx beam sweeping (CATT, MTK, Intel, ZTE, HW) - Option 3. For FR1, no impact of UE rx beam sweeping on RSTD measurement period shall be considered. For FR2, at least for the case when the QCL information is available to the UE, there is no impact of UE rx beam sweeping on RSTD measurement period. (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#6 | Sub-topic 3-6 Measurement period with multiple PRS periodicities Tentative agreements: None Candidate options: - Option 1. Use the maximum PRS resource periodicity among all PRS resources (QC, MTK, HW) - Option 2. LMF needs not to provide multiple PRS periodicity to UE with a single layer at least (Intel) - Option 3. FFS (Ericsson) - discuss this after we agree on a measurement period for the same PRS periodicity Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#7 | Sub-topic 3-7 Measurement period with multiple PRS frequency layers Tentative agreements: None Candidate options: - Option 1. Measurement periods adds up for each frequency layer that is configured to be measured (QC, Intel, HW) - Decide after single frequency layer requirement is settled (HW) - Option 2. Measurement period is scaled by number of frequency layers (CATT, MTK) - Option 3. Based on CSSF (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#8 | Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Tentative agreements: None Candidate options: - Option 1. No (QC, CATT, HW) - measurement period will be defined for cases when PRS occasions are not dropped for any reason (QC, HW) - Option 2. Yes (Ericsson, Intel) - When the RSTD measurement period is extended, the increase in the measurement period depends at least on the periodicity of the PRS resource which has non-available occasions and the number of such non-available PRS occasions, but may further depend on the measurement gap configuration, etc. Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#9 | Sub-topic 3-9 Measurement period extension due to HO Tentative agreements: None Candidate options: 3-9-1: Extension of measurement period due to HO - Option 1. TRSTD FDD, NR,HO=TRSTD FDD, NR+k*TPRS+THO ms (CATT, Intel, Ericsson, HW) - k is the number of times the handover occurs during TRSTD FDD, NR,HO - THO is the time during which the RSTD measurement may not be possible due to handover. 3-9-2: Measurement period exceeding responseTime - Option 1. Extension of positioning measurement period due to HO shall not exceed responseTime. UE behaviour in such case is FFS. (QC, Intel) - Option 2. No RAN4 impact (Ericsson) - Option 3. Measurement period requirements on measurement period is independent of responseTime (HW) - if the configured responseTime is smaller than the measurement period, UE should still report but does not need to meet the accuracy requirements, or the report can be only for a subset of measurable resources Recommendations for 2nd round: For 3-9-1, check if option 1 is agreeable. For 3-9-2, further discuss in 2nd round. |
| Sub-topic#10 | Sub-topic 3-10 Calculation of PRS occasion duration Tentative agreements: None Candidate options: - Option 1. RAN4 to define RSTD measurement period based on Type 2 PRS duration calculation. RSTD measurement period for Type 1 PRS duration calculation shall be no longer than Type 2. (QC, Intel, CATT, MTK) - Option 2. Most straightforward way is to use the same (type 1 or type 2) as UE used to report {N,T}, but open to discuss further (HW) - Option 3. Measurement period does not need to depend on this (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 3-1 Basic number of PRS occasions Sub-topic 3-2 Periodicity of PRS occasions Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability Sub-topic 3-4 Sampling and processing time for baseline requirements Sub-topic 3-5 Rx beam sweeping in FR2 Sub-topic 3-6 Measurement period with multiple PRS periodicities Sub-topic 3-7 Measurement period with multiple PRS frequency layers Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Sub-topic 3-9 Measurement period extension due to HO Sub-topic 3-10 Calculation of PRS occasion duration |
| Qualcomm | Sub-topic 3-1: support option 1 with X=4 (similar to LTE). Option 2 is not agreeable as it assumes cross-occasion combining which is impractical and was not even the assumption in LTE. We reiterate again the terminology of option 2 is confusing and not consistent with terminology in 38.211 and 38.214. Sub-topic 3-2: Support option 1. The UE capability T should be considered. Regarding Intel’s comment in the 1st round, it cannot be guaranteed from NW side that the PRS configuration is met according to UE capability. PRS is typically a broadcast signal intended for a large population of UEs with different capabilities. So for some UE’s, configured PRS may exceed their capabilities. Sub-topic 3-3: We can agree to option 1a as it accounts for the case when assistance data is received just after a PRS occasion passed by and so UE has to wait until next PRS occasion. Regarding MTK comment: don’t understand why the scaling factor changes when Tprs > T. The number of resource per slot is a separate UE capability and does not change with relation to {N,T} no matter how large or small Tprs is. Regarding Ericsson’s comment: all of these formulations are based on measurements with MG as stated in our Tdoc. All UE capabilities in RAN1 are only defined for measurement with MG so far. Sub-topic 3-4: support option 1. Option 2 does not make sense to us. Sub-topic 3-5: we cannot agree to option 3 based on our comments from 1st round. We think option 1 with UE capability is a reasonable compromise. If this capability is not set for a UE, it uses option 2 method. We note again that high slot repetitions (e.g., 32) that RAN1 has provisioned for PRS becomes rather useless with option 2 and positioning performance will not meet the latency requirements outlined in SI phase. Sub-topic 3-6: Support option 1. Option 2 is not correct. UE can be configured with two PRS periods; one for tracking purpose and one for acquisition. These are all discussed in RAN1 and is not precluded in specifications Sub-topic 3-7: Support option 1. We don’t understand MTK comment from 1st round. summing the measurement period of each frequency layer also means that UE does one frequency layer processing at a time. Sub-topic 3-8: Support option 1. It is paradoxical that Intel wants to downscope the important positioning topics and yet supports option 2 in this case. Sub-topic 3-9-1: option 1 is ok. Sub-topic 3-9-2: We can agree to option 2. Sub-topic 3-10: support option 1. We think this is reasonable and type 1 will typically not be longer than Type 1 in terms of measurement period. Regarding Huawei’s example from the 1st round, we cannot understand how the example can actually happen in real life. Some further clarification would be helpful. Regarding Ericsson’s comment: measurement period depends on every UE capability signaling that is provisioned in RAN1. |
| CATT | Sub-topic 3-1 Basic number of PRS occasions Support option 1. Sub-topic 3-2 Periodicity of PRS occasions Support option 1. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability Support option 1a or option 3. Sub-topic 3-4 Sampling and processing time for baseline requirements Support option 1. Sub-topic 3-5 Rx beam sweeping in FR2 Support option 2. Sub-topic 3-6 Measurement period with multiple PRS periodicities Support option 1. Sub-topic 3-7 Measurement period with multiple PRS frequency layers We are fine with option 1 or option 2. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Support option 1. Sub-topic 3-9 Measurement period extension due to HO For 3-9-1, Support option 1. For 3-9-2, Support option 3. Sub-topic 3-10 Calculation of PRS occasion duration For 3-9-1, Support option 2. First define the unified requirement based on capability {N,T}, when the difference due to calculation type is realized, the subscript like Ntype1 and Ntype2 can be used. |
| MTK | Sub-topic 3-1: Option 1 Sub-topic 3-2: Option 1 Sub-topic 3-3: We update option 2 as follows: - LPRSN×NPRSslotN', if Tprs = T - LPRSN×NPRSslotN'×TTprs , of Tprs > T - FFS for Tprs < T The reason for bullet 1 is as what we explain in first round discussion: for example, if LPRS=2N and NPRSslot=2N', then the required PRS occasion should be scaled by 4 since the PRS resources for UE to process is 4 times larger than UE’s PRS processing capability. If this is not true, then in our understanding the term NPRSslotN' is not needed at all as UE is not expected to process more than N' PRS resources. The reason for bullet 2 is that - UE buffering capability is the same regardless of Tprs, so the factor LPRSN remains unchanged - TPRS>T means that UE is given more time to process PRS. For example, consider Tprs = 2T and NPRSslot=2N'. In this case, UE can process N’ PRS resources within T ms. Since UE is given 2T time for PRS processing, UE can then process 2N’ PRS resource within 2T ms. The second term NPRSslotN'×TTprs will then equal to 1 in this case. Again, if this is not true, then in our understanding the term NPRSslotN' is not needed at all as UE is not expected to process more than N' PRS resources. The reason for bullet 3 is that - It is not confirmed that UE support (N, T) also means UE supports down scaling of (N, T), so we suggest it as FFS. Sub-topic 3-4: Option 1 Sub-topic 3-5: Option 2 Sub-topic 3-6: Option 1 Sub-topic 3-7: We understand the point and can support option 1 Sub-topic 3-8: Option 1 Sub-topic 3-9-1: Option 1 Sub-topic 3-9-2: Option 3 Sub-topic 3-10: Option 1 |
| Huawei | Sub-topic 3-1 Basic number of PRS occasions Support option 1 with X=4. Sub-topic 3-2 Periodicity of PRS occasions Support option 1. To Intel on option 2, the capability is per UE while PRS configuration in assistance data can be broadcasted and can be common for all UEs. We do not expect LMF will tailor PRS configuration including Tprs for each individual UE. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability We can support LPRSN×NPRSslotN' for all possible relation between Tprs and T, provided that the requirements do not apply for resources spanning over two sampling periods of N We agree with MTK that the two scaling factors need to be multiplied. For Tprs>T, we need more time to check if UE should be required to process more resources per slot than its reported capability. For Tprs<T, we understand UE should only be required to measure every T and down scaling is not assumed. Sub-topic 3-4 Sampling and processing time for baseline requirements Option 1. Sub-topic 3-5 Rx beam sweeping in FR2 We support option 2, but we are also fine with QC proposal to define a UE capability. Sub-topic 3-6 Measurement period with multiple PRS periodicities Option 1. Sub-topic 3-7 Measurement period with multiple PRS frequency layers Option 1. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period Option 1. Sub-topic 3-9 Measurement period extension due to HO Sub-topic 3-9-1: Option 1 Sub-topic 3-9-2: Option 3 Sub-topic 3-10 Calculation of PRS occasion duration Option 2. Response to QC, we understand it is up to UE to choose which type (1 or 2) is assumed when reporting {N,T}, and the reported N can be different for type 1 or type 2 for the same UE implementation. So the duration should be calculated based on the type UE chooses for reporting. With option 1, the requirements may be unnecessarily tightened or relaxed for UE reporting with type 1. |
| Intel | Sub-topic 3-1 Basic number of PRS occasions The margin of “4” can be bracketed we can check how much needed with further evaluation. ] Sub-topic 3-2 Periodicity of PRS occasions We can support Option 1 if NW misaligned with UE’s capability. Sub-topic 3-3 Scaling of PRS occasions due to UE processing capability If the assume the minimum duration for PRS processing is Ti, the option 1 or 1a is reasonable. Regarding to the sub-topic 3-4, if additional “T” is used, the option 1a shall exclude one T. Sub-topic 3-4 Sampling and processing time for baseline requirements Support Option 1. Sub-topic 3-5 Rx beam sweeping in FR2 Support Option 2 Sub-topic 3-6 Measurement period with multiple PRS periodicities Support Option 1. Sub-topic 3-7 Measurement period with multiple PRS frequency layers Support Option 1. Sub-topic 3-8 Whether PRS occasion dropping due to SSB collision should be accounted in measurement period For the technical point itself, we support Option 2. But this is also related to the discussion of RRM impacts in 217.Can be down-scoped if we can reach consensus in Rel16. 3-9-1: Extension of measurement period due to HO Option 1. 3-9-2: Measurement period exceeding responseTime Option 1 can avoid too long HO Sub-topic 3-10 Calculation of PRS occasion duration Option 2 and 3 is same. |
| Ericsson | 3-1: do not agree with option 1 3-2: do not agree with option 1, there are issues as discussed in in GTW 3-3: need to solve the issues above first 3-4: needs further discussion, cannot accept option 1 in this meeting 3-5: GTW agreement 3-6: further investigate, cannot accept option 1 in this meeting 3-7: based on CSSF 3-8: option 2 3-9-1: option 1 3-9-2: option 2 3-10: further discussion is needed |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #4: Measurement period for UE Rx-Tx time difference

The topic is for measurement period requirements for Rx-Tx time difference measurement. The proximity between PRS and SRS is also addressed under this topic. Based on the submitted papers, the framework of the requirements can be same as RSTD requirements, so in this topic only the specific issues for Rx-Tx time difference are addressed.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006170 | Qualcomm Incorporated | Observation 6. It is not necessary to make the UE Rx-Tx time difference measurement period a function of TSRS. Observation 2. UE motion, UE clock drift and gNB clock drift can result in significant errors for measurements that are performed apart in time but are all used to generate the same position fix. Proposal 7. The measurement requirements for UE Rx-Tx timing difference is applicable only if the configured parameters SRS-Slot-offset and SRS-Periodicity for SRS resource for positioning are such that any SRS transmission is within [-25, 25] msec of at least one DL PRS resource of each of the TRPs in the assistance data. Observation 3. SRS for positioning is configured on-demand and released when positioning session is completed. With sparse PRS periods, there is sufficient capacity for SRS transmission both within and outside of the [-25, 25]ms range of every PRS period. |
| R4-2006234 | CATT | Proposal 4: UE Rx-Tx time difference measurement period is not dependent on SRS periodicity. |
| R4-2007845 | Huawei, HiSilicon | Proposal 1: SRS periodicity is not accounted in UE Rx-Tx time difference measurement period. Proposal 5: Rx-Tx timing difference measurement period requirements apply provided that there is at least one SRS transmission within the measurement period. |
| R4-2007943 | Ericsson | - Observation 1: SRS and PRS are configured by different network nodes (serving cell and LMF, respectively). - Observation 2: SRS and PRS may be on different frequencies (inter-frequency UE Rx-TX). - Observation 3: UE Rx-Tx measurement period depends on the periodicity of SRS signals. - Observation 4: UE Rx-Tx measurement period depends on the periodicity of PRS signals. - Proposal 2: UE Rx-Tx measurement period depends on max(PRS periodicity, SRS periodicity). - Proposal 3: The requirements for UE Rx-Tx apply regardless of the time separation between SRS and PRS. - Proposal 4: If the closest subframes #i and #j are separated by more than ½ subframe, the UE shall compensate for the difference in the received timing of radio frame #i used for TUE-TX estimation and the subframe #j.Proposal 5: UE Rx-Tx measurement period is extended by T, to compensate for the number of PRS occasions dropped due to their overlap with SSB symbols. - The maximum for the allowed extension T shall be limited. - Proposal 6: UE Rx-Tx measurement period is extended due to dropped SRS. - Proposal 7: When the measurement period is extended, the increase in the measurement period depends at least on the periodicity of the PRS/SRS resource which has non-available occasions and the number of such non-available PRS/SRS occasions, but may further depend on the measurement gap configuration, DRX, etc. - The extension is proportional to max(TPRS,TSRS)*X, where TPRS and TSRS are the PRS periodicity and SRS periodicity, respectively, and X is the number of time intervals of length max(TPRS,TSRS) where at least one of PRS and SRS is dropped. - Proposal 8: If the number of PRS/SRS occasions not available at the UE exceeds an acceptable limit, the measurement can be dropped, i.e., no further extension of the measurement period is allowed. - Proposal 9: The requirements for UE Rx-Tx time difference measurement apply, provided the NTA offset has not changed during the measurement period. - Proposal 10: When no measurement gaps are used, the UE Rx-Tx measurement period can be defined as: T = ceil(NPRS,req / KPRS) max(TPRS, TSRS) + T, where - KPRS= LPRS/CombSizeN ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, - LPRS is the number of PRS symbols pe slot, - T is the extension due to dropped signals (see section 2.2.3), - NPRS,req comb pattern realizations are required for a UE Rx-Tx time difference measurement. - Proposal 11: When measurement gaps are used, K´PRS (K´PRS≤KPRS) is the number of comb realizations within the effective measurement time of a measurement gap, and the measurement period becomes: T = ceil(NPRS,req / K´PRS) max(TPRS, TSRS, MGRP) CSSF + T. - Proposal 12: When the on-going UE Rx-Tx measurement continues under a serving cell change (according to the earlier agreement), the UE Rx-Tx time difference measurement period is extended, and the extension depends on the number of serving cell changes and on the corresponding interruption time. - Proposal 19: The UE shall continue the UE Rx-Tx time difference measurement during which timing adjustment for its UL transmissions, autonomous adjustment or based on configured TA, occurred one or more times. |
| R4-2006557 | Intel Corporation | Observation 2: UE Rx-Tx measurement time does NOT depend SRS periodicity but SRS proximity (e.g. the closest “j”th UL SRS subframe to UE reception time for “i”th DL PRS subframe). Observation 3: UE Rx-Tx time difference measurement period delay relies on DL PRS reception and processing time only because SRS transmission timing can be pre-known by UE before UE detected DL PRS timing successfully. Proposal 2: UE Rx-Tx measurement delay depends on PRS periodicity, which can be same as that of PRS RSTD [2]. |

Open issues summary

Sub-topic 4-1 Whether SRS periodicity should be accounted in UE Rx-Tx time difference measurement period

- Option 1. No (QC, CATT, HW, Intel)

- Option 2. Yes (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period

- Option 1. Yes (Ericsson)

- Option2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 4-3 Measurement period with HO

- Option 1. When the UE Rx-Tx time difference measurement period is extended, the extension depends on the number of serving cell changes and on the corresponding interruption time (Ericsson)

- Option 2. UE should re-start the Rx-Tx time difference measurement after cell change (HW)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 4-4-1 Measurement period with TA change: UE behavior

- Option 1. The UE shall continue the UE Rx-Tx time difference measurement during which timing adjustment for its UL transmissions, autonomous adjustment or based on configured TA, occurred one or more times (Ericsson)

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 4-4-2 Measurement period with NTA_offset change: Requirements applicability

- Option 1. The accuracy requirements for UE Rx-Tx time difference measurement apply, provided the NTA offset has not changed during the measurement period. (Ericsson)

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 4-5 Proximity between SRS and PRS

- Option 1. The measurement requirements for UE Rx-Tx timing difference is applicable only if the configured parameters SRS-Slot-offset and SRS-Periodicity for SRS resource for positioning are such that any SRS transmission is within [-25, 25] msec of at least one DL PRS resource of each of the TRPs in the assistance data.  (QC)

- Option 2. Proposal 5: Rx-Tx timing difference measurement period requirements apply provided that there is at least one SRS transmission within the measurement period. (HW)

- Option 3. The requirements for UE Rx-Tx apply regardless of the time separation between SRS and PRS. If the closest subframes #i and #j are separated by more than ½ subframe, the UE shall compensate for the difference in the received timing of radio frame #i used for TUE-TX estimation and the subframe #j. (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period We support option 2. Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period We support option 1. Sub-topic 4-3 Measurement period with HO We support Option 1. We do not understand Option 2 – we already had an agreement from RAN4#93: - If the cell change occurs on the serving cell where the SRS is configured then after the serving cell change: - the UE shall restart the UE Rx-Tx time difference measurement; - otherwise the UE shall continue the ongoing UE Rx-Tx time difference measurement. Sub-topic 4-4-1 Measurement period with TA change: UE behavior Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Support option 1. This condition can better be specified under the accuracy requirement (see also topic #8) i.e. UE Rx-Tx accuracy applies provided NTA_offset does not change over the UE Rx-Tx measurement period. Sub-topic 4-5 Proximity between SRS and PRS We support option 3 |
| Qualcomm | Subtopic 4-1: Support option 1 Subtopic 4-2: Not needed. Same comment as in subtopic 3-8. Subtopic 4-3: We can support option 1 based on previous agreements. Subtopic 4-4-1: We cannot support option 1 which bundles all timing adjustment (gNB initiated or UE initiated) in the same way. UE autonomous timing adjustment has to meet certain min/max slew limits whereas TA update by serving gNB is a sudden jump. They cannot be treated the same way. Moreover, for this issue whatever is decided to be applicable to UE has to be applicable to gNB Rx-Tx time difference as well. Subtopic 4-4-2: Option 1 is reasonable. Subtopic 4-5: Support option 1. Option 3 is merely a suggestion that has no place in the specification. What UE needs to do is meet the transmit timing accuracy requirements and how/if UE does compensation is an implementation issue. For option 2, we have questions for Huawei: the measurement period can last several seconds depending on need for MG, number of resources to be measured, and UE capability. We also note that Huawei’s suggestion for Rx beam sweeping further prolongs the measurement period. How is it possible to just have one SRS transmission for the entire measurement period? Also, given that there is reciprocity between UE Rx-Tx and gNB Rx-Tx time difference measurement, is Huawei suggesting that one SRS transmission from UE is sufficient for gNB to meet the measurement accuracy requirements? We have addressed the concerns about the impact to SRS capacity in our paper. We are also open to other suggestions that is beneficial to both gNB and UE Rx-Tx time difference measurements (as option 1 is). For instance, same PRS and SRS periodicity. But we cannot agree to having one SRS transmission for the entire measurement period. In our view, this means measurement accuracy requirements have to be defined very loosely which effectively kills multi-RTT method. |
| Intel | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Support Option 1 Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period UE Rx-Tx time difference measurement period delay relies on DL PRS reception and processing time only because SRS transmission timing can be pre-known by UE before UE detected DL PRS timing successfully. So we doesn’t think SRS dropping shall include such delay requirements. Sub-topic 4-3 Measurement period with HO Support Option 1. Sub-topic 4-4 -1Measurement period with TA change The option 1 is fine for us because the TA message can be maintained by UE and LMF. - Sub-topic 4-4-2 Measurement period with NTA_offset change: Requirements applicability NTA_offset will not impact the final location estimation in NR if both UE Rx-Tx and gNB Rx-Tx time difference measurement reports are aligned to included it or not. So we don’t think option 1 is necessary because NTA_offset can be out of the reporting results. Sub-topic 4-5 Proximity between SRS and PRS We support Option 2. |
| CATT | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Support option 1. Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Since we support option 1 in sub-topic 4-1, the SRS periodicity is not accounted in Rx-Tx time difference measurement period, the SRS dropping should be also not accounted. Sub-topic 4-3 Measurement period with HO Support option 2. I think option 2 is not conflicted with previous agreement, HO behavior is a serving cell change and according to the agreement, UE shall restart the measurement in this case, Sub-topic 4-4-1 Measurement period with TA change: UE behavior Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability NTA_offset is a constant value when the FR and UL band is determined, I don’t understand when the NTA_offset will change. Sub-topic 4-5 Proximity between SRS and PRS Support option 2. |
| Huawei | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Option 1. By definition in 38.215, the Tx timing is not dependent on SRS transmission. Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Option 2, same as for 3-8. Sub-topic 4-3 Measurement period with HO Option 2. On option 1, at cell changes, the UL timing will also change (DL reference is changed, TA may also be changed) even the SRS configuration is same in the two cells. We think UE should re-start the measurement. Sub-topic 4-4-1 Measurement period with TA change: UE behavior We think option 1 is reasonable for measurement period, but UE does not need to meet the accuracy. For gNB side, as gNB is not aware of TA change in a neighbor cell UE, gNB will continue the measurement, so we think the same should be applied for UE side. Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability We do not see the need to have this, because in our view UE is not required to meet accuracy requirements if there is TA change, which also includes change of NTA offset. Also, we think NTA offset is fixed once the network is deployed, so it is a corner case to have NTA offset change during the Rx-Tx measurement period. Sub-topic 4-5 Proximity between SRS and PRS Option 2. On option 1, we can understand the motivation to have proximity conditions from positioning performance side, but this will cause restrictions to the network. In particular, what we are concerned is the SRS capability for positioning UE instead of regular UE, and that will limit the use of multi-RTT especially when PRS periodicity is large. Om option 3, we think it is not addressing the same issue. UE should do this compensation per definition of Rx-Tx. In response to Qualcomm question: what we are proposing is to ensure at least one SRS in the measurement period, mainly to account for the case aperiodic SRS is used for multi-RTT, but we are not proposing to limit it to one. We are also open to discuss possible restrictions on SRS and PRS periodicity, and to us it is easier for network compared to limit the SRS offset around PRS. |
| MTK | Sub-topic 4-1: option 1 Sub-topic 4-2: Option 2, no need to consider it Sub-topic 4-3: option 2 Sub-topic 4-4-1: UE can continue Rx-Tx time difference measurement if TA changes. However, we suggest RAN4 not to define test cases for this scenario Sub-topic 4-4-1: Agree with CATT’s view Sub-topic 4-5: We support option 2. |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

| Sub-topic#1 | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Tentative agreements: None Candidate options: - Option 1. No (QC, CATT, HW, Intel, MTK) - Option 2. Yes (Ericsson) Recommendations for 2nd round: Check if option 1 is agreeable. |
| --- | --- |
| Sub-topic#2 | Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Tentative agreements: None Candidate options: - Option 1. Yes (Ericsson) - Option2. No (QC, Intel, CATT, HW, MTK) Recommendations for 2nd round: Check if option 2 is agreeable. |
| Sub-topic#3 | Sub-topic 4-3 Measurement period with HO Tentative agreements: None Candidate options: - Option 1. When the UE Rx-Tx time difference measurement period is extended, the extension depends on the number of serving cell changes and on the corresponding interruption time (Ericsson, QC, Intel) - Option 2. UE should re-start the Rx-Tx time difference measurement after cell change (HW, CATT, MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#4-1 | Sub-topic 4-4-1 Measurement period with TA change: UE behavior Tentative agreements: None Candidate options: - Option 1. The UE shall continue the UE Rx-Tx time difference measurement during which timing adjustment for its UL transmissions, autonomous adjustment or based on configured TA, occurred one or more times (Ericsson, Intel, HW, MTK) - No test case for this case (MTK) - Option 2. FFS (QC) Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#4-2 | Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Tentative agreements: None Candidate options: - Option 1. The accuracy requirements for UE Rx-Tx time difference measurement apply, provided the NTA offset has not changed during the measurement period. (Ericsson, QC) - Option 2. Not necessary to capture in specification (Intel, CATT, HW, MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#5 | Sub-topic 4-5 Proximity between SRS and PRS Tentative agreements: None Candidate options: - Option 1. The measurement requirements for UE Rx-Tx timing difference is applicable only if the configured parameters SRS-Slot-offset and SRS-Periodicity for SRS resource for positioning are such that any SRS transmission is within [-25, 25] msec of at least one DL PRS resource of each of the TRPs in the assistance data. (QC) - Option 2. Rx-Tx timing difference measurement period requirements apply provided that there is at least one SRS transmission within the measurement period. (HW, Intel, CATT, MTK) - Option 3. The requirements for UE Rx-Tx apply regardless of the time separation between SRS and PRS. If the closest subframes #i and #j are separated by more than ½ subframe, the UE shall compensate for the difference in the received timing of radio frame #i used for TUE-TX estimation and the subframe #j. (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Sub-topic 4-3 Measurement period with HO Sub-topic 4-4-1 Measurement period with TA change: UE behavior Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Sub-topic 4-5 Proximity between SRS and PRS |
| Qualcomm | Sub-topic 4-1: support option 1 based on definition of UE Rx-Tx in 38.215. Sub-topic 4-2: support option 2. Similar to the same issue in RSTD. Sub-topic 4-3: support option 1. To Huawei’s comment from 1st round: if UL timing changes but SRS and PRS periods are close to each other (which is what we propose in sub-topic 4-5) and the measurement accuracy is based on one sample (which is what QC/HW support), then continuing with the measurement after HO should be possible. Without these two conditions, we cannot agree to option 1 though. Sub-topic 4-4-1: we cannot agree on option 1 as it bundles two different types of adjustments together. One is based on autonomous adjustment with slew rate limits and one is gNB-issues TA. Furthermore, this issue should also be viewed together with gNB Rx-Tx time difference requirements. Sub-topic 4-4-2: we support option 2. Sub-topic 4-5: As mentioned in the 1st round, we cannot support option 2 as it makes UE Rx-Tx time difference requirements applicable with only one SRS transmission during the entire measurement period which is unreasonable in our view. Option 3 is also not agreeable as it misses the mark on challenges of measuring Rx-Tx time difference. Apart from time drift, we also like to mention the case when UE is mobile. Even with pedestrian speed mobility, meeting decent accuracy requirements for Rx-Tx is challenging if SRS period is too large compared to PRS period. |
| CATT | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Support option 1. Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Support option 2. Sub-topic 4-3 Measurement period with HO Support option 2. Sub-topic 4-4-1 Measurement period with TA change: UE behavior Support option 1 but no accuracy requirement is needed in this case. Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Support option 2. Sub-topic 4-5 Proximity between SRS and PRS Support option 2. |
| Huawei | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Option 1. Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Option 2. Sub-topic 4-3 Measurement period with HO FFS, the issue needs to be considered together with gNB measurement. Sub-topic 4-4-1 Measurement period with TA change: UE behavior FFS, the issue needs to be considered together with gNB measurement. Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Option 2. Sub-topic 4-5 Proximity between SRS and PRS FFS, we still think option 1 is too restrictive on network side, and may ultimately limit the usage of multi-RTT in real deployment. |
| Intel | Sub-topic 4-1 Whether SRS periodicity should be accounted in Rx-Tx time difference measurement period Support Option 1 Sub-topic 4-2 Whether SRS dropping should be accounted in measurement period Support Option 2 Sub-topic 4-3 Measurement period with HO Support Option 1. Keep align with that of RSTD with HO. Sub-topic 4-4-1 Measurement period with TA change: UE behavior Support Option 1 Sub-topic 4-4-2 Measurement period with NTA_offset change : Requirements applicability Support option 2 Sub-topic 4-5 Proximity between SRS and PRS Support Option 2 |
| MTK | Sub-topic 4-3: Support option 2. It is expected TA would be changed after cell change. The serving cell site would be changed accordingly. As such, the old Rx-Tx measurements are no longer valid. We would like to clarify that this is an discussion on UE behavior, and is not related to how accuracy requirements should be defined. Sub-topic 4-4-1: Support option 1 but no accuracy requirement is needed in this case. Sub-topic 4-4-2: Support option 2 Sub-topic 4-5: Support option 2 |
| Ericsson | 4-1: still option 2 4-2: still option 1 4-3: option 1 4-4-1: option 1 4-4-2: option 1 4-5: option 3 |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #5: Measurement capability

The topic is for measurement capability requirements, e.g. number of TRPs, PRS resource sets or PRS resources UE shall measure per frequency layer, TRP or resource set, for RSTD and Rx-Tx time difference measurements. The number of RSTD or Rx-Tx time difference measurements UE shall be able to measure per TRP pair or per TRP is also in the scope. Based on the submitted papers, the proposals for these two measurements are quite similar.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Observation 1. Minimum capabilities defined in RAN1 are more than sufficient to make DL-TDOA work in any scenario. Proposal 4. RAN4 to not define a minimum value for number of RSTD measurements that UE shall be capable of reporting. Proposal 5. If the DL-PRS assistance data provides the UE with a number of DL-PRS resources beyond its capabilities, the UE assumes the DL-PRS resources in the assistance data are sorted in a decreasing order of priority. The UE is expected to measure and report the PRS resources according to its capability and the decreasing order of priority and meet the corresponding measurement requirements. Observation 1. Minimum capabilities defined in RAN1 are more than sufficient to make multi-RTT work in any scenario. Proposal 5. RAN4 to not define a minimum value for number of UE Rx-Tx time difference measurements that UE shall be capable of reporting. |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 8: RAN4 not to define capability requirements for X2…X7. The need for measurement capability requirements for RSTD, which is addressed in our companion paper [2], can also apply for Rx-Tx time difference. |
| R4-2007944 | Ericsson | - Proposal 9: During the RSTD measurement period T specified in TS 38.133, the UE shall be able to perform RSTD measurements for up to MPRS PRS resources per PRS resource set (MPRS≤X4=64) in up to MPRSset per TRP (MPRSset≤X3=2) for up to MTRP TRPs per carrier frequency (MTRP≤X2=64). |

Open issues summary

Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement

- Option 1. No (QC, HW)

  - Reported capabilities defined by RAN1 are sufficient

- Option 2. Yes (Ericsson)

  - During the RSTD measurement period T specified in TS 38.133, the UE shall be able to perform RSTD measurements for up to MPRS PRS resources per PRS resource set (MPRS≤X4=64) in up to MPRSset per TRP (MPRSset≤X3=2) for up to MTRP TRPs per carrier frequency (MTRP≤X2=64).

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities

- Option 1. UE assumes the DL-PRS resources in the assistance data are sorted in a decreasing order of priority. The UE is expected to measure and report the PRS resources according to its capability and the decreasing order of priority and meet the corresponding measurement requirements (QC)

- Option2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability

- Option 1. If the time span of a DL PRS resource instance (i.e., the time duration spanned after repetition by DL-PRS-ResourceRepetitionFactor) ± its corresponding DL-PRS-expectedRSTD-uncertainty is greater N, where N is the duration of DL PRS symbols in ms that UE is capable of processing, measurement requirements do not apply. (QC, HW)

- Option2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement RAN4 still needs to define the numbers for which the requirements in 38.133 apply. These numbers can be FFS and may or may not be different from those in the signaling. Further clarification: X1-X7 is just the RAN1 parameter list, will not be defined in any RAN1 specification, but will be the maximum numbers for the corresponding signaling ranges for parameters configured by the network, which are now under discussion in RAN2. We always have some ranges for RAN2 parameters but we still have RAN4 requirements and the parameter settings/ranges for which the requirements apply. The same should also be done for the signaled parameters corresponding to X1-X7 ranges defined by RAN1. Furthermore, we need to discuss in RAN4 the relation of the RAN4 requirements to the corresponding UE capabilities, e.g., one requirement for all capabilities or one requirement per capability (two extreme cases), etc. Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Further discussion is needed Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Needs further discussion. At least there should not be relation between the search window and the repetitions, etc. |
| Qualcomm | Subtopic 5-1: Support option 1. We do not agree with Ericsson’s comment above. Measurement period can be parametrically formulated and work for any number of PRS resources. In fact, option 2 is not saying anything new compared to RAN1 agreements and is redundant. Subtopic 5-2: Support option 1. This was how LTE-OTDOA was and the priority is established in the list of assistance data. Subtopic 5-3: Support option 1.UE can process different PRS resources in different times/chunks but UE should not be required to process one PRS resource in multiple segments and stitch them together. |
| Intel | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Support Option 1 Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities In our views, if the DL-PRS assistance data provides the UE with a number of DL-PRS resources beyond its capabilities, the UE can freely to select the DL-PRS resources used for measurement. It can be total up to UE implementation. No need to restrict the UE behaviour indeed. Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability As our understanding, LMF’s configuration shall follow UE’s capability. Otherwise, the measurement requirements can be not applicable. The question to the proponents of Option 1, if the measurement latency requirements is defined based on “T” which is up to UE itself, the problem here is impossible to be happened, why need we such requirements (exceeding UE processing capability) ? |
| CATT | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Support option 1. It has been defined in RAN1. Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Support option 1. |
| Huawei | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Option 1. Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities We understand this is an issue for RAN1 or RAN2. For RAN4 requirements, UE is only required to measure according to its indicated capability. Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Option 1. |
| MTK | Sub-topic 5-1: Option 1 Sub-topic 5-2: Option 1 Sub-topic 5-3: Support option 1. We note that in practice UE cannot still measure part of configured PRS resource within the PRS resource instance. Supporting option 1 simply means that RAN4 will not define requirements and test cases for this scenario. |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Tentative agreements: None Candidate options: - Option 1. No (QC, HW, Intel, CATT, MTK) - Reported capabilities defined by RAN1 are sufficient - Option 2. Yes (Ericsson) - During the RSTD measurement period T specified in TS 38.133, the UE shall be able to perform RSTD measurements for up to MPRS PRS resources per PRS resource set (MPRS≤X4=64) in up to MPRSset per TRP (MPRSset≤X3=2) for up to MTRP TRPs per carrier frequency (MTRP≤X2=64). Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#2 | Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Tentative agreements: None Candidate options: - Option 1. UE assumes the DL-PRS resources in the assistance data are sorted in a decreasing order of priority. The UE is expected to measure and report the PRS resources according to its capability and the decreasing order of priority and meet the corresponding measurement requirements (QC) - Option 2. UE can freely to select the DL-PRS resources used for measurement. It can be total up to UE implementation (Intel) - Option 3. The issue is for RAN1/2, no need to discuss in RAN4 (HW) - Option 4. FFS (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#3 | Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Tentative agreements: None Candidate options: - Option 1. If the time span of a DL PRS resource instance (i.e., the time duration spanned after repetition by DL-PRS-ResourceRepetitionFactor) ± its corresponding DL-PRS-expectedRSTD-uncertainty is greater N, where N is the duration of DL PRS symbols in ms that UE is capable of processing, measurement requirements do not apply. (QC, HW, CATT, MTK) - Option 2. LMF’s configuration shall follow UE’s capability. (Intel) - Option 3. FFS (Ericsson) Recommendations for 2nd round: Check if option 1 is agreeable. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability |
| Qualcomm | Sub-topic 5-1: Support option 1. Option 2 does not even say anything new compared to RAN1 agreement because it says “up to”. We can agree to define the requirements based on minimum possible signaled value of each parameter (e.g., if min possible value of X4 = N, then requirements will be defined for X4=N). Sub-topic 5-2: We can support option 2 as well. It is important to note that even if resources are sorted in decreasing order of priority in the list, UE may not be able to measure according to the priority list due to resource muting pattern, or if resource goes through a fade. Our intention is to ensure that UE should not abandon measurements. Because UE can measure a subset of the resources (e.g., first N) and report those and measurement requirements should apply to this subset. Sub-topic 5-3: Support option 1. As discussed in previous sub-topics, option 2 cannot be guaranteed. For example, Also, regarding Ericsson’s comment from 1st round, cross-occasion combining is not practically possible and was never the baseline in LTE. |
| CATT | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Support option 1. Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Support option 3. We think the UE behavior in option 2 is reasonable, but this is no need to define in RAN4 spec. Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Support option 1. |
| Huawei | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Option 1. Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Option 3. For LTE the related UE behavior is defined in 36.355 and we do not see why in NR there will be RAN4 impact. Sub-topic 5-3 Requirements when duration of a single PRS resource exceeding UE processing capability Option 1. |
| Intel | Sub-topic 5-1 Whether RAN4 to define capability requirements for PRS measurement Support Option 1 Sub-topic 5-2 Requirements when PRS configuration in assistance data exceeds reported capabilities Support Option 2 and Option 3. Some questions on the reported capability: which types of these capability? |
| Ericsson | 5-1: Same comment as in 1st round 5-2: needs further discussion, the network cannot guarantee since PRS configuration is cell specific 5-3: needs further discussion, the network cannot guarantee since PRS configuration is cell specific. Do not agree with option 1. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #6: Reporting criteria

The topic is for reporting criteria for RSTD and Rx-Tx time difference measurements. Based on the submitted papers, the proposals for these two measurements are quite similar.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Proposal 6. Ecat = 1 for RSTD per positioning session, which includes RSTD measurements and PRS-RSRP measurements (if configured with RSTD and supported by UE). Proposal 6. Ecat = 1 for UE Rx-Tx time difference per positioning session, which includes UE Rx-Tx time difference measurements and PRS-RSRP measurements (if configured with UE Rx-Tx time difference and supported by UE). |
| R4-2006232 R4-2006234 | CATT | Proposal 8: For RSTD measurement, Ecat = 1 is defined per positioning session which includes the RSTD measurement and PRS-RSRP measurement. Proposal 5: For UE Rx-Tx time difference measurement, Ecat = 1 is defined per positioning session which includes the UE Rx-Tx time difference measurement and PRS-RSRP measurement. |
| R4-2006561 | Intel Corporation | Change #1: NR DL RSTD measurement reporting criteria is introduced Change #2: NR DL RSTD measurement requirements are introduced in Section 9.8. |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 10: Ecat = 1 for RSTD per positioning session, which includes RSTD measurements and PRS-RSRP measurements (if configured with RSTD and supported by UE). The reporting criteria requirements for RSTD, which is addressed in our companion paper [2], can also apply for Rx-Tx time difference. |
| R4-2007944 R4-2007943 | Ericsson | - Proposal 5: Ecat=1 for intra-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per intra-frequency layer. - Proposal 6: Ecat=1 for inter-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per inter-frequency layer. - Proposal 17: Ecat=1 for intra-frequency UE Rx-Tx measurements, 1 report capable of UE Rx-Tx measurements and also PRS-RSRP measurements (when configured for multi-RTT) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per intra-frequency layer. - Proposal 18: Ecat=1 for inter-frequency UE Rx-Tx measurements, 1 report capable of UE Rx-Tx measurements and also PRS-RSRP measurements (when configured for multi-RTT) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per inter-frequency layer. |
| R4-2007958 | Ericsson | Adding reporting criteria for NR RSTD |
| R4-2007959 | Ericsson | Adding reporting criteria for NR UE Rx-Tx |

Open issues summary

Sub-topic 6-1 Reporting criteria for RSTD measurement

The sub-topic is for RSTD, and the agreement can be applied to Rx-Tx time difference.

- Option 1. Ecat = 1 for RSTD per positioning session, which includes RSTD measurements and PRS-RSRP measurements (if configured with RSTD and supported by UE). (QC, CATT, HW)

- Option 2. Reporting criteria as follows (Ericsson)

  - Ecat=1 for intra-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per intra-frequency layer.

  - Ecat=1 for inter-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per inter-frequency layer.

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 6-1 Reporting criteria for RSTD measurement At least we need intra- and inter-frequency reporting criteria, per frequency layer. Then, in LTE 1 report was limited to 16 cells. We need something similar for NR, but the exact wording for the specification can be further discussed. |
| Qualcomm | Subtopic 6-1: Option 1. No need for specifying intra-f vs inter-f or any lower limit for the number of resources. All of these are separately signaled via capability signaling. |
| Intel | Sub-topic 6-1 Reporting criteria for RSTD measurement We also prefer to define the criteria for both intra and inter measurement at least. |
| CATT | Sub-topic 6-1 Reporting criteria for RSTD measurement Support option 1. |
| Huawei | Sub-topic 6-1 Reporting criteria for RSTD measurement Option 1, same comment as Qualcomm. |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006561 (Intel) | Ericsson: We need to first resolve the technical issues, we cannot agree on the CR in this meeting, |
|  | Intel: Reply Ericsson’s comments: yes, we fully agree this principle. But so far this meeting this last meeting for this WI. And even with the 1Q extension, there is one meeting left. So it is better to include more contents as possible for CR. We may update the CRs based on the agreements by the end of this meeting. |
|  | Huawei: we suggest to use one CR for introducing reporting criteria for all PRS measurements, based on technical conclusions. |
| R4-2007958 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: cannot be agreed based on comments in 6-1. |
|  | Huawei: we suggest to use one CR for introducing reporting criteria for all PRS measurements, based on technical conclusions. |
| R4-2007959 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: cannot be agreed based on comments in 6-1. |
|  | Huawei: we suggest to use one CR for introducing reporting criteria for all PRS measurements, based on technical conclusions. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 6-1 Reporting criteria for RSTD measurement Tentative agreements: None Candidate options: - Option 1. Ecat = 1 for RSTD per positioning session, which includes RSTD measurements and PRS-RSRP measurements (if configured with RSTD and supported by UE). (QC, CATT, HW) - Option 2. Reporting criteria as follows (Ericsson) - Ecat=1 for intra-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per intra-frequency layer. - Ecat=1 for inter-frequency RSTD measurements, 1 report capable of RSTD measurements and also PRS-RSRP measurements (when configured for DL-TDOA) on at least X2 TRPs, X3 PRS resource sets per TRP, and X4 PRS resources per PRS resource set, per inter-frequency layer. - Option 3. At least we need intra- and inter-frequency reporting criteria (Ericsson, Intel) Recommendations for 2nd round: Further discuss in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 6-1 Reporting criteria for RSTD measurement |
| Qualcomm | Support option 1. We don’t understand why reporting criteria has to be defined separately for intra-f and inter-f. There are two capabilities for intra-vs-inter. Number of frequency layers that UE supports and a separate capability siganling provisioned in RAN1 list. |
| CATT | Sub-topic 6-1 Reporting criteria for RSTD measurement Support option 1. |
| Huawei | Sub-topic 6-1 Reporting criteria for RSTD measurement Option 1. |
| Intel | Sub-topic 6-1 Reporting criteria for RSTD measurement According to GTW agreements, we support Option 1 . |
| Ericsson | 6-1: it should not be per positioning session |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #7: Side conditions

The topic is for side conditions for RSTD and Rx-Tx time difference measurements.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Proposal 14. For FR2, the side condition for PRS SNR of reference cell to be -3 dB and the side condition for PRS SNR of neighbor cells to be -10 dB. Proposal 11. No side condition is needed for serving cell in UE Rx-Tx timing difference measurement. |
| R4-2006232 R4-2006234 | CATT | Proposal 12: Side conditions for PRS RSTD measurements in FR2 are defined same as those in FR1, i.e. -13dB for neighbour cell and -6dB for serving cell. Proposal 6： The side condition of serving cell in UE Rx-Tx time difference shall be -6dB. |
| R4-2006304 | MediaTek inc. | Proposal 1: The side conditions for FR2 PRS-RSTD measurements are the same as FR1 |
| R4-2006556 | Intel Corporation | Proposal 1: SINR side conditions for PRS-RSTD in FR2 can be • PRS Es/Iot = -6 dB for reference cell and • PRS Es/Iot = -13 dB for neighbor cells |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 15: Side condition for RSTD accuracy requirements in FR2 is PRS Es/Iot of -3 dB for reference cell and -10 dB for neighbor cells. Proposal 2: The Rx-Tx time difference side condition is only defined for neighbor cell. |
| R4-2007944 R4-2007943 | Ericsson | • Proposal 1: RSTD side conditions for neighbour and reference cell in FR2: same as for FR1. • Proposal 1: Serving cell side condition for UE Rx-Tx: -3 dB, for FR1 and FR2. |

Open issues summary

Sub-topic 7-1 Side condition for RSTD measurement in FR2

- Option 1. PRS SNR of reference cell to be -3 dB and the side condition for PRS SNR of neighbor cells to be -10 dB. (QC, HW)

- Option 2. Side conditions for PRS RSTD measurements in FR2 are defined same as those in FR1, i.e. -13dB for neighbour cell and -6dB for serving cell. (CATT, MTK, Intel, Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 7-2 Whether to define side condition for serving cell in UE Rx-Tx time difference measurement

Background:

agreements from RAN4#94-e-Bis:

- Serving cell:

  - Option 1. -6 dB

  - Option 2. Serving cell side condition for UE Rx-Tx: -3 dB, for FR1 and FR2.

  - Option 3. Not needed

- Reference cell:

  - Not needed.

Proposals:

- Option 1. No (QC, HW)

- Option 2. Yes (CATT, Ericsson)

  - CATT: -6dB

  - Ericsson: -3dB

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 7-1 Side condition for RSTD measurement in FR2 We support option 2. Sub-topic 7-2 Whether to define side condition for serving cell in Rx-Tx time difference measurement We support option 2. |
| Qualcomm | Subtopic 7-1: Support option 1. Subtopic 7-2: Support option 1. |
| Intel | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Support Option 2. We can deprioritize this discussion in the performance part if we can’t achieve any agreements. Sub-topic 7-2 Whether to define side condition for serving cell in Rx-Tx time difference measurement Support option 1. |
| CATT | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Support option 2. Sub-topic 7-2 Whether to define side condition for serving cell in Rx-Tx time difference measurement Support option 2. UE Rx-Tx time difference is performed for single cell which can be serving cell and neighbor cell, and the power of receive signal is different for serving cell and neighbor cell, so the side condition should be defined separately. |
| Huawei | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Option 1. Sub-topic 7-2 Whether to define side condition for serving cell in Rx-Tx time difference measurement Option 1. |
| MTK | Sub-topic 7-1: Support option 2 Sub-topic 7-2: Support option 2 with -6 dB. |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Tentative agreements: None Candidate options: - Option 1. PRS SNR of reference cell to be -3 dB and the side condition for PRS SNR of neighbor cells to be -10 dB. (QC, HW) - Option 2. Side conditions for PRS RSTD measurements in FR2 are defined same as those in FR1, i.e. -13dB for neighbour cell and -6dB for serving cell. (CATT, MTK, Intel, Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#2 | Sub-topic 7-2 Whether to define side condition for serving cell in UE Rx-Tx time difference measurement Tentative agreements: None Candidate options: - Option 1. No (QC, HW, Intel) - Option 2. Yes (CATT, Ericsson, MTK) - CATT, MTK: -6dB - Ericsson: -3dB Recommendations for 2nd round: Defer the discussion on this topic after we have conclusion on sub-topic 8-5. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 7-1 Side condition for RSTD measurement in FR2 |
| Qualcomm | Sub-topic 7-1: option 1 based on system level and link level simulation results. |
| CATT | Support option 2. |
| Huawei | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Option 1. |
| Intel | Sub-topic 7-1 Side condition for RSTD measurement in FR2 Support Option 2. But it can be FFS in the performance part. |
| MTK | Sub-topic 7-1: Option 2 |
| Ericsson | 7-1: Option 2 7-2: option 2 |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #8: Measurement accuracy

The topic is for measurement accuracy for RSTD and Rx-Tx time difference measurements.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Proposal 13. PRS-RSTD measurement accuracy requirements to be defined using samples from only one DL PRS resource repetition for reference and neighbor, i.e., the time duration spanned by one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor. Combining measurements across PRS repetitions, if possible and available, to improve performance should be left to UE implementation. Proposal 8. The number of samples, N, used to define the accuracy requirements of Rx-Tx timing difference measurement to be the same as the number of samples used to define the accuracy requirements for RSTD measurements. Proposal 9. Only one set of accuracy requirements applicable to both serving and neighbor cells to be defined. Proposal 10. During the measurement period of each Rx-Tx measurement, i.e., from beginning of the PRS occasion to the end of its corresponding SRS occasion, a TA change shall make measurement accuracy requirements inapplicable. |
| R4-2006232 R4-2006234 | CATT | Proposal 13: The accuracy requirements of RSTD measurement is defined based on one sample. Proposal 14: The accuracy requirements shall be agnostic to comb pattern. Proposal 7：UE Rx-Tx time difference accuracy does not apply when TA change occurs. Proposal 8：No separate accuracy requirements for serving cell and neighbour cell are defined. |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 16: RSTD accuracy requirements are defined based on single shot measurement. Proposal 17: Applicable of RSTD accuracy requirements is not impacted by HO. Proposal 3: Rx-Tx timing difference accuracy requirements do not apply in case of cell change or TA change. Proposal 4: Define one set of Rx-Tx timing difference accuracy for all cells. |
| R4-2007944 R4-2007943 | Ericsson | - Proposal 16: The applicable accuracy requirements for RSTD measurements under cell change: - For intra-frequency HO, intra-frequency accuracy applies; - For inter-frequency HO, - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies, - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies, - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies. - Proposal 17: RAN4 specifies at least the RSTD accuracy requirements (+/-Y dB) under the assumption of using the same antenna panel for receiving both the reference and neighbor PRS resources. - Proposal 18: For different antenna panels within the same RSTD measurement, the RSTD accuracy can be specified as +/-Y+Y dB. - Proposal 13: Applicable accuracy requirements for UE Rx-Tx measurements under cell change: - For intra-frequency serving cell change, intra-frequency accuracy applies; - For inter-frequency serving cell change, - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies, - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies, - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies. - Proposal 20: The UE shall meet the same accuracy requirements, regardless of whether timing adjustment occurred or not. - Proposal 21: The UE can compensate the measurement by the amount of the timing adjustment applied at the UE during the UE Rx-Tx measurement period. |
| R4-2007117 | Nokia, Nokia Shanghai Bell | Proposal 2: Specify UE Rx-Tx time difference accuracy requirements for serving cell and for neighbor cell separately (Option 1). |

Open issues summary

Sub-topic 8-1 Number of samples for RSTD accuracy

- Option 1. 1 sample. (QC, CATT, HW)

- Option 2 (Ericsson). The accuracy and the number of occasions (see subtopic 3-1) depends on the required number of comb realizations (NPRS,req) which is derived from simulations (e.g., see table below); the number of occasions is then ceil(NPRS,req / KPRS) where KPRS= LPRS/CombSizeN  ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, LPRS is the number of PRS symbols per slot.

| FRx, SCS, number of PRBs | Minimum number of comb realizations, NPRS,req |
| --- | --- |
| FR1, 15 kHz, ≤52 PRBs (10 MHz) | 4 |
| FR1, 15 kHz, ≤104 PRBs (20 MHz) | 2 |
| FR1, 15 kHz, >104 PRBs | 1 |
| FR1, 30 kHz, ≤48 PRBs (20 MHz) | 4 |
| FR1, 30 kHz, ≤132PRBs (30 MHz) | 2 |
| FR1, 30 kHz, ≤272 PRBs (100 MHz) | 1 |
| FR2, 120 kHz, ≤32 PRBs (50 MHz) | 4 |
| FR2, 120 kHz, >32 PRBs | 1 |

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb

- Option 1. The accuracy requirements shall be agnostic to comb pattern (CATT)

- Option 2 (Ericsson). No, see Option 2 in sub-topic 8-1. There are different comb parameters, e.g., one is the basic comb pattern and another one is comb size. Within a slot, the pattern can have different comb sizes (effectively multiple repetitions within a slot), which has to be accounted for in the requirements.

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO

- Option 1. No. (HW)

- Option 2. Yes. (Ericsson)

  - For intra-frequency HO, intra-frequency accuracy applies;

  - For inter-frequency HO,

    - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies,

    - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies,

    - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies.

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 8-4 Assumption on antenna panel

- Option 1. Consider antenna panel in defining accuracy requirements (Ericsson)

  - RAN4 specifies at least the RSTD accuracy requirements (+/-Y dB) under the assumption of using the same antenna panel for receiving both the reference and neighbor PRS resources.

  - Proposal 18: For different antenna panels within the same RSTD measurement, the RSTD accuracy can be specified as +/-Y+Y dB.

- Option 2. Other

Recommended WF:  Further discussion needed. Collect companies’ views.

Sub-topic 8-5 Serving vs. neighbour for UE Rx-Tx time difference accuracy

- Option 1. Only one set of accuracy requirements applicable to both serving and neighbor cells to be defined. (QC, CATT, HW)

- Option 2. Specify UE Rx-Tx time difference accuracy requirements for serving cell and for neighbor cell separately (Nokia, Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views. This sub-topic is related to the side condition discussion

Sub-topic 8-6 Applicability of UE Rx-Tx time difference accuracy requirements with TA change

- Option 1. The accuracy requirements do not apply, if during the measurement period of each Rx-Tx measurement, one or more TA changes occurred. (QC, CATT, HW)

- Option 2. The UE shall meet the same accuracy requirements, regardless of whether timing adjustment occurred or not. The UE can compensate the measurement by the amount of the timing adjustment applied at the UE during the UE Rx-Tx measurement period. (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO

- Option 1. The accuracy requirements do not apply, if during the measurement period of each Rx-Tx measurement, one or more cell changes occurred (HW)

- Option 2. Similar to LTE, applicable accuracy requirements for UE Rx-Tx measurements under cell change (Ericsson)

  - For intra-frequency serving cell change, intra-frequency accuracy applies;

  - For inter-frequency serving cell change,

    - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies,

    - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies,

    - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies.

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy

Based on the discussion from last meeting, the number of samples are Rx-Tx time difference measurements supposed to be discussed after we have link level simulation results. On the other hand, there are proposals submitted on this sub-topic. As the issue also impacts the core requirements, from moderator perspective, companies are still encouraged to provide views in this meeting

- Option 1. The number of samples, N, used to define the accuracy requirements of Rx-Tx timing difference measurement to be the same as the number of samples used to define the accuracy requirements for RSTD measurements (QC)

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 8-1 Number of samples for RSTD accuracy The term “sample” is ambiguous since the accuracy will depend on the comb size, repetitions, etc. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb We support option 2. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO We support option 2. We need the rules on which accuracy applies, similar to LTE. If the type of the measurement changes after HO, then the applicable accuracy requirements change too. Sub-topic 8-4 Assumption on antenna panel We support option 1. Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy We support option 2. Better accuracy should be expected for the serving cell. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO We support option 2. We need the rules on which accuracy applies. If the type of the measurement changes after HO, then the applicable accuracy requirements change too. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy We need to agree on simulation assumptions and see the results first. |
| Qualcomm | Subtopic 8-1: Support option 1. The term “sample” is clearly defined in our paper for so many meetings: the time duration spanned by one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor. In our view, the new invented terminology in option 2 is confusing (what is comb realization?) and the formulation dictates a very specific UE implementation which is not necessary. Subtopic 8-2: Support option 1 (which in our view means given all other conditions being the same, comb pattern has no role in accuracy requirements and this has been shown by a majority of simulation results). Option 2 is not agreeable based per our comments in subtopic 8-1. Subtopic 8-3: We cannot decide this until we have concluded the definition of intra-freq and inter-freq and also the scenarios when UE needs MG for positioning. Subtopic 8-4: We cannot agree to option 1. RSTD accuracy requirements are defined in units of time. What does it even mean to associate it with a dB value? Subtopic 8-5: Support option 1. There is no justification to have separate accuracy requirements. Subtopic 8-6: Option 1. Subtopic 8-7: We cannot decide this until we have concluded the definition of intra-freq and inter-freq and also the scenarios when UE needs MG for positioning. |
| Intel | Sub-topic 8-1 Number of samples for RSTD accuracy Support Option 1 Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb According to our simulation results, the combsize difference will introduce some performance difference. So we prefer to differentiate the accuracy requirement regarding to combsize. But this can be FFS. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Support Option 1 Sub-topic 8-4 Assumption on antenna panel Can be FFS Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy In our view, this is related to the discussion 7-2. If the side condition is for neighbor cell only, option 1 shall be fine. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Can be FFS Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Can be FFS. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Support Option 1 |
| CATT | Sub-topic 8-1 Number of samples for RSTD accuracy Support option 1. In my understanding, 1 PRS sample includes its repetition, the accuracy should depend on the number of repetition realizations not the comb realizations. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Support option 1. Option 2 may have a misunderstanding for comb size, according to RAN1’s agreement below, a comb pattern includes number of symbols and combSize N(values are {2,4,6,12}). The multiple repetitions are represented by the configuration of DL-PRS-ResourceRepetitionFactor According to several companies’ simulation results, the accuracy is not different for different comb size(2,4,6 or 12) Agreement: - At least the following DL PRS RE patterns, with comb size N equal to number of symbols M are supported (figures for information) - Comb-2: Symbols {0, 1} have relative RE offsets {0, 1} - Comb-4: Symbols {0, 1, 2, 3} have relative RE offsets {0, 2, 1, 3} - Comb-6: Symbols {0, 1, 2, 3, 4, 5} have relative RE offsets {0, 3, 1, 4, 2, 5} Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO In my understanding, the accuracy requirement depends on the state after HO, if it is inter-frequency measurement after HO, inter-frequency measurement applys. Sub-topic 8-4 Assumption on antenna panel Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Support option 1. Different side condition can be defined for serving cell and neighbor cell. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Support option 1. Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Support option 1. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Support option 1. |
| Huawei | Sub-topic 8-1 Number of samples for RSTD accuracy Option 1. Same comment as for 3-1. We suggest to account the required number of repetitions in the side condition of accuracy requirements. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Option 1. On option 2, we can understand the point from Ericsson. So far, RAN4 simulation has been done with e.g. 2 symbol for comb-2, but comb-2 can be also configured with a larger number of symbols per slot, and we are open to study on the performance impact, but still, it is considered as configuration for side condition. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Option 1. Regardless of intra/inter-frequency definition, or need for MG, we think the RSTD accuracy should not be impacted due to serving cell change. Sub-topic 8-4 Assumption on antenna panel We suggest to defer the discussion to Perf part given the tight timeline for core part completion. Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Option 1. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Option 1, same as in LTE. We do not think UE shall compensate the UL timing change for Rx-Tx measurement. Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Option 1, we think UE would re-start Rx-Tx measurement in case of HO, so no accuracy requirement should apply. After the new measurement period, UE should meet the corresponding accuracy requirements. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Option 1. |
| MTK | Sub-topic 8-1: Option 1 Sub-topic 8-2: Accuracy requirements may depend on comb pattern Sub-topic 8-3: Agree with QC’s view Sub-topic 8-4: Don’t understand the intention of this proposal. We have already considered side conditions for defining requirements, why need to consider antenna panels? Sub-topic 8-5: Option 1 Sub-topic 8-6: Option 1 Sub-topic 8-7: Agree with QC’s view Sub-topic 8-8: Option 1 |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 8-1 Number of samples for RSTD accuracy Tentative agreements: None Candidate options: - Option 1. 1 sample. (QC, CATT, HW, Intel, MTK) - Option 2 (Ericsson). The accuracy and the number of occasions (see subtopic 3-1) depends on the required number of comb realizations (NPRS,req) which is derived from simulations (e.g., see table below); the number of occasions is then ceil(NPRS,req / KPRS) where KPRS= LPRS/CombSizeN ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, LPRS is the number of PRS symbols per slot. Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#2 | Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Tentative agreements: None Candidate options: - Option 1. The accuracy requirements shall be agnostic to comb pattern (CATT, QC, HW) - Option 2. No, see Option 2 in sub-topic 8-1. There are different comb parameters, e.g., one is the basic comb pattern and another one is comb size. Within a slot, the pattern can have different comb sizes (effectively multiple repetitions within a slot), which has to be accounted for in the requirements. (Ericsson) - Option 3. FFS (Intel, MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#3 | Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Tentative agreements: None Candidate options: - Option 1. No. (HW, Intel) - Option 2. Yes. (Ericsson, CATT) - For intra-frequency HO, intra-frequency accuracy applies; - For inter-frequency HO, - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies, - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies, - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies. - Option 3. FFS (QC, MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#4 | Sub-topic 8-4 Assumption on antenna panel Tentative agreements: None Candidate options: - Option 1. Consider antenna panel in defining accuracy requirements (Ericsson) - RAN4 specifies at least the RSTD accuracy requirements (+/-Y dB) under the assumption of using the same antenna panel for receiving both the reference and neighbor PRS resources. - Proposal 18: For different antenna panels within the same RSTD measurement, the RSTD accuracy can be specified as +/-Y+Y dB. - Option 2. FFS (QC, Intel, HW) - Option 3. No need to discuss (MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#5 | Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Tentative agreements: None Candidate options: - Option 1. Only one set of accuracy requirements applicable to both serving and neighbor cells to be defined. (QC, CATT, HW, Intel, MTK) - Option 2. Specify UE Rx-Tx time difference accuracy requirements for serving cell and for neighbor cell separately (Nokia, Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#6 | Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Tentative agreements: None Candidate options: - Option 1. The accuracy requirements do not apply, if during the measurement period of each Rx-Tx measurement, one or more TA changes occurred. (QC, CATT, HW, MTK) - Option 2. The UE shall meet the same accuracy requirements, regardless of whether timing adjustment occurred or not. The UE can compensate the measurement by the amount of the timing adjustment applied at the UE during the UE Rx-Tx measurement period. (Ericsson) - Option 3. FFS (Intel) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#7 | Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Tentative agreements: None Candidate options: - Option 1. The accuracy requirements do not apply, if during the measurement period of each Rx-Tx measurement, one or more cell changes occurred (HW, CATT) - Option 2. Similar to LTE, applicable accuracy requirements for UE Rx-Tx measurements under cell change (Ericsson) - For intra-frequency serving cell change, intra-frequency accuracy applies; - For inter-frequency serving cell change, - When the measured inter-frequency becomes a serving carrier frequency: inter-frequency accuracy applies, - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies, - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies. - Option 3. FFS (QC, Intel, MTK) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#8 | Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Tentative agreements: None Candidate options: - Option 1. The number of samples, N, used to define the accuracy requirements of Rx-Tx timing difference measurement to be the same as the number of samples used to define the accuracy requirements for RSTD measurements (QC, Intel, CATT, HW, MTK) - Option 2. Discuss after we see simulation results (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 8-1 Number of samples for RSTD accuracy Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Sub-topic 8-4 Assumption on antenna panel Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy |
| Qualcomm | Sub-topic 8-1: Support option 1. Cross-occasion combining should not be the baseline. Sub-topic 8-2: Support option 1. There is comb size which refers to frequency domain arrangement of PRS RE’s (comb-2, comb-4, …) and then number of PRS symbols within a slot. Num symb > comb is allowed but we should not define requirements for the same reason that we don’t define requirements for different power boost levels. The parameter num Symb was not even in the simulation assumptions so why are we talking about it now? Sub-topic 8-3: since now RAN1 has agreed to define capabilities and requirements for MG based measurement only, we support option 1. Sub-topic 8-4: Option 1 is not clear to us. Why is RSTD (a timing measurement) tied to a dB value? Sub-topic 8-5: support option 1. Sub-topic 8-6: we support option 1. Moreover, this is even more important for gNB Rx-Tx and the same requirements should be specified for both. Sub-topic 8-7: same comment as in sub-topic 8-3. Sub-topic 8-8: support option 1. We don’t see how this would change based on simulation results. For both RSTD and Rx-Tx, UE estimates TOA. In one case UE subtracts two TOA’s (RSTD) and in the other Tx timing is subtracted. For both, UE is estimating the same metric! |
| CATT | Sub-topic 8-1 Number of samples for RSTD accuracy Support option 1. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Support option 1. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Support option 2 with a small correction. - For intra-frequency HO, intra-frequency accuracy applies; - For inter-frequency HO, - When the measured inter-frequency becomes a serving carrier frequency: intra-frequency accuracy applies, - When the measured inter-frequency remains inter-frequency: inter-frequency accuracy applies, - When the measured intra-frequency becomes inter-frequency: inter-frequency accuracy applies. Sub-topic 8-4 Assumption on antenna panel Support option 3, there is no need to do this assumption. Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Support option 1. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Support option 1. Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Support option 1. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Support option 1. |
| Huawei | Sub-topic 8-1 Number of samples for RSTD accuracy Option 1. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb FFS, we can further check the impact of number of symbols from simulation. This is an issue for Perf part and brought up for the first time. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Option 1. Sub-topic 8-4 Assumption on antenna panel FFS, This is an issue for Perf part and brought up for the first time. Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Option 1. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Option 1. Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Option 1, it is same issue as TA change in 8-6. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Option 1. |
| Intel | Sub-topic 8-1 Number of samples for RSTD accuracy Support Option 1. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Can be FFS. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Support Option 1 Sub-topic 8-4 Assumption on antenna panel Can be FFS in the performance part Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy |
| MTK | Sub-topic 8-1 Number of samples for RSTD accuracy Support Option 1. Sub-topic 8-2 Whether RSTD accuracy is agnostic to comb Can be FFS. Sub-topic 8-3 Whether applicable RSTD accuracy requirement is impacted by HO Option 3. We understand that PRS must be measured within MG. However, HO means that there exists UE mobility (i.e., UE moves from one cell to another cell). It is not clear yet whether we would consider UE mobility for defining RSTD accuracy requirements, and if considered, what would be the UE speed. Therefore, we would like to keep it FFS. Sub-topic 8-4 Assumption on antenna panel Support option 3, there is no need to do this assumption. Sub-topic 8-5 Serving vs. neighbour for Rx-Tx time difference accuracy Support option 1. Sub-topic 8-6 Applicability of Rx-Tx time difference accuracy requirements with TA change Support option 1. Sub-topic 8-7 Whether applicable Rx-Tx time difference accuracy requirement is impacted by HO Support option 1. Sub-topic 8-8 Number of samples for Rx-Tx time difference accuracy Support option 1. |
| Ericsson | 8-1: see our comments from GTW session 8-2: not, and perhaps not to the number of repetitions, depending on the occasion definition 8-3: yes 8-4: option 1 8-5: option 2 8-6: ok with option 1 8-7: yes, we need to define them 8-8: same comments as for RSTD |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #9: Measurement report mapping

The topic is for remaining issues in report mapping for RSTD and Rx-Tx time difference measurements.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006168 R4-2006170 | Qualcomm Incorporated | Proposal 2. The relationship between LMF recommended value (k1) and the UE selected value (k2) to be: k1≤k2≤log214Tcmini(BWPRS,i) Where i is the positioning layer frequency layer index of all RSTD measurements of the same TRP pair. Proposal 3. In FR1, k1>0. Proposal 2. The relationship between LMF recommended value (k1) and the UE selected value (k2) to be: k1≤k2≤log214Tcmini(BWPRS,i) Where i is the positioning layer frequency layer index of all Rx-Tx time difference measurements of the same TRP. Proposal 3. In FR1, k1>0. Proposal 4. For UE Rx-Tx time difference reporting, signaling of NTA,Offset is not necessary. |
| R4-2006232 R4-2006234 | CATT | Proposal 1: UE selected parameter k2 is larger than or equal to k1. Proposal 2: The range of k is {2,3,4,5} in FR1. Proposal 1: UE selected parameter k2 is larger than or equal to k1. Proposal 2: The range of k is {2,3,4,5} in FR1. Proposal 3: UE Rx-Tx time difference report mapping should be independent on NTA_offset and NTA_offset is no need to be signalled to LMF. |
| R4-2006304 | MediaTek inc. | Proposal 4: k2 >= k1, as long as UE meets the accuracy requirements |
| R4-2006556 R4-2006557 | Intel Corporation | Proposal 5: LMF and UE can choose k1 and k2 independently from {0,1,2,3,4,5}. Observation 1: NTA_offset will not impact the location estimation in NR if both UE Rx-Tx and gNB Rx-Tx time difference measurement reports are aligned to included it or not. Proposal 1: Both UE and gNB Rx-Tx timing difference reporting results and table shall be independent on NTA,Offset. |
| R4-2007841 R4-2007845 | Huawei, HiSilicon | Proposal 18: RAN4 not to define relationship between k1 and k2, or separate range of k1/k2 for FR1 and FR2. Proposal 6: There is no need for UE to signal NTA,Offset to LMF. |
| R4-2007945 R4-2007997 | Ericsson | RSTD: - Proposal 1: The values for k are within the range [kmin,kmax], where for FR1 - kmin=3, - kmax=5. - Proposal 2: No separate table for FR1 are needed. Tables for all k={0,1,2,3,4,5} are specified, but their applicability is clarified in the text. - Proposal 3: k2 (selected by UE) shall not exceed k1 (configured by LMF). - Proposal 4. k2 (selected by UE) shall be within [kmin, kmax]. - Proposal 5: For absolute RSTD measurement report mapping, agree on the measurement report mapping tables according to Tables 1 –6. - Observation: For differential reporting, for all k≠0, the achievable range is smaller than the agreed range [0,8191Tc], and it’s even different depending on k (Option 1 in Tables 7 – 12). - Proposal 6: Ensure the same range for all k’s, e.g., by increasing the last reporting interval to 8192 for all k’s. - Proposal 7: Agree on measurement report mapping tables based on the range from 0 to +8192 Tc, according to Tables 7 –12 (Option 2 in the tables). UE Rx-Tx: - Proposal #1: Both absolute and differential UE Rx-Tx mappings in FR1 are applicable for 3 ≤ k ≤5. - Proposal #2: Both absolute and differential UE Rx-Tx mappings in FR2 are applicable for 0 ≤ k ≤5. - Proposal #3: If any of Rx and Tx components in UE Rx-Tx measurement belong to FR1 then the UE Rx-Tx mapping is applicable for 3 ≤ k ≤5. - Proposal #4: The UE selected k value shall not exceed timingReportingGranularityFactor configured by the LMF. - Proposal #5: Differential UE Rx-Tx mapping is defined from 0 to 8192 over all reportable values with uniform granularity of Tc*2k for all values except the largest value in the range. |
| R4-2007946 | Ericsson | Measurement report mapping for RSTD is added |
| R4-2007998 | Ericsson | Absolute UE Rx-Tx Measurement Report Mapping for k=0, 1, 2, 3, 4 and 5 Differential UE Rx-Tx Measurement Report Mapping for k=0, 1, 2, 3, 4 and 5 The mapping is based on the agreements in LS to RAN2 approved in R4-2005845. |
| R4-2007949 | Ericsson | Observation 1: With all equal step sizes, the maximum range [-8175, 8175] solves the bits issue, but the mapping is not fully symmetric for k≥2 and not fully covering the range for all k’s. Observation 2: To solve the issue in Observation 1, the edge (lowest and highest) intervals for path can be slightly increased for k≥2 to cover up to -8175 and 8175, respectively. No new LS to RAN2 is needed with this approach. Observation 3: Alternatively to the solution in Observation 2, the range [-8160, 8160], which is a multiple of 32, can be considered, but that would change the number of reports and require an LS to RAN2. Proposal 1: Agree on Option 2 in Tables 1-6 for implementing in TS 38.133. |
| R4-2007950 | Ericsson | Report mapping for additional path reporting for RSTD is added |
| R4-2007951 | Ericsson | Report mapping for additional path reporting for UE Rx-Tx is added |
| R4-2007995 | Ericsson | - Observation#1: In FR1, gNB configures one of the three possible values ofwhich impacts the UE Rx-Tx mapping. - Observation#2: included in the UE Rx-Tx measurement needs to be eliminated at LMF to accurately estimate UE position. - Observation#3: gNB Rx-Tx and UE Rx-Tx measurements are gNB and UE capabilities. Therefore, not all gNB and UE may support gNB Rx-Tx and UE Rx-Tx measurements respectively. - Observation#4: It is up to LMF implementation whether to use gNB Rx-Tx and/or UE Rx-Tx measurements for UE positioning. - Observation#5: Based on observations # 3 and #4, LMF cannot always cancel out NTA offset by combining gNB Rx-Tx and UE Rx-Tx time difference measurement results. - Observation #6: LMF needs to be aware of the NTA offset used by the UE for deriving the UE reported UE Rx-Tx measurement value. - Proposal #1: At least in FR1 the UE signals the information about the NTA offset along with UE Rx-Tx measured value for enabling LMF to derive the reported value. |
| R4-2007996 | Ericsson | LS on impact of NTA offset on UE Rx-Tx time difference measurement |

Open issues summary

Sub-topic 9-1 Relation between k1 and k2

The sub-topic is about the relation between UE selected parameter k2 and network recommended value k1. Based on submitted papers, all interested companies have the same proposal for RSTD and Rx-Tx time difference.

- Option 1. k1≤k2≤log214Tcmini(BWPRS,i)  (QC)

  - i is the positioning layer frequency layer index of all RSTD measurements of the same TRP pair.

- Option 2. k2 >= k1. (CATT, MTK)

- Option 3. RAN4 not to define relationship between k1 and k2 (Intel, HW)

- Option 4. k2 <= k1 (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 9-2 Applicable values for k in FR1

Common understanding is that all values {0,1,2,3,4,5} for k are applicable for FR2

- Option 1. {1,2,3,4,5} (QC)

- Option 2. {2,3,4,5} (CATT)

- Option 3. {0,1,2,3,4,5} (Intel, HW)

- Option 4. {3,4,5} (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 9-3 Whether signaling of NTA_offset along with Rx-Tx time difference to LMF is needed

- Option 1. No (QC, CATT, HW, Intel)

- Option 2. Yes (Ericsson)

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference

Two mapping tables for k=4 and k=5 are copied from the discussion paper R4-2007945 for illustration of option 1 and option 2.

- Proposal. Ensure the same range for all k’s, e.g., by increasing the last reporting interval to 8192 for all k’s. Agree on measurement report mapping tables based on the range from 0 to +8192 Tc, according to Tables 7 –12 (Option 2) (Ericsson)

- k=4, resolution 16Tc

| Reported Quantity Value DIFFRSTD_i | Option 1: RSTD, maximum range [0,8191] | Option 2: RSTD, range [0,8192] | Unit |
| --- | --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 16 | 0 RSTD < 16 | Tc |
| DIFFRSTD_0001 | 16 RSTD < 32 | 16 RSTD < 32 | Tc |
| DIFFRSTD_0002 | 32 RSTD < 48 | 32 RSTD < 48 | Tc |
|  |  |  | … |
| DIFFRSTD_509 | 8144 RSTD < 8160 | 8144 RSTD < 8160 | Tc |
| DIFFRSTD_510 | 8160 RSTD < 8176 | 8160 RSTD < 8192 | Tc |
| DIFFRSTD_511 | 8176 RSTD | 8192 RSTD | Tc |

- k=5, resolution 32Tc

| Reported Quantity Value DIFFRSTD_i | Option 1: RSTD, maximum range [0,8191] for k=0 | Option 2: RSTD, range [0,8192] | Unit |
| --- | --- | --- | --- |
| DIFFRSTD_0000 | 0 RSTD < 32 | 0 RSTD < 32 | Tc |
| DIFFRSTD_0001 | 32 RSTD < 64 | 32 RSTD < 64 | Tc |
| DIFFRSTD_0002 | 64 RSTD < 96 | 64 RSTD < 96 | Tc |
|  |  |  | … |
| DIFFRSTD_253 | 8096 RSTD < 8128 | 8096 RSTD < 8128 | Tc |
| DIFFRSTD_254 | 8128 RSTD < 8160 | 8128 RSTD < 8192 | Tc |
| DIFFRSTD_255 | 8160 RSTD | 8192 RSTD | Tc |

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 9-5 Report mapping for additional path

Two mapping tables for k=4 and k=5 are copied from the discussion paper R4-2007945 for illustration of option 1 and option 2.

- Proposal. The edge (lowest and highest) intervals for path can be slightly increased for k≥2 to cover up to -8175 and 8175, respectively. Agree on Option 2 in Tables (Ericsson)

- k=4, resolution 16Tc

| Reported Quantity Value, path_i | Measured Quantity Value, path |  | Unit |
| --- | --- | --- | --- |
|  | Option 1 | Option 2 |  |
| path_0000 | path < -8175 | path < -8175 | Tc |
| path_0001 | -8175 path < -8159 | -8175 path < -8152 | Tc |
| path_0002 | -8159 path < -8143 | -8152 path < -8136 | Tc |
|  |  |  | … |
| path_511 | -15 path < 1 | -8 path < 8 | Tc |
| … | … | … | … |
| path_1020 | 8129 path < 8145 | 8136 path < 8152 | Tc |
| path_1021 | 8145 path < 8161 | 8152 path < 8175 | Tc |
| path_1022 | 8161 path | 8175 path | Tc |

- k=5, resolution 32Tc

| Reported Quantity Value, path_i | Measured Quantity Value, path |  | Unit |
| --- | --- | --- | --- |
|  | Option 1 | Option 2 |  |
| path_000 | path < -8175 | path < -8175 | Tc |
| path_001 | -8175 path < -8143 | -8175 path < -8128 | Tc |
| path_002 | -8143 path < -8111 | -8128 path < -8096 | Tc |
|  |  |  | … |
| path_256 | -15 path < 17 | 0 path < 32 | Tc |
| … | … | … | … |
| path_509 | 8081 path < 8113 | 8096 path < 8128 | Tc |
| path_510 | 8113 path < 8145 | 8128 path < 8175 | Tc |
| path_511 | 8145 path | 8175 path | Tc |

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 9-1 Relation between k1 and k2 We support option 4. The UE should not use a more coarse mapping than what is configured by the network. Sub-topic 9-2 Applicable values for k in FR1 We support option 4. Could accept option 2. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed NTA_offset is included (embedded) in the reported UE Rx-Tx time difference measured value. Any one of 3 possible NTA_offset values is configured by gNB in a cell in FR1. Therefore, LMF does not know which NTA_offset is used in the cell where the UE Rx-Tx is measured. Some companies claim LMF can cancel NTA_offset by combining UE Rx-Tx and gNB Rx-Tx; but this is not always possible for several reasons 1) UE Rx-Tx and gNB Rx-Tx are capabilities i.e. gNB on whose signals UE Rx-Tx is measured may not support gNB Rx-Tx. This means LMF can use only UE Rx-Tx 2) It is up to LMF to decide whether to use UE Rx-Tx and/or gNB Rx-Tx for positioning. It is also claimed by a company that comparison between UE Rx-Tx measured from different cells can eliminate NTA_offset. This is not technically correct and may lead to further error. The reason is that NTA_offset is configured per cell basis. This means UE Rx-Tx measured on cell1 and cell2 may use different NTA_offset. In our view knowledge about NTA_offset in LMF is critical to ensure positioning based on UE Rx-Tx can be accurately estimated by LMF. Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference We prefer option 2 in the tables (last column) Sub-topic 9-5 Report mapping for additional path We prefer option 2 in the tables (last column) |
| Qualcomm | Subtopic 9-1: UE shall meet the accuracy requirements and hence k1 and k2 has to be set according to the PRS BW (which is why log214Tcmini(BWPRS,i) appears in our proposal. But UE does not need to do better than what LMF sets. We cannot agree to option 4. Subtopic 9-2: We can agree to option 1 or option 3. Subtopic 9-3: Option 1. Subtopic 9-4: We prefer option 1 first column (in our view, the last entries are not useful anyway) Subtopic 9-5: We prefer option 1 first column (addition positive path is even more limited in usage) |
| Intel | Sub-topic 9-1 Relation between k1 and k2 Support Option 3. This allow maximum UE flexibility (either for power saving or improve accuracy) Sub-topic 9-2 Applicable values for k in FR1 No strong preference. But regarding to same overhead singling, the much wider range is better. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Support Option 1. Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Prefer Option 1 no any other LS is needed. Sub-topic 9-5 Report mapping for additional path Prefer Option 1 no any other LS is needed. |
| CATT | Sub-topic 9-1 Relation between k1 and k2 Based on the definition of k1 and k2, we suggest that UE should choose a granularity k2 larger than or equal to LMF recommended k1. After the analysis, we found the value is decided by UE actually, so the LMF recommended k1 is not very necessary. UE shall report based on the finest granularity it can achieve. Sub-topic 9-2 Applicable values for k in FR1 Support option 2 or option 3. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Support option 1. NTA_offset is a constant value and have no impact on measurement accuracy, there is no need to be signaled to LMF. |
| Huawei | Sub-topic 9-1 Relation between k1 and k2 Option 3, but we can also compromise to option 2. On option 1, we think the upper bound is not needed. The accuracy requirements will be defined depending on BW, and UE has to meet the requirement, and that should be sufficient. Also, what happens if NW configures k1 larger than the proposed upper bound? Sub-topic 9-2 Applicable values for k in FR1 Option 3, we do not see the clear benefit to have such limits. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Option 1. Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Slightly prefer option 1. 32 Tc (9182-8160) is not a big issue for the reporting range of ~4.16us. Sub-topic 9-5 Report mapping for additional path Slightly prefer option 1. 30 Tc (8175-8145) is not a big issue for the reporting range of ~4.16us. |
| MTK | Sub-topic 9-1: Support option 2 or option 3. For both options, UE can adjust its k2 as long as accuracy requirements are met Sub-topic 9-3: Option 1 |

CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007946 (Ericsson) | Intel: some contents without agreements so far needs to be updated. |
|  | CATT: the value range of k and differential reporting range should be aligned with conclusion in this meeting. |
|  | Qualcomm: The rules for k2 cannot be agreed. Also, for differential reporting, no need to mention the rules again (even if it was agreeable, which is not) since RAN4 already agreed to use the same k2 for differential and absolute. |
| R4-2007998 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: in addition to comments for 7946, no need to have first absolute and second absolute values and formulating how differential is calculated. We can simply say in table caption that reference is smallest value. |
| R4-2007950 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: same comments as 7946 |
|  | Huawei: suggest to merge with 7946 as both are for RSTD. |
| R4-2007951 (Ericsson) | Company A |
|  | Company B |
|  | Qualcomm: same comments as in RSTD CR. |
|  | Huawei: suggest to merge with 7998 as both are for Rx-Tx. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 9-1 Relation between k1 and k2 Tentative agreements: None Candidate options: - Option 1. k1≤k2≤log214Tcmini(BWPRS,i) (QC) - i is the positioning layer frequency layer index of all RSTD measurements of the same TRP pair. - Option 2. k2 >= k1. (CATT, MTK, HW) - Option 3. RAN4 not to define relationship between k1 and k2 (Intel, HW, MTK) - Option 4. k2 <= k1 (Ericsson) - Option 5. UE shall report based on the finest granularity it can achieve (CATT) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#1 | Sub-topic 9-2 Applicable values for k in FR1 Tentative agreements: None Candidate options: - Option 1. {1,2,3,4,5} (QC) - Option 2. {2,3,4,5} (CATT, Ericsson) - Option 3. {0,1,2,3,4,5} (Intel, HW, QC, CATT) - Option 4. {3,4,5} (Ericsson) Recommendations for 2nd round: Check if option 3 is agreeable. |
| Sub-topic#1 | Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Tentative agreements: None Candidate options: - Option 1. No (QC, CATT, HW, Intel, MTK) - Option 2. Yes (Ericsson) Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#1 | Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Tentative agreements: None Candidate options: - Option 1. Allow different ranges for different k values (QC, Intel, HW) - Option 2. Ensure the same range for all k’s, e.g., by increasing the last reporting interval to 8192 for all k values (Ericsson) Recommendations for 2nd round: Check if option 1 is agreeable. |
| Sub-topic#1 | Sub-topic 9-5 Report mapping for additional path Tentative agreements: None Candidate options: - Option 1. Allow different ranges for different k values (QC, Intel, HW) - Option 2. Ensure the same range for all k’s, e.g., by increasing the last reporting interval to +/- 8175 for all k values (Ericsson) Recommendations for 2nd round: Check if option 1 is agreeable. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Discussion on 2nd round (if applicable)

| Company | Comments |
| --- | --- |
|  | Sub-topic 9-1 Relation between k1 and k2 Sub-topic 9-2 Applicable values for k in FR1 Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Sub-topic 9-5 Report mapping for additional path |
| Qualcomm | Sub-topic 9-1: We cannot support option 4. Option 1,2,3 are ok to us. Option 5 is not ok since as long as UE meets the accuracy requirements, there is no need to do anything extra. To Huawei’s comment on Option 1 in first round “. Also, what happens if NW configures k1 larger than the proposed upper bound?” for exactly the same reason as to prevent LMF from misconfiguring an inappropriate k1, we need the upper bound rule. Sub-topic 9-2: Option 3 is agreeable. Sub-topic 9-3: Option 1. Sub-topic 9-4: option 1. The different ranges variation is insignificant. Sub-topic 9-5: option 1. The different ranges variation is insignificant. |
| CATT | Sub-topic 9-1 Relation between k1 and k2 Support option 2 or option 5. Sub-topic 9-2 Applicable values for k in FR1 Support option 2 or option 3. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Support option 1. Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Support option 1. Sub-topic 9-5 Report mapping for additional path Support option 1. |
| Huawei | Sub-topic 9-1 Relation between k1 and k2 Option 2 or option 3. To us what matters is that UE meets the accuracy requirements which are defined based on PRS BW. Sub-topic 9-2 Applicable values for k in FR1 Option 3. We do not see the need to have such restriction. Again, what matters is that UE meets the accuracy requirements which are defined based on PRS BW. Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Option 1. Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Option 1. Sub-topic 9-5 Report mapping for additional path Option 1. |
| Intel | Sub-topic 9-1 Relation between k1 and k2 Support Option 3. If k1 and k2 is up to NW and UE respectively, why we need such restriction? Sub-topic 9-2 Applicable values for k in FR1 We can support Option 1,2,3 Sub-topic 9-3 Whether signaling of NTA offset along with Rx-Tx time difference to LMF is needed Support Option 1 Sub-topic 9-4 Report mapping for differential RSTD and Rx-Tx time difference Option 1 introduce less impacts on RAN2 Sub-topic 9-5 Report mapping for additional path Support Option 1 |
| Ericsson | 9-1: option 4 9-2: option 4 or could compromise to option 2. Not option 3. 9-3: option 2 9-4: option 2 9-5: option 2 |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #10: Link level simulation

The topic is for link level simulation assumptions and results for PRS measurement.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006237 | CATT | So in this paper, we give our suggestions on simulation assumption for TOA measurement based on [2] to evaluate UE Rx-Tx time difference accuracy. |
| R4-2007954 | Ericsson | In this contribution, link-level simulation assumptions for UE Rx-Tx for NR positioning are proposed. |
| R4-2006171 | Qualcomm | In this paper, we present simulation assumptions for UE Rx-Tx time difference measurement. Some of the assumptions are common to RSTD/RSRP link-level simulation assumptions [2]. |
| R4-2006559 | Intel Corporation | Observation 1: When UE measuring UE Rx-Tx time difference, the transmission timing shall be perfectly known by UE. Proposal 1: The following performance metric are proposed for link-level studies: • CDF of UE Rx-Tx time difference estimation error |
| R4-2007941 | Ericsson | - Observation 1: In the studied scenarios, multiple repetitions are very beneficial for small bandwidths, in both FR1 and FR2. - Observation 2: at least 4 repetitions or comb realizations are needed in the accuracy requirements for small bandwidths. - Proposal 1: The proposals for RSTD accuracy requirements can be collected according to the format: Number of comb realizations (realization=repetition or increased comb size within a slot). <FRx, SCS, number of PRBs>; X. - Proposal 2: The proposals for RSTD accuracy requirements can be collected according to the format: FRx, SCS, number of PRBs; Minimum number of comb realizations. FR1, 15 kHz, ≤52 PRBs (10 MHz); 4. FR1, 15 kHz, ≤104 PRBs (20 MHz); 2. FR1, 15 kHz, >104 PRBs; 1. FR1, 30 kHz, ≤48 PRBs (20 MHz); 4. FR1, 30 kHz, ≤132PRBs (30 MHz); 2. FR1, 30 kHz, ≤272 PRBs (100 MHz); 1. FR2, 120 kHz, ≤32 PRBs (50 MHz); 4. FR2, 120 kHz, >32 PRBs; 1. |
| R4-2007942 | Ericsson | - Observation: Multiple repetitions (or more generally comb realizations) provide gains; 16 repetitions are necessary at Es/Iot=-13 dB. - Proposal 1: The proposals on the number of comb realization for PRS-RSRP accuracy requirements can be collected according to the format: FRx, SCS, number of PRBs; Minimum number of comb realizations. Es/Iot≥-3; Es/Iot≥-6; Es/Iot≥-13. FR1, 15 kHz, ≤52 PRBs (10 MHz); X; Y; Z. - Proposal 2: The proposals on the number of comb realization for PRS-RSRP accuracy requirements can be collected according to the format: FRx, SCS, number of PRBs; Minimum number of comb realizations. Es/Iot≥-3; Es/Iot≥-6; Es/Iot≥-13. FR1, 15 kHz, ≤52 PRBs (10 MHz); 2; 4; 16. FR1, 15 kHz, ≤104 PRBs (20 MHz); 2; 2; 8. FR1, 15 kHz, >104 PRBs; 1; 1; 1. FR1, 30 kHz, ≤48 PRBs (20 MHz); 2; 4; 16. FR1, 30 kHz, ≤132PRBs (30 MHz); 2; 2; 8. FR1, 30 kHz, ≤272 PRBs (100 MHz); 1; 1; 1. FR2, 120 kHz, ≤32 PRBs (50 MHz); 2; 4; 16. FR2, 120 kHz, >32 PRBs; 1; 1; 1. |

Open issues summary

Sub-topic 10-1 Link level simulation assumption for UE Rx-Tx time difference

There are 4 papers suggesting the simulation assumption for Rx-Tx time difference. The proposals for many parameters are similar, while the differences are mainly related to following parameters. Interested companies are encouraged to provide views on the following parameters.

- Number of cells (1, 2 or 3)

- Es/Iot side condition for serving/reference cell and neighbor cell

- Number of samples

- Need for assumption on UE location

- Need for assumption on reference cell and expected RSTD uncertainty

- Whether and how Tx timing error is simulated

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 10-2 Methodology for summarizing RSTD simulation results

- Option 1. The proposals for RSTD accuracy requirements can be collected according to the format (Ericsson)

| FRx, SCS, number of PRBs | Minimum number of comb realizations |
| --- | --- |
| FR1, 15 kHz, ≤52 PRBs (10 MHz) | 4 |
| FR1, 15 kHz, ≤104 PRBs (20 MHz) | 2 |
| FR1, 15 kHz, >104 PRBs | 1 |
| FR1, 30 kHz, ≤48 PRBs (20 MHz) | 4 |
| FR1, 30 kHz, ≤132PRBs (30 MHz) | 2 |
| FR1, 30 kHz, ≤272 PRBs (100 MHz) | 1 |
| FR2, 120 kHz, ≤32 PRBs (50 MHz) | 4 |
| FR2, 120 kHz, >32 PRBs | 1 |

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 10-3 Methodology for summarizing PRS-RSRP simulation results

- Option 1. The proposals on the number of comb realization for PRS-RSRP accuracy requirements can be collected according to the format (Ericsson)

| FRx, SCS, number of PRBs | Minimum number of comb realizations |  |  |
| --- | --- | --- | --- |
|  | Es/Iot≥-3 | Es/Iot≥-6 | Es/Iot≥-13 |
| FR1, 15 kHz, ≤52 PRBs (10 MHz) | 2 | 4 | 16 |
| FR1, 15 kHz, ≤104 PRBs (20 MHz) | 2 | 2 | 8 |
| FR1, 15 kHz, >104 PRBs | 1 | 1 | 1 |
| FR1, 30 kHz, ≤48 PRBs (20 MHz) | 2 | 4 | 16 |
| FR1, 30 kHz, ≤132PRBs (30 MHz) | 2 | 2 | 8 |
| FR1, 30 kHz, ≤272 PRBs (100 MHz) | 1 | 1 | 1 |
| FR2, 120 kHz, ≤32 PRBs (50 MHz) | 2 | 4 | 16 |
| FR2, 120 kHz, >32 PRBs | 1 | 1 | 1 |

- Option 2. Other

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 10-1 Link level simulation assumption for Rx-Tx time difference 1 cell is sufficient Side condition: -3 dB serving cell and -13 dB neighbor cell Measurement period should be given along with periodicity, repetitions, etc. Sub-topic 10-2 Methodology for collecting simulation results Support option 1 Sub-topic 10-3 Methodology for collecting PRS-RSRP simulation results Support option 1 |
| Qualcomm | Subtopic 10-1: If we configure two cells with different side conditions, the same simulation can be run and give results for both. This manages simulation times better than having 1 cell per simulation and run it twice. Side condition is the same side condition as in RSTD. If some companies want to simulate -3 dB, they can also do it. It can be included in the sim assumptions. UE location is needed to pin down the exact TOA. Number of samples for deriving CDF is needed to define accuracy requirements. ExpectedRSTD and uncertainty (which exists even for Rx-Tx) is needed to narrow down the search window. No need to model Tx timing. This is very complicated and does not align among companies. Subtopic 10-2: The new terminology is not clear to us and we cannot agree to option 1. Subtopic 10-3: Same comment as 10-2. |
| Intel | Sub-topic 10-1 Link level simulation assumption for Rx-Tx time difference Focus on the difference part with the simulation assumption with RSTD, e.g. how to model UE transmission error. In our view, we can take the ideal UE transmission. Sub-topic 10-2 Methodology for collecting simulation results In our view, firstly we try to collect the results for all combinations including in the previous assumption as possible. Then we can justify which groups requirements are needed. Sub-topic 10-3 Methodology for collecting PRS-RSRP simulation results Same as 10-2 |
| CATT | Sub-topic 10-1 Link level simulation assumption for Rx-Tx time difference Number of cells: 2 or 3 cells, at least 1 interference cell is defined Es/Iot side condition: follow the conclusion of sub-topic 7-2, if no conclusion is reached, [-6,-13dB] and [-3,-10] should both be simulated. Tx timing: no need to simulated Sub-topic 10-2 Methodology for collecting simulation results Don’t agree option 1. It can be collected as number of repetition realization and the side condition should also be represented. Sub-topic 10-3 Methodology for collecting PRS-RSRP simulation results Same as sub-topic 10-2, shall be the number of repetition realization |
| Huawei | Sub-topic 10-1 Link level simulation assumption for Rx-Tx time difference Suggest to use 3 cells, same as RSTD and RSRP simulation. We do not see particular reason to change the setup, since it will not impact the performance. 3-cell is a valid scenario in deployment, and there is no PRS-to-PRS interference. Suggest to use -13dB for FR1 and -10dB for FR2, as we propose to only define one set of conditions. Suggest simulate 1 sample same as for RSTD, with different repetitions. We not see the need for UE location. For RSTD simulation, the TOAs are also measured and we do not have UE location modeled. Suggest to not model Tx timing error. Sub-topic 10-2 Methodology for collecting simulation results On option 1, we still need to understand what the target accuracy is when we determine the min number of “comb realizations”. Again, we suggest to have the required number of repetitions or comb realizations as side condition instead of the accuracy requirements. Sub-topic 10-3 Methodology for collecting PRS-RSRP simulation results Same as 10-2. |

CRs/TPs comments collection

| Sub-topic#1 | Sub-topic 9-1 Relation between k1 and k2 Tentative agreements: None Candidate options: - Option 1. k1≤k2≤log214Tcmini(BWPRS,i) (QC) - i is the positioning layer frequency layer index of all RSTD measurements of the same TRP pair. - Option 2. k2 >= k1. (CATT, MTK, HW) - Option 3. RAN4 not to define relationship between k1 and k2 (Intel, HW, MTK) - Option 4. k2 <= k1 (Ericsson) - Option 5. UE shall report based on the finest granularity it can achieve (CATT) Recommendations for 2nd round: Further discuss in 2nd round. |
| --- | --- |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 10-1 Link level simulation assumption for Rx-Tx time difference Tentative agreements: None Open issues: - Number of cells (1, 2 or 3) - 1 cell (Ericsson) - 2 cells (QC) - 2 or 3 cells, at least one interference cell (CATT) - 3 cells, same as RSTD - Es/Iot side condition for serving/reference cell and neighbor cell - -3 for serving and -13 for neighbor (Ericsson) - Same as for RSTD (QC, CATT, HW) - Number of samples - Needed (Ericsson, QC, HW) - Need for assumption on UE location - Yes (QC) - No, same as RSTD (HW) - Need for assumption on reference cell and expected RSTD uncertainty - Yes (QC) - Whether and how Tx timing error is simulated - No (QC, Intel, CATT, HW) Recommendations for 2nd round: Revise one of the submitted papers on simulation assumptions, and discuss directly based on the revision. |
| Sub-topic#1 | Sub-topic 10-2 Methodology for collecting RSTD simulation results Tentative agreements: None Candidate options: Proposal for RSTD accuracy requirements to collected according to the format in R4-2007941 - Option 1: Yes (Ericsson) - Option 2. No (QC, CATT, HW) Recommendations for 2nd round: No further discussion on this sub-topic. Continue the discussion based on sub-topic 3-1 and 8-1. |
| Sub-topic#1 | Sub-topic 10-3 Methodology for collecting PRS-RSRP simulation results Tentative agreements: None Candidate options: Proposal for PRS-RSRP accuracy requirements to collected according to the format in R4-2007941 - Option 1: Yes (Ericsson) - Option 2. No (QC, CATT, HW) Recommendations for 2nd round: No further discussion on this sub-topic. Continue the discussion based on sub-topic 3-1 and 8-1. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Topic #11: Scheduling restriction and need for measurement gaps

The topic is for whether PRS measurement in FR1 causes scheduling restriction, and the conditions where PRS measurement can be measured without gaps.

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006173 | Qualcomm Incorporated | Proposal 5. RAN4 to introduce a UE capability signalling for scheduling restrictions on PRS symbols in FR1: if enabled, UE does not expect any scheduling restrictions in PRS symbols in FR1. Proposal 6a: For FR1 and when PRS is within serving cell active BWP and also the SCS/CP of the positioning frequency layer is same as those of serving cell active BWP, UE shall be able to measure PRS without measurement gap. Proposal 6b: Otherwise, UE shall request measurement gaps for PRS measurement and is required to meet the PRS measurement requirements only when it is provided with measurement gaps for PRS measurement. |
| R4-2006232 | CATT | Proposal 9: The restriction should be implemented by NW which means gNB should not configure PRS on the same symbol where SSB is transmitted. But the UE receiving and processing is no need to be restricted. Proposal 10: in FR1, UE can measure a PRS layer without MG if the following conditions are met, otherwise UE should measure PRS layer with MG. • The PRS layer is within UE active BWP, and • The SCS of the PRS layer is same as SCS of UE active BWP or UE support mixed SCS, and • The CP of the PRS layer is same as CP of UE active BWP In FR2, the measurement gap is always needed. |
| R4-2006306 | MediaTek inc. | Proposal 5: UE behavior on scheduling restriction in FR1 shall be the same that for FR2, i.e., UE drops PRS if there exist other DL channels |
| R4-2006556 | Intel Corporation | Proposal 6: The scheduling restriction regarding PRS symbols in both FR1 and FR2 is needed. Observation 5: When measurement gap is needed for PRS measurement shall not rely on FR1/FR2. Proposal 2: Regarding to the consistent definitions for NR measurement, the intra/inter frequency PRS measurement can be defined as: Definition; Need gap or Not. intra-frequency; DL PRS resources of all measured reference/neighbor/serving cells/TRPs have - same SCS and CP type - same centre frequency - same point-A - same configured PRS BW; NO: If BW of PRS to be measured is contained in the active BWP.. Yes: If BW of PRS to be measured is not contained in the active BWP.. inter-frequency; DL PRS resources of the measured neighbor/reference/serving cells/TRPs is different in any of - SCS and CP type - centre frequency - point-A - configured PRS BW; Yes. |
| R4-2007841 | Huawei, HiSilicon | Proposal 11: PRS measurement should not cause any scheduling restriction. If UE cannot do simultaneous PRS measurement and data reception, PRS should be measured with MG. Proposal 12: UE should be able to measure a PRS layer without MG if the following conditions are met, otherwise UE is assumed to measure the PRS layer with MG. - The PRS layer is in FR1 and within UE active BWP, and - The SCS/CP of the PRS layer is same as SCS/CP of UE active BWP, or UE supports mixed numerology between PRS and UE active BWP. Proposal 13: RAN4 to define a new UE capability to indicate if UE supports concurrent PRS measurement and data reception in the UE active BWP with a different numerology. |
| R4-2007842 | Huawei, HiSilicon | LS on UE behaviours for concurrent receiving of PRS and DL signal/channel. |
| R4-2007944 R4-2007943 | Ericsson | Proposal 2: Intra/inter-frequency RSTD measurement definitions are as in Table 1. Definition; Need for measurement gaps Note 1. Intra-frequency; - The center frequency of PRS BW is the center frequency of a serving cell SSB - The SCS PRS is the same as that of a serving cell SSB NOTE: for RSTD, the above conditions are met for both reference and the other DL link; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Inter-frequency; if at least one of the two conditions above is not met NOTE: for RSTD, this applies for at least one of the reference and the other DL link; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. NOTE 1: For RSTD configured for a reference link and the other DL link, measurement gaps may be needed for both DL links, for one of the two DL links, or for none of the two DL links, depending for which of the two DL links the conditions are met.. Proposal 3: The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not: - not needed: Measured PRS bandwidth is fully within the active BWP of the UE - needed: Measured PRS bandwidth is not fully within the active BWP of the UE For RSTD configured for a reference link and the other DL link, measurement gaps may be needed for both DL links, for one of the two DL links, or for none of the two DL links, depending for which of the two DL links this condition is met. Proposal 14: Intra- and inter-frequency UE Rx-Tx measurements are defined according to the table below. Definition; Need for measurement gaps. Intra-frequency; - The center frequency of PRS BW is the center frequency of a serving cell SSB - The SCS PRS is the same as that of a serving cell SSB; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Inter-frequency; if at least one of the two conditions above is not met; not needed; Measured PRS bandwidth is fully within the active BWP of the UE. needed; Measured PRS bandwidth is not fully within the active BWP of the UE. Proposal 15: The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not: - Not needed: Measured PRS bandwidth is fully within the active BWP of the UE - Needed: Measured PRS bandwidth is not fully within the active BWP of the UE |
| R4-2007117 | Nokia, Nokia Shanghai Bell | Proposal 1: Apply the RAN1 rule for scheduling restriction of PRS symbols with other DL signals and channels in FR2, also for FR1. |

Open issues summary

Sub-topic 11-1 Need for measurement gaps

- Option 1 (Ericsson): The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not:

  - Not needed: Measured PRS bandwidth is fully within the active BWP of the UE

  - Needed: Measured PRS bandwidth is not fully within the active BWP of the UE

- Option 1a (Intel):

  - For intra-frequency, same as option 1;

  - For inter-frequency, gap is always needed.

- Option 2 (CATT, QC):

  - In FR1, UE can measure a PRS layer without MG if the following conditions are met, otherwise UE should measure PRS layer with MG.

    - The PRS layer is within UE active BWP, and

    - The SCS of the PRS layer is same as SCS of UE active BWP or UE support mixed SCS, and

    - The CP of the PRS layer is same as CP of UE active BWP

  - In FR2, the measurement gap is always needed.

- Option 3 (HW): UE should be able to measure a PRS layer without MG if the following conditions are met, otherwise UE is assumed to measure the PRS layer with MG.

  - The PRS layer is in FR1 and within UE active BWP, and

  - The SCS/CP of the PRS layer is same as SCS/CP of UE active BWP, or UE supports mixed numerology between PRS and UE active BWP.

  - RAN4 to define a new UE capability to indicate if UE supports concurrent PRS measurement and data reception in the UE active BWP with a different numerology.

Recommended WF: Further discussion needed. Collect companies’ views.

Sub-topic 11-2 Scheduling restriction in FR1

Based on the submitted papers it seems companies referred to different UE behaviours when talking about the term ‘scheduling restriction’, e.g. some companies considered it as interruption to data scheduling due to PRS measurement, some companies considered it as PRS dropping due to data scheduling, while some companies considered it as dropping of PRS measurement due to SSB measurement. Therefore, companies’ views are not listed for this sub-topic, but companies are encouraged to provide comments for the following questions. It is further noted that the issue pf concurrent PRS and RRM measurements are treated in email#217, so it is not considered in this sub-topic.

Scenario to be addressed (from R1-1913522): PRS frequency layer is within UE active BWP, and measurement gap for PRS measurement is not configured.

- Question 1. Based on sub-topic 11-1, if UE cannot measure the PRS frequency layer without measurement gaps, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols?

- Question 2. Based on sub-topic 11-1, if UE cannot measure the PRS frequency layer without measurement gaps, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols?

- Question 3. Based on sub-topic 11-1, if UE can measure the PRS frequency layer without measurement gaps, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols?

- Question 4. Based on sub-topic 11-1, if UE can measure the PRS frequency layer without measurement gaps, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols?

Recommended WF: Further discussion needed. Collect companies’ views.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Sub-topic 11-1 Need for measurement gaps Option 1 Sub-topic 11-2 Scheduling restriction in FR1 The UE does not need gaps if the PRS BW is fully within UE active BWP. |
| Qualcomm | Subtopic 11-1 Support option 2. Option 1 disregards the potential SCS/CP difference. We can agree to option 3 if there is consensus to define this new UE capability (which in our view is of lower priority and urgency compared to all the other existing issues). Subtopic 11-2: Q1: if UE cannot measure PRS without MG, UE will not measure PRS and hence will not cause interruption to DL signals/channels. Q2: if UE cannot measure PRS without MG, UE will not measure PRS. So UE will drop PRS. Q3: if UE can measure PRS without MG, it should be allowed to cause interruptions to other DL signals/channels since processing of PRS and other DL signals/channels in the same symbol should not be required. Q4: if UE can measure PRS without MG, it should be allowd to drop PRS since processing of PRS and other DL signals/channels in the same symbol should not be required. Based on our answers to Q3 and Q4, scheduling of PRS and other DL signals/channels in the same symbol should not be allowed to ensure UE has a well-defined behavior. |
| Intel | - Sub-topic 11-1 Need for measurement gaps Prefer Option 1a. But if we flatten the definition of intra/inter measurement definition proposed in R4-2006556, Option 1a is quite similar to Opt2(for FR1 at least) and Opt3. Sub-topic 11-2 Scheduling restriction in FR1 In order to resolve the problem of PRS measurement colliding with other RRM/DL signal, there are potential two ways : one is PRS measurement with gap, the other is to define the scheduling restriction to RRM/DL signals. So, our understanding is: Q1: not allowed Q2: up to which reference signal will be prioritized. In our proposal, PRS can be deprioritized and dropped. Q3: not allowed Q4: up to which DL signal will be prioritized. |
| CATT | Sub-topic 11-1 Need for measurement gaps Support option 2. Sub-topic 11-2 Scheduling restriction in FR1 - Based on sub-topic, 11-1, if UE can measure PRS without gap, then it has same numerology with active BWP and can receive PRS and other signals simultaneously. |
| Huawei | Sub-topic 11-1 Need for measurement gaps Option 3. We think both the frequency location w.r.t. active BWP and the SCS w.r.t. active BWP needs to be considered. We also suggest to define the new capability which is similar as for SSB measurement. Sub-topic 11-2 Scheduling restriction in FR1 Q1: not allowed Q2: allowed Q3: not allowed, in our view, if UE cannot support concurrent reception of serving cell data and PRS, it should request MG for PRS measurement. Q4: not allowed, in our view, if UE cannot support concurrent reception of serving cell data and PRS, it should request MG for PRS measurement. |
| MTK | Sub-topic 11-1: support option 3 Sub-topic 11-2: Support QC’s view |

CRs/TPs comments collection

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Sub-topic 11-1 Need for measurement gaps Tentative agreements: None Candidate options: - Option 1 (Ericsson): The need for measurement gaps is determined by whether the measured PRS bandwidth is fully within the active BWP of the UE or not: - Not needed: Measured PRS bandwidth is fully within the active BWP of the UE - Needed: Measured PRS bandwidth is not fully within the active BWP of the UE - Option 1a (Intel): - For intra-frequency, same as option 1; - For inter-frequency, gap is always needed. - Option 2 (CATT, QC): - In FR1, UE can measure a PRS layer without MG if the following conditions are met, otherwise UE should measure PRS layer with MG. - The PRS layer is within UE active BWP, and - The SCS of the PRS layer is same as SCS of UE active BWP or UE support mixed SCS, and - The CP of the PRS layer is same as CP of UE active BWP - In FR2, the measurement gap is always needed. - Option 3 (HW, QC, MTK): UE should be able to measure a PRS layer without MG if the following conditions are met, otherwise UE is assumed to measure the PRS layer with MG. - The PRS layer is in FR1 and within UE active BWP, and - The SCS/CP of the PRS layer is same as SCS/CP of UE active BWP, or UE supports mixed numerology between PRS and UE active BWP. - RAN4 to define a new UE capability to indicate if UE supports concurrent PRS measurement and data reception in the UE active BWP with a different numerology. Recommendations for 2nd round: Further discuss in 2nd round. |
| Sub-topic#2 | Sub-topic 11-2 Scheduling restriction in FR1 Tentative agreements: None Candidate options: Question 1. Based on sub-topic 11-1, if UE cannot measure the PRS frequency layer without measurement gaps, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols? - No (QC, Intel, HW, MTK) Question 2. Based on sub-topic 11-1, if UE cannot measure the PRS frequency layer without measurement gaps, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols? - Yes (QC, Intel, HW, MTK) Question 3. Based on sub-topic 11-1, if UE can measure the PRS frequency layer without measurement gaps, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols? - Yes (QC, MTK) - No (Intel, CATT, HW) Question 4. Based on sub-topic 11-1, if UE can measure the PRS frequency layer without measurement gaps, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols? - Yes (QC, MTK) - No (CATT, HW) - Up to which DL signal will be prioritized (Intel) Recommendations for 2nd round: Further discuss in 2nd round. |

LS for all topics are listed here as well as the simulation assumptions (for approval). All of them are suggested to be return to after 1st round.

| R4-2007996 | LS on impact of NTA offset on UE Rx-Tx time difference measurement | Ericsson |
| --- | --- | --- |
| R4-2007842 | [draft] reply LS on agreements related to NR Positioning | Huawei, HiSilicon |
| R4-2006237 | Link level simulation assumption for UE RX-Tx time difference | CATT |
| R4-2007954 | Link-level simulation assumptions for UE Rx-Tx | Ericsson |
| R4-2006171 | Link-level simulation assumptions for UE Rx-Tx time difference measurements | Qualcomm Incorporated |
| R4-2006559 | Link-level simulation assumptions for UE Rx-Tx time difference measurement | Intel Corporation |

A new Tdoc for WF is requested for all topics

|  | WF on requirements for RSTD and UE Rx-Tx time difference measurement | Huawei, HiSilicon |
| --- | --- | --- |

CRs/TPs

CRs for all topics are listed here. All of them are suggested to be return to after 1st round.

| R4-2006560 | CR to TS 38.133: Structure for NR positioning RRM requirements | Intel Corporation |
| --- | --- | --- |
| R4-2007955 | Positioning measurement accuracy requirements structure in section 10 | Ericsson |
| R4-2006561 | CR to TS 38.133: PRS RSTD requirements | Intel Corporation |
| R4-2007958 | Reporting criteria for NR RSTD | Ericsson |
| R4-2007959 | Reporting criteria for NR UE Rx-Tx | Ericsson |
| R4-2007946 | Measurement report mapping for PRS RSTD | Ericsson |
| R4-2007950 | Additional path report mapping for RSTD | Ericsson |
| R4-2007998 | UE Rx-Tx Measurement Report Mapping in NR in 38.133 | Ericsson |
| R4-2007951 | Additional path report mapping for UE Rx-Tx | Ericsson |

Discussion on 2nd round (if applicable)

For sub-topic 11-1, moderator would ask companies to consider the latest RAN1 agreements:

Agreement: - UE is not expected to process DL PRS without configuration of measurement gap in Rel-16 - RAN1 assumes that no RAN4 requirements are to be defined for the case w/o configured measurement gap in Release 16 - Inform RAN4 about this agreement

Moderator would like to ask companies whether the following proposal is agreeable for 11-1:

It is up to UE implementation to request measurement gaps for PRS measurement.

If above is agreeable, moderator would ask companies to provide views on the following updated questions related to 11-2

Question 1. If UE requested measurement gaps but gaps are not configured, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols?

Question 2. If UE requested measurement gaps but gaps are not configured, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols?

Question 3. If UE did not request measurement gaps, is the UE allowed to cause interruption to DL signals/channels due to PRS measurement in the same symbols?

Question 4. If UE did not request measurement gaps, is the UE allowed to drop PRS measurement due to reception of DL signal/channels in the same symbols?

| Company | Comments |
| --- | --- |
|  | Sub-topic 11-1 Need for measurement gaps Sub-topic 11-2 Scheduling restriction in FR1 |
| Qualcomm | Sub-topic 11-1: we can agree that requirements should be defined based on measurements within MG. Q1: if UE requested MG, but not granted MG, then positioning measurement should not occur and no interruption on DL signals/channels should be allowed. Q2: if UE requested MG but not granted MG, positioning measurement requirements do not apply. Q3: RAN4 should not discuss this case or define any requirements in this scenario. Q4: same answer as in Q3. |
| CATT | Sub-topic 11-1 Need for measurement gaps Moderator suggestion is acceptable for us. Sub-topic 11-2 Scheduling restriction in FR1 Q1: No Q2: Yes Q3: No, the case UE did not request gap is not considered in R16 according to RAN1 agreement, the interruption to other DL signal due to PRS is not allowed. Q4: No, same as Q3. |
| Huawei | Sub-topic 11-1 Need for measurement gaps Support that It is up to UE implementation to request measurement gaps for PRS measurement. Sub-topic 11-2 Scheduling restriction in FR1 Q1: No Q2: Yes Q3: No, otherwise UE should just request gaps. Q4: No, otherwise UE should just request gaps. |
| Intel | Sub-topic 11-1 Need for measurement gaps Based on GTW agreement, it is up to UE implementation to request measurement gaps for PRS measurement. Sub-topic 11-2 Scheduling restriction in FR1 Q1: No Q2: Yes Q3: No, otherwise UE should just request gaps. Q4: No, otherwise UE should just request gaps. |
| MTK | Sub-topic 11-1 Need for measurement gaps Agree with moderator’s proposal Sub-topic 11-2 Scheduling restriction in FR1 Agree with HW’s view. |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

Suggestions for all open Tdocs in this email thread are as follows:

| R4-2007996 | LS on impact of NTA offset on UE Rx-Tx time difference measurement | Ericsson | Revised to R4-2009119 |
| --- | --- | --- | --- |
| R4-2009119 | LS on impact of NTA offset on UE Rx-Tx time difference measurement | Ericsson | Return to (ongoing discussion with CATT) |
| R4-2007842 | [draft] reply LS on agreements related to NR Positioning | Huawei, HiSilicon | Noted |
| R4-2009111 | LS on intra/inter-frequency measurement for NR positioning | Huawei, HiSilicon | Agreeable |
| R4-2006237 | Link level simulation assumption for UE RX-Tx time difference | CATT | Noted |
| R4-2007954 | Link-level simulation assumptions for UE Rx-Tx | Ericsson | Noted |
| R4-2006171 | Link-level simulation assumptions for UE Rx-Tx time difference measurements | Qualcomm Incorporated | Revised to R4-2009127 |
| R4-2009127 | Link-level simulation assumptions for UE Rx-Tx time difference measurements | Qualcomm Incorporated | Agreeable |
| R4-2006559 | Link-level simulation assumptions for UE Rx-Tx time difference measurement | Intel Corporation | Noted |
| R4-2008664 | WF on requirements for RSTD and UE Rx-Tx time difference measurement | Huawei, HiSilicon | Agreeable |
| R4-2006560 | CR to TS 38.133: Structure for NR positioning RRM requirements | Intel Corporation | Noted |
| R4-2007955 | Positioning measurement accuracy requirements structure in section 10 | Ericsson | Revised to R4-2009126 |
| R4-2009126 | Positioning measurement accuracy requirements structure in section 10 | Ericsson | Agreeable (there are some sub-sections with titles only, not sure if this is an issue for ITU submission, but technically CR is agreeable) |
| R4-2006561 | CR to TS 38.133: PRS RSTD requirements | Intel Corporation | Noted |
| R4-2007958 | Reporting criteria for NR RSTD | Ericsson | Revised to R4-200xxxx |
| R4-200xxxx | Reporting criteria for NR RSTD | Ericsson | Agreeable |
| R4-2007959 | Reporting criteria for NR UE Rx-Tx | Ericsson | Merged |
| R4-2007946 | Measurement report mapping for PRS RSTD | Ericsson | Revised to R4-200xxxx |
| R4-200xxxx | Measurement report mapping for PRS RSTD | Ericsson | Agreeable |
| R4-2007950 | Additional path report mapping for RSTD | Ericsson | Merged |
| R4-2007998 | UE Rx-Tx Measurement Report Mapping in NR in 38.133 | Ericsson | Revised to R4-200xxxx |
| R4-200xxxx | UE Rx-Tx Measurement Report Mapping in NR in 38.133 | Ericsson | Agreeable |
| R4-2007951 | Additional path report mapping for UE Rx-Tx | Ericsson | Merged |