3GPP TSG-RAN WG4 Meeting # 95-e 												R4- 2009094

Electronic Meeting, 25 May – 5 June, 2020

Agenda item:			12.3

Source:	Ericsson

Title:	Email discussion summary for [94e Bis][232]LTE_RRM_Maintenance

Document for:	Information

# Introduction

Maintenance of LTE specifications, focussed on at least addressing TBDs, FFSs and square brackets in release 15 and release 16 core specifications for ITU specification submission.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Conclude on any open technical issues and identify in principle agreeable CRs which may be agreed or revised in 2nd round

- 2nd round: Review draft revised CRs to get an agreeable set of maintenance CRs at the end of second round which address at least the needs of the ITU specification submission

# Topic #1: euCA requirements finalization

Main technical topic overview. Closure of outstanding R15 euCA technical issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007146 | Nokia, Nokia Shanghai Bell | Proposal 1: UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping. Proposal 2: Clarify in 36.133 that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells. |
| R4-2007147.zip | Nokia, Nokia Shanghai Bell | Cat F Rel 15 CR related to P1 in R4-2007146 |
| R4-2007148 | Nokia, Nokia Shanghai Bell | Cat A Rel 15 CR related to P1 in R4-2007146 |
| R4-2007149.zip | Nokia, Nokia Shanghai Bell | Cat F Rel 15 CR related to P2 in R4-2007146 |
| R4-2007150 | Nokia, Nokia Shanghai Bell | Cat A Rel 15 CR related to P2 in R4-2007146 |

## Open issues summary

- Number of carriers to measure

- Snon-intrasearch

### Sub-topic 1-1 : number of carriers to measure

Sub-topic description: euCA requirements finalization

Open issues and candidate options before e-meeting: (from current 36.133):  A UE which supports ca-IdleModeMeasurements shall support the idle mode CA measurements of at least [1, 2 or 3] inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier.

Issue 1-1: Required number of carriers to measure

- Proposals

  - Option 1: : UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping.

  - Option 2: Other?

- Recommended WF

  - Check if proposal 1/option 1 (from R4-2007146, Nokia) is agreeable and if so, discussion can take place on corresponding CR wording.

### Sub-topic 1-2 : s-NonIntraSearch

Sub-topic description: s-NonIntraSearch clarifications

Open issues and candidate options before e-meeting: 36.133 section 4.9.2.2 refers to requirements in section 4.2.2.4, in which the s-NonIntraSearch may be applied.

Issue 1-2: TBA

- Proposals

  - Option 1: clarify in the RAN4 specification 36.133 section 4.9.2.2, that the s-NonIntraSearch thresholds do not apply to carriers configured for early measurement reporting

  - Option 2: Other?

- Recommended WF

  - Check if proposal 2/option 1 (from R4-2007146, Nokia) is agreeable and if so, discussion can take place on corresponding CR wording.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Qualcomm | Subtopic 1-1: we support option 1. Subtopic 1-2: we support option 1. |
| Ericsson | Subtopic 1-1: Support proposal in option 1 Subtopic 1-2: Support proposal in option 1 |
| Huawei | We would like to clarify the implication of option 1. The existing agreement says: For overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply. Then in 4.2.2.4, requirements are defined based on s-NonIntraSearch If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below. So, is option 1 conflicting with the existing requirements, or what is the impact to the UE requirements from option 1? |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007147 | Ericsson: CR is fine for us |
| R4-2007149 | Ericsson: CR is fine for us |
|  | Huawei: We have concern with this CR as it complicates UE measurement behavior in idle mode. With the change, when serving cell is above s-NonIntraSearch, UE is required to measure - high priority carriers not configured for EMR with Thigher_priority_search, and - high priority carriers configured for EMR with Tmeasure,EUTRAN_Inter, and - equal/low priority carriers configured with EMR with Tmeasure,EUTRAN_Inter. So the measurements are done with two intervals (Thigher_priority_search and Tmeasure,EUTRAN_Inter) which is reasonable in our view. The number of carriers (Kcarrier and Nlayers) will be impacted by this change. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Subtopic 1-1 (number of carriers) : It is agreed that the UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping. This agreement resolves issue with square brackets/tentative values for 36.133 ITU-R submission. Candidate options: Recommendations for 2nd round: Subtopic 1-2 (s-NonIntraSearch): Proponents should try to address Huawei’s request for clarification on the proposal in 2nd round. CR can be returned to after further discussion. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2007147 | Agreeable |
| R4-2007149 | Return to |

## Discussion on 2nd round (if applicable)

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Open issues |
| --- | --- |
| Sub-topic#1 | Subtopic 1-2 (s-NonIntraSearch): Proponents should try to address Huawei’s request for clarification on the proposal in 2nd round. CR can be returned to after further discussion. |

## Companies views’ collection for 2nd round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Sub topic 1-1: Sub topic 1-2: …. Others: |
| MTK | (Rapporteur note : Added 1/6/2020 after completion of 1st round discussion) Sorry, we didn’t notice that there was a euCA discussion in 1st round. Sub topic 1-1: It is unclear what the option 1 means: UE shall support the idle mode CA measurements of at least 2 inter-frequency carriers of which zero or one inter-frequency carrier may be a non-overlapping carrier and at most 1 inter-frequency carrier can be overlapping. Why we need to separate “at least” for non-overlapping carrier and “at most” for overlapping carrier? Why we need to say “of which zero or one inter-frequency carrier may be a non-overlapping carrier”? It is very confusing. So I could only interpret the meaning based on the corresponding CR R4-2007147, it seems that UE is requested to measure at least 1 overlapping and at least 1 non-overlapping carrier. As we clarify in our Tdoc R4-2006886. It is very hard to determine the values before we clarify the corresponding UE behavior. It will be more clear if companies can first confirm whether we have consensus on the following rules (not necessarily to be specified in the spec, but at least we should mark it as an agreement in the Chairmannote): no RAN4 requirement is defined (up to UE implementation) when any of limitations shown in following table, measurement capability for re-selection purpose and EMR purpose, is exceeded Case 2; FDD E-UTRA A≤3, (A=aR +aO+aN); TDD E-UTRA B≤3, (B=bR +bO+bN); FDD UTRA C≤3, (C=cR); TDD UTRA D≤3, (D=dR); GSM E≤32, (E=eR); cdma2000 1x F≤3, (F=fR); HRPD G≤3, (G=gR); NR H≤8, (H=hR +hO+hN); Total carriers N≤10, (K= kO+ kN). Only for Re-selection; aR; bR; cR; dR; eR; fR; gR; hR. Overlapping EMR; aO; bO; N.A; N.A; N.A; N.A; N.A; hO; kO≤1 (kO=aO+bO +hO). Non-overlapping EMR; aN; bN; N.A; N.A; N.A; N.A; N.A; hN; kN≤1 (kN=aN+bN +hN). We think that CR and above clarification should be agreed as a package; otherwise, companies will interpret the CR in different way. Subtopic 1-2: We share the same concern with Huawei. For overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply. Then in 4.2.2.4, requirements are defined based on s-NonIntraSearch If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2. If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below. So, from UE perspective, the early measurement reporting will be definitely impacted by the s-NonIntraSearch. We also need to clarify what is the impact if the only 1 configured overlapping EMR is an equal/low priority carrier for re-selection and serving quality is larger than s-NonIntraSearch, does UE still need to measure/report it or not? |
| Nokia | This is closely related to the NR discussion. Please find some background information and references to the current requirements development: Background (Rel-15 EMR): - euCA did in fact have same discussion. - Started before RAN2 decision and initially RAN4 did account the search thresholds in Rel-15 - RAN2 then decided on the note under discussion: - NOTE 1: The fields s-NonIntraSearchP and s-NonIntraSearchQ in SIB2 do not affect the idle/inactive UE measurement procedures. How the UE performs idle/inactive measurements is up to UE implementation as long as the requirements in TS 38.133 [14] are met for measurement reporting - RAN4 meeting Chongqing it was agreed to align the RAN4 euCA requirements with the RAN2 decision stating the inter-frequency search thresholds (S-measure) are not applied for carrier configured for early measurement reporting (R4-1912845) - In RAN4 meeting in Reno discussions continued related to: - number of non-overlapping carriers - Clarify RAN4 specification that the fields s-NonIntraSearch in SystemInformationBlockType3 do not affect measurements of inter-frequency CA candidate cells - Due to time these were agreed to discuss offline and return in the following meeting (Athens 2020) which then did not include euCA on the Agenda. - In R4-1914463/64 (Reno meeting) the changes from the agreement to align with the RAN2 specification are visible. Based on the Huawei input it seems that the concern is related to measurement of higher priority carriers when these are not configured as EMR carrier. However, hiher priority search is as such a minimum requirement and UE may always search more often than minimum requirements. Hence, if it is too complex to support different search periodicities, the UE can search higher priority carriers more often while T331 is running. |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007149 |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2007149 | Huawei indicated concern on the CR in 1st round. Nokia provided feedback on the concerns just before the 2nd round deadline. In view of the limited time for discussion of the feedback and the fact that this CR does not address any ITU square bracket / TBD issue, the moderator recommendation is postponed, |
| R4-2007147 | This CR was agreeable in first round. MTK provided further feedback on the agreed CR during 2nd round, related to the UE behaviour if the number of configured overlapped or non-overlapped carriers is exceeded. Common understanding for this case is that there are no requirements, This CR is needed to address square brackets for ITU submission. Hence moderator recommendation is to reconfirm 1st round decision that CR is agreed. |

# Topic #2: Rel-13 maintance

Release 13 essential corrections

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007424.zip | Rohde & Schwarz | Cat F Rel 13 CR In SchedulingRequest-Config-DEFAULT tables of tests A.7.3.50/51/54/55/58/58 the CR-ConfigIndex changed from 0 (5 ms periodicity) to 5 (20 ms periodicity). |
| R4-2007425 | Rohde & Schwarz | Cat A Rel-14 CR shadow for 7424 |
| R4-2007426 | Rohde & Schwarz | Cat A Rel-15 CR shadow for 7424 |
| R4-2007427 | Rohde & Schwarz | Cat A Rel-16 CR shadow for 7424 |
| R4-2007724.zip | Huawei, Hisilicon | Cat F Rel 13 CR : Add the clarification that “provided that the cell is at least YdB better ranked, where ‘Y’ is specified in Table 4.6.2.6-3” |
| R4-2007725 | Huawei, Hisilicon | Cat A Rel-14 CR shadow for 7724 |
| R4-2007726 | Huawei, Hisilicon | Cat A Rel-15 CR shadow for 7724 |
| R4-2007727 | Huawei, Hisilicon | Cat A Rel-16 CR shadow for 7724 |
| R4-2006082 | Anritsu | Cat F Rel13 CR : Delete patrameter of sr-Configindex in Table A.7.3.55.1-4. |
| R4-2006083 | Anritsu | Cat A rel 14 CR shadow for 6082 |
| R4-2006084 | Anritsu | Cat A rel 15 CR shadow for 6082 |
| R4-2006085 | Anritsu | Cat A rel 16 CR shadow for 6082 |
| R4-2006086 | Anritsu | Cat F rel 13 CRChange a reference to the RMC in the associated test as follows: A.8.1.26 PDSCH parameters in Table A.8.1.26.1-2 R.11 HD-FDD -> R.25 HD-FDD |
| R4-2006087 | Anritsu | Cat A rel 14 CR shadow for 6086 |
| R4-2006088 | Anritsu | Cat A rel 14 CR shadow for 6086 |
| R4-2006089 | Anritsu | Cat A rel 14 CR shadow for 6086 |

## Open issues summary

Discussion of updates in CRs

### Sub-topic 2-1

Sub-topic description: Rel-13 essential corrections

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CRs directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| XXX | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007424 | Qualcomm: we can agree to this CR. Ericsson: the changes is similar to that in R4-2006082 although they are in different sections. It is good to align them. R&S: As per offline comments from Anritsu the SR_Index we propose, does not lead to 20ms periodicity (ref. TS 36.213 Table 10.1.5-1). As such I request a revision of this CR. The values intended in the revision would be to the default ones which RAN5 (TS 38.508) is using for testing. These would be: SR_Index 27 for FDD and 32 for TDD, both leading to SR periodicity of 20ms. Any other views for the revision? Anritsu: Thank you R&S for considering our proposal. However I’m afraid that I was not clear enough in my offline e-mail. Our intention was: sr_ConfigIndex is 30 for FDD and 27 for TDD, which we assume will follow the default value in TS 36.508. Huawei: We are fine with the changes, but we are not quite clear about the test procedure described in the reason of change. From our understanding, UE will sent SR for UL grant for RRC reconfiguration complete and UE will apply the RRC reconfiguration and monitor the UL grant in the “new MPDCCH” upon receiving the RRC request. As described in the reason of change, TE will not apply the reconfiguration until receiving the RRC complete messages, which means even with a longer SR period, TE will not allocate UL grant using the “new MPDCCH” and UE still cannot receive the UL grant. |
| R4-2007724 | Qualcomm: we can agree to this CR. Ericsson: This CR is fine to us. |
| R4-2006082 | Qualcomm: would it be possible to recommend an SR index for RAN5? Removing this from test config makes RAN5 the responsible group for determining this and we’re not sure if this is the right procedure. |
|  | Ericsson: the changes is similar to that in R4-2007424 although they are in different sections. It is good to align them. |
|  | R&S: As per offline discussions with Anritsu, the issue addressed in this CR can be covered in #7424 and its revision, thus I think the CR is not required. |
| R4-2006086 | Qualcomm: we can agree to this CR. |
|  | Ericsson: this CR is fine to us as well. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2007424 | To be revised |
| R4-2007724 | Agreeable |
| R4-2006082 | Merged |
| R4-2006086 | Agreeable |

## Discussion on 2nd round (if applicable)

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-20086576 (revision of R4-2007424) | R&S: Revised, SR_Index 30 for FDD, 27 for TDD, cover page updated to resolve the confusion spotted by HW. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008656 | Moderator recommendation : Agreed |

# Topic #3: Rel-14 maintance

Main technical topic overview. Rel-14 essential corrections

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2007876 | Huawei, HiSilicon | Cat F Rel-14 CR : Change the inter-frequency reselection margin for eMTC to 8dB for normal and enhanced coverage for RSRP based, and 5.5dB for RSRQ based. A redundant note is also removed. |
| R4-2007877 | Huawei, HiSilicon | Cat A Rel-15 CR shadow for 7876 |
| R4-2007878 | Huawei, HiSilicon | Cat A Rel-16 CR shadow for 7876 |

## Open issues summary

Discussion of updates in CRs

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2007876 | Qualcomm: we can agree to this CR. Ericsson: we are also fine with this CR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2007876 | Agreeable |

## Discussion on 2nd round (if applicable)

Not applicable

## Summary on 2nd round (if applicable)

Not applicable; CR agreed in first round

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

Not applicable

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| XXX | Based on 2nd round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

# Topic #4: Rel-15 maintance

Main technical topic overview. Discussion of release 15 clean up, excluding euCA which is discussed under topic 1.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006966 | Ericsson | CAT F Rel 15 CR : All tentative values in square brackets are finalised except for values which are expected to be addressed by other release 15 CRs in RAN4#95 All TBD in specifications are addressed |
| R4-2006968 | Ericsson | Cat F Rel 15 CR / Clause A.8.7A renamed from TBD to E-UTRAN FDD – UTRAN TDD Measurements. Spelling corrections. |
| R4-2006969 | Ericsson | Cat A Rel 16 CR to 6968 |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006966 | Qualcomm: we can agree to this CR. Nokia: we can agree to this CR Moderator (Ericsson) : No comments were received in the first round email discussion on this release 15 36.133 clean up CR, but it contains the same error as commented for the release 16 CR R4-2006967 in table 8.1.2.1-1 and needs to be revised |
| R4-2006968 | Qualcomm: we can agree to this CR. Nokia: we can agree to this CR |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2006966 | To be revised |
| R4-2006968 | Agreeable |

## Discussion on 2nd round (if applicable)

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2008657 (revision of R4-2006966) |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008657 | Comments from first round addressed; moderator recommendation is Agreed |

# Topic #5: Rel-16 maintance

Main technical topic overview. Release 16 clean up.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2006967 | Ericsson | CAT F Rel 16 CR : All tentative values in square brackets are finalised except for values which are expected to be addressed by other release 15 CRs in RAN4#95 All TBD in specifications are addressed |
| R4-2007118 | Nokia, Nokia Shanghai Bell | Cat F Rel 16 CR : Subclause references are corrected in subclauses 5.2.2.4.1 , 5.2.2.5.1 and 5.3.2.2.1 Typo corrected in subclause 5.5.2.3.2. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1

Sub-topic description:

Open issues and candidate options before e-meeting:

Issue 2-1: Please provide comments on CR directly

- Proposals

  - Option 1: TBA

  - Option 2: TBA

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Rapporteur | Please provide comments on CRs directly |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2006967 | Nokia: in 8.1.2.1-1 GP Id#9 the MGL has been deleted completely. Otherwise the CR is agreeable. |
|  | Qualcomm: we can agree to this CR. |
| R4-2007118 | Ericsson : We can agree to this CR |
|  | Qualcomm: we can agree to this CR. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Candidate options: Recommendations for 2nd round: |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2006967 | To be revised |
| R4-2007118 | Agreeable |

## Discussion on 2nd round (if applicable)

| CR/TP number | Comments collection |
| --- | --- |
| R4-2008658 (revision of R4-2006967) |  |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2008658 | Comments from first round addressed. CR R4-2006976 split into Cat A and Cat F part and both shared on reflector. R4-2008658 contains the cat F changes for R16 and is recommended to be agreed |
| R4-2009107 | This contains category A changes for R16. A draft has been shared on the reflector email thread for checking with no comments received. Recommended to be uploaded as official Tdoc after R15 cat F CR is agreed and then handled as per other cat A CRs |