3GPP TSG-RAN WG4 Meeting #96-e		R4-2009721

E-meeting, 17 – 28 August, 2020

Agenda item:	7.6.4

Source:	Intel Corporation

Title:	Discussion on PDCCH-WUS requirements

Document for:	Discussion

# Introduction

In RAN4 #95-e WF on power saving demodulation was approved [1].

- Issue 1: Whether to introduce joint test for PDCCH-WUS during DRX OFF and PDCCH during DRX ON for power saving UE? - Option 1: To define a joint test case for PDCCH-WUS in DRX OFF and PDCCH in DRX ON. - Option 2: No new requirements are needed.

In this paper we provide view on demodulation requirements for NR power saving WI.

# Discussion

As a part of Rel-16 NR power saving, PDCCH-WUS signal was introduced to inform UE about upcoming normal PDCCH and PDSCH signals and, as results, trigger the “Wake Up” during DRX ON state. In Figure 1 we provide the example of this processing.

Figure 1. Example of UE operation with PDCCH-WUS

From physical channel design point of view, PDCCH-WUS is same as normal PDCCH. Therefore, UE can use similar demodulation processing for PDCCH-WUS as for Normal PDCCH, which is already verified by Rel-15 requirements.

Observation #1: No new demodulation processing is verified in PDCCH-WUS test.

Another open question is testing time. Based on TS 38.521-4 [3], the minimum test time for FR1 Normal PDCCH requirements is about 20-100 seconds under assumption that each slot is allocated by PDCCH. Same time, for PDCCH-WUS test we will have one PDCCH transmission within duration of DRX cycle. In paper with simulation assumption [2], it was suggested to consider 80 ms DRX cycle. Under such conditions, test time will increase to 0.4-2.2 hours in case we keep target BLER equal to 10-2.

Observation #2: PDCCH-WUS requirements are not feasible from test time perspective.

Based on TS 38.521-4 [3], the following procedure is considered for Normal PDCCH requirements:

Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pmdsg is the ratio (statDTX)/(NACK+ACK+statDTX).

Taking into account that there is no specific feedback in case UE receive or not PDCCH-WUS, statDTXs will include impact from PDCCH-WUS decoding fail and Normal PDCCH decoding fail (in case UE successfully decodes PDCCH-WUS). Therefore, we cannot separately verify correct PDCCH-WUS processing.

Observation #3: Existing test methodology does not allow to verify whether decoding of PDCCH-WUS has impact of Normal PDCCH/PDSCH.

Proposal 1:	Do not define demodulation requirements for PDCCH-WUS verification.

# Conclusion

In this paper we provided view on demodulation requirements for power saving and made the following observations and proposal:

Observation #1: No new demodulation processing is verified in PDCCH-WUS test.

Observation #2: PDCCH-WUS requirements are not feasible from test time perspective.

Observation #3: Existing test methodology does not allow to verify whether decoding of PDCCH-WUS has impact of Normal PDCCH/PDSCH.

Proposal 1:	Do not define demodulation requirements for PDCCH-WUS verification.

# References

- R4-2008802 “WF on power saving demodulation”, CATT, RAN4 #95-e, May 2020.

- R4-2008803 “Simulation assumption for PDCCH-WUS test”, Mediatek, RAN4 #95-e, May 2020.

- TS 38.521-4 “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance”