3GPP TSG-RAN WG4 Meeting # 96-e 	                                              R4-2009741

Electronic Meeting, 17-28 Aug., 2020

Agenda Item:	7.7.2.1.1

Source:	Intel Corporation

Title:	Further Discussion on NR PRS RSTD Requirements

Document for:		Discussion

# Introduction

In the last RAN4 meeting, UE PRS RSTD requirements were discussed, and some agreements achieved [1]. In this contribution we will provide further considerations on open issues below.

- SINR side condition

- Measurement period requirements

- Measurement report mapping

# SINR Side Condition

In the last meeting the following agreement for PRS RSTD measurement SINR side condition was achieved [1].

- Side condition for PRS-RSTD in FR2 to be: - Option 1. PRS SNR of reference cell to be -3 dB and the side condition for PRS SNR of neighbor cells to be -10 dB. - Option 2. Side conditions for PRS RSTD measurements in FR2 are defined same as those in FR1, i.e. -13dB for neighbour cell and -6dB for serving cell.

In principle the SINR side condition applied shall be up to the probability that UE can detect the sufficient neighbor cells for the measurements. Thus, from the results of the system level simulation provided in R4#93 [2][3][4] for FR2 more than 95% UEs can hear > 2 strongest cells/TRPs when SINR of reference cell and neighbor cells are up to -6dB and -13dB respectively.

However, in the last meeting, some concerns were raised because of the worse measurement performance with the SINR side condition of Option 1 especially under TDL-C propagation channel. Therefore, we also provide some simulation results for FR2 with SINR below.

- Option 1: PRS Es/Iot = -6 dB for reference and PRS Es/Iot = -13 dB for neighbour

The other simulation assumptions can be found in [5] also.

The simulation results for FR2 is given in Figure 1 below.

Figure 1. RSTD simulation results in FR2

From the results in Figure 1, it was noted the performance of PRS RSTD can be quite diverse depending on UE detection algorithm. For an example, if the first arrived path in case of multiple path can be detected correctly, RSTD performance can achieve significant improvements.

Observation 1: RSTD measurement accuracy will be less than [-40.69ns, 89.51ns] under TDL-C propagation channel.

In comparison with existing LTE requirements (e.g. minimum RSTD error required is about 128ns (4Ts) [6]), NR PRS-RSTD measurement performance with SINR=[-6,-13]dB for reference cell and neighbor cells in FR2 can be much better to meet the LTE’s RSTD measurement accuracy requirements.

Therefore, it is safe to propose that:

Proposal 1: SINR side conditions for PRS-RSTD in FR2 can be

- PRS Es/Iot = -6 dB for reference cell and

- PRS Es/Iot = -13 dB for neighbor cells

# Measurement Reporting Period

# Basic PRS RSTD measurement period

According to the previous RAN4’s discussion [1], the following factors composing the baseline formulation of PRS RSTD measurement requirement shall be further considered in detail.

- Basic number of PRS occasions

There are two alternatives for this parameter were captured in WF [1].

Candidate options: - Option 1. A certain number (X) of PRS occasions, X=4 - Option 2. ceil(NPRS,req / KPRS) - NPRS,req is the comb pattern realizations that are required for RSTD measurement, derived from simulation results - KPRS= LPRS/CombSizeN ResourceRepetitionFactor is the number of comb pattern realizations within a single TPRS, - LPRS is the number of PRS symbols per slot

Firstly, it was noted that there was some ambiguity of the definition of PRS occasion in the last meeting. In our views, similarly a PRS occasion in LTE which can occupy multiple consecutive subframes (e.g. 1,2,4,6). And from the realistic UE implementation perspective, UE PRS measurement is based on the correlation over all PRS symbols which may be repeated. That is a PRS occasion in NR can include all slots occupied by PRS resource after the repetition indeed.

And as agreed in [1], the basic number of PRS occasions (X) per PRS resource needed to meet the accuracy requirements accounting for fading channel, AGC settling, and muting pattern. Therefore, the extra margin beside the number of samples which are used to conduct the corresponding accuracy requirements based RAN4’s simulation results is necessary. As the start point, “4” PRS occasions can be assumed.

On the other hand, for Option2 above it will introduce too complicated requirements potentially. For an instance, if the separated accuracy performance requirements defined by the comb size, repetition factor and number of symbols per slot, there are more than “4x7x4 = 112” requirements, which is extremely increase RAN4’s work loading and further realistic testing efforts.

Observation 2: The option defining RSTD measurement relying on more specific PRS parameters (e.g. comb size, repetition factors, e.t.c. ) will increase RAN4 work loading and further testing efforts significantly.

Therefore, As the start point, [4] PRS occasions can be assumed

Proposal 2: [4] PRS occasions can be assumed as the basic number of PRS occasion.

  - A PRS occasion is the time duration in which there is one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor ± its corresponding DL-PRS-expectedRSTD-uncertainty.

- Periodicity of PRS occasion

Also in [1], there are two options for the periodicity of PRS occasions which can be used as the minimum basic time unit per PRS occasion.

Candidate options: - Option 1. maxT, TPRS,MGRP - Option 2. maxTPRS,MGRP

The network configured TPRS is broadcasted to all UEs in a cell. Thus, according to RAN1 agreements on how to define UE processing capability (“T”), it is possible that there is misalignment between the network configured TPRS and UE specific T. As a result, from RAN4 perspective, the measurement delay requirements shall depend on the larger interval of available PRSs. Thus maxT, TPRS,MGRP shall be taken as the basic timing interval for PRS measurement latency requirements.

Proposal 3: The basic timing interval to be used to define PRS measurement period can be:

maxT, TPRS,MGRP

- Scaling of PRS occasion s due to UE processing capability

In [], the candidate options for this parameter as below.

- FFS: Scaling of PRS occasions due to UE processing capability - Option 1. { max LPRSN,NPRSslotN' } - Option 2. Depends on T and Tprs - LPRSN×NPRSslotN', if Tprs <= T - LPRSN×NPRSslotN'×TTprs , of Tprs > T - Option 3. NPRSslotN' - other options not precluded - FFS: requirements do not apply for resources spanning over two sampling periods of N - FFS: requirements apply provided that the configured PRS duration K is no larger than N

For convenience the finalized RAN1 agreements on PRS processing capability are self-contained below.

Agreement: - For UE DL PRS processing capability, - UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE - Additionally, UE reports new parameter, number of DL PRS resources that UE can process in a slot, which is reported per SCS per band. - Values: {1, 2, 4, 8, 12, 16, 32, 64} - The following sets of values for N, T and B are supported - Values for N = {0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50} ms - Values for T = {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms - Values for maximum BW reported by UE = {5, 10, 20, 40, 50, 80, 100, 200, 400} MHz Agreement: The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS

Regarding to the impact due to UE processing capability of (N, T), it is much straightforward to enlarge the measurement period when the duration of total PRS resource within a PRS occasion (denoted by “LPRS”) is longer than N. Hence the exact scaling factor because of (N, T) can be represented as : LPRSN

Moreover, as the additional UE processing capability depending on a shorter measurement window (per slot) was defined as N', the measurement time for PRS symbols within a slot will be extended if the duration of total PRS symbols within a slot (denoted by NPRSslot) is out of UE’s reported capability N'. That is, if the total measurement duration needed for NPRSslot PRS resource is less than N' which indicated by the other UE PRS processing  capability, the measurement period for LPRS resource can rely on UE capability (N,T) only. Otherwise the measurement period shall be scaled by NPRSslotN' also as illustrated in the figure below.

Figure 2. PRS RSTD measurement period scaling due to UE processing capability

Proposal 4: The scaling factor because of UE PRS processing capability can be represented by

Ms=NPRSslotN'LPRSN

- Sampling and processing time for baseline requirements

In the last meeting, the following candidate options for sampling and processing time were also agreed.

- FFS: Sampling and processing time - Option 1: T - Option 2.NPRS=NTTPRS - other options not precluded

From UE implementation (e.g. the computation complexity needed the necessary processing time ) the processing time for PRS resources within a PRS periodicity (TPRS) , which denoted as NPRS in symbol or slot up to defitnion of duration time, can be conducted by:NPRS=NTTPRS

However, from RAN4 requirements perspective, it is better to consider the minimum sampling and processing time can be based on UE capability “T” . Hence, we can propose that

Proposal 5: The basic sampling and processing time unit for PRS measurement can be “T” which is indicated by UE processing capability.

# Further scaling factors for RSTD measurement period

Beside the above basic formulation of RSTD measurement requirements, the further scaling factors shall be taken count into the total requirements, including:

- Rx beam sweeping in FR2

In the last meeting it was also discussed whether or how to extend the measurement period to account for Rx beam sweeping in FR2. And the following agreements were achieved.

- For FR1, no impact of UE RX beam sweeping on RSTD measurement period shall be considered - For FR2 - Use Rx beam sweeping factor similar way as in SSB-based RRM (i.e. no intra-PRS-occasion Rx beam sweeping) for the case when the QCL information is not available to the UE - FFS how to handle the case when the QCL information is available to the UE - Option 1: Do not perform RX beam sweeping - Option 2: Perform RX beam sweeping - Solutions with more advanced RX beam sweeping can be considered in the future releases

In our view, in [6,TS38.133] there are multiple requirements to define the additional delay because of RX beam sweeping , e.g. the requirements for SCell activation delay in FR2. It is noted that in order to guarantee the known conditions for Rx beam both the assistance data (e.g. “DL-PRS-QCL-Info”) and the detectable associated reference signal (e.g. SSB) are required. Similarly, for PRS RSTD measurement in FR2 UE need NOT to perform RX beam sweeping given both QCL information known and associated SSB detectable.

Proposal 6: For PRS RSTD measurement in FR2, UE need NOT to perform RX beam sweeping when

- DL-PRS-QCL-Info is available for UE and

- Associated reference signal (e.g. SSB) is detectable

- Multiple PRS periodicities

The options for the multiple PRS periodicities can be:

- FFS Measurement period with multiple PRS periodicities - Option 1. Use the maximum PRS resource periodicity among all PRS resources - Option 2. LMF needs not to provide multiple PRS periodicity to UE with a single layer at least - other options not precluded

As RAN1/2 has no any explicit restriction on the different PRS periodicities for the PRS resource within a same PRS measurement layer, the maximum PRS resource periodicity can be assumed when RAN4 defining the PRS measurement period.

Proposal 7: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

- Mmultiple PRS frequency layers

In [1], it was agreed that :

- FFS Measurement period with multiple PRS frequency layers - Option 1. Measurement periods adds up for each frequency layer that is configured to be measured - Option 2. Measurement period is scaled by number of frequency layers - Option 3. Based on CSSF

If UE needs to be able to perform the measurement on the multiple positioning frequency layer at the same time, the processing complexity and memory consumption can easily become multiple of the number of layers to be considered, unless some limitations are set. As RAN1 agreed “UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16”, the measurement on the multiple layers shall be in sequential way. That is the total measurement delay on the multiple positioning frequency layer shall be sum of each layer measurement duration totally.

Proposal 8: In case of multiple PRS layers the measurement delay can be:

TRSTD_multiple_layer, NR=i=1NPRS_layerTRSTD_perlayer, NR,i

Where in,

TRSTD_perlayer, NR,i is the measurement delay for ith positioning frequency layer.

- PRS occasion dropping due to SSB

In the last meeting, it was agreed that

- FFS Whether PRS occasion dropping due to SSB collision should be accounted in measurement period - Option 1. Yes - Option 2. No, measurement period will be defined for cases when PRS occasions are not dropped for any reason

And according to the agreements in the last RAN plenary meeting[7, RP-201343], the open issue of concurrent PRS and RRM measurement will be down-scoped. Hence, the requirement in Rel16 can be applicable for the case without concurrent PRS and RRM measurement.

Proposal 9: The requirement in Rel16 need not account the PRS occasion dropping due to PRS and RRM measurement happened simultaneously.

- Measurement period extension for the gap sharing

On the other hand, for the measurements delay with gap, the scaling factor regarding to the measurement gap sharing shall be counted. In our view, the same scaling factor for the gap sharing as the SSB intra/inter measurement can be applied for RSTD measurement also.

And in order to minimize RAN4 efforts, the equal splitting scheme of gap sharing for NR DL RSTD measurement can be assumed in Rel16. Therefore, we can conclude that:

Proposal 10: For RSTD measurement with gap, the measurement delay extension with the similar gap sharing scaling factor as NR SA measurements for the equal splitting gap scheme shall be considered, e.g.

- CSSFRSTD = CSSFwithin_gap,i which is defined in clause 9.1.5.2.2 of TS38.133 [6] for the inter-frequency SSB measurements with equal splitting gap sharing scheme

In summary, we can propose that:

Proposal 11: The RSTD measurement delay can be defined as:

TRSTD, NR=KPRS_RXBeam*i=1NPRS_layer{ CCSFRSTD,i *TRSTD_perlayer, NRi}

Where in ,

- i is the index of PRS positioning frequency measurement layer

- NPRS_layer is the number of positioning frequency layers

- KPRS_RXBeam is the scaling factor for FR2 RX beam sweeping,

  - If QCL information is known by UE and the associated reference signal (e.g. SSB) is detectable, KPRS_RXBeam can be 1. Otherwise it is [8] for UE supporting FR2 power class 1 and [24/5] for UE supporting FR2 power class 2/3/4

- For ith positioning frequency layer

TRSTD_perlayer, NR(i)=maxTPRSi ，Ti ，MGP⋅Msi

  - TPRSi is the PRS resource set periodicity

  - Msi is the scaling factor because of UE PRS processing capability limitation

Msi=NPRS,islotNi'LPRS,iNi

    - PRS resource duration within a PRS resource occasion is denoted by “LPRS,i"

    - PRS symbol duration per a slot is denoted by “NPRS,islot"

    - UE DL PRS processing capability is indicated by {Ni, Ti}  and {Ni, '} defined in TS38.214

- CSSFRSTD,i is the scaling factor for the RSTD measurements on the “i”th positioning frequency layer , which can be same as CSSFwithin_gap,i defined in clause 9.1.5.2.2 of TS38.133 [9] for the equal splitting gap sharing scheme only

# Measurement Report Mapping

In the last RAN4 meeting, there were significant progress achieved for RSTD reporting mapping below [1].

- LMF provides a recommended k value (k1). UE selects parameter k (k2) and informs to the LMF. Further discuss the relation between UE selected parameter k2 and network recommended value k1. - Further discuss for RSTD and UE Rx-Tx time difference reporting: - Relationship between k1 and k2 - Range of k1 and k2 in FR1 - Agreements on RSTD and UE Rx-Tx time difference report mapping tables captured in LS to RAN2 (R4-2005415)

Hereby, we provide further considerations on the open issues above in this section.

As RAN1’s discussion, k1 was configured by LMF mainly rely on PRS resource bandwidth. Meanwhile, from RAN4 perspective k2 will be selected by UE regarding to the possible performance improvements and power consumption.

Therefore, in order to allow UE’s flexibility UE can freely decide k2’s value if the applicable measurement accuracy requirement can be guaranteed.

Proposal 12: LMF and UE can choose k1 and k2 independently from {0,1,2,3,4,5}.

# Conclusion

In this contribution, some considerations on NR PRS RSTD measurement requirements are provided and the following observations and proposals can be drawn:

Observation 1: RSTD measurement accuracy will be less than [-40.69ns, 89.51ns] under TDL-C propagation channel.

Proposal 1: SINR side conditions for PRS-RSTD in FR2 can be

- PRS Es/Iot = -6 dB for reference cell and

- PRS Es/Iot = -13 dB for neighbor cells

Observation 2: The option to define RSTD measurement relying on more specific PRS parameters (e.g. comb size, repetition factors, e.t.c) will increase RAN4 work loading and further testing efforts significantly.

Proposal 2: [4] PRS occasions can be assumed as the basic number of PRS occasion.

  - A PRS occasion is the time duration in which there is one DL PRS resource after repetition by DL-PRS-ResourceRepetitionFactor ± its corresponding DL-PRS-expectedRSTD-uncertainty.

Proposal 3: The basic timing interval to be used to define PRS measurement period can be:

maxT, TPRS,MGRP

Proposal 4: The scaling factor because of UE PRS processing capability can be represented by

Ms=NPRSslotN'LPRSN

Proposal 5: The basic sampling and processing time unit for PRS measurement can be “T” which is indicated by UE processing capability.

Proposal 6: For PRS RSTD measurement in FR2, UE need NOT to perform RX beam sweeping when

- DL-PRS-QCL-Info is available for UE and

- Associated reference signal (e.g. SSB) is detectable

Proposal 7: Use the maximum PRS resource periodicity among all PRS resource in a same positioning frequency layer.

Proposal 8: In case of multiple PRS layers the measurement delay can be:

TRSTD_multiple_layer, NR=i=1NPRS_layerTRSTD_perlayer, NR,i

Where in,

TRSTD_perlayer, NR,i is the measurement delay for ith positioning frequency layer.

Proposal 9: The requirement in Rel16 need not account the PRS occasion dropping due to PRS and RRM measurement happened simultaneously.

Proposal 10: For RSTD measurement with gap, the measurement delay extension with the similar gap sharing scaling factor as NR SA measurements for the equal splitting gap scheme shall be considered, e.g.

- CSSFRSTD = CSSFwithin_gap,i which is defined in clause 9.1.5.2.2 of TS38.133 [6] for the inter-frequency SSB measurements with equal splitting gap sharing scheme

Proposal 11: The RSTD measurement delay can be defined as:

TRSTD, NR=KPRS_RXBeam*i=1NPRS_layer{ CCSFRSTD,i *TRSTD_perlayer, NRi}

Where in,

- i is the index of positioning frequency measurement layer

- NPRS_layer is the number of positioning frequency layers

- KPRS_RXBeam is the scaling factor for FR2 RX beam sweeping,

  - If QCL information is known by UE and the associated reference signal (e.g. SSB) is detectable, KPRS_RXBeam can be 1. Otherwise it is [8] for UE supporting FR2 power class 1 and [24/5] for UE supporting FR2 power class 2/3/4

- For ith positioning frequency layer

TRSTD_perlayer, NR(i)=maxTPRSi ，Ti ，MGP⋅Msi

  - TPRSi is the PRS resource set periodicity

  - Msi is the scaling factor because of UE PRS processing capability limitation

Msi=NPRS,islotNi'LPRS,iNi

    - PRS resource duration within a PRS resource occasion is denoted by “LPRS,i"

    - PRS symbol duration per a slot is denoted by “NPRS,islot"

    - UE DL PRS processing capability is indicated by {Ni, Ti}  and {Ni, '} defined in TS38.214

- CSSFRSTD,i is the scaling factor for the RSTD measurements on the “i”th positioning frequency layer , which can be same as CSSFwithin_gap,i defined in clause 9.1.5.2.2 of TS38.133 [9] for the equal splitting gap sharing scheme only

Proposal 12: LMF and UE can choose k1 and k2 independently from {0,1,2,3,4,5}.

# References

- R4-2008664, “WF on requirements for RSTD and UE Rx-Tx time difference measurement”

- R4-1913487,  “System-level evaluations for PRS-RSTD and PRS-RSRP”

- R4-1913262, “System level Simulation Results for PRS-RSTD and PRS-RSRP Core Requirement”

- R4-1914957, “Further simulation results for NR positioning”

- R4-2002287, “Updated link level simulation assumption for RSTD and RSRP”

- 3GPP TS 38.133 v16.4.0: "NR: Requirements for support of radio resource management "

- 3GPP TS 36.133 v15.7.0: "E-UTRA: Requirements for support of radio resource management "

- R4-2005355, WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement

- R1-2002770, Summary of the RAN1 WG e-mail discussion [100b-e-NR-Pos-01]

- 3GPP TS38.104 v16.0.0, “NR; Base Station (BS) radio transmission and reception”