3GPP TSG-RAN WG4 Meeting # 96-e                                                            R4-2009783

Electronic Meeting, 17 – 28 Aug., 2020

Source: 	CATT

Title: 	Simulation results for NR PUSCH UL timing adjustment demodulation requirement

Agenda item:	7.15.3.2.3

Document for:	Discussion

- Introduction

In RAN4#95-e meeting, adding additional SCS/CBW combinations for ULTA in WF [1] was approved as follows:

- Additional SCS/CBW combinations - Add simulation assumptions for 5MHz CBW/15KHz SCS and 10Mhz CBW/30KHz SCS to requirements for agreed UL timing adjustment scenarios and use applicability rule to only test supported SCS/CBW combinations.

This paper will provide our evaluation results for NR PUSCH UL timing for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS for simulation alignment.

- Evaluation results

The agreed test parameters[1, 2, 3, 4] are summarized and shown in Table A-1 in Annex.

The evaluation results for  NR PUSCH UL timing adjustment are shown in Table 2-1.

Table 2-1 Evaluation results for NR PUSCH UL timing adjustment for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS

| Scenario | CBW | SCS | l0 | MCS | A | Δω | PUSCH mapping type | SNR @70% of maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Ideal(dB) | Impairment(dB) |
| Scenario Y | 5MHz | 15KHz | 2 | 16 | 10us | 0.13s-1 | Type A | 6.18 | 8.68 |
|  | 5MHz | 15KHz | 0 | 16 | 10us | 0.13s-1 | Type B | 6.15 | 8.65 |
|  | 10MHz | 30KHz | 2 | 16 | 5us | 0.26s-1 | Type A | 6.18 | 8.68 |
|  | 10MHz | 30KHz | 0 | 16 | 5us | 0.26s-1 | Type B | 6.16 | 8.66 |
| Scenario Z | 5MHz | 15KHz | 2 | 16 | 10us | 0.18s-1 | Type A | 6.29 | 8.79 |
|  | 5MHz | 15KHz | 0 | 16 | 10us | 0.18s-1 | Type B | 6.27 | 8.77 |
|  | 10MHz | 30KHz | 2 | 16 | 5us | 0.36s-1 | Type A | 6.29 | 8.79 |
|  | 10MHz | 30KHz | 0 | 16 | 5us | 0.36s-1 | Type B | 6.27 | 8.77 |

- Conclusion

This paper provides our evaluation results for NR PUSCH UL timing for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS for simulation alignment.

- References

- R4-2008821,WF on Rel-16 NR HST BS demodulation requirements, Nokia, Nokia Shanghai Bell,RAN4#95-e

- R4-2005536, WF on Rel-16 NR HSTUL TA BS demodulation requirements, ZTE, RAN4#94-ebis

- R4-1915886，Way forward on NR HST PUSCH demodulation requirements，Huawei, HiSilicon，RAN4#93

- R4-2002405，WF on Rel-16 NR HST BS demodulation requirements，Nokia, Nokia Shanghai Bell, RAN4#94-e

- Annex

Table A-1 Test parameters for NR PUSCH UL timing adjustment for 5MHz CBW/15KHz SCS and 10MHz CBW/30KHz SCS

| Parameter | value |
| --- | --- |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A and type B |
| MCS index | MCS 16 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | Type A: l0 = 2 Type B: l0 = 0 |
| SCS and BW | 15kHz: 5MHz; 30kHz: 10MHz |
| RB assignment | 15kHz SCS/5MHz CBW: 10 continuously allocated RBs for each UE 30kHz SCS/10MHz CBW: 10 continuously allocated RBs for each UE |
| Starting PRB index | 15kHz SCS/5MHz CBW: Moving UE 0, Stationary UE 10 30kHz SCS/10MHz CBW: Moving UE 0, Stationary UE 10 |
| Carrier frequency (GHz) | 15kHz SCS: 2.1GHz 30kHz SCS: 3.6GHz |
| Propagation condition | Scenario Y: AWGN 15kHz SCS: A=10us, △ω=0.13s-1 30kHz SCS: A=5us, △ω=0.26s-1 Scenario Z: AWGN 15kHz SCS: A=10us, △ω=0.18s-1 30kHz SCS: A=5us, △ω=0.36s-1 |
| SRS transmission(optional) | Disabled |
| Testing metric | SNR @70% of maximum throughput |