3GPP TSG-RAN WG4 Meeting #96-e	R4-2009863

Electronic Meeting, 17 Aug. – 28 Aug. 2020

Source: 	Intel Corporation

Title: 					RRM requirements for inter-band CA in FR2

Agenda Item: 			7.13.1.5

Document for:	Discussion

# 1	Introduction

The way forward [1] on the RRM requirements for FR2 inter-band CA was agreed in RAN4#95-e. Here are the open issues summarized in the end of WF:

RAN4 needs to further study on the following open issues - The interruption requirements for CBM UE. - The scheduling restrictions and measurement restrictions due to incorrect network configuration - The unknown SCell activation requirement for CBM UE in case2. - Case 2: SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR2.

In our contribution we will discuss these open issues.

# 2	Discussion

## 2.1 Interruption requirements

During the last meeting the following options were for We have the following agreement for CBM UE [1]:

The following options are considered on how to define the interruption requirements for FR2 inter-band CA for CBM UE. - Option 1: the existing interruption requirements of intra-band CA can be applied. - Option 2: the interruption requirements can be defined as the current interruption with adding a SMTC duration which is the longest SMTC duration among all the serving cells in this FR2 band pair. - Option 3: RAN4 RRM need feedback on the RF architectures of common beam UEs from RF session, e.g. in different band combinations.

The exact architecture other than beam (shared phaseshifters) is not specified in RAN4 RF sessions. It is up to UE implementation. There is no need to wait for feedback from RF session. Depending on implementation (shared RF chain or independent RF chains) either existing inter-band CA or existing intra-band CA interruption requirements can be applied. To support both implementations the most conservative requirement should be chosen which is the existing interruption requirements of intra-band CA.

According to TS38.133 Section 8.2.2 both Option 1 and Option 2 describe completely the same interruption requirements

Proposal 1: For the UEs with common beam management in FR2 inter-band CA the existing interruption requirements of intra-band CA can be applied.

## 2.2 Scheduling restrictions

We have the following agreement during the last meeting [1]:

Scheduling restriction requirements for IBM UE - There are no scheduling restrictions on one FR2 band due to RLM/BFD/CBD/L1-RSRP measurements being performed on another FR2 band under the following conditions: - network does not configure simultaneous UL/DL between two FR2 bands if the UE does not have such capability of simultaneousRxTxInterBandCA. - network does not configure mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL. - network does not configure mixed numerology between SSB and data on two FR2 bands if the UE does not have such capability of simultaneousRxDataSSB-DiffNumerology in FR2. - The scheduling availability requirements for FR2 inter-band CA scenario shall be introduced to clarify there is no scheduling restriction if UE uses independent beam. FFS whether to define the scheduling restrictions for the following cases for both IBM UE and CBM UE. - Case 1: network configures simultaneous UL/DL between between two FR2 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA. - Case 2: network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL. - Case 3: network configures mixed numerology between SSB and data on two FR2 bands if the UE does not have the capability of simultaneousRxDataSSB-DiffNumerology in FR2.

Let’s consider the aforementioned three cases.

### 2.2.1 Case 1 and Case 2

Case 1 and case 2 are related to UE capabilities. Two FR2 bands can be configured with simultaneous UL/DL or mixed numerology only when UE indicates its support. There is no need to define scheduling restrictions for these cases.

Proposal 2: Do not define the scheduling restrictions for the cases when network configures simultaneous UL/DL between two FR2 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA and when network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL.

### 2.2.2 Case 3

Note 1: we need to mention that according to TS38.306 simultaneousRxDataSSB-DiffNumerology capability is related to intra-band CA and cannot be directly applied to inter-band case.

Following Note 1, talking about simultaneousRxDataSSB-DiffNumerology capability we mean that data and SSB are received within one band and therefore have same RX beam. However, for FR2 bands RX beam sweeping is assumed to be performed on SSB what significantly complicates concurrent data reception.

Observation 1: simultaneousRxDataSSB-DiffNumerology capability is not applicable for FR2.

Observation 2: configuring FR2 intra-band CA with SSB on one CC and data on another CC will lead to significant performance degradation.

CBM UEs

Similarly to what we have just discussed, for FR2 inter-band CA in case if for CBM UE network configures simultaneous SSB on one FR2 band and data on another FR2 band, data reception will be complicated due to necessity of having common RX beam with SSB where RX beam sweeping is performed.

Proposal 3: For CBM UEs the scheduling restriction applies on one FR2 band due to SS-RSRP/SS-RSRQ/SS-SINR measurements and SSB based RLM/BFD/CBD/L1-RSRP measurement being performed on another FR2 band

IBM UEs

Discussing case 3 we need to consider two sub-cases:

Case 3.1: network configures inter-band CA with data on one FR2 band and SSB on another FR2 band. All the CCs of the first band use one SCS and all the CCs of the second band use another SCS. See Figure 1 for clarification. These sub-case does not need any scheduling restrictions as UE shall perform independent FFT for the bands.

Data SCS 120 kHz SSB SCS 240 kHz Band A Band B CC1 CC2

Figure 1. Inter-band CA example for Case 3.1

Proposal 4: For IBM UEs do not define the scheduling restrictions for the case when network configures mixed numerology between SSB and data on two FR2 bands but keeps the same numerology withing each band.

Case 3.2: network configures inter-band CA with data on one FR2 band and both SSB and data on another FR2 band. See Figure 2 for clarification. In this case we don’t care about numerology. Following Observation 2 scheduling restrictions for the second band (Band B in the figure) should be applied.

Data SSB Band A Band B CC1 CC3 Data CC2

Figure 2. Inter-band CA example for Case 3.2

Proposal 5: For IBM UEs the scheduling restriction applies on one CC due to SS-RSRP/SS-RSRQ/SS-SINR measurements and SSB based RLM/BFD/CBD/L1-RSRP measurement being performed on another CC of the same FR2 band.

## 2.3 Measurements restrictions

The following agreement was captured in [1]:

For CBM UEs in FR2 inter-band CA, the existing measurement restriction requirements for FR2 is applied for the RLM/BFD/CBD/L1-RSRP measurements being performed on different FR2 bands. Measurement restriction requirement for FR2 inter-band CA with IBM - There are no measurement restrictions for the RLM/BFD/CBD/L1-RSRP measurements being performed on different FR2 bands under the following conditions: - network does not configure mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL. - network does not configure mixed numerology between SSB and CSI-RS on two FR2 bands if the UE does not have such capability of simultaneousRxDataSSB-DiffNumerology in FR2. FFS whether to define the measurement restrictions for the following cases for both IBM and CBM. - Case 1: network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception of with different numerologies between FR2 CCs in DL. - Case 2: network configures mixed numerology between SSB and CSI-RS on two FR2 bands if the UE does not have the capability of simultaneousRxDataSSB-DiffNumerology in FR2.

Similar to the scheduling restrictions, there is no need to define measurements restrictions for Case 1. Two FR2 bands can be configured with mixed numerology only when UE indicates its support.

Proposal 6: Do not define the measurement restrictions for the case when network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception of with different numerologies between FR2 CCs in DL

Using the same logic as for scheduling restrictions the following proposals are made:

Proposal 7: For CBM UEs the scheduling restriction applies when the SSB for RLM, BFD, CBD or L1- RSRP measurement is in the same OFDM symbol as the CSI-RS for RLM, BFD, CBD or L1- RSRP measurement

Proposal 8: For IBM UEs Do not define the measurement restrictions for the case when network configures mixed numerology between SSB and CSI-RS on two FR2 bands but keeps the same numerology withing each band.

Proposal 9: For IBM UEs the measurement restriction applies when the SSB for RLM, BFD, CBD or L1- RSRP measurement on one CC is in the same OFDM symbol as the CSI-RS for RLM, BFD, CBD or L1- RSRP measurement on another CC of the same FR2 band.

## 2.4 SCell activation delay requirements

We have the following agreement during the last meeting [1]:

SCell activation requirement for case 2: SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR2 - For CBM UEs in the Case 2, if the target SCell is known, the existing known SCell requirement in the case of“SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1” shall be applied. - For CBM UEs in the Case 2, if the target SCell is unknown, FFS the SCell activation delay requirements. - Option 1: the existing SCell activation delay requirements for FR1+FR2 CA without L1-RSRP measurement delay can be reused. - Other options are not precluded.

We share the opinion that in case of common beam management gNB for all CCs are co-located. In this case measurements on the active serving cell can be reused for the unknown SCell in another FR2 band. So we can remove L1-RSRP measurement delay from the SCell activation delay and agree with the aforementioned Option 1.

Proposal 10: In the case when SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR2 and the target SCell is unknown the existing SCell activation delay requirements for FR1+FR2 CA without L1-RSRP measurement delay can be reused.

# 3	Conclusion

In this contribution, we discussed the RRM impact of inter-band CA in FR2. After discussion, the following conclusions are provided:

Proposal 1: For the UEs with common beam management in FR2 inter-band CA the existing interruption requirements of intra-band CA can be applied.

Proposal 2: Do not define the scheduling restrictions for the cases when network configures simultaneous UL/DL between two FR2 bands if the UE does not have the capability of supporting simultaneousRxTxInterBandCA and when network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception with two different numerologies between FR2 CCs in DL.

Observation 1: simultaneousRxDataSSB-DiffNumerology capability is not applicable for FR2.

Observation 2: configuring FR2 intra-band CA with SSB on one CC and data on another CC will lead to significant performance degradation.

Proposal 3: For CBM UEs the scheduling restriction applies on one FR2 band due to SS-RSRP/SS-RSRQ/SS-SINR measurements and SSB based RLM/BFD/CBD/L1-RSRP measurement being performed on another FR2 band

Proposal 4: For IBM UEs do not define the scheduling restrictions for the case when network configures mixed numerology between SSB and data on two FR2 bands but keeps the same numerology withing each band.

Proposal 5: For IBM UEs the scheduling restriction applies on one CC due to SS-RSRP/SS-RSRQ/SS-SINR measurements and SSB based RLM/BFD/CBD/L1-RSRP measurement being performed on another CC of the same FR2 band.

Proposal 6: Do not define the measurement restrictions for the case when network configures mixed numerology on two FR2 CCs if the UE does not have the capability of supporting simultaneous reception of with different numerologies between FR2 CCs in DL

Proposal 7: For CBM UEs the scheduling restriction applies when the SSB for RLM, BFD, CBD or L1- RSRP measurement is in the same OFDM symbol as the CSI-RS for RLM, BFD, CBD or L1- RSRP measurement

Proposal 8: For IBM UEs Do not define the measurement restrictions for the case when network configures mixed numerology between SSB and CSI-RS on two FR2 bands but keeps the same numerology withing each band.

Proposal 9: For IBM UEs the measurement restriction applies when the SSB for RLM, BFD, CBD or L1- RSRP measurement on one CC is in the same OFDM symbol as the CSI-RS for RLM, BFD, CBD or L1- RSRP measurement on another CC of the same FR2 band.

Proposal 10: In the case when SCell being activated belongs to FR2 and if there is no active serving cell on that FR2 band provided that PCell or PSCell is FR2 and the target SCell is unknown the existing SCell activation delay requirements for FR1+FR2 CA without L1-RSRP measurement delay can be reused.

# 4	References

- R4-2008998, Way forward on FR2 inter-band CA RRM requirements, Huawei