3GPP TSG-RAN WG4 Meeting # 96-e                   	       R4-2009889

# Electronic Meeting, 17th – 28th Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0947 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for introducing the PDSCH RMC configuration in cell re-selection test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The PDSCH RMC configuration of Cell2 is missing in test cases A.6.1.1.1.2-3, A.6.1.1.2.2-3, A.7.1.1.1.2-3 and A.7.1.1.2.2-3. It should be introduced to align with RAN5 specification and it shall be the same as Cell1’s configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the PDSCH RMC configuration of Cell2 in test cases A.6.1.1.1.2-3, A.6.1.1.2.2-3, A.7.1.1.1.2-3 and A.7.1.1.2.2-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH RMC configuration of Cell2 in test cases A.6.1.1.1.2-3, A.6.1.1.2.2-3, A.7.1.1.1.2-3 and A.7.1.1.2.2-3 are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.1.1.1.2-3, A.6.1.1.2.2-3, A.7.1.1.1.2-3 and A.7.1.1.2.2-3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | Y | N | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### >>>>>Start of Change 1<<<<<

##### A.6.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.1.2-1, A.6.1.1.1.2-2 and A.6.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2, 3 | Cell2 |  |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2, 3 | 40 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2, 3 | 15 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 16 | -3.11 | 2.79 | -infinity | 2.79 | -3.11 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 16 | 13 | 16 | -infinity | 16 | 13 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 2 | -82 | -85 | -82 | -infinity | -82 | -85 |
|  |  | 3 | -79 | -82 | -79 | -infinity | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -53.94 | -52.21 | -52.21 | pecified in Cell 1 columns- |  |  |
|  | dBm/9.36 MHz | 2 | -53.94 | -52.21 | -52.21 |  |  |  |
|  | dBm/38.16 MHz | 3 | -47.85 | -46.12 | -46.12 |  |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Sintrasearch | dB | 1, 2, 3 | N50 |  |  | N50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

### >>>>>End of Change 1<<<<<

### >>>>>Start of Change 2<<<<<

##### A.6.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.6.1.1.2.2-1, A.6.1.1.2.2-2 and A.6.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1.

Table A.6.1.1.2.2-1: Supported test configurations

| Configuration | Description of serving cell | Description of target cell |
| --- | --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.6.1.1.2.2-2: General test parameters for FR1 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2, 3 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 |  |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 15 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2, 3 | 75 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.2.2-3: Cell specific test parameters for FR1 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1 | N/A |  |  | N/A |  |  |
|  |  | 2 | TDDConf.1.1 |  |  | TDDConf.1.1 |  |  |
|  |  | 3 | TDDConf.2.1 |  |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  |  | SR.1.1 FDD |  |  |
|  |  | 2 | SR.1.1 TDD |  |  | SR.1.1 TDD |  |  |
|  |  | 3 | SR.2.1 TDD |  |  | SR.2.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  |  | CR.1.1 FDD |  |  |
|  |  | 2 | CR.1.1 TDD |  |  | CR.1.1 TDD |  |  |
|  |  | 3 | CR.2.1 TDD |  |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  |  | CCR.1.1 FDD |  |  |
|  |  | 2 | CCR.1.1 TDD |  |  | CCR.1.1 TDD |  |  |
|  |  | 3 | CCR.2.1 TDD |  |  | CCR.2.1 TDD |  |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2, 3 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  |  | -140 |  |  |
|  |  | 3 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2, 3 | SS-RSRP |  |  | SS-RSRP |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |  |  |
|  |  | 2 | -98 |  |  |  |  |  |
|  |  | 3 | -95 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -98 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
|  | dB | 1 | 14 | 14 | 14 | -4 | -infinity | 12 |
|  |  | 2 |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 2 | -84 | -84 | -84 | -102 | -infinity | -86 |
|  |  | 3 | -81 | -81 | -81 | -99 | -infinity | -83 |
| Io | dBm/9.36 MHz | 1 | -55.88 | -55.88 | -55.88 | -68.60 | -infinity | -57.78 |
|  | dBm/9.36 MHz | 2 | -55.88 | -55.88 | -55.88 | -68.60 | -infinity | -57.78 |
|  | dBm/38.16 MHz | 3 | -49.79 | -49.79 | -49.79 | -62.50 | -infinity | -51.69 |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 | 0 | 0 |
| Snonintrasearch | dB | 1, 2, 3 | 50 |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2, 3 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2, 3 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2, 3 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

### >>>>>End of Change 2<<<<<

### >>>>>Start of Change 3<<<<<

##### A.7.1.1.1.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.7.1.1.1.2-1, A.7.1.1.1.2-2 and A.7.1.1.1.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Only cell 1 is already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.7.1.1.1.2-2: General test parameters for intra frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell1 |  |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T2 end condition | Active cell |  | 1, 2 | Cell2 |  |
|  | Neighbour cells |  | 1, 2 | Cell1 |  |
| Final condition | Visited cell |  | 1, 2 | Cell1 |  |
| RF Channel Number |  |  | 1, 2 | 1 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | >7 | During T1, Cell 2 shall be powered off, and during the off time the physical cell identity shall be changed, The intention is to ensure that Cell 2 has not been detected by the UE prior to the start of period T2 |
| T2 |  | s | 1, 2 | 135 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |
| T3 |  | s | 1, 2 | 35 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.1.2-3: Cell specific test parameters for intra frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
|  |  | 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
|  |  | 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
|  |  | 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| SSB configuration |  | 1 | SSB.3 FR2 |  |  | SSB.7 FR2 |  |  |
|  |  | 2 | SSB.4 FR2 |  |  | SSB.8 FR2 |  |  |
| OCNG Pattern |  | 1, 2 | OP.4 |  |  | OP.4 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  |  |  |  |
|  |  | 2 | -90 |  |  |  |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | -3 | 1.5 | -infinity | 1.5 | -3 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -96 | -91.5 | -infinity | -91.5 | -96 |
|  |  | 2 | -82 | -93 | -88.5 | -infinity | -88.5 | -93 |
| Io on SSB symbols of each cell | dBm/95.04 MHz | 1 | -59.37 | -63.40 | -62.47 | -64.01 | -62.47 | -63.40 |
|  |  | 2 | -57.18 | -62.86 | -61.67 | -64.01 | -61.67 | -62.86 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

### >>>>>End of Change 3<<<<<

### >>>>>Start of Change 4<<<<<

##### A.7.1.1.2.2	Test Parameters

The test scenario comprises of 2 cells on 2 different NR carriers respectively as given in tables A.7.1.1.2.2-1, A.7.1.1.2.2-2 and A.7.1.1.2.2-3. The test consists of three successive time periods, with time duration of T1, T2, and T3 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas and cell 2 is of higher priority than cell 1. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.7.1.1.2.2-1: Supported test configurations

| Configuration | Description for serving cell | Description for target cell |
| --- | --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |  |

Table A.7.1.1.2.2-2: General test parameters for FR2 inter frequency NR cell re-selection test case

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2 | Cell2 | The UE camps on cell 2 in the initial phase and during T1 period the UE reselects to cell 1 |
| T1 end condition | Active cell |  | 1, 2 | Cell1 | The UE shall perform reselection to cell 1 during T1 |
|  | Neighbour cells |  | 1, 2 | Cell2 |  |
| T3 end condition | Active cell |  | 1, 2 | Cell2 | The UE shall perform reselection to cell 2 with higher priority during T3 |
| RF Channel Number |  |  | 1, 2 | 1, 2 |  |
| Time offset between cells |  |  | 1, 2 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
|  |  |  | 2 | SSB.2 FR2 |  |
| SMTC configuration |  |  | 1, 2 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2 | 1.28 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2 | 190 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2 | Not configured |  |
| T1 |  | s | 1, 2 | 35 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2 | >7 | During T2, cell 2 shall be powered off, and during the off time the physical cell identity shall be changed. The intention is to ensure that cell 2 has not been detected by the UE prior to the start of period T3. |
| T3 |  | s | 1, 2 | 95 | T3 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.7.1.1.2.2-3: Cell specific test parameters for FR2 inter frequency NR cell re-selection test case in AWGN

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| PDSCH RMC configuration |  | 1, 2 | SR.3.1 TDD |  |  | SR.3.1 TDD |  |  |
| RMSI CORESET parameters |  | 1, 2 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| RMSI CORESET RMC configuration |  | 1, 2 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| OCNG Pattern |  | 1, 2 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1, 2 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1, 2 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1, 2 | SSB |  |  | SSB |  |  |
| Qrxlevmin | dBm/SCS | 1 | -140 |  |  | -140 |  |  |
|  |  | 2 | -137 |  |  | -137 |  |  |
| Pcompensation | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qhysts | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Qoffsets, n | dB | 1, 2 | 0 |  |  | 0 |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | 1, 2 | SS-RSRP |  |  | SS-RSRP |  |  |
| AoA setup |  | 1, 2 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -93 |  |  | -93 |  |  |
|  |  | 2 | -90 |  |  | -90 |  |  |
| Note2 | dBm/15 kHz | 1 | -102 |  |  | -102 |  |  |
|  |  | 2 |  |  |  |  |  |  |
|  | dB | 1 | 8 | 8 | 8 | -3 | -infinity | 8 |
|  |  | 2 |  |  |  |  |  |  |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -85 | -85 | -96 | -infinity | -85 |
|  |  | 2 | -82 | -82 | -82 | -93 | -infinity | -82 |
| Io | dBm/95.04 MHz | 1 | -55.37 | -55.37 | -55.37 | -62.25 | -infinity | -55.37 |
|  |  | 2 | -52.37 | -52.37 | -52.37 | -59.25 | -infinity | -52.37 |
| Treselection | s | 1, 2 | 0 | 0 | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | 1, 2 | 50 |  |  | Not sent |  |  |
| Threshx, high | dB | 1, 2 | 48 |  |  | 48 |  |  |
| Threshserving, low | dB | 1, 2 | 44 |  |  | 44 |  |  |
| Threshx, low | dB | 1, 2 | 50 |  |  | 50 |  |  |
| Propagation Condition |  | 1, 2 | AWGN |  |  | AWGN |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

### >>>>>End of Change 4<<<<<