3GPP TSG-RAN WG4 Meeting #96-e	R4-2009893

# Electronic Meeting, 17th – 28th Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0949 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 for measurement capability of IDLE mode DC/CA measurement (Section 4.2.2.1, 4.4.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1. In WF agreed in RAN4 #95e, the agreed measurement capability is total number of LTE inter-RAT EMR carriers ≤7, not total number of FDD E-UTRA inter-RAT carriers ≤7 and total number of TDD E-UTRA inter-RAT carriers ≤7. 2. The measurement capabilities for UE supporting inter-freq. or inter-RAT EMR measurement in NR IDLE/INACTIVE mode are specified in different sections 4.2.2.1 and 4.3.2.2. A clarification must be added to show that measurement capabilities in section 4.2.2.1 and in section 4.3.2.2 should be simultaneously followed. 3. RAN2’s capability names have been updated to idleInactiveEUTRA-MeasReport-r16 and idleInactiveNR-MeasReport-r16. 4. For UE supporting idleInactiveEUTRA-MeasReport-r16, the requirement should be specified in the section “measurements of inter-RAT DC candidate cells,” not in the section “measurements of inter-frequency CA/DC candidate cells” |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Modifiy that for UE supporting idleInactiveNR-MeasReport-r16endc-IdleInactiveMeasurements-r16 or idleInactiveEUTRA-MeasReport-r16 shall be capable of monitoring at least: - Depending on UE capability, 7 FDD and TDD E-UTRA inter-RAT carriers. 2. Add “In addition to the requirements defined in section 4.2.2.1,” to clarify that measurement capabilities in section 4.2.2.1 and in section 4.3.2.2 should be simultaneously followed. 3. Udate the capability name of inter-freq. EMR measurement as idleInactiveEUTRA-MeasReport-r16 4. Separate the inter-freq. EMR measurement and inter-RAT EMR measurement in NR IDLE/INACTIVE mode into 2 different sections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. There exists mismatch between agreed measurement capability in RAN4 #95e and in current spec - The relation between measurement capabilities specified in section 4.2.2.1 and measurement capabilities specified in section 4.3.2.2 are unclear - The capability name of inter-freq. EMR measurement is wrong - The requirement for UE supporting idleInactiveEUTRA-MeasReport-r16 is wrongly captured in the section “measurements of inter-frequency CA/DC candidate cells” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.1, 4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change 1------------------------------------------------------------

#### 4.2.2.1	UE measurement capability

For idle mode cell re-selection purposes, and for UE supporting idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 NR CA and MR-DC measurement, the UE shall be capable of monitoring at least:

-	Intra-frequency carrier, and

-	Depending on UE capability, 7 NR inter-frequency carriers, and

-	Depending on UE capability, 7 FDD E-UTRA inter-RAT carriers, and

-	Depending on UE capability, 7 TDD E-UTRA inter-RAT carriers.

In addition to the requirements defined above, a UE supporting E-UTRA measurements in RRC_IDLE state shall be capable of monitoring a total of at least 14 carrier frequency layers, which includes serving layer, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

For Idle mode NR-CA and MR-DC measurements on overlapping carriers, the UE supporting idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall be capable of monitoring at least:

-	Depending on UE capability, 7 NR inter-frequency carriers, and

-	Depending on UE capability, 7 FDD and TDD E-UTRA inter-RAT carriers.

Note:	the definition of overlapping and non-overlapping carriers for Idle mode CA/DC measurements is in section 4.3.

In addition to the requirements defined above, a UE supporting idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 measurements in RRC_IDLE state shall be capable of monitoring a total of at least 13 carrier frequency layers, comprising of any above defined combination of E-UTRA FDD, E-UTRA TDD and NR layers.

---------------------------------------------------------- End of Change 1-----------------------------------------------------------------

------------------------------------------------------- Beginning of Change 2------------------------------------------------------------

### 4.4.2	Measurement Requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, overlapping and non-overlapping carriers..

#### 4.4.2.1	Detected cell requirement during state transition and Idle mode

This subclause defines the requirements for the detected cell status for the idle mode CA/DC measurement when UE transitions from RRC Connected mode to Idle mode and after UE has entered Idle mode. The requirements are applicable to an NE-DC and NR carrier aggregation capable UE which has been configured with one or more of following, one or more SCells, one E-UTRAN PSCell or one or more downlink E-UTRAN SCells during the Connected mode and which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16. The requirements are applicable for SCell(s) and E-UTRAN FDD and TDD PSCell and SCells.

Upon releasing the connection and if the UE has been configured with idle mode CA measurement reporting, following requirements apply concerning the detected cells in Connected mode upon state transitioning to Idle mode and during Idle mode:

-	A cell which is detected cell in Connected mode prior to connection release, shall remain detected after UE has entered Idle mode and during Idle mode, provided that the following conditions are met:

-	The UE has been provided with a list of cells and/or carrier frequencies for early measurement reporting by dedicated RRC signaling and

-	The detected cell is among the list of cells or on a carrier frequency provided for early measurement reporting, and

-	The UE is provided with a valid timer T331 by dedicated RRC signaling, and

-	The detected cell and SSBs remains detectable until UE reconnect to the network and transmits the early measurement report, and

-	The carrier frequency of the detected cell and the carrier frequency of the serving cell are among the supported band combination of the UE.

An inter-RAT E-UTRAN cell is considered detectable according to RSRP, RSRP Ês/Iot, SCH_RP and SCH Ês/Iot defined in Annex B.1.1 and Annex B.1.2 in [36.133] for a corresponding Band. An inter-frequency cell is considered detectable according to the conditions in Annex B.1.2 and B.1.3 for a corresponding band.

#### 4.4.2.2	Measurements of inter-frequency CA/DC candidate cells

While T331 is running, the UE shall perform measurement on the configured overlapping and non-overlapping inter-frequency for idle mode measurement reporting according to the UE measurement capability.

In addition to the requirements defined in section 4.2.2.1, a UE which supports idleInactiveNR-MeasReport-r16 shall support the idle mode CA measurements of at least 7 overlapping inter-frequency carriers and non-overlapping inter-frequency carriers. UE shall be able to support at least 7 overlapping inter-frequency carriers.

For overlapping carriers, the inter-frequency measurement requirements in section 4.2.2.4 apply.

For overlapping inter-frequency carriers, the UE shall be capable of performing SS-RSRP and SS-RSRQ measurements of the overlapping carriers, and the UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the overlapping carriers to higher layers, with measurement accuracy as specified in sub-clauses [38.133] and [38.133], respectively.

The UE shall be able to report idle mode CA/DC measurements when idle mode CA/DC measurement reporting is requested by the network.

#### 4.4.2.2A	Measurements of inter-RAT E-UTRAN DC candidate cells

While T331 is running, the UE shall perform measurement on the configured overlapping and non-overlapping inter-RAT carriers for idle mode measurement reporting according to the UE measurement capability.

In addition to the requirements defined in section 4.2.2.1, a UE which supports idleInactiveEUTRA-MeasReport-r16 shall be able to monitor at least 7 overlapping inter-RAT carriers. UE shall be able to support at least 1 non-overlapping Inter-RAT carrier.

For overlapping carriers, the inter-RAT measurement requirements in section 4.2.2.5 apply.

For overlapping inter-RAT carriers, the UE shall be capable of performing RSRP and RSRQ measurements of the overlapping carriers, and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the overlapping carriers to higher layers, with measurement accuracy as specified in sub-clauses in [36.133] and [36.133], respectively.

The UE shall be able to report idle mode DC measurements when idle mode DC measurement reporting is requested by the network.

#### 4.4.2.3	Measurements on serving cell

The UE shall measure the RSRP and RSRQ level of the serving cell and evaluate the cell selection criterion S defined in section 4.2.2.2 and the UE physical layer shall be capable of reporting RSRP and RSRQ measurements of the serving cell to higher layers, with measurement accuracy as specified in [38.133]

------------------------------------------------------------ End of Change ----------------------------------------------------------------