3GPP TSG-RAN WG4 Meeting #96-e                                                          R4-2009926

Electronic Meeting, 17 - 28 August 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.21-11: Addition of DC configuration for DC_2A-66A_n28A

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_2A-66A_n28A as defined in WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_2-66_n28

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-66A_n28A | DC_2A_n28A DC_66A_n28A |

### 5.x.2	Co-existence studies

Co-existence studies on DC_28_n66 and DC_2_n28 show the following own Rx impact of the 3rd band:

- IMD4 by dual UL of Band 66 and Band n28 may fall into Band 2 DL.

### 5.x.3	∆TIB and ∆RIB values

For DC_2-66_n28, the TIB,c and RIB,c values are reused from the LTE combination CA_2-66-28, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-n28 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.6 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-n28 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n28 | 0.2 |

### 5.x.4	REFSENS requirements

The MSD value for the impact on Band 2 DL by dual uplink of Band 66 + Band n28 stated in 5.x.2 is shown in the following table.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n28A | 2 | 1880 | 5 | 25 | 1960 | 11.0 | FDD | IMD4 |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | n28 | 740 | 5 | 25 | 795 | N/A | FDD | N/A |

##### ---End of changes---

# Reference

[1]	RP-201369, “New WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, RAN#88-e