3GPP RAN WG4 Meeting #96-e		R4-2009960

Electronic meeting, 17th – 28th August 2020

Agenda item:	13.1.1

Source:	Apple Inc.

Title:	Remaining issues with the test methodology for high DL power and low UL power test cases

WI/SI:	FS_FR2_enhTestMethods

Release:	Rel-17

Document for:	Discussion

# Introduction

The FR2 UE RF specification TS38.101-2 [1] relies on the technical report on NR test methodology TR38.810 [2] to define the allowed test methods, measurement procedures, and initial assessments of measurement uncertainty.  The inclusion of large relaxations to the high DL power and low UL power test cases has been identified as the cause of a number of challenges related to correct characterization of UE performance [3-12]: especially for regulatory facing test cases.  The new study on FR2 RF test method enhancement addresses this gap with the following objective [20]:

1.	Define test methodology for high DL power and low UL power test cases

-	Considering path loss reduction, measurement antenna gain improvement, DUT positioning improvement, and MU improvement

-	Considering NFTF (defined in Clause 5.2 of TR38.810) and direct near field test methodologies as possible alternative methods

-	Other approaches are not precluded

-	Study preliminary assessment of measurement uncertainty of new alternative methods

This paper provides our views on the high DL power / low UL power objective.

# Discussion

Following a review by RAN5 of test cases which require high DL signal power and low UL signal power, it can be observed that some test cases require up to 34 dB and 30.4 dB relaxation for DL and UL, respectively [15].  The table below itemizes these relaxations.

| Test Case | Test Specification | Testability Issue | RAN5 Decision | Related documents |
| --- | --- | --- | --- | --- |
| Maximum input level | 38.521-2 38.521-3(EN-DC with FR2) | High DL Power (Note1) | Required relaxation is 26dB for n257/n258/n261 and 34dB for n260. RAN5 decided not to test on all FR2 bands. | R5-185805 |
| Adjacent channel selectivity |  |  | Required relaxation for the interferer of Case 2 is 26dB for n257/n258/n261 and 34dB for n260. Do not test ACS case 2 since with relaxation it is same as case 1, focus on case 1. | R5-195134 |
| Transmit OFF power |  | Low UL Power(Note 2) | Required relaxation is agreed as 30.4dB for 400MHz for n257/n258/n261 with 1dB SNR impact, for n260 for 400MHz BW further discussion is required. Not to test or test with relaxation is TBD. | R5-187273 R5-187274 R5-188063 R5-188065 R5-195135 |
| (Receiver) Spurious emissions |  |  | Estimated SNR is smaller than -15 dB, required relaxation larger than 25 dB depending on frequency range. Not to test or test with relaxation is TBD. |  |
| Spurious emission band UE co-existence |  |  | Whether relaxation is required depends on further analysis of achievable SNR and assesment of MU. |  |
| Adjacent channel leakage ratio |  |  | Whether relaxation is required depends on further analysis of achievable SNR and assesment of MU. |  |
| Minimum output power |  |  | Whether relaxation is required depends on further analysis of achievable SNR and assesment of MU. |  |
| Note 1 : Testability issue due to the upper limit of downlink power achievable from the test system. Note 2 : Testability issue due to the lower limit of measurable power level by the test system. Note 3 : This table does not list all of the test cases with testability issues. RAN5 continue to study the testability of remaining TRx test cases and treatment of them. |  |  |  |  |

Since some of these requirements verify UE compliance with regulatory requirements, the relaxations identified for Rel-15 need to be reduced.

Because the high DL power / low UL power objective allows the consideration of alternative methodologies, it may be useful to progress the work in two tracks:  one examining possible enhancements to the permitted test methods listed in TR38.810 and another focusing on alternative methods.

Proposal 1: Progress the work on the high DL power / low UL power objective in two tracks: one examining possible enhancements to the permitted test methods listed in TR38.810 and another focusing on alternative methods.

In general, we are are interested in enabling lower MU systems and NF systems which may not support all features of conformance test.  Coupled with OEM declarations, enhanced test methods may need to perform beam peak search over a subset of the sphere.

# Conclusions

In this contribution we have shared views on the high DL power / low UL power objective and made the following proposal:

Proposal 1: Progress the work on the high DL power / low UL power objective in two tracks: one examining possible enhancements to the permitted test methods listed in TR38.810 and another focusing on alternative methods.

# References

- 3GPP TS38.101-2, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone,” V15.5.0, 3GPP RAN #83, March 2019

- 3GPP TR38.810, “NR; Study on test methods,” V16.2.0, 3GPP RAN #83, March 2019

- R4-1900213, “Impact of polarization basis mismatch between test equipment and UE on the beam correspondence test procedure,” Apple Inc., 3GPP RAN4 #90, February 2019

- R4-1900712, “On EIRPs of different link polarization for spherical coverage and beam correspondence tolerance,” Samsung

- R4-1901970, “Polarization mismatch impact on power class requirements,” Intel Corporation

- R4-1901821, “On EIS test metric for FR2,” Huawei

- R4-1901822, “Draft CR on EIS test metric for TR 38.810,” Huawei

- R4-1900671, “On FR2 EIS”, Qualcomm

- R4-1900503, “On Simultaneous Transmission/Reception of multiple polarizations by TE,” Keysight, R&S

- R4-1904192, “On Polarization Mismatch Impact,” Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #90bis

- R5-195415, “38.508-1 implementation of FR2 UL demod OTA tests using single pol Rx TE,” Qualcomm, 3GPP RAN5 #83, May 2019

- R4-1713660, “On the environmental conditions for FR2”, Rohde &Schwarz, 3GPP TSG RAN WG4 Meeting #85, December 2017

- R4-1814841, “On FR2 Temperature Testing”, Keysight Technologies, 3GPP TSG-RAN WG4 #89 Meeting, November 2018

- R4-1904191, “On ETC FR2 EIRP and EIS Testing”, Keysight Technologies, 3GPP TSG-RAN WG4 Meeting #90bis

- R5-195196, “5G-NR FR2 Transmitter & Receiver Testability Issues – Regional Regulatory Bodies Input Required,” 3GPP RAN5 #83, May 2019

- RP-190761, "NR RF requirement enhancements for frequency range 2 (FR2)"

- RP-191852, “New SID: Study on enhanced test methods for FR2 UEs,” Apple Inc., CAICT

- R5-193516, “TSG RAN WG5 Action Points after RAN5#82 and RAN5#5 5G-NR Ad hoc”

- 3GPP TS37.544, “UTRA and E-UTRA User Equipment (UE) Over The Air (OTA) performance; Conformance testing,” V14.6.0, June 2018

- RP-192322, “New SID: Study on enhanced test methods for FR2,” Apple Inc., CAICT, 3GPP RAN #85, September 2019