3GPP RAN-WG4 Meeting #96-e		R4-2009966

Electronic meeting, 17th – 28th August 20

Agenda item:	7.1.2.2

Source:	Apple Inc.

Title:	ACS requirements for NR-U

WI/SI:	NR_unlic-Core

Release:	Rel-16

Document for:	Approval

# 1	Introduction

The discussion on the definition of the level on ACS in NR-U is still on-going for single carrier and intra-band CA. During the last meeting the interferer bandwidth was defined but it was not possible to find consensus on the ACS level. This contribution provides our view on the definition of ACS requirement for single carrier and intra-band contiguous CA in NR-U for band n46.

# 2	Discussion

## 2.1	Adjacent Channel Selectivity

In our view it is important to consider the adjacent channel interference ratio (ACIR) on the definition of the ACS value for NR-U, which describes the interference level generated by interfering system to victim system. ACIR is defined as the ratio of the total transmitted power from the base station or the UE to the total interference power affecting a victim receiver. ACLR is used to define the radio performance of transmitter, while ACS reflects the receiver radio performance. Table 1 captures the ACS levels and ACLR limits at 20 MHz for LTE LAA, WiFi 802.11ax and NR FR1 (>= 3300 MHz).

Table 1: ACS levels and ACLR limits

| LTE LAA | WiFi | NR FR1 | NR - U |
| --- | --- | --- | --- |
| ACS level = 27 dB ACLR limit = 45 dB | ACS level = 13 dB ACLR limit = 26 dB | ACS level = 33 dB ACLR limit = 45 dB | ACS = TBD ACLR limit = 35 dB |

The difference between ACS and ACLR is in LTE LAA 18 dB, in Wifi 13 dB, in NR 12 dB. In our view, in NR-U the difference between ACS level and ACLR level shall not be larger than 12 dB. The significant difference between ACLR in NR-U and ACLR in LTE/NR can have an impact on the receiver testing, if the ACS is not adjusted accordingly.

We are proposing an ACS value of 23 dB for 20 MHz, which results in an ACIR of 23 dB when considering an ACLR value of 35 dB. Assuming the ACS value of 23 dB, the difference between ACS and ACLR in NR-U would be 12 dB, which is the same difference as in NR.

For the estimation of ACS values in NR, the value was scaled in terms of wanted bandwidth and interferer bandwidth, applying the equation below.

ACS=ACSSC-10*log10BWwantedBWinterferer

Table 2: Single Carrier ACS levels for NR-U

|  | 20 MHz | 40 MHz | 60 MHz | 80 MHz |
| --- | --- | --- | --- | --- |
| ACS level | 23 dB | 20 dB | 18 dB | 17 dB |

The ACS level for 20 MHz wanted channel bandwidth is defined as 23 dB. For the interferer bandwidth we are considering the agreed jammer BW of 20 MHz [1]. ACS level has been scaled with the equation above in order to maintain the PSD. In Table 2 we provide the ACS levels from wanted channel bandwidth from 20 MHz to 80 MHz.

Proposal 1:	NR-U ACS level values for single carrier shall be defined as in Table 2.

For intra-band contiguous CA we have considered in the contribution the BW classes B, C, D, E and I that are listed as part of the band combinations for NR-U in Rel-16. As for the interferer BW, we are using 20 MHz for BW Class B, C, D, E and I.

Table 3: ACS level for intra-band contiguous CA for NR-U

|  | B | C | D | E | I |
| --- | --- | --- | --- | --- | --- |
| ACS level | 16 dB | 13 dB | 11 dB | 10 dB | 12 dB |

Table 4: Test parameters for intra-band contiguous CA for NR-U

| Rx Parameter | Units | NR-U CA bandwidth class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | I |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |  |  |  |
| PInterferer | dBm | Aggregated power + 14.5 dB | Aggregated power + 11.5 dB | Aggregated power + 9.5 dB | Aggregated power + 8.5 dB | Aggregated power + 10.5 dB |
| BWInterferer | MHz | 20 |  |  |  |  |
| FInterferer (offset) | MHz | 10 + Foffset / -10 - Foffset |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 . NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Proposal 2:	RAN4 shall define the ACS requirements for intra-band contiguous CA as provided in Table 3 and Table 4.

# 3	Conclusions

This contribution provides our view on ACS requirement for NR-U. In summary, we have made the following proposals:

Proposal 1:	NR-U ACS level values for single carrier shall be defined as in Table 2.

Proposal 2:	RAN4 shall define the ACS requirements for intra-band contiguous CA as provided in Table 3 and Table 4.

# 4	References

R4-2008435, “WF on NR-U ACS and blocking requirements”, Apple, 3GPP RAN4 #95-e, June 2020