3GPP TSG-WG4 Meeting #96-e	R4-2010025

# Electronic Meeting, 17th – 28th August, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR |  | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS 38.101-1: Clarification of UE external components |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ V2X_NRSL |  |  |  |  |  | Date: |  |  | 2020-30-7 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In RAN4 the common understanding is that the UE antenna connector is located at the output of the UE as indicated in figure G.1-1. - In V2X applications due to the large form factor there may be an external vehicle antenna connector placed away from the UE. In this scenario there is potential for uncertainty as to whether the transceiver specifications are referenced to the UE antenna connector or the vehicle antenna connector. - To resolve this ambiguity wording has to be inserted into TS38.101-1 to clarify the reference point for the transceiver specifications as being at the UE antenna connector and to also state that any components external to the UE used to move the UE antenna connector to any other location will not be considered in these specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification that components external to the UE will not be considered in transceiver specifications |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For certain implementations where external components are used there will be ambiguity as to the reference point for the transceiver specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 7.1, Annex G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Changes>

## 6		Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed. For V2X applications the transmitter characteristics are specified at UE antenna connector without any external components that may be used to relocate the antenna away from the UE. An example of a V2X UE integration block diagram is illustrated in annex G.

Transmitter requirements for UL MIMO operation apply when the UE transmits on 2 ports on the same CDM group. The UE may use higher MPR values outside this limitation.

The applicability of transmitter requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

## <End of Changes>

## <Start of Changes>

## 7				Receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity). For V2X applications the receiver characteristics are specified at UE antenna connector without any external components that may be used to relocate the antenna away from the UE. An example of a V2X UE integration block diagram is illustrated in annex G.

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective clauses below.

The applicability of receiver requirements for Band n90 is in accordance with that for Band n41; a UE supporting Band n90 shall meet the minimum requirements for Band n41.

<End of Changes>

## <Start of Changes>

Annex G (informative):

Figure G.1-1 shows a V2X integration block diagram with an example set of components external to the UE such as cables and compensator devices.

Cable

Compensator

Vehicle Antenna Connector

UE

Cable

UE Antenna Connector

PA, LNA, Filters

Figure G.1-1 V2X UE integration block diagram

## <End of Changes>