3GPP TSG-RAN WG4 Meeting #96-e                                                                R4-2010035

Electronic Meeting, 17 Aug. – 28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 0966 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on active TCI state switch test case (section A.5.5.8.1, A.7.5.8.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2020-07-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN4 had already agreed a CR on active TCI state in core requirement. The UE shall be able to receive PDCCH with the old TCI state until slot n+ THARQ + 3ms. There is a mismatch between the test case and the core requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the test case to verify UE receiving PDCCH with the old TCI state until slot n+ THARQ + 3ms. - Editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Section A.5.5.8.1 and A.7.5.8.1 are incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.5.8.1, A.7.5.8.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

A.5.5.8.1	MAC-CE based active TCI state switch

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state

A.5.5.8.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10.3Supported test configurations are shown in Table A.5.5.8.1.1.1-1.

The test scenario comprises of one E-UTRA PCell (Cell 1), and one NR PSCell (Cell 2) as given in Table A.5.5.8.1.1.1-2. Cell-specific parameters of E-UTRA PCell are specified in Table A.3.7.2.1-1 and Cell-specific parameters of NR PSCell is specified in Table A.5.5.8.1.1.1-3 below. The OTA related test parameters for FR2 is shown in Table A.5.5.8.1.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PSCell (Cell 2) to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC), and Cell 2 (PSCell) on radio channel 2 (PSCC).

-	UE is configured with 2 different TCI states for PSCell, PDCCH TCI state 0 (QCL’d to SSB0) and TCIstate 1 (QCL’d to SSB1), in Cell 2 before starting the test.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state

The test consists of two time periods, T1 and T2. During T1 only SSB to which PDCCH-TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PSCell on TCI state 0 till n+ THARQ +3 ms.The test equipment also verifies the TCI state switch time in PSCell by scheduling the UE on TCI state 1 after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc) .

Table A.5.5.8.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.5.8.1.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| E-UTRA RF Channel Number |  | 1 | One E-UTRA radio channel is used for this test |
| NR RF Channel Number |  | 2 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| Active PSCell |  | Cell 2 | PSCell on RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | For both PCell and PSCell |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |

Table A.5.5.8.1.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 2 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.5.5.8.1.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |
|  |  | T1 | T2 | T1 | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |
|  |  | AoA1 |  | AoA2 |  |
| NocNote 1 | dBm/15 kHz | [-92.1] |  |  |  |
| NocNote 1 | dBm/SCS | [-83.1] |  |  |  |
| Ês/Noc | dB | 1 | 1 | 1 | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 | -82.1 | -82.1 | -82.1 |
| IoNote2,Note6 | dBm/95.04 MHz Note4 | -54.94 | -54.94 | -54.94 | -54.94 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. |  |  |  |  |  |

A.5.5.8.1.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB

End of Change 1

Start of Change 2

A.7.5.8.1	MAC-CE based active TCI state switch

A.7.5.8.1.1	NR PCell FR2 active TCI state switch for a known TCI state

A.7.5.8.1.1.1	Test Purpose and Environment

The purpose of this test is to verify the active TCI state switch delay requirement defined in clause 8.10.3. Supported test configuration is shown in Table A.7.5.8.1.1.1-1.

The test scenario comprises of one NR PCell (Cell 1) as given in Table A.7.5.8.1.1.1-2. Cell-specific parameters of NR PCell are specified in Table A.7.5.8.1.1.1-3 below. The OTA related test parameters for FR2 are shown in Table A.7.5.8.1.1.1-4.

PDCCHs indicating new transmissions shall be sent continuously on PCell to ensure that the UE would have ACK/NACK sending.

Before the test starts,

-	UE is connected to Cell 1 (PCell) on radio channel 1 (PCC).

-	UE is configured with 2 different TCI states for PCell, PDCCH TCI state 0 (QCL’d to SSB0) and TCIstate 1 (QCL’d to SSB1), in Cell 1 before starting the test.

-	UE is indicated in TCI state 0 as the active PDCCH TCI state

The test consists of two time periods, T1 and T2. During T1 only SSB to which PDCCH-TCI-state0 is QCL’d is transmitted. At the beginning of T2, the SSB corresponding to TCI state 1 starts transmitting. The UE is configured to provide periodic L1-RSRP reports. In slot n which is within 1280ms of UE providing L1-RSRP report with results for both SSB0 and SSB1, UE receives a MAC-CE command indicating a switch to TCI state 1. tci-PresentInDCI is not configured in the PDSCH configuration, i.e. TCI state for the PDSCH is identical to the PDCCH TCI state.

The test equipment verifies that UE can be scheduled on PCell on TCI state 0 till n+ THARQ +3 ms.The test equipment also verifies the TCI state switch time in PCell by scheduling the UE on TCI state 1 after n+ THARQ +3 ms + (Tfirst-SSB + TSSB-proc) .

Table A.7.5.8.1.1.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.8.1.1.1-2: General test parameters for TCI state switch

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1 | One NR radio channel is used for this test |
| Active PCell |  | Cell 1 | PCell on RF channel number 1. |
| CP length |  | Normal |  |
| DRX |  | OFF |  |
| Cell-individual offset for cells on RF channel number 1 | dB | 0 | Individual offset for cells on PCC. |
| Cell-individual offset for cells on RF channel number 2 | dB | 0 | Individual offset for cells on PSCC. |
| Cell2 timing offset to cell1 | s | 3 | Synchronous EN-DC |
| T1 | s | [0.2] |  |
| T2 | s | [0.2] |  |

Table A.7.5.8.1.1.1-3: NR Cell specific test parameters for TCI state switch

| Parameter | Unit | Cell 1 |
| --- | --- | --- |
| Frequency Range |  | FR2 |
| Duplex mode |  | TDD |
| TDD configuration |  | TDDConf.3.1 |
| BWchannel |  | 100 MHz: NRB,c = 66 |
| Initial DL BWP Configuration |  | DLBWP.0.2 |
| Dedicated DL BWP Configuration |  | DLBWP.1.1 |
| Initial UL BWP Configuration |  | ULBWP.0.2 |
| Dedicated UL BWP Configuration |  | ULBWP.1.1 |
| PDSCH Reference measurement channel |  | SR.3.1 TDD |
| RMSI CORESET parameters |  | CR.3.1 TDD |
| Dedicated CORESET parameters |  | CCR.3.1 TDD |
| OCNG Patterns |  | OP.1 |
| SSB Configuration |  | SSB.1 FR2 |
| SMTC Configuration |  | SMTC.1 |
| TCI State 0 |  | TCI.State.0 |
| TCI State 1 |  | TCI.State.1 |
| TRS Configuration |  | TRS.2.1 TDD |
| Correlation Matrix and Antenna Configuration |  | 1x2 Low |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |
| Propagation Condition |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.5.8.1.1.1-4: OTA related test parameters for TCI state switch

| Parameter | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | SSB0 |  | SSB1 |  |  |
|  |  | T1 | T2 | T1 |  | T2 |
| Angle of arrival configuration |  | Setup 3 according to clause A.3.15.3 |  |  |  |  |
|  |  | AoA1 |  |  | AoA2 |  |
| NocNote 1 | dBm/15 kHz | [-92.1] |  |  |  |  |
| NocNote 1 | dBm/SCS | [-83.1] |  |  |  |  |
| Ês/Noc | dB | 1 | 1 | -Infinity |  | 1 |
| SS-RSRP Note 2 | dBm/120 kHz Note3 | -82.1 | -82.1 | -Infinity |  | -82.1 |
| IoNote2 | dBm/95.04 MHz Note4 | -54.9 | -54.9 | -54.9 |  | -54.9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. Note 2: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the center of the quiet zone. |  |  |  |  |  |  |

A.7.5.8.1.1.2	Test Requirements

During T2, UE shall send L1-RSRP report with results for both SSB0 and SSB1.

After receiving MAC-CE command in slot n, UE shall:

-	be able to continue to receive on TCI state 0 till   n+ THARQ +3 ms

-	be able to start receiving on TCI state 1 after n+ THARQ +5 ms + Tfirst-SSB

End of Change 2