3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2010087

# Electronic Meeting, 17-28 Aug., 2020

Source:	LG Electronics

Title:	Discussion of Test Cases for NR V2X RRM

Agenda item:	7.3.6.2

Document for:	Discussion

Introduction

List of test cases for NR V2X RRM is introduced in [1]. Based on the list, we provide our view on related configuration, parameters and test setup as starting point of discussion.

Discussion

Comparing to LTE-V2X, main difference of Rel-16 NR V2X is to support different SCS such as 15kHz, 30kHz and 60kHz in FR1. In addition, V2X UE resource pre-emption and re-evaluation are specified in L1 SL-RSRP measurement requirements.

Except for the differences, almost requirements is similar to LTE-V2X. Therefore, the test cases in LTE-V2X can be considered as baseline for introducing the test cases for NR V2X RRM.

Proposal 1: Consider test cases in LTE-V2X as baseline for introducing test cases in NR V2X RRM.

Related core requirements, the following tests are needed to be introduced.

Proposal 2: Introduce the following test cases in NR V2X RRM.

- UE Transmit Timing

  - Test for GNSS as synchronization reference source

  - Test for FR1 NR Cell as synchronization reference source

  - Test for E-UTRAN Cell as synchronization reference source

  - Test for SyncRef UE as synchronization reference source

- Initiation/Cease of SLSS Transmissions

  - Test for FR1 NR Cell as synchronization reference source without gap under non-DRX

  - Test for E-UTRAN Cell as synchronization reference source

- Selection / Reselection of V2X Synchronization Reference Source

  - Test for GNSS configured as the highest priority

  - Test for FR1 NR Cell configured as the highest priority

- L1 SL-RSRP measurements

  - Test for V2X UE Autonomous Resource

  - Test for V2X UE Resource Pre-emption

  - Test for V2X UE Resource Re-evaluation

- Congestion Control measurements

  - Test for Congestion Control Measurement

- Interruption

  - Test for interruptions to WAN due to V2X Sidelink Communication

For each test case, considering all SCSs is not recommended in aspect of increasing number of test cases. To reduce test cases, same approach as Rel-15 NR test case is recommended such as to set the supported test configuration and test parameters in each test.

And, related configurations such as reference resource pool configuration, SL reference measurement channels for PSCCH and PSSCH, SL BWP configurations, SL OCNG and S-SSB configurations need to be introduced.

Proposal 3: Define supported test configuration and test parameters in each test as Rel-15 NR test case.

Proposal 4: Define test related configurations in new sub-section of A.3 RRM test configurations.

Example,

A.3	     RRM test configurations

A.3.19    V2X sidelink communication

A.3.19.1	Introduction

A.3.19.2	Reference resource pool configuration

A.3.19.3	SL Reference measurement channels

A.3.19.4	SL BWP configurations

A.3.19.5	SL Generic OFDMA Channel Noise Generator (SLOCNG)

A.3.19.6	S-SSB Configuration for FR1

The sub-section 2.1 and 2.2 show the examples for proposal 2 and proposal 3 as starting point of discussion.

Proposal 5: Consider examples in 2.1 and 2.2 as starting point of discussion for RRM test configuration.

Example of supported test configuration and test parameters for V2X UE transmit timing.

  - Test for GNSS as synchronization reference source

    - Supported Test Configurations

| Configuration | Description |
| --- | --- |
|  | NR V2X |
| 1 | NR V2X SCS 15kHz, BW 10MHz |
| 2 | NR V2X SCS 30kHz, BW 40MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

    - Test parameters

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2 | None |  |
| Active V2X UE |  |  |  |  |
| NR RF Channel Number |  | 1,2 | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 1 | 10 |  |
|  |  | 2 | 40 |  |
| V2X SL configuration |  | 1,2 | [TBD] |  |
| SL Tx Pool |  |  |  |  |
| Bitmap of SL Tx resource pool |  | 1,2 | [TBD] |  |
| SL BWP Configuration |  | 1,2 | [TBD] |  |
| PSCCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| PSSCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| Propagation condition |  |  | AWGN |  |

  - Test for FR1 NR cell as synchronization reference source

    - Supported Test Configurations

| Configuration | Description |  |
| --- | --- | --- |
|  | NR cell | NR V2X |
| 1 | NR FDD, SSB SCS 15 kHz, data SCS 15 kHz, BW 10 MHz | NR V2X SCS 15kHz, BW 10MHz |
| 2 | NR TDD, SSB SCS 30 kHz, data SCS 30 kHz, BW 40 MHz | NR V2X SCS 30kHz, BW 40MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

    - Test parameters

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2 | Table X.X |  |
| Active V2X UE |  |  |  |  |
| NR RF Channel Number |  | 1,2 | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 1 | 10 |  |
|  |  | 2 | 40 |  |
| V2X SL configuration |  | 1,2 | [TBD] |  |
| SL Tx Pool |  |  |  |  |
| Bitmap of SL Tx resource pool |  | 1,2 | [TBD] |  |
| SL BWP Configuration |  | 1,2 | [TBD] |  |
| PSCCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| PSSCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| Propagation condition |  |  | AWGN |  |

For parameter for active cell, corresponding Table X.X needs to be introduced.

  - Test for E-UTRAN cell as synchronization reference source

    - Supported Test Configurations

| Configuration | Description |  |
| --- | --- | --- |
|  | E-UTRAN cell | NR V2X |
| 1 | LTE FDD, BW 10 MHz | NR V2X SCS 15kHz, BW 10MHz |
| 2 | LTE TDD, BW 10 MHz | NR V2X SCS 30kHz, BW 40MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

    - Test parameters

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1,2 | Table X.Y |  |
| Active V2X UE |  |  |  |  |
| NR RF Channel Number |  | 1,2 | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 1 | 10 |  |
|  |  | 2 | 40 |  |
| V2X SL configuration |  | 1,2 | [TBD] |  |
| SL Tx Pool |  |  |  |  |
| Bitmap of SL Tx resource pool |  | 1,2 | [TBD] |  |
| SL BWP Configuration |  | 1,2 | [TBD] |  |
| PSCCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| PSSCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| Propagation condition |  |  | AWGN |  |

For parameter for active cell, corresponding Table X.Y needs to be introduced.

  - Test for SyncRef UE as synchronization reference source

    - Supported Test Configurations

| Configuration | Description |  |
| --- | --- | --- |
|  | SyncRef UE | NR V2X |
| 1 | NR V2X SCS 15kHz, BW 10MHz | NR V2X SCS 15kHz, BW 10MHz |
| 2 | NR V2X SCS 30kHz, BW 40MHz | NR V2X SCS 30kHz, BW 40MHz |
| Note: The UE is only required to be tested in one of the supported test configurations |  |  |

    - Test parameters

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active SyncRef UE |  | 1,2 | Table X.Z |  |
| Active V2X UE |  |  |  |  |
| NR RF Channel Number |  | 1,2 | 1 |  |
| Channel Bandwidth (BWchannel) | MHz | 1 | 10 |  |
|  |  | 2 | 40 |  |
| V2X SL configuration |  | 1,2 | [TBD] |  |
| SL Tx Pool |  |  |  |  |
| Bitmap of SL Tx resource pool |  | 1,2 | [TBD] |  |
| SL BWP Configuration |  | 1,2 | [TBD] |  |
| PSCCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| PSSCH RMC Configuration |  | 1 | [TBD] |  |
|  |  | 2 | [TBD] |  |
| Propagation condition |  |  | AWGN |  |

For parameter for active cell, corresponding Table X.Z needs to be introduced.

Example of test related configurations

As test related configurations, there are reference resource pool configuration, SL reference measurement channels, SL BWP configurations, SL OCNG and S-SSB configurations.

Table 2.2-1 shows that configurations are applied to which test cases.

Table 2.2-1: RRM test configurations for NR V2X

| Test Config. | V2X UE Transmit timing | Initiation/Cease of SLSS Transmission | Selection / Reselection of V2X Synchronization Reference Source | L1 SL-RSRP Measurement | Congestion Control measurements | Interruption |
| --- | --- | --- | --- | --- | --- | --- |
| Reference resource pool pre-configuration | O | O | O | - | - | - |
| Reference resource pool configuration | O | O | O | O | O | O |
| SL reference measurement channels | O | O | - | O | O | O |
| SL OCNG configuration | - | - | - | O | - | - |
| S-SSB configurations | O | O | O | - | - | - |

Here,

- Reference resource pool configuration is introduced similarly as LTE-V2X.

- SL reference measurement channels are introduced for PSCCH and PSSCH as follows.

  - PSCCH RMC : {CBW, RBs, SCS} = {10MHz, 10RBs, 15kHz}, {40MHz, 10RBs, 30kHz}

  - PSSCH RMC : {CBW, RBs, SCS} = {10MHz, 50RBs, 15kHz}, {10MHz, 10RBs, 15kHz}

{40MHz, 50RBs, 30kHz}, {40MHz, 10RBs, 30kHz}

- SL OCNG configuration is introduced for all unused REs similarly as LTE-V2X.

- S-SSB configurations are introduced with {CBW, SCS} = {10MHz, 15kHz}, {40MHz, 30kHz}

Detailed examples are as follows.

  - Reference resource pool configuration

    - Resource pool pre-configuration for V2X SL communication

| Parameter | Unit | Value/remark | Comment |
| --- | --- | --- | --- |
| Reference channel |  | V2XSLConf.1.1 |  |
| SL Rx Pool |  |  |  |
| Bitmap of SL Rx resource pool |  | 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 | sl-TimeResource = 100 |
| SL Subchannel Size | RB | 10 |  |
| Number of SL Subchannel |  | 1 |  |
| StartRB of SL Subchannel | RB index | 0 |  |
| SL Tx Pool |  |  |  |
| Bitmap of SL Tx resource pool |  | 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 1111111111_1111111111 | sl-TimeResource = 100 |
| SL Subchannel Size | RB | 10 |  |
| Number of SL Subchannel |  | 1 |  |
| StartRB of SL Subchannel | RB index | 0 |  |

    - Resource pool configuration for V2X SL communication

| Parameter | Unit | Value/remark | Comment |
| --- | --- | --- | --- |
| Reference channel |  | V2XSLConf.2.1 |  |
| SL Rx Pool |  |  |  |
| Bitmap of SL Rx resource pool |  | 1111111111_1111111111 | sl-TimeResource = 20 |
| SL Subchannel Size | RB | 50 |  |
| Number of SL Subchannel |  | 1 |  |
| StartRB of SL Subchannel | RB index | 0 |  |
| SL Tx Pool |  |  |  |
| Bitmap of SL Tx resource pool |  | 1111111111_1111110000 | sl-TimeResource = 20 |
| SL Subchannel Size | RB | 50 |  |
| Number of SL Subchannel |  | 1 |  |
| StartRB of SL Subchannel | RB index | 0 |  |
| SL Tx Pool Exception |  |  |  |
| Bitmap of SL Tx resource pool |  | 0000000000_0000001111 | sl-TimeResource = 20 |
| SL Subchannel Size | RB | 50 |  |
| Number of SL Subchannel |  | 1 |  |
| StartRB of SL Subchannel | RB index | 0 |  |
| SL-UE-SelectedConfig |  |  |  |
| SL UE speed threshold | km/h | 200 | SL UE speed = 0 km/h |
| Minimum MCS |  | 4 |  |
| Maximum MCS |  | 25 |  |
| MinSubChannelNumPSSCH |  | 1 |  |
| MinSubChannelNumPSSCH |  | 1 |  |
| MaxTxTransNumPSSCH |  | 1 |  |
| SL-UE-SelectedConfigRP |  |  |  |
| PSSCH-RSRP Threshold |  | 1 | -128dBm |

  - SL Reference measurement channels

    - PSCCH Reference Measurement Channels for SCS=15kHz

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | SLCR.1.1 HD |
| Channel bandwidth |  | MHz | 10 |
| Subcarrier spacing |  | kHz | 15 |
| Number of transmitter antennas |  |  | 1 |
| Allocated PSCCH resource blocks |  |  | 10 |
| Number of PSCCH symbols in a slot |  |  | 2 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 37 |
| Information Bit | SCI Format |  | 1-A |
|  | Priority |  | As set by higher layers |
|  | Frequency resource assignment Note1 |  | Set as per PSSCH RB allocation specific in the test |
|  | Time resource assignment Note1 |  | Set as per PSSCH RB allocation specific in the test |
|  | Resource reservation period |  | 0 |
|  | DMRS pattern |  | Set as the DMRS pattern specified in the test |
|  | 2nd stage SCI Format |  | 2-A |
|  | Beta_offset indicator |  | 0 |
|  | Number of DMRS port |  | 0 |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | Additional MCS table indicator |  | 0 |
|  | PSFCH overhead indication |  | 0 |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits |  | Bits | 360 |
| Note 1: UE is allowed to autonomously select the un-used or redundant bits/code-points in SCI format 1-A |  |  |  |

    - PSCCH Reference Measurement Channels for SCS=30kHz

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | SLCR.2.1 HD |
| Channel bandwidth |  | MHz | 40 |
| Subcarrier spacing |  | kHz | 30 |
| Number of transmitter antennas |  |  | 1 |
| Allocated PSCCH resource blocks |  |  | 10 |
| Number of PSCCH symbols in a slot |  |  | 2 |
| Modulation |  |  | QPSK |
| Information Bit Payload (without CRC) |  | Bits | 37 |
| Information Bit | SCI Format |  | 1-A |
|  | Priority |  | As set by higher layers |
|  | Frequency resource assignment Note1 |  | Set as per PSSCH RB allocation specific in the test |
|  | Time resource assignment Note1 |  | Set as per PSSCH RB allocation specific in the test |
|  | Resource reservation period |  | 0 |
|  | DMRS pattern |  | Set as the DMRS pattern specified in the test |
|  | 2nd stage SCI Format |  | 2-A |
|  | Beta_offset indicator |  | 0 |
|  | Number of DMRS port |  | 0 |
|  | Modulation and coding scheme |  | Set as the PSSCH MCS specified in the test |
|  | Additional MCS table indicator |  | 0 |
|  | PSFCH overhead indication |  | 0 |
|  | Reserved bits |  | Set all these bits to 0 |
| Transport block CRC |  | Bits | 24 |
| Binary Channel Bits |  | Bits | 360 |
| Note 1: UE is allowed to autonomously select the un-used or redundant bits/code-points in SCI format 1-A |  |  |  |

    - PSSCH Reference Measurement Channels for SCS=15kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | SLSR.1.1 HD | SLSR.1.2 HD |
| Sidelink resource allocation mode |  | 2 | 2 |
| Channel bandwidth | MHz | 10 | 10 |
| Allocated PSSCH resource blocks |  | 50 | 10 |
| startSLsymbolsNote1 |  | 2 | 2 |
| lengthSLsymbols Note1 |  | 9 | 9 |
| Number of PSSCH DM-RS |  | 2 | 2 |
| Modulation |  | QPSK | QPSK |
| Target Code Rate |  | 1/3 | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 3240 | 640 |
| Transport block CRC | Bits | 24 | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 | 0 |
| Binary Channel Bits (see Note 2) | Bits | 9600 | 1920 |
| Note 1: startSLsymbols and lengthSLsymbols are specified in 8.1.2.1 in TS39.214 [26]. Note 2: Binary channel bits calculated under assumption of 9 symbols per slot. Note 3: PSSCH is not scheduled in slots containing S-SSB according to the S-SSB configuration used in the test. S-SSB configurations are defined in clause A.3.19.6. |  |  |  |

    - PSSCH Reference Measurement Channels for SCS=30kHz

| Parameter | Unit | Value |  |
| --- | --- | --- | --- |
| Reference channel |  | SLSR.1.1 HD | SLSR.1.2 HD |
| Sidelink resource allocation mode |  | 2 | 2 |
| Channel bandwidth | MHz | 40 | 40 |
| Allocated PSSCH resource blocks |  | 50 | 10 |
| startSLsymbolsNote1 |  | 2 | 2 |
| lengthSLsymbols Note1 |  | 9 | 9 |
| Number of PSSCH DM-RS |  | 2 | 2 |
| Modulation |  | QPSK | QPSK |
| Target Code Rate |  | 1/3 | 1/3 |
| Information Bit Payload (Transport block size) | Bits | 3240 | 640 |
| Transport block CRC | Bits | 24 | 24 |
| Number of PSSCH HARQ retransmissions |  | 0 | 0 |
| Binary Channel Bits (see Note 2) | Bits | 9600 | 1920 |
| Note 1: startSLsymbols and lengthSLsymbols are specified in 8.1.2.1 in TS39.214 [26]. Note 2: Binary channel bits calculated under assumption of 9 symbols per slot. |  |  |  |

  - SL BWP configurations

| BWP Parameters | Unit | Values |  |  |
| --- | --- | --- | --- | --- |
| Reference BWP |  | SLBWP.1.1 |  |  |
| Starting PRB index |  | 0 |  |  |
| Bandwidth | RB | Same as RF channel defined in each test |  |  |

  - SL Generic OFDMA Channel Noise Generator (SLOCNG)

    - SLOCNG pattern 1: Generic SL OCNG pattern for all unused REs

| OCNG Parameters | Control Region | Data Region |
| --- | --- | --- |
| Resource allocation | Unused REs Note 1 | Unused REs Note 2 |
| Channel | PSCCH | PSSCH |
| Contents | Virtual UE IDs | Uncorrelated pseudo random QPSK modulated data |
| Antenna transmission scheme | Same as used in PSCCH RMC | Same as used in PSSCH RMC |
| Subcarrier spacing | Same as used in PSCCH RMC | Same as used in PSSCH RMC |
| Code rate | Same as used in PSCCH RMC | Same as used in PSSCH RMC |
| Transmit Power(Allocation ) | Relative power level [dB] |  |
| 0 - 9 | 18 | 18 |
| 20 - 29 | 18 | 18 |
| 30 - 39 | 18 | 18 |
| 40 - 49 | 18 | 18 |
| CP length | Same as used in PSCCH RMC | Same as used in PSSCH RMC |
| Note 1: REs not used in the scheduled PSCCH for the UE under test. Note 2: REs not allocated to any physical channels, S-SSB or any other reference signal within the channel bandwidth under test. Note 3: These physical resource blocks are assigned to an arbitrary number of virtual UEs with one PSSCH per virtual UE; The parameter is used to scale the power of PSCCH and the power of PSSCH. Note 4: Value of is applicable to slots not used for V2X sidelink transmissions. |  |  |

  - S-SSB Configuration for FR1

    - S-SSB pattern 1 in FR1: S-SSB allocation for SSB SCS=15 kHz in 10 MHz

| S-SSB Parameters | Values |  |
| --- | --- | --- |
| Reference S-SSB | S-SSB.1.1 FR1 | S-SSB.1.2 FR1 |
| Channel bandwidth | 10 MHz |  |
| S-SSB SCS | 15 kHz |  |
| S-SSB periodicity (TS-SSB) | 160 ms |  |
| Number of S-SSBs per SS-burst | 1 |  |
| S-SS/PSBCH block index | 0 |  |
| Symbol numbers containing S-SSB Note 2 | 13 |  |
| Slot numbers containing S-SSB Note 2 | 0 |  |
| SFN containing S-SSB | SFN mod (TS-SSB/10ms) = 0 |  |
| RB numbers containing S-SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+10)Note 1 |  |
| Slot offset from the start of S-SSB period to 1st S-SSB | 0 | 1 |
| Note 1: RBs containing S-SSB can be configured in any frequency location within the bandwidth. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

    - S-SSB pattern 2 in FR1: S-SSB allocation for SSB SCS=30 kHz in 40 MHz

| S-SSB Parameters | Values |  |
| --- | --- | --- |
| Reference S-SSB | S-SSB.2.1 FR1 | S-SSB.2.2 FR1 |
| Channel bandwidth | 40 MHz |  |
| S-SSB SCS | 30 kHz |  |
| S-SSB periodicity (TS-SSB) | 160 ms |  |
| Number of S-SSBs per SS-burst | 1 |  |
| S-SS/PSBCH block index | 0 |  |
| Symbol numbers containing S-SSB Note 2 | 13 |  |
| Slot numbers containing S-SSB Note 2 | 0 |  |
| SFN containing S-SSB | SFN mod (TS-SSB/10ms) = 0 |  |
| RB numbers containing S-SSB within channel BW | (RBJ, RBJ+1,.…, RBJ+10)Note 1 |  |
| Slot offset from the start of S-SSB period to 1st S-SSB | 0 | 1 |
| Note 1: RBs containing S-SSB can be configured in any frequency location within the bandwidth. Note 2: These values have been derived from other parameters for information purposes (as per TS 38.213 [3]). They are not settable parameters themselves. |  |  |

Conclusion

In this paper, we provided our views on the RRM test cases for NR V2X, and the examples related to configuration and test parameters. Based on these, our proposals are as follows.

Proposal 1: Consider test cases in LTE-V2X as baseline for introducing test cases in NR V2X RRM.

Proposal 2: Introduce the following test cases in NR V2X RRM.

- UE Transmit Timing

  - Test for GNSS as synchronization reference source

  - Test for FR1 NR Cell as synchronization reference source

  - Test for E-UTRAN Cell as synchronization reference source

  - Test for SyncRef UE as synchronization reference source

- Initiation/Cease of SLSS Transmissions

  - Test for FR1 NR Cell as synchronization reference source without gap under non-DRX

  - Test for E-UTRAN Cell as synchronization reference source

- Selection / Reselection of V2X Synchronization Reference Source

  - Test for GNSS configured as the highest priority

  - Test for FR1 NR Cell configured as the highest priority

- L1 SL-RSRP measurements

  - Test for V2X UE Autonomous Resource

  - Test for V2X UE Resource Pre-emption

  - Test for V2X UE Resource Re-evaluation

- Congestion Control measurements

  - Test for Congestion Control Measurement

- Interruption

  - Test for interruptions to WAN due to V2X Sidelink Communication

Proposal 3: Define supported test configuration and test parameters in each test as Rel-15 NR test case.

Proposal 4: Define test related configurations in new sub-section of A.3 RRM test configurations.

Example,

A.3	     RRM test configurations

A.3.19    V2X sidelink communication

A.3.19.1	Introduction

A.3.19.2	Reference resource pool configuration

A.3.19.3	SL Reference measurement channels

A.3.19.4	SL BWP configurations

A.3.19.5	SL Generic OFDMA Channel Noise Generator (SLOCNG)

A.3.19.6	S-SSB Configuration for FR1

Proposal 5: Consider examples in 2.1 and 2.2 as starting point of discussion for RRM test configuration.

- Reference

[1] R4-2010086, “Work Plan and List of Test Cases for NR V2X RRM”, LG Electronics