TSG-RAN Working Group 4 (Radio) meeting #96-E	R4-2010180

Electronic Meeting, 17th – 28th August 2020

Source:	Ericsson

Title:	TP to TR 38.921: Addition of BS antenna model and parameters in subclause 4.2.3 and subclause 8.1

Agenda item:	11.1.4

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#95-e) base station antenna array parameters needed for the simulation campaign part of the SI was agreed [1, 2, 3].

In this contribution a text proposal has been created based on agreements from last meeting regarding base stations antenna array simulation parameters applicable for the simulation campaign part of the SI. The text proposal is attached at the end on this contribution and is presented for approval.

# Discussion

The antenna parameters agreed last meeting in [1] is captured in Table 2-1.

Table 2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) |
| dh (m) | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 |
| Total Radiated Power (dBm) | 46 | 46 | [41 or 37] |
| Mechanical downtilt (deg.) | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: TRP includes the powers from two polarizations |  |  |  |

To align with parameters for the frequency range below 6 GHz, communicated to ITU-R in [2] it is suggested to add a separate parameter for mechanical tilt. The values for mechanical tilt are deployment specific and aligned with the earlier mentioned LS response.

The array antenna model developed for AAS BS in TR 38.840 has been improved and captured in TR 38.820. For report completeness and also according to 3GPP drafting rules the array antenna model is copied from TR 38.820 into TR 38.921.

# Conclusion

A complete text proposal for TR 38.921 including the array antenna model and applicable BS antenna parameters is attached at the end of this contribution. The text proposal adds array antenna model in subclause 8.1.1 and array antenna parameters in subclause 8.1.2. For BS a reference is added in subclause 4.2.3 to antenna characteristics to be used during the simulations.

The text proposal is presented for approval.

# References

[1]	R4-2008926, “WF on BS Antenna parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz”, Huawei, Nokia

[2] 	R4-2008924, “LS on Parameters of terrestrial component of IMT for sharing and compatibility studies in preparation for WRC-23 (below 5 GHz)”, Ericsson

[3]	R4-2008966, “Email discussion summary for [95e][138] FS_6425_10500MHz _NR”, Ericsson

TEXT PROPOSAL:

### 4.2.3		Antenna and beam forming pattern modelling

The BS antenna is modelled as described in subclause 8.1.1 using parameters for different BS deployments listed in subclause 8.1.2.

TEXT PROPOSAL:

## 8.1	BS antenna characteristics

### 8.1.1		Array antenna model

In Table 8.1.1-1, the parameters used by the parameterized array antenna model are described.

Table 8.1.1-1: Parameters of the parameterized array antenna model

| Parameter | Symbol | Unit |
| --- | --- | --- |
| Front to back ratio | Am | dB |
| Side lobe suppression | SLAv | dB |
| Horizontal HPBW | 3dB | Degrees |
| Vertical HPBW | 3dB | Degrees |
| Array element peak gain | GE,max | dBi |
| Number of radiating elements rows and columns | (M, N) | Integer |
| Horizontal element separation | dh | m |
| Vertical element separation | dv | m |
| Electrical down-tilt angle | etilt | Degrees |
| Electrical scan angle | escan | Degrees |

The parameterized antenna model is built around array antenna model where the element factor, array factor and linear phase progressing is characterized as described by equations in Table 8.1.1-2.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πn-1dvλcosθ+m-1dhλsinθsinφ wm,n=1MNexpj2πn-1dvλsinθetilt-m-1dhλcosθetiltsinφescan | dBi |

### 8.1.2		Array antenna parameters

In Table 8.1.2-1, base station array antenna parameters for different deployment scenarios is listed. Element parameters have been selected to produce correct element peak gain.

Table 8.1.2-1: BS array antenna parameters

| Parameter | Macro Sub-urban | Macro Urban | Micro Urban |
| --- | --- | --- | --- |
| Am (dB) | 30 | 30 | 30 |
| SLAv (dB) | 30 | 30 | 30 |
| 3dB (deg.) | 90 | 90 | 90 |
| 3dB (deg.) | 65 | 90 | 90 |
| GE,max (dBi) | 6.4 | 5.5 | 5.5 |
| LE (dB) | 2.0 | 2.0 | 2.0 |
| (M, N) | (16, 8) | (16, 8) | (8,8) |
| dh (m) | 0.5 | 0.5 | 0.5 |
| dv (m) | 0.7 | 0.5 | 0.5 |
| Horizontal coverage range (deg.) | +/- 60 | +/- 60 | +/- 60 |
| Vertical coverage range (deg.) | 90 to 100 | 90 to 120 | 90 to 120 |
| Total Radiated Power (dBm) | 46 | 46 | [41 or 37] |
| Mechanical downtilt (deg.) | 6 | 10 | N/A |
| Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: TRP includes the powers from two polarizations |  |  |  |