3GPP TSG-RAN WG4 Meeting #96-e                                                R4-2010215

# Electronic Meeting, 17-28 Aug., 2020

Agenda Item:	7.1.5.10

Source:	MediaTek Inc.

Title:	Discussion on measurement requirements for NR-U

Document for:		Discussion

# Introduction

In this paper, we provide our view on measurement requirements for NR-U, including

- UE monitoring capabilities for QCLed SSBs

- Scheduling restriction for SS-RSRP, SS-RSRQ, SS-SINR

- RSSI measurements: the definition of intra-frequency and inter-frequency RSSI measurement, and RSSI measurement period, scheduling restriction for RSSI

# UE monitoring capabilities for QCLed SSBs

This section also has been discussed in our companion paper [3]. In the previous meeting, RAN4 sent LS [1] to RAN1 about whether monitoring all candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index is mandatory for UEs. RAN1’s reply LS regarding the monitoring capability [2] is as follows.

RAN1 would like to thank RAN4 for their LS [1] related to NR-U SSB monitoring capabilities.

Related to the four questions asked by RAN4, RAN1 feedback is as follows.

[Question 1] Provide feedback whether monitoring within a given discovery burst transmission window all candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index is mandatory for UEs.

[RAN1 answer] During RAN1 discussion, we did not reach consensus on how to set N1 and N2 values. However, it is RAN1 understanding that RAN4 may choose not to define different RLM/RRM performance requirements corresponding to different N1/N2 capabilities. Hence, assuming a single RLM/RRM performance requirement, the introduction of N1/N2 UE capabilities is not necessary. It is RAN1 understanding that how many candidate SS/PBCH block indexes corresponding to the same SS/PBCH block index the UE should monitor in a given discovery burst transmission window can be left as UE implementation, as long as the single RLM/RRM performance requirement is met.

As a consequence, RAN1 has agreed that from RAN1 perspective, N1 and N2 should not be defined as UE capabilities.

Observation 1: UE is not mandated to monitoring all candidate SBIs corresponding to the same SBI, within a given discovery burst transmission window.

Observation 2: It is RAN1 understanding that how many candidate SBIs corresponding to the same SBI the UE should monitor in a given discovery burst transmission window can be left as UE implementation, as long as the single RLM/RRM performance requirement is met.

In many RAN4 NR-U RRM requirements, the evaluation/measurement period depends on the number of “not available SSBs”. However, the observed number of “not available SSBs” will be impacted by the different UE implementations. For examples, the UEs monitoring 1 position (candidate SBI) will observe larger number of “not available SSBs” than the UEs monitoring N>1 positions. Currently, there is no capability signaling for the number of positions to monitor, so this capability is unknown to network. As a result, UE and network would have different understanding on the time period in requirements.

To void this misunderstanding, the requirement can be specified based on certain predefined numbers, without introducing the variable N1/N2 UE capabilities.

For example, one option is to define “available SSB” based on the assumption that UE is capable to monitor at least one candidate SBI corresponding to the same SBI within the discovery burst transmission window. In this way, the UEs monitoring 1 or more positions can meet the requirement.

Proposal 1: In a given discovery burst transmission window, UE is required to monitor at least one candidate SBI corresponding to the same SBI.

Since the observed number of “not available SSBs” will be impacted by this assumption, this clarification should be captured in the terminology for unavailable SSB/SMTC occasions.

Proposal 2: Capture the number of candidate SBIs corresponding to the same SBI UE should monitor to in the terminology for unavailable SSB/SMTC occasions.

# Scheduling restriction during SS-RSRP, SS-SINR, and SS-RSRQ measurement

In the last meeting, the following agreements regarding the scheduling restriction in NR-U have been agreed [4]:

- RAN4 to define scheduling restrictions during SS-RSRP, SS-SINR and SS-RSRQ measurements in NR-U

- In NR-U, scheduling restriction should depend on the signaling of deriveSSB_IndexFromCell.

- In NR-U, the scheduling restriction of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH (clause 9.2.5.3.2 in TS 38.133) is applicable.

  - FFS: scheduling restriction to intra-band and inter-band CA.

Similar to R15, in FR1, the scheduling restriction due to one CC shall not apply to other CCs on the other bands.

Proposal 3: In FR1 inter-band CA, the scheduling restriction due to one CC shall not apply to other CCs on the other bands.

There are 2 reasons to apply scheduling restriction in FR1 in R15

- Due to UL/DL collision, as captured in 9.2.5.3.1 “Scheduling availability of UE performing measurements in TDD bands on FR1” in 38.133.

- Due to different subcarrier spacing, as captured in 9.2.5.3.2 “Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1”

Regarding UL/DL collision, the case of “deriveSSB_IndexFromCell is not enabled” was not considered in R15 TDD bands. However, in NR-U, the asynchronous deployment has been considered and it has agreed the scheduling restriction should depend on the signaling of deriveSSB_IndexFromCell, the scheduling restriction in analogue to R15 requirement is proposed as follows.

Proposal 4: If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within DRS window duration.

# RSSI measurement

# 4-1 Intra-frequency and inter-frequency RSSI definition

Regarding RSSI intra-frequency and inter-frequency definition, the following options are for further discussion [4].

- Issue 3-1-1: Intra-frequency and inter-frequency RSSI definition

- An intra-frequency RSSI measurement is defined when:

  - RSSI channel BW is contained within the channel/carrier BW of the UE.

- Further study whether to include SCS conditions into the definition and how to handle RSSI measurements under assumption of different SCS in RSSI, active BWP, etc.

- Options discussed in RAN4 95:

  - Option 2a:  RMTC configured SCS is the same as the SCS of active BWP

  - Option 2b: the SCS of the RSSI measurement is the same as the SCS of an intra-frequency SSB or CSI-RS

  - Option 2c: No additional condition is needed.

  - Option 2d: the SCS configured for the RSSI measurement is the same as the SCS of a serving cell, where the SCS of a serving cell is FFS.

The conditioning on SCS is necessary as it would introduce measurement duration of fractional symbols. We support Option 2a. Regarding the Option 2b, although the SCS are the same for RSSI and inter-freq. SSB, if the RSSI BW is larger than SSB BW and the RMTC SCS is different from the BWP, the gap will still be required.

Proposal 5: An intra-frequency RSSI measurement shall meet the condition that the RMTC configured SCS is the same as the SCS of active BWP (Option 2a).

# 4-2 RSSI measurement period

The CSSF definition for RSSI intra-/inter-frequency measurement has been agreed as follows [4].

- Issue 3-2-1: CSSF definition

  - At least for CSSF within measurement gaps, CSSF needs to be adapted for NR-U to account for RSSI measurements in RMTC in addition to other NR-U measurements in SMTC.

  - FFS: whether CSSF needs to be adapted for CSSF outside measurement gaps.

Regarding the CSSF outside gap, it should further discuss whether to allow the case that RMTC is overlapping with SMTC. If the overlapping is not allowed, then there is no sharing issue between RMTC and SMTC, i.e. CSSF outside gap is not necessary for RSSI measurements.

Proposal 6: For a RSSI measurement outside measurement gap, measurement period is scaled up by CSSFoutside_gap, if RMTC is allowed to be overlapped with SMTC. CSSFoutside_gap is not required if RMTC is not allowed to be overlapped with SMTC

It can be observed that if RMTC and SMTC are non-overlapping in time then the load/interference can be more accurately measured. Besides, if it excludes the overlapping case, the measurement period can be simplified, as the sharing between SMTC and RMTC is not necessary.

Proposal 7: No requirement for the RMTC which is overlapping with SMTC.

# 4-3 Scheduling restriction due to RSSI measurement

Similar to the discussion in section 3, the async deployment has to be considered. Thus the scheduling restriction to avoid UL/DL collision is proposed as below:

Proposal 8: If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within RMTC window duration.

As discussed in the definition of intra-frequency RSSI measurement, if the SCS configured in RMTC is higher than the SCS of data, then it will lead to measurement duration of fractional data-symbols. Thus, the scheduling restriction shall be applied the measurement duration of fractional data-symbols

Proposal 9: Scheduling restriction shall apply on the RMTC window duration if the SCS configured in RMTC is different from the SCS of data.

# Summary

In this paper, measurement requirements for NR-U have been discussed. We have the following observations and proposals:

Proposal 1: In a given discovery burst transmission window, UE is required to monitor at least one candidate SBI corresponding to the same SBI.

Proposal 2: Capture the number of candidate SBIs corresponding to the same SBI UE should monitor to in the terminology for unavailable SSB/SMTC occasions.

Proposal 3: In FR1 inter-band CA, the scheduling restriction due to one CC shall not apply to other CCs on the other bands.

Proposal 4: If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within DRS window duration.

Proposal 5: An intra-frequency RSSI measurement shall meet the condition that the RMTC configured SCS is the same as the SCS of active BWP (Option 2a).

Proposal 6: For a RSSI measurement outside measurement gap, measurement period is scaled up by CSSFoutside_gap

Proposal 7: No requirement for the RMTC which is overlapping with SMTC.

Proposal 8: If deriveSSB_IndexFromCell is not enabled the UE is not expected to transmit PUCCH/PUSCH/SRS on all symbols within RMTC window duration.

Proposal 9: Scheduling restriction shall apply on the RMTC window duration if the SCS configured in RMTC is different from the SCS of data.

# References

[1] R1-2003274/R4-2005418, “LS on NR-U SSB monitoring capabilities”, Nokia, RAN4

[2] R1-2004992, Reply LS on NR-U SSB monitoring capabilities, Nokia.

[3] R4-2010214, Discussion on RLM requirement for NR-U, MediaTek Inc.

[4] R4-2009249 WF on NR-U RRM (Part 3), Nokia.