3GPP RAN WG4 Meeting #96e		R4-2011532

Electronic meeting, 17th – 28th August 2020

Agenda item:	9.3.2

Source:	Apple Inc, Comcast

Title:	LTE/NR spectrum sharing in band 48/n48 frequency range

WI/SI:	NR_n48_LTE_48_coex-Core

Release:	Rel-16

Document for:	Decision

# 1	Introduction

Dynamic spectrum sharing (DSS) is an important feature that allows for sharing existing spectrum between the LTE and NR carriers, thus enabling smoother transition from LTE and faster adoption of NR. There are several prerequisites for sharing LTE and NR within the same frequency band, such as aligned sub-carrier grids and no overlaps between the LTE reference signals and the NR synchronisation blocks. As an example, the following WI was agreed to enable dynamic spectrum sharing in band 41/n41 frequency range [1].

After the RAN#86 meeting, a new WI was agreed [2] aiming to analyse and introduce, if needed, changes to support dynamic spectrum sharing in band 48/n48 frequency range. This WI was further revised during the RAN#88 meeting accounting for the fact that the CBRS Alliance agreed to add 15kHz SCS as one of the mandatory options [3]. From the agreed WI perspective, the following issues have to be addressed: channel raster, UL shift, synchronisation pattern. In this discussion paper we elaborate further on each of the aforementioned points suggesting further changes to band n48 to enable DSS between LTE and NR.

# 2	Channel raster for spectrum sharing in band 48/n48

## 2.1	Background

To align LTE and NR carrier, both carriers have to be configured with the same centre frequencies, which in turn should be an integer multiple of the channel raster step of the corresponding LTE and NR bands. While the LTE band 48 has a channel raster of 100kHz, the corresponding NR band n48 follows so-called SCS based channel raster, which is either 15 or 30kHz (refer also to Annex B). Based on that, one of the solutions not requiring any standardization changes would be to consider FREF values at step of 300kHz for those carriers where DSS operation between LTE and NR is anticipated. For other carriers, using exclusively either LTE or NR, there will be no restrictions on how they can be configured.

Observation 1a:	300kHz raster does not require any further standardization changes and thus can be used to align LTE and NR centre frequencies for those carriers where DSS operation is needed.

Even though the solution with FREF values at step of 300kHz does not require any further standardization changes in 3GPP, it is worth noting that unlike other licensed bands where an operator receives a particular fixed chunk of spectrum, band 48/n48 spectrum is managed by so-called SAS entity. It allocates spectrum to operators accounting for their requests, total number of operators working in a certain geographical area, and also other SAS entities. In other words, an operator simply does not know in advance which spectrum will be allocated, its size and the centre frequency.

Figure 2.1-1: Overview of the CBRS band (3GPP band 48/n48).

Figure 2.1-1 illustrates that the CBRS band 3550-3650 MHz frequency range is logically partitioned into ten 10MHz channels to ease spectrum allocation and management process by the SAS entity and also between the SAS entities. Referring back to Observation 1a, even though a solution based on the 300kHz raster could be theoretically possible, it becomes almost unrealistic in the real-life deployments because not all the 10MHz channels are on the 300kHz raster, and some channels could be simply busy by incumbent services. Figure 2.1-2 illustrates further this problem by showing exemplary 10MHz and 20MHz allocations within 100MHz spectrum allocation of the band 48/n48. As can be seen from the top-most figure, there are only certain channels, which centre frequencies are aligned on the 300kHz raster (highlighted in red for the sake of simplicity). As an example, the following centre frequencies are aligned on the 300kHz raster – 3555, 3585, 3615, 3645 – and thus can be used for DSS between LTE and NR. However, as can be seen from the figure, there are only four 10MHz channels in 100MHz range that are aligned on the 300kHz raster. The problem gets even worse for 20MHz channels because there is only one channel with centre frequency 3600 that is aligned on the 300kHz raster.

Figure 2.1-2: Exemplary 10MHz and 20MHz channel allocations with centre frequencies.

Even though one could anticipate that the SAS entity might take care of allocating spectrum in a preferred way, there is no guarantee that the corresponding channel will be free, i.e. not taken by another SAS entity or not allocated to another operator. In other words, the preferred channel still could be busy irrespective of the operator’s preference to get a particular channel.  Another potential problem is when a particular carrier initially uses only LTE with 100kHz raster, but then an operator decides to enable DSS thus requiring 300kHz raster alignment. In this case, the corresponding carrier centre frequency will have to be re-allocated potentially impacting other operators’ centre frequencies in the whole band 48/n48 spectrum range. As an example, 40MHz chunk of spectrum can be easily and fairly shared between four operators with 100kHz channel raster when only LTE is used, i.e. each operator will get 10MHz. However, once an operator decides to use NR and DSS requiring the channel raster to be an integer multiple of 300kHz, it might be impossible to accommodate four 10MHz channels without violating minimum guard band sizes.

Observation 1b:	Band 48/n48 spectrum is managed by the SAS entity, and thus an operator cannot be sure that allocated spectrum will be on the 300kHz raster.

## 2.2	Applying -/+100kHz shift to the centre frequency

### 2.2.1	Overview of solution

One of the approaches to align LTE and NR sub-carriers grids is to apply when needed -/+100kHz shift to the centre frequency. If the spectrum range allocated by SAS is not on the 300kHz raster, then the channel centre frequency can be shifted right or left, as exemplified on Figure 2.2-1 and 2.2-2. If the allocated spectrum is on the 300kHz raster, e.g. 3550 – 3560 MHz, then no shift would be needed. If an operator receives spectrum which is not on the 300kHz raster, e.g. 3560 – 3570 MHz, then it is possible to shift the centre frequency to the closest 300kHz raster which in this case would be 3564.9MHz. It is worth noting that shifting the centre frequency, when and if needed, will be decided by gNB and does not impact the SAS entity.

Figure 2.2-1: Applying -/+100kHz shift to the centre frequency (10MHz@30kHz SCS).

Figure 2.2-2: Applying -/+100kHz shift to the centre frequency (10MHz@15kHz SCS).

As already mentioned earlier, applying -/+100kHz shift will be needed only to those channels, centre frequencies of which is not on the 300kHz raster, i.e. is not on the common raster between LTE 100kHz raster and the NR 15/30kHz SCS based raster. The table below presents several band 48/n48 10MHz channels, indicating when the shift would be applied, and which LTE ARFCN and NR ARFCN values would be used.  As an example, centre frequencies 3555MHz and 3585MHz are aligned on the 300kHz raster and thus there is no need to shift it by -/+100kHz. On the contrary to it, centre frequency 3565MHz is not aligned on the 300kHz raster, and the closest one is 3564.9MHz. The centre frequency 3575MHz is not aligned in the 300kHz raster either and the closest one would be 3575.1MHz.

Observation 1c:	If the allocated frequency range is not strictly aligned on the 300kHz raster, then it is possible to shift the centre frequency by -/+100kHz to the closest NR ARFCN raster point.

Observation 1d:	The centre frequency shift will be applied by gNB, which does not require any changes in RAN WG4; and it does not impact SAS operation.

Table 2.2-3: Exemplary 10MHz channel centre frequencies aligned on the common LTE and NR raster

| Channel (MHz) | Centre frequency (MHz) | LTE ARFCN | NR ARFCN | MOD (300kHz) |
| --- | --- | --- | --- | --- |
| 3550-3560 | 3554.9 | 55289 | x | 2 |
|  | 3555 | 55290 | 637000 | 0 |
|  | 3555.1 | 55291 | x | 1 |
| 3560-3570 | 3564.9 | 55389 | 637660 | 0 |
|  | 3565 | 55390 | x | 1 |
|  | 3565.1 | 55391 | x | 2 |
| 3570-3580 | 3574.9 | 55489 | x | 1 |
|  | 3575 | 55490 | x | 2 |
|  | 3575.1 | 55491 | 638340 | 0 |
| 3580-3590 | 3584.9 | 55589 | x | 2 |
|  | 3585 | 55590 | 639000 | 0 |
|  | 3585.1 | 55591 | x | 1 |

While it seems like a feasible solution not requiring any further specification changes, it bears noting that by shifting centre frequency by -/+100kHz we will impact guard bands effectively reducing their size. Even worse, shrunk guard bands will not meet the minimum requirements, as specified in TS 38.101-1. Referring back to Figure 2.2-1 and Figure 2.2-2, the minimum guard band for 10MHz@30kHz SCS channel is 665kHz, while after shifting the centre frequency it will become 580kHz. And the problem becomes even more severe for smaller channel bandwidth and 15kHz SCS as show in Figure 2.2-2, in which the guard band will become 220kHz which is much smaller than required minimum of 312.5kHz. Thus, there is no guarantee that regulatory emission requirements will be met.

Observation 1e:	Shifting the centre frequency -/+100kHz will impact guard bands, and thus there is no guarantee that emission requirements still can be met.

Table 2.2-4: Minimum guardband for each UE channel bandwidth and SCS (kHz) (Table 5.3.3-1 from TS 38.101-1)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 552.5 | 692.5 | N/A | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 905 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | 1310 | 1290 | 1610 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

To understand the severity of the problem caused by shifting the centre frequency, we conduct a series of simulations. We analyse the required MPR needed in a normal case with standard guard bands and when the centre frequency is shifted resulting in a smaller guard band, and for which because of that the larger power reduction might be needed. The following assumptions have been made for this analysis:

-	the UE maximum power is 23dBm;

-	we focus on 5-20MHz channel bandwidth at both 15kHz and 30kHz SCS because they are more vulnerable to centre frequency shift due to smaller guard bands;

-	since band n48 has a special NS_27 value that enforces a more stringent spectral emission mask, we present results separately for NS_01 and NS_27;

-	since NS_27 activates more stringent emission requirements for frequency ranges outside band n48, we simulate separate NS_27 "lower" and NS_27 "inner" cases; while NS_27 "inner" always has the centre frequency in the middle of the band n48, NS_27 "lower" has the centre frequency so that the transmission channel is right at the band edge;

-	as in similar methodologies, we present results for edge, outer, and inner RB.

Table 2.2-5 and 2.2-6 below present a summary in a form of absolute power back-off in dB, i.e. the one that has to be applied to meet emission requirements when the centre frequency is shifted. Detailed results can be found in Appendix C-E, in which for every simulation case we present separately figures with the absolute power back-off values needed to meet emission requirements at the RB level granularity.

The key points of the simulation results can be summarised as follows:

-	The required power back-off (without shift and shift) varies depending on the transmission channel and guard bands;

-	As anticipated, the required power back-off is higher for scenarios with smaller channel bandwidth and/or smaller SCS. While this difference is noticeable for 5-10MHz@15kHz SCS, it becomes quite marginal already for 20MHz@15kHz SCS;

-	DFT-s-OFDM requires smaller power back-off when compared to CP-OFDM;

-	Due to more stringent requirements associated with NS_27, a case when the transmission channel is right at the band edge, NS_27 (lower), requires higher power back-off;

-	Edge RB allocations typically require larger power back-off only with small channel bandwidth when guard bands are small;

-	As the transmission channel become larger, it is the outer RB allocations that will require additional power back-off whereas edge RB allocations might need no power reduction at all;

-	Almost in all cases of CP-OFDM, larger channel bandwidth might require power back-off even for inner allocations;

-	It is important to emphasize the required power back-off with the centre frequency shift is still within existing MPR margins allowed for PC3, i.e. we do not need to specify additional power back-off on top of what TS 38.101-1 specifies in Table 6.2.2-1 (see also Annex G);

-	For CP-OFDM results, all the inner allocations are within 1.5dB MPR limits and edge allocations are within 3dB MPR limits even when the shift as applied. The reason why outer allocations require slightly more than 3dB MPR in our simulations is because we use a conservative PA model. As for the NS_27 lower results and 20MHz channel, TS 38.101-1 defines MPR of 6dB for channel bandwidth larger than 10MHz, i.e. even when the 20MHz channel centre frequency is shifted, the MPR results are still within the 6dB margin;

-	For DFT-s- OFDM, all the inner allocations are within 0dB MPR margin for 5-10MHz channel, and same applies to the edge allocations which are within the 1dB MPR margin. Only outer allocations require slightly higher MPR, but similarly to CP-OFDM this is due the conservative PA model and simulation impairments.

Table 2.2-5: Required power back-off without and with the centre frequency shift (CP-OFDM).

| Channel@SCS | RB Allocation | Required power back-off [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NS_01 |  | NS_27 (inner) |  | NS_27 (lower) |  |
|  |  | no shift | shift | no shift | shift | no shift | shift |
| 5MHz@15kHz | Edge | 0.2 | 2.6 | 0.4 | 2.3 | 0.2 | 2.2 |
|  | Inner | 0 | 0.3 | 0 | 0.4 | 0 | 0.2 |
|  | Outer | 3.3 | 3.4 | 3.3 | 3.3 | 3.2 | 3.4 |
| 10MHz@15kHz | Edge | 0 | 1.3 | 0 | 1.3 | 0 | 1.2 |
|  | Inner | 0 | 0.4 | 0 | 0.4 | 0 | 0.1 |
|  | Outer | 3.4 | 3.5 | 3.4 | 3.5 | 3.4 | 3.5 |
| 20MHz@15kHz | Edge | 0 | 0 | 0 | 0 | 2.2 | 2.2 |
|  | Inner | 0 | 0.1 | 0 | 0.1 | 2.3 | 2.3 |
|  | Outer | 3.3 | 3.4 | 3.3 | 3.4 | 6 | 6 |
| 10MHz@30kHz | Edge | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Outer | 3 | 3.2 | 3.2 | 3.4 | 3.2 | 3.3 |
| 20MHz@30kHz | Edge | 0 | 0 | 0 | 0 | 2.1 | 2.2 |
|  | Inner | 0 | 0 | 0 | 0 | 2.2 | 2.2 |
|  | Outer | 3.3 | 3.3 | 3.3 | 3.3 | 5.7 | 5.7 |

Table 2.2-6: Required power back-off without and with the centre frequency shift (DFT-s-OFDM).

| Channel@SCS | RB Allocation | Required power back-off [dB] |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | NS_01 |  | NS_27 (inner) |  | NS_27 (lower) |  |
|  |  | no shift | shift | no shift | shift | no shift | shift |
| 5MHz@15kHz | Edge | 0 | 1.1 | 0 | 1 | 0 | 0.9 |
|  | Inner | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Outer | 1.1 | 1.3 | 1.2 | 1.3 | 1.2 | 1.4 |
| 10MHz@15kHz | Edge | 0 | 0.6 | 0 | 0.7 | 0 | 0.6 |
|  | Inner | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Outer | 1.4 | 1.5 | 1.4 | 1.4 | 1.4 | 1.5 |
| 20MHz@15kHz | Edge | 0 | 0 | 0 | 0 | 1.6 | 1.6 |
|  | Inner | 0 | 0 | 0 | 0 | 1.7 | 1.7 |
|  | Outer | 1.6 | 1.6 | 1.6 | 1.6 | 4.5 | 4.5 |
| 10MHz@30kHz | Edge | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0 | 0 | 0 | 0 |
|  | Outer | 1.2 | 1.3 | 1.2 | 1.1 | 1.1 | 1.2 |
| 20MHz@30kHz | Edge | 0 | 0 | 0 | 0 | 1.6 | 1.6 |
|  | Inner | 0 | 0 | 0 | 0 | 1.6 | 1.6 |
|  | Outer | 1.2 | 1.3 | 1.2 | 1.3 | 4.2 | 4.3 |

Observation 1f:	The required power back-off is increased for certain allocations when the centre frequency is shifted by -/+100kHz, but the maximum required power back-off for inner or outer region can still be considered to be inside the PC3 MPR.

Observation 1g:	Blanking of edge RBs is not sufficient because it is not only edge, but also inner and outer allocations that need some power back-off, especially for small transmission channel bandwidth and/or sub-carrier spacing.

One of concerns raised during the discussions is that shifting the centre frequency and applying additional power back-off will lead to tightening of MPR requirements and/or worsen UL performance. As explained above, additional power back-off that a UE needs to apply is still within existing MPR limits. As for worsen UL performance, accounting for the fact that commercial band n48 deployments will broadcast NS_27 flag, the largest MPR "degradation" observed in our simulations is 1.9dB for 5MHz@15kHz SCS and 1.3dB for 10MHz@15kHz SCS (refer to Annex F that presents tables with delta MPR values). However, this will happen only for those UEs which: a) are e.g. at the cell edge and, b) which are transmitting at the maximum power, and c) which are scheduled by the network with edge/outer RBs. Since the network knows when the centre frequency is applied, it can always schedule cell edge UEs so that its extra power back-off remains at the reasonably low level.

Proposal 1a:	Adopt a solution based on the -/+100kHz shift of the centre frequency.

### 2.2.2	Specification impact

Shifting the centre frequency by -/+100kHz does not require any big changes into the existing design and can be adopted easily with quite marginal specification changes impacting only RAN WG4. As already commented earlier, since we align LTE and NR sub-carrier grids by finding the closest raster point that is common for both LTE and NR, no further changes are need with regards to the raster design. It is effectively up to the operator preference and the base station implementation which centre frequency should be chosen.

As for the required power back-off to meet all the emission requirements when the guard bands are smaller, we can tackle separately DL and UL directions.

-	DL direction. In general, we can assume that it can be left up to the base station implementation how to handle shrunk guard bands, especially accounting for the fact that the base station controls and knows whether the guard bands are smaller or not. It is also commonly the case that base stations have better filtering techniques and thus no power back-off maybe be needed at all. And if needed, it will be up to the base station design how much it will be.

-	UL direction. As for the UL transmission, our preliminary understanding that a UE has to be informed whether guard bands are smaller or not because the UE does not know it, and thus cannot make a decision when to apply power back-off. Of course, one can devise some rule to "guess" that the centre frequency is shifted, but this is not reliable and might put further restrictions on deployment scenarios. As one particular example, SAS may allocate 20MHz to the operator, e.g. 3550-3570MHz, and if the operator decides to use DSS in 3560-3570MHz frequency range with the shifted centre frequency of 3564.9MHz, then still no power back-off could be applied as higher emissions will impact another carrier that belongs to the same operator. Based on that we anticipate some impact to TS 38.101-1.

Proposal 1b:	Introduce signalling to indicate explicitly to the UE that the centre frequency is shifted and thus the guard bands are smaller.

What should be discussed further in RAN WG4 is whether it is sufficient to have just an indication to the UE that the guard bands are smaller or whether we should also standardise maximum allowable power back-off values similar to the ones presented in Table 2.2-5 and 2.2-6. As explained earlier, even though shrunk guard bands require extra power back-off, it is still within existing MPR margins i.e. we do not need to introduce any A-MPR. From that perspective it can be left to UE implementation to decide by how much it has to back-off its power to meet all the requirements with shrunk guard bands because it will anyway be within the existing limits allowed by the specification.

Proposal 1c:	Since extra power back-off needed for shrunk guard bands is still within existing MPR limits, it can be left up to the UE implementation.

If for any reason RAN WG4 concludes that it is beneficial to standardise those values, we can accommodate power back-off values based on the following principles:

-	We round up back-off values in 0.5dB step;

-	Since we round up values, we do not differentiate between CP-OFDM and DFT-s-OFDM results considering only CP-OFDM as it has higher back-off values;

-	Since it is anticipated that NS_27 will be always set in cells operating in band n48, we consider only NS_27 results;

-	For the sake of simplicity, we do not differentiate between NS_27 for band n48 edge and inner allocations i.e. we consider the highest back-off value amongst them;

Table 2.2-7: Exemplary allowed power back-off needed to compensate for shrunk guard band

| Channel@SCS | RB Allocation |  |
| --- | --- | --- |
| 5MHz@15kHz | Edge | 2 |
|  | Inner | 0.5 |
|  | Outer | 1.5 |
| 10MHz@15kHz | Edge | 1.5 |
|  | Inner | 0.5 |
|  | Outer | 1.5 |
| 20MHz@15kHz | Edge | 0.5 |
|  | Inner | 1 |
|  | Outer | 1 |
| 10MHz@30kHz | Edge | 0 |
|  | Inner | 0 |
|  | Outer | 1 |
| 20MHz@30kHz | Edge | 0.5 |
|  | Inner | 0.5 |
|  | Outer | 1 |

# 3	UL shift for LTE/NR spectrum sharing in band 48/n48

## 3.1	Background

As already mentioned in section 2, one of the prerequisites for dynamic spectrum sharing between the LTE and NR carriers is the system ability to align sub-carrier grids of the LTE and NR systems. While DL grid alignment is ensured with the common raster points, UL sub-carrier grid alignment requires an additional higher layer parameter was added to the system information to indicate UL channel raster shift. As can be seen from Figure 3-1, having a common and aligned sub-carrier grid, the base station can decide flexibly how many RBs should be allocated for LTE or NR UEs. And if both LTE and NR use 15kHz SCS, then no inter-numerology guard-band is needed.

Figure 3-1: Exemplary sub-carrier grid with 15kHz LTE and 15kHz NR RBs.

However, as follows from TR 38.822 and TS 38.101-1, this parameter is mandatory only for the FDD and SUL bands. In other words, a UE camped on the TDD bands is not anticipated to act on the corresponding parameter if it is signalled by the network. The problem was firstly identified for bands 41/n41 for LTE/NR spectrum sharing WI [1] and RAN WG4 agreed to introduce a new band to circumvent around the fact that channel raster shift is not a mandatory parameter for the NR TDD bands and to introduce new channel raster values. Referring to Table 5.2-1 from TS 38.101-1, band n90 was introduced which is effectively the same band as n41, but the network deploying LTE and NR on the same carrier will use band n90 to prevent legacy terminals from camping on that carrier. In turn, a UE supporting band n90 has to support UL channel raster shift parameter.

Referring to the latest agreements from the CBRS Alliance, both 15kHz and 30kHz SCS are the mandatory supported configuration. In other words, at least 15kHz SCS is a straightforward use case and configuration for dynamic spectrum sharing between the LTE and NR carriers. As for 30kHz SCS, according to RAN WG4 specifications, there are no particular requirements for the mixed-numerology deployments, and as Figure 3-2 presents, a particular network implementation may follow different strategies. If the network uses the common sub-carrier grid spanning over LTE and NR allocations, then it would be quite easy to ensure that inter-numerology guard band is either 15kHz or 30kHz SCS RB, or even zero RBs. The latter case is also feasible because even though 15kHz and 30kHz numerologies are not orthogonal, it will just cause additional emission to the neighbouring RB. And since the scheduling is done by the base station, the latter will be aware of it. As an example, if LTE and NR transmissions do not take 100% of UL resources, then it is quite natural to make an inter-numerology guard-band as it comes for free. For the 100% load scenario, it can be up to the network implementation and scheduling whether to schedule all RBs catering for better spectrum utilization (e.g. best effort service) or avoid unnecessary emissions to the neighbouring RB.

Figure 3-2: Exemplary sub-carrier grid for 15kHz LTE and 30kHz NR RBs.

Thus, enabling UL shift on band n48 will allow sub-carrier grid alignment which in turn can be used by the network for better resource utilization. And even though it can be viewed as a marginal optimization for 30kHz SCS and large carrier sizes, it will be a more noticeable improvement for 5 or 10MHz carriers which can be allocated by the SAS entity, especially accounting for the fact that 30kHz SCS RB will provide worse spectrum utilization than 15kHz SCS.

Observation 2a:	UL shift is needed for 15kHz SCS deployments to align UL sub-carrier grids between LTE and NR required for dynamic spectrum sharing.

Observation 2b:	UL shift can be viewed as a nonessential feature for 30kHz SCS if some inter-numerology guard band is always used by the network, but still can help to achieve better resource utilisation.

## 3.2	Potential solutions

As a summary of our observations, our view is that it is possible to contemplate further two major approaches: a) UL shift if a mandatory parameter for 15kHz SCS and is not supported for 30kHz SCS; or b) UL shift is a mandatory parameter for 15kHz SCS and can be optionally supported by a UE for 30kHz SCS.

a)	UL shift is a mandatory parameter for 15kHz SCS and is not supported at all for 30kHz SCS. UL shift can be considered as a mandatory feature at least for 15kHz SCS. Following the same considerations as for other FDD bands and TDD band 41/n41, enabling UL shift will help DSS deployments for narrow channels that can be allocated by the SAS entity and for which an operator will apply 15kHz SCS. With this approach we also assume that UL shift is not supported at all for 30kHz SCS.

b)	UL shift is a mandatory parameter for 15kHz SCS and can be optionally supported for 30kHz SCS. This option is similar to option a), but at the same time we can keep UL shift as an optional feature for 30kHz SCS. The only downside of this approach is that it is not entirely clear what the overall system behaviour will be in a situation when the network enables UL shift for 30kHz SCS, but a UE does not support it. On the one hand, there is nothing that prevents a UE from camping on that cell; but it does not make much sense either because UL grid will be shifted. The most straightforward solution would be to prevent a UE from camping on the cell if its system information indicates UL shift and a UE does not support it. However, this behaviour is not captured now in RAN WG2 specifications, i.e. there is no rule for the UE side to consider the cell as barred if a UE does not support UL shift, but the network activates it.

Referring back to the approaches a) and b), one can see that they have a common denominator. Irrespective of the fact whether UL shift is optional for 30kHz SCS or mandated only for 15kHz SCS, it is better to clarify UE behaviour for the case when a UE camps on the cell, configuration of which enables UL shift for the given SCS, but a UE does not support.

Proposal 2a:	Introduce UL shift as a mandatory UE feature at least for 15kHz SCS (on band n48).

Proposal 2b:	Ask RAN WG2 to clarify UE behaviour when a UE camps on the cell with enabled UL shift, but a UE does not support UL shift functionality for a particular SCS.

# 4	Sync pattern for spectrum sharing in band 48/n48

## 4.1	Background

As discussed during the previous RAN4 meetings, there can be overlaps between the LTE CRS and NR SSB transmissions. As can be seen from the Figure 4-1 below, existing band n48 synchronization pattern C can potentially work for 1 and 2 port LTE transmission because the corresponding NR SSB can be transmitted in LTE OFDM symbols #1-2 and #8-9, even though there might be some performance impact because NR SSB still collides with LTE CRS in symbols #4 and #11. However, if LTE uses 4 antenna port transmission, then all the sync pattern C NR SSB transmission opportunities overlap with the LTE CRS symbols. One solution to this problem is to introduce sync pattern B for band n48. Referring to the same Figure 4-1, the sync pattern B NR SSB can be transmitted in LTE OFDM symbols #2-3 even if the 4-port LTE transmission is used.

Observation 3a:	NR sync pattern C can work with 1-2 port LTE deployments, but 4-port LTE CRS transmission will always collide with NR SSB.

Figure 4-1: LTE CRS and NR SSB transmissions for 1ms sub-frame.

It also worth noting that 4-port LTE transmission is a widely deployed feature, which is also commonly supported by different UE vendors. Figure 4-2 below shows MIMO activation statistics gathered from three major US carriers for the LTE 4x4 MIMO capable devices. The statistics is based on total number of 2x2 and 4x4 MIMO activations for the period of one month, which was collected from 4x4 MIMO capable UEs and divided by the number of UEs, i.e. it can be construed as average number of LTE MIMO activations per UE. As can be seen from the picture, in all the major networks LTE 4x4 MIMO is being aggressively activated (and reasons for only 2x2 MIMO activation are multi-fold, e.g. coverage, site support, etc). It is also worth noting that since every operator has a different number of UEs, deployments, and MIMO activation strategies, the figure naturally shows different average number of MIMO activations per a UE. Nevertheless, the relative ratio between 2x2 and 4x4 MIMO is very close across all the operators showing importance of the 4-port LTE transmission modes.

Observation 3b:	4-port LTE transmission modes are widely supported by UEs and used in commercial deployments.

Figure 4-2: Average number of LTE MIMO activations.

Furthermore, since band n48 spectrum is managed dynamically by the SAS entity, an operator cannot be sure that he will have a large channel bandwidth; and in certain cases, the allocated spectrum can be rather small. In this case 4-port LTE transmission becomes quite a crucial feature allowing for better spectrum utilisation of scarce radio resources.

Observation 3c:	Since the band n48 spectrum is managed by the SAS entity and allocated channels can be rather small, the 4-port LTE transmission feature becomes even more important allowing to achieve better spectrum utilisation.

## 4.2	Potential solutions

One of the potential solutions to avoid LTE CRS and NR SSB overlaps is to leverage LTE MBSFN sub-frames so that LTE CRS can be muted in those sub-frames where NR SSB is expected. However, the problem is that LTE MBSFN configuration allows to mute only specific sub-frames, which are not the ones where NR SSB can be sent. Figure 4-3 below presents TDD configurations #1 and #2 considered by the CBRS alliance for band 48/n48. The LTE MBSFN can be enabled for sub-frames #3-4 and #7-8; but the NR SSB can be transmitted only in the first symbols of the half-sub-frame.

Figure 4-3: LTE MBSFN and NR SSB transmissions for 10ms frame.

Observation 3d:	NR sync pattern B can work with 4-port LTE deployments.

Observation 3e:	Since candidate LTE MBSFN sub-frames do not overlap with OFDM symbols where NR SSB is transmitted, LTE MBSFN cannot be considered as a viable solution to avoid overlaps (unless some further changes are introduced impacting other WGs).

Another workaround option, which has been raised by some companies, is the possibility to of employing LTE downlink transmission mode 9 (TM9) to avoid the problems of interference with CRS for LTE UEs. However, there are several deployment and technical viability ies issues with this option. Firstly, the use of TM9 is a scheduling decision and a trade-off between reduction of system capacity reduction due to additional overhead of multi-layer transmissions and increasing in user throughput. Hence, even for a TM9 capable UE, this transmission mode will not be configured by base-station unless reduced SINR conditions and sustained UE traffic requirements justify its usage. Secondly, all LTE UEs use CRS to perform acquisition and channel estimation, irrespective of configured transmission mode, and it cannot be generally assumed that all b48/n48 UEs will support TM9 in a given network. Finally, network equipment supporting 4-port CRS configuration are not able to natively support 8-layer transmissions, hence, there is little purpose and support for in 4-port radio equipment with for TM9.

Based on the presented considerations, the most flexible solution would be to consider sync pattern B for band n48 so that 4-port LTE deployments can also be used. However, as discussed earlier in RAN WG4, there are several concerns with that approach which mostly relate to the fact that band n48 overlaps with other bands, e.g. band n77 and band n78, as can be seen from Figure 2.2-2. It should be noted that there is also C-band, for implementation of which band n77 was chosen. However, since the actual C-band does not overlap with band n48, there should be no concern with it.

Observation 3f:	C-band does not overlap with band n48, even if it is implemented by band n77.

Observation 3g:	The main concern is band n77/n78 overlapping with band n48.

Firstly, even though band n77 and n78 overlap with band n48, this happens only for 150MHz in 3550-3700MHz frequency range. So, potential performance impact will be limited and will not concern the whole band n77/n78. Furthermore, band n48 and n77/78 are deployed in different geographical areas. It means that a UE knows from its SIM subscription and geographical location which patterns it should be probe while scanning for a particular range. As an example, a UE with EU subscription located in EU will not probe for pattern B while scanning 3550-3700MHz range. Similarly, a UE with US subscription in US will probe for both patterns B and C while scanning the 3550-3700MHz frequency range. All these optimisations are already used by UEs, which instead of trying blindly different RATs and frequencies, use the subscription information to decide which RAT should be probed first and for which frequencies. In the worst-case scenario, a UE might unnecessarily probe for both patterns B and C, which will happen only in the overlapping range of 3550-3700MHz, outside of which only pattern C can be used. Nevertheless, our understanding is that it will not cause any serious problem. Having different sync patterns in the same frequency range might increase initial search time and cell selection process during the cold start, but once a UE camps on the operator serving cell, it will follow cell re-selection process and frequencies indicated by a particular operator. In the worst-case scenario, even if a UE fails to camp on e.g. band n48 or n77 because it does not find the synchronization signal, it will search for other bands and RATs.

Another concern expressed during previous meetings was increased power consumption. However, overall power consumption while searching for SSB is rather marginal when compared to the UE power consumption when it exchanges data with the network in the CONNECTED state. It becomes an issue only when a UE constantly and continuously searches for available networks potentially probing not only different NR bands, but also different RATs. In this case however adding sync pattern B to band n48 will have only marginal effect.

Figure 4-4: 3GPP NR bands n48, n77, n78, and the C-band.

During RAN4#95 meeting it was discussed whether synchronisation pattern B can be added directly to band n48 or whether a new band shall be instantiated for this purpose. In the latter case, there were concerns that if a new band is introduced then it might segment further UE ecosystem. However, operators using CBRS are likely also have other band support requirements that are shared with handset manufactures. Similarly, it will be normal to expect that handset manufacturers will try to support different variants and/or superset of band requirements; including the support both bands on a continuing basis if that is deemed necessary by market requirements and business drivers. Additionally, we note that synchronization pattern B is a cleaner solution as it will also improve the performance for other deployment modes, not just 4-port LTE CRS operation. It has also been recently requested and being evaluated for other new TDD bands, such as n38 and n40.

Thus, it should be discussed further which approach is better.

Proposal 3a:	Adopt synchronisation pattern B.

Proposal 3b:	It should be discussed further whether sync pattern B can be added to band n48 definition or whether a new band shall be introduced.

Finally, if synchronization pattern B cannot be agreed now as a solution for band n48 or a new band, we kindly request RAN WG4 to discuss and agree to supporting synchronization pattern B as a common way forward support DSS on mid-band TDD bands.

Proposal 3c:	If synchronisation pattern B cannot be agreed now, we ask RAN WG4 to devise a solution that would enable dynamic spectrum sharing between NR and LTE using 4-port CRS transmission.

# 3	Conclusions

In this discussion paper we have presented our further views on several issues for dynamic spectrum sharing between LTE and NR on band n48. As a summary of our findings, since it is clear that an operator cannot rely on the fact the SAS entity will allocate the frequency range aligned on the 300kHz raster, some solution is needed. Amongst possible approaches identified at the moment, it seems that shifting centre frequency -/+100kHz can achieve required alignment with quite marginal specification changes and resulting impact to the overall system performance. Furthermore, since the CBRS alliance has added 15kHz SCS as one of the mandatory deployment options, it is also crucial to ensure that UL sub-carrier grids are also aligned. As for the synchronisation pattern, our view is that adoption of pattern B is cleanest solution to the problem, but we also encourage companies to look deeper into this problem to devise a solution that would enable dynamic spectrum sharing between NR and LTE using 4-port CRS transmission.

Proposal 1a:	Adopt a solution based on the -/+100kHz shift of the centre frequency.

Proposal 1b:	Introduce signalling to indicate explicitly to the UE that the centre frequency is shifted and thus the guard bands are smaller.

Proposal 1c:	Since extra power back-off needed for shrunk guard bands is still within existing MPR limits, it can be left up to the UE implementation.

Proposal 2a:	Introduce UL shift as a mandatory UE feature at least for 15kHz SCS (on band n48).

Proposal 2b:	Ask RAN WG2 to clarify UE behaviour when a UE camps on the cell with enabled UL shift, but a UE does not support UL shift functionality for a particular SCS.

Proposal 3a:	Adopt synchronisation pattern B.

Proposal 3b:	It should be discussed further whether sync pattern B can be added to band n48 definition or whether a new band shall be introduced.

Proposal 3c:	If synchronisation pattern B cannot be agreed now, we ask RAN WG4 to devise a solution that would enable dynamic spectrum sharing between NR and LTE using 4-port CRS transmission.

# 4	References

RP-182883, "New WI proposal: LTE/NR spectrum sharing in Band 41/n41", KDDI Corporation

RP-192427, "New WID: LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc.

RP-201359, "Revised WID: LTE/NR spectrum sharing in band 48/n48 frequency range", Apple Inc, Comcast

R4-2008288, "LTE/NR spectrum sharing in band 48/n48 frequency range (channel raster)", Apple Inc.

R4-2006337, "LTE/NR spectrum sharing in band 48/n48 frequency range (UL shift)", Apple Inc.

R4-2008289, "LTE/NR spectrum sharing in band 48/n48 frequency range (sync pattern)", Apple Inc.

R4-2006588, "Views on band 48/n48 spectrum sharing", Nokia, Nokia Shanghai Bell

R4-2006871, "Views on dynamic spectrum sharing between LTE band 48 and NR band n48", Google Inc.

R4-2007086, "DSS in band n48", OPPO

R4-2007441, "On LTE/NR spectrum sharing in band 48/n48", Intel Corporation

R4-2007791, "UL shift for LTE/NR spectrum sharing in band 48/n48", Ericsson

R4-2008962, "Email discussion summary for [95e][134] NR_n48_LTE_48_coex", Moderator (Apple)

Annex A (excerpt from TS 38.101-1)

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz | FDD |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n5 | 824 MHz – 849 MHz | 869 MHz – 894 MHz | FDD |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz | FDD |
| n8 | 880 MHz – 915 MHz | 925 MHz – 960 MHz | FDD |
| n12 | 699 MHz – 716 MHz | 729 MHz – 746 MHz | FDD |
| n14 | 788 MHz – 798 MHz | 758 MHz – 768 MHz | FDD |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n20 | 832 MHz – 862 MHz | 791 MHz – 821 MHz | FDD |
| n25 | 1850 MHz – 1915 MHz | 1930 MHz – 1995 MHz | FDD |
| n28 | 703 MHz – 748 MHz | 758 MHz – 803 MHz | FDD |
| n303 | 2305 Mhz – 2315 MHz | 2350 MHz – 2360 MHz | FDD |
| n34 | 2010 MHz – 2025 MHz | 2010 MHz – 2025 MHz | TDD |
| n38 | 2570 MHz – 2620 MHz | 2570 MHz – 2620 MHz | TDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |
| n40 | 2300 MHz – 2400 MHz | 2300 MHz – 2400 MHz | TDD |
| n41 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD |
| n48 | 3550 MHz – 3700 MHz | 3550 MHz – 3700 MHz | TDD |
| n50 | 1432 MHz – 1517 MHz | 1432 MHz – 1517 MHz | TDD1 |
| n51 | 1427 MHz – 1432 MHz | 1427 MHz – 1432 MHz | TDD |
| n65 | 1920 MHz – 2010 MHz | 2110 MHz – 2200 MHz | FDD4 |
| n66 | 1710 MHz – 1780 MHz | 2110 MHz – 2200 MHz | FDD |
| n70 | 1695 MHz – 1710 MHz | 1995 MHz – 2020 MHz | FDD |
| n71 | 663 MHz – 698 MHz | 617 MHz – 652 MHz | FDD |
| n74 | 1427 MHz – 1470 MHz | 1475 MHz – 1518 MHz | FDD |
| n75 | N/A | 1432 MHz – 1517 MHz | SDL |
| n76 | N/A | 1427 MHz – 1432 MHz | SDL |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |
| n78 | 3300 MHz – 3800 MHz | 3300 MHz – 3800 MHz | TDD |
| n79 | 4400 MHz – 5000 MHz | 4400 MHz – 5000 MHz | TDD |
| n80 | 1710 MHz – 1785 MHz | N/A | SUL |
| n81 | 880 MHz – 915 MHz | N/A | SUL |
| n82 | 832 MHz – 862 MHz | N/A | SUL |
| n83 | 703 MHz – 748 MHz | N/A | SUL |
| n84 | 1920 MHz – 1980 MHz | N/A | SUL |
| n86 | 1710 MHz – 1780 MHz | N/A | SUL |
| n90 | 2496 MHz – 2690 MHz | 2496 MHz – 2690 MHz | TDD5 |
| NOTE 1: UE that complies with the NR Band n50 minimum requirements in this specification shall also comply with the NR Band n51 minimum requirements. NOTE 2: UE that complies with the NR Band n75 minimum requirements in this specification shall also comply with the NR Band n76 minimum requirements. NOTE 3: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 4: A UE that complies with the NR Band n65 minimum requirements in this specification shall also comply with the NR Band n1 minimum requirements. NOTE 5: Unless otherwise stated, the applicability of requirements for Band [n90] is in accordance with that for Band n41; a UE supporting Band [n90] shall meet the requirements for Band n41. |  |  |  |

Annex B (excerpt from TS 38.101-1):

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n1 | 100 | 384000 – <20> – 396000 | 422000 – <20> – 434000 |
| n2 | 100 | 370000 – <20> – 382000 | 386000 – <20> – 398000 |
| n3 | 100 | 342000 – <20> – 357000 | 361000 – <20> – 376000 |
| n5 | 100 | 164800 – <20> – 169800 | 173800 – <20> – 178800 |
| n7 | 100 | 500000 – <20> – 514000 | 524000 – <20> – 538000 |
| n8 | 100 | 176000 – <20> – 183000 | 185000 – <20> – 192000 |
| n12 | 100 | 139800 – <20> – 143200 | 145800 – <20> – 149200 |
| n14 | 100 | 157600 – <20> – 159600 | 151600 – <20> – 153600 |
| n18 | 100 | 163000 – <20> – 166000 | 172000 – <20> – 175000 |
| n20 | 100 | 166400 – <20> – 172400 | 158200 – <20> – 164200 |
| n25 | 100 | 370000 – <20> – 383000 | 386000 – <20> – 399000 |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
| n30 | 100 | 461000 – <20> – 463000 | 470000 – <20> – 472000 |
| n34 | 100 | 402000 – <20> – 405000 | 402000 – <20> – 405000 |
| n38 | 100 | 514000 – <20> – 524000 | 514000 – <20> – 524000 |
| n39 | 100 | 376000 – <20> – 384000 | 376000 – <20> – 384000 |
| n40 | 100 | 460000 – <20> – 480000 | 460000 – <20> – 480000 |
| n41 | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
| n48 | 15 | 636667 – <1> – 646666 | 636667 – <1> – 646666 |
|  | 30 | 636668 – <2> – 646666 | 636668 – <2> – 646666 |
| n50 | 100 | 286400 – <20> – 303400 | 286400 – <20> – 303400 |
| n51 | 100 | 285400 – <20> – 286400 | 285400 – <20> – 286400 |
| n65 | 100 | 384000 – <20> – 402000 | 422000 – <20> – 440000 |
| n66 | 100 | 342000 – <20> – 356000 | 422000 – <20> – 440000 |
| n70 | 100 | 339000 – <20> – 342000 | 399000 – <20> – 404000 |
| n71 | 100 | 132600 – <20> – 139600 | 123400 – <20> – 130400 |
| n74 | 100 | 285400 – <20> – 294000 | 295000 – <20> – 303600 |
| n75 | 100 | N/A | 286400 – <20> – 303400 |
| n76 | 100 | N/A | 285400 – <20> – 286400 |
| n77 | 15 | 620000 – <1> – 680000 | 620000 – <1> – 680000 |
|  | 30 | 620000 – <2> – 680000 | 620000 – <2> – 680000 |
| n78 | 15 | 620000 – <1> – 653333 | 620000 – <1> – 653333 |
|  | 30 | 620000 – <2> – 653332 | 620000 – <2> – 653332 |
| n79 | 15 | 693334 – <1> – 733333 | 693334 – <1> – 733333 |
|  | 30 | 693334 – <2> – 733332 | 693334 – <2> – 733332 |
| n80 | 100 | 342000 – <20> – 357000 | N/A |
| n81 | 100 | 176000 – <20> – 183000 | N/A |
| n82 | 100 | 166400 – <20> – 172400 | N/A |
| n83 | 100 | 140600 – <20> –149600 | N/A |
| n84 | 100 | 384000 – <20> – 396000 | N/A |
| n86 | 100 | 342000 – <20> – 356000 | N/A |
| [n90] | 15 | 499200 – <3> – 537999 | 499200 – <3> – 537999 |
|  | 30 | 499200 – <6> – 537996 | 499200 – <6> – 537996 |
|  | 100 | 499200 – <20> – 538000 | 499200 – <20> – 538000 |

Annex C: 5MHz@15kHz SCS (CP-OFDM)

Figure C-1: NS_01 without shift (left) and with shift (right).

Figure C-2: NS_27 inner band without shift (left) and with shift (right).

Figure C-3: NS_27 lower edge without shift (left) and with shift (right).

Annex D: 10MHz@15kHz SCS (CP-OFDM)

Figure D-1: NS_01 without shift (left) and with shift (right).

Figure D-2: NS_27 inner band without shift (left) and with shift (right).

Figure D-3: NS_27 lower edge without shift (left) and with shift (right).

Annex E: 20MHz@15kHz SCS (CP-OFDM)

Figure E-1: NS_01 without shift (left) and with shift (right).

Figure E-2: NS_27 inner band without shift (left) and with shift (right).

Figure E-3: NS_27 lower edge without shift (left) and with shift (right).

Annex F: Summary of extra power back-off

Table F-1: Power back-off needed to compensate for shrunk guard band (CP-OFDM).

| Channel@SCS | RB Allocation | NS_01 | NS_27 (inner) | NS_27 (lower) |
| --- | --- | --- | --- | --- |
| 5MHz@15kHz | Edge | 2.4 | 1.9 | 1.9 |
|  | Inner | 0.3 | 0.2 | 0.4 |
|  | Outer | 1.1 | 1.1 | 1 |
| 10MHz@15kHz | Edge | 1.3 | 1.3 | 1.2 |
|  | Inner | 0.4 | 0.4 | 0.1 |
|  | Outer | 1.1 | 1.2 | 1.2 |
| 20MHz@15kHz | Edge | 0 | 0 | 0.1 |
|  | Inner | 0.1 | 0.1 | 0.6 |
|  | Outer | 1 | 1 | 0.5 |
| 10MHz@30kHz | Edge | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0 |
|  | Outer | 0.8 | 0.5 | 0.8 |
| 20MHz@30kHz | Edge | 0 | 0 | 0.1 |
|  | Inner | 0 | 0 | 0.3 |
|  | Outer | 0.4 | 0.6 | 0.3 |

Table F-2: Power back-off needed to compensate for shrunk guard band (DFT-s-OFDM).

| Channel@SCS | RB Allocation | NS_01 | NS_27 (inner) | NS_27 (lower) |
| --- | --- | --- | --- | --- |
| 5MHz@15kHz | Edge | 1.1 | 1.0 | 0.9 |
|  | Inner | 0 | 0 | 0 |
|  | Outer | 0.5 | 0.5 | 0.4 |
| 10MHz@15kHz | Edge | 0.6 | 0.7 | 0.6 |
|  | Inner | 0 | 0 | 0 |
|  | Outer | 0.7 | 0.5 | 0.6 |
| 20MHz@15kHz | Edge | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0.3 |
|  | Outer | 0.4 | 0.5 | 0.9 |
| 10MHz@30kHz | Edge | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0 |
|  | Outer | 0.5 | 0.5 | 0.4 |
| 20MHz@30kHz | Edge | 0 | 0 | 0 |
|  | Inner | 0 | 0 | 0.2 |
|  | Outer | 0.2 | 0.2 | 0.5 |

Annex G: Existing maximum power reduction

Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 | ≤ 0.52 | 02 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0 dB MPR is 26 dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40 % of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |