3GPP TSG-RAN WG4 Meeting # 96-e												             R4-2011543

Electronic Meeting, 17th – 28th August, 2020

Agenda item:			7.3.3.1, 7.3.3.2, 7.3.4

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [96e][110] 5G_V2X_NRSL_UE_Concurrent

Document for:	Information

# Introduction

This email thread discuss NR V2X con-current operation.  The contributions are scattered in agenda 7.3.3.1, 7.3.3.2, 7.3.4, which includes:

- Topic #1: Con-current operation requirements and clarification

  - Con-current operation definition

  - con-current band combination

  - requirements spec structure

- Topic #2: Tx requirements:

  - Switching period between NR SL and LTE SL in ITS band

  - delta Tib

  - Pcmax

- Topic #3: Rx requirements:

  - delta Rib and REFSENS

# Topic #1: Con-current operation requirements and clarification

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010290 | vivo | Proposal 1: Option b is preferred as con-current operation definition. Option b, i.e., “The simultaneous transmission and reception of sidelink and Uu interfaces where operation is agnostic of the service used on each interface” Proposal 2: Band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) can be aligned as intra-band EN-V2X operation instead of intra-band con-current operation. Proposal 3: Include all the con-current requirements for NR V2X to TS 38.101-3. |
| R4-2010605 | Xiaomi | Observation 1: No scheduling or configuring resource allocation has been used in LTE V2X concurrent operation band definition. Proposal 1: Use option b as definition of concurrent operation. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 1-1: Con-current operation definition

Candidate options of con-current operation definition in RAN4#95e-bis 1st round discussion (R4-2008451)

- Option 1: only the band combination where the Uu schedules/configures SL by semi-persistent way can be specified as concurrent operation (Huawei)

- Option 2: the band combination where the Uu schedules/configures SL can be specified as concurrent operation

- Option 3: the band combinations where the Uu schedules/configures SL can be specified and which is agnostic as to the actual service being delivered on Uu and Sidelink interfaces (LGE, Huawei)

- Option 4: The simultaneous transmission and reception of sidelink and Uu interfaces where operation is agnostic of the service used on each interface (CATT, LGE, Vodafone, Qualcomm).

- Option 4a: the band combination where the Uu interworks with SL can be specified as concurrent operation (vivo)

Tentative con-current operation definition in the specification

- Con-current operation: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.

Moderator’s recommendation:

- Recommended WF

  - Further check if the definition adopted tentatively in the spec is aligned with RAN1 and RAN2 relative capability discussion

  - Other definitions are not excluded

### Issue 1-2: Con-current operation band combination

Whether band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) should be considered as intra-band con-current operation

- Option 1: Band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) can be aligned as intra-band EN-V2X operation instead of intra-band con-current operation

- Option 2: keep V2X_47_n47 as intra-band con-current band combination

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 1-3: con-current requirements spec

Whether need to put con-current operation requirements in both TS 38.101-1 and TS 38.101-3

- Option 1: Keep current con-current requirements as it is in both specs

- Option 2: Include all the con-current requirements for NR V2X to TS 38.101-3

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

## Companies views’ collection for 1st round

### Open issues

| Issues | Company Comments |
| --- | --- |
| 1-1: Con-current operation definition | LGE: based on agreements at last RAN4 meeting, we would like to keep the existing con-current operation definitions. Xiaomi: Option 4 is the same as stated in Xiaomi’s paper option b. We prefer option 4. vivo: Ok with the tentative con-current operation’s definition. CATT: Prefer to keep the definition as it is. Huawei, HiSilicon: In some cases, we see simultaneous Tx and Rx are not possible, but we can accept to focus on the RF requirements for the band combination. However, the wording should be changed to align with the RAN2 specification. In RRC connected mode, which service is used in which band is clearly indicated and even the RAN4 requirements are also specified for SL or Uu differently for the supported band combination. Thus agnostic operation is not correct, which should be revised and the proposed changes are: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is aware of the service used on each interface. OPPO: Option 4. Futurewei: Perhaps some clarification is needed. The concurrent operation, in general, have some interaction with each other, at least aware of some basic information about SL. As NR SL handles multitude of services, can the concurrently operating other RAT be agnostic all the time? Vodafone: Still prefer option 4. Qualcomm: Option4: We would like to stay with last meeting’s agreement of “The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface.” |
| 1-2: Con-current operation band combination | LGE: prefer to change as follow Option 1: Band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) can be aligned as intra-band EN-V2X operation instead of intra-band con-current operation Xiaomi: Option 1. Anyway based on the definition in issue 1-1, it should be option 1 in this issue. Vivo: Option 1 is preferred. CATT: Prefer Option 1. As discussed in previous meetings, LTE V2X and NR V2X operate as TDM mode. Based on the definition of con-current operation, TDM should not be considered as simultaneous transmission. Also, both Uu and SL are involved in the definition. Huawei, HiSilicon: Option 1 is ok. V2X_(n)47AA and V2X_47A_n47A in the spec should be changed to DC_(n)47AA and DC_47A_n47A accordingly. OPPO: Option 1. Futurewei: Option 1. Qualcomm: Option 1: We think that this option best fits the band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) |
| 1-3: con-current requirements spec | LGE: prefer option1 to keep the consistency with NR DC - Option 1: Keep current con-current requirements as it is in both specs Xiaomi: Prefer option 1. Vivo: For the con-current definition, both two RATs including Uu and SL are involved. The requirements for two RATs inter-working with each other should be captured in TS 38.101-3. It is inconvenient to separate the requirements into TS 38.101-1 and 38.101-3. Option 2 is preferred. CATT: Prefer Option 1: Keep current con-current requirements as it is in both specs NR-DC requirements where only NR involved are specified in 38.101-1, while EN-DC and NE-DC requirements where both LTE and NR are involved are specified in 38.101-3. Based on this principle, the existing arrangement in both specs would be preferred. Huawei, HiSilicon: Option 1 is preferred. Futurewei: Option 1: Keep current con-current requirements as it is in both specs Qualcomm: Option1: It makes sense to capture all con-current requirements for NR V2X in cases where there is con-currency with NR bands in TS38.101-1. However, in cases where NR V2X is concurrent with LTE bands or LTE V2X is concurrent with NR bands it should be captured in TS38.101-3. |
| Others |  |

### CRs/TPs comments collection

Major close-to-finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| XXX | Company A |
|  | Company B |
| YYY | Company A |
|  | Company B |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic#1 | Issue 1-1: Con-current operation definition Tentative agreements: Most companies favor the tentative definition in the spec, i.e. option1 below. Candidate options: Option 1: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is agnostic of the service used on each interface. Option 2: The simultaneous transmission and reception of sidelink and Uu interfaces while operation is aware of the service used on each interface. Recommendations for 2nd round: Keep the definition unchanged and send LS to RAN1 and RAN2 to see if the definition is aligned with their understanding and make sure there is no conflict among groups. Issue 1-2: Con-current operation band combination Tentative agreements: Band combination of NR V2X SL (at n47) + LTE V2X SL (at B47) can be aligned as intra-band EN-V2X operation instead of intra-band con-current operation. Recommendations for 2nd round: DC_(n)47AA and DC_47A_n47A in the spec should be changed to V2X_(n)47AA and V2X_47A_n47A, and the changes will be reflected in the CR for TS 38.101-3. Issue 1-3: Con-current requirements spec Candidate options: Option 1: Keep current con-current requirements as it is in both specs. Option 2: Include all the con-current requirements for NR V2X to TS 38.101-3. Recommendations for 2nd round: Most companies prefer option 1. It is suggested to go with option 1. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | LS on definition of NR V2X con-current operation | Huawei, HiSilicon |
| #2 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #2: Tx requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009829 | CATT | Switching period is placed at NR slot and the switching time is 150us. |
| R4-2010138 | LG Electronics | Switching period is placed at NR slot and the switching time is 150us. |
| R4-2010290 | vivo | Proposal 4: Option2 is preferred for switching period position. Option 2: The whole switching time including switching period as well as transient periods shall be placed at the previous E-UTRA sub-frame or NR slot Observation 1: The configured power class for inter-band con-current operations would exceed 23dBm. Proposal 5: Add a restriction on PCMAX_H for inter-band con-current operation as follows: PCMAX_H (p,q) = Min{10 log10 [pCMAX_H,c, Uu (p) + pCMAX_H,c,V2X (q)], PPowerclass, con-current} where PPowerclass, con-current is the power class defined for con-current operation. |
| R4-2010457 | CATT | Proposal 1: 150us switching time can be used for intra-band switching between LTE V2X and NR V2X. Proposal 2: The time mask for the switching between LTE V2X and NR V2X can be specified as Figure 1 and Figure 2. |
| R4-2010605 | Xiaomi | Observation 2: Similar to CA UL switching, UE capability can help to indicate the switching time and switching period location. Proposal 2: Choose option 2 to locate the switching period. |
| R4-2010819 | Huawei, HiSilicon | Observation 1: No clear benefit for a longer switching time under the scheduling restriction condition. Observation 2: The whole switching period together with transient period should be put on one side on LTE subframe or NR slot to avoid more wasted resource. Observation 3: It’s not reasonable to put the switching period only at the NR V2X side. Observation 4: Due to the scheduling restriction, no essential difference for options to put the switching period at either LTE sub-frame or NR slot. Proposal: It is proposed to agree on the time masks for switching between E-UTRA SL and NR SL as in Figure 2. |
| R4-2010014 | Qualcomm | Proposal 5: Do not specify switching time RF requirement, use RRM requirement in 38.113 clause 12.9.1 to verify the LTE-NR Tx switching requirement. Proposal 6: LTE-NR Tx Switching time requirement is specified as a package in the following: - Switching time requirement in RF spec (38.101) is 150us - If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement. - If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1. Observation 1: Regardless of switch position being chosen, the interruption is at least one slot on NR or one subframe on LTE, and at most one slot on NR AND one subframe on LTE. Observation 2: For switch happens at (1) slot/subframe n and S us before n+1 subframe/slot boundary, or (2) slot n and less than g us before n+1 subframe boundary or later, one slot on NR or one subframe LTE interruption is achievable. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 2-1: Switching period

Candidate options

- Option 1: Switching period is placed at NR slot and the guard symbol at LTE side can also be utilized

- Option 2: The whole switching time including switching period as well as transient periods shall be placed at the previous E-UTRA sub-frame or NR slot

- Option 3: Do not specify switching time RF requirement, use RRM requirement in 38.113 clause 12.9.1 to verify the LTE-NR Tx switching requirement.

- Option 4: LTE-NR Tx Switching time requirement is specified as a package in the following:

  - Switching time requirement in RF spec (38.101) is 150us

  - If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement.

  - If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1.

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 2-2: delta Tib

Candidate options

- Option 1: 0.2dB Delta Tib values for V2X_20A_n38A

- Option 2: 0dB Delta Tib values for V2X_20A_n38A.

- Option 3: other values are not excluded.

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 2-3: Pcmax

Candidate options

- Option 1: Add a restriction on PCMAX_H for inter-band con-current operation

- Option 2: Keep the requirement as it is

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

## Companies views’ collection for 1st round

### Open issues

| Issues | Company Comments |
| --- | --- |
| 2-1: Switching period | LGE : prefer option 1 since align with 5G V2X WI objectives in Rel-16. In the Justification and objective (RP-200129), the NR V2X service is restricted for the advanced 5G service when two RAT V2X service are operated in ITS spectrum as follow In 3. Justification TSG RAN has already agreed in TR 38.913 that it is not intended for NR V2X to replace the services offered by LTE V2X. Instead, the NR V2X shall complement LTE V2X for advanced V2X services and support interworking with LTE V2X. In 4. Objectives coexistence with other V2X technologies in the adjacent channel in ITS spectrum in 5.9 GHz, without assuming that 5.9 GHz spectrum will be universally available nor that it will be universally available in sufficient quantity to support NR V2X advanced use cases. - Option 1: Switching period is placed at NR slot and the guard symbol at LTE side can also be utilized Xiaomi: Option 2. We believe that the switching here only occurs in certain circumstance as better QoS or more important service, i.e safety. Then the switching time located in previous carrier should be a better choice. Vivo: We stick to Option 2. CATT: Prefer Option 1. Swtching period should be placed at NR slot and the guard period at LTE side should be used. Huawei, HiSilicon: Preference is Option 2. As agreed in RRM spec, one NR slot or LTE sub-frame is restricted for transmission or reception if switching is occurred at this slot or sub-frame, thus option 1 should be avoided as the switching across the boundary between LTE-V and NR-V. For option 4, for the case “Support of fewer than 14 consecutive sidelink symbols in a slot”, in the RAN1 spec it specifies that “SL transmissions can start from a first symbol indicated by startSLsymbols and be within a number of consecutive symbols indicated by lengthSLsymbols”, in other words, it means it is impossible that switching of less than 14 symbols between LTE SL and NR SL can happen in the ITS band. For the left case in option 4, no essential difference compared to option 2. Futurewei: Option 2 Qualcomm: Option3 is first choice and option4 is the second choice if option3 is not acceptable. Option 1 and option2 do not address the fact that there was an agreement in RRM session in RAN4#95-e on scheduling where at least one subframe or slot is not expected to receive or transmit data when switching occurs. Any proposal for switching should address this important point. We only see option3 or option 4 as viable solutions given the RRM agreement which in effect makes the switching time between NR and LTE TX at least one slot or subframe. As pointed out in our contribution bullet 2 of option 4 is an optional feature which a UE may or may not support. Bullet 3 of option 4 clearly mentions the switching time requirement as being satisfied when clause 12.9.1 of 38.133 is met, but option 1 or 2 does not mention this explicitly. Our first selection is option 3 and as a compromise we propose option 4 |
| 2-2: Delta Tib | LGE: prefer option 1. To protect V2X service or safety message in licensed band, RAN4 need to use HTF. Then MSD is quite reduced by 3rd harmonic impact. In our measured results, about 6dB is better than without HTF case when consider shared antenna RF architecture. - Option 1: 0.2dB Delta Tib values for V2X_20A_n38A Huawei, HiSilicon: either option 1 or option 2 is ok for us. Qualcomm: Option 3: 0.3dB Delta Tib values for V2X_20A_n38A. The guidance in 37.716-11-11 should be followed for DC_20_n38 |
| 2-3: Pcmax | LGE: Do not need to restricted Pcmax _H for inter-band con-current V2X operation. CATT: Prefer Option 1. The current version is aligned with the formula for LTE V2X in 36.101. However, the restriction for inter-band con-current V2X operation should be adopted to cap the maximum power like what has been done in CA scenarios. Huawei, HiSilicon: Option 2, no need to restrict Pcmax _H. As mentioned by CATT, the formula is aligned with LTE V2X, and the restriction actually was lifted for LTE V2X in Rel-15 with the group agreement. Qualcomm: Option 2: keep the requirement as it is |
| Others | Vivo: If Pcmax_H would exceed PC3, then we think it is needed to add a restriction of PPowerclass, con-current on PCMAX_H. Companies can check whether Pcmax_H is an issue. Huawei, HiSilicon: No restriction is needed. |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2009824 CR for 38.101-3 switching period, CATT | Qualcomm: Do not agree with adding switching diagrams in section 6.3C.2 in light of the RRM agreements related to switching not sure if this diagram applies |
| R4-2009829 CR for 38.886 switching period, CATT | Qualcomm: Do not agree with adding switching diagrams in section 8.1.7.3 in light of the RRM agreements related to switching not sure if this diagram applies |
| R4-2010138 LGE | Qualcomm : table 6.2C.4.2-1 delta Tibc, table 7.3C.2.3-2 delta Ribc should follow the figures in 37.716-11-11 for DC_20_n38. Do not agree with adding switching diagrams in section 6.3C.2 in light of the RRM agreements related to switching not sure if this diagram applies. Table 7.3.2.3-1 may have to be updated based on target SNR, RB number and NF decisions that are agreed at this meeting |
| R4-2010289 vivo | Company A |
|  | Company B |
| R4-2010821 Huawei, HiSilicon | Qualcomm : Do not agree with adding switching diagrams in section 6.3C.2 in light of the RRM agreements related to switching not sure if this diagram applies. The MSD figures in table 7.3C.2.3.1-1 may have to be updated based on target SNR, RB number and NF decisions that are agreed at this meeting |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #2 | Issue 2-1: switching period Candidate options: - Option 1: Switching period is placed at NR slot and the guard symbol at LTE side can also be utilized - Option 2: The whole switching time including switching period as well as transient periods shall be placed at the previous E-UTRA sub-frame or NR slot - Option 3: Do not specify switching time RF requirement, use RRM requirement in 38.113 clause 12.9.1 to verify the LTE-NR Tx switching requirement. - Option 4: LTE-NR Tx Switching time requirement is specified as a package in the following: - Switching time requirement in RF spec (38.101) is 150us - If UE supports this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement can be verified by configuring smaller number of consecutive symbols in a slot, and UE has to satisfy the 150us switching time requirement. - If UE doesn’t support this feature: “Support of fewer than 14 consecutive sidelink symbols in a slot”, the switching time requirement is considered as satisfied when UE satisfied the corresponding scheduling restriction requirement in 38.133 clause 12.9.1. Recommendations for 2nd round: Continue the discussion in 2nd round based on the WF on switching period. Issue 2-2: delta Tib Candidate delta Tib options for V2X_20A_n38A: - Option 1: 0.2dB - Option 2: 0dB - Option 3:0.3dB. Recommendations for 2nd round: Continue discussion for option 1 (0.2dB) and option 3 (0.3dB). The value will be discussed in the draft CR in 2nd round. Issue 2-3: Pcmax Candidate options: - Option 1: Add a restriction on PCMAX_H for inter-band con-current operation - Option 2: Keep the requirement as it is Recommendations for 2nd round: The restriction was already lifted for LTE, it is suggested to keep the requirement as it is. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on NR-V2X switching period | CATT |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |
| R4-2009824 (CATT) | Noted |
| R4-2010821 (Huawei) | draft CR for 38.101-3, used for 2nd round discussion to be revised in R4-20xxxxx |
| R4-2009829 (CATT) | CR for TR 38.886, revised based on the 2nd round discussion to be revised in R4-20xxxxx |
| R4-2010138 (LGE) | CR for 38.101-3, revised based on the 2nd round progress to be revised in R4-20xxxxx |
| R4-2010289 (vivo) | CR for 38.101-3, revised based on the 2nd round progress to be revised in R4-20xxxxx |

## Discussion on 2nd round (if applicable)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |

# Topic #3: Rx requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009577 | Xiaomi | Do not recommend using the HTF for the NR V2X band combination V2X_20_n38 and give the MSD values as shown in Table 2 under the case without HTF. |
| R4-2010136 | LG Electronics | Proposal 1: RAN4 allow 0.2dB Delta Tib/Rib values for V2X_20A_n38A UE. This is applied to the maximum output power and REFSENS relaxation as same LTE V2X UE. Proposal 2: RAN4 specify MSD levels for 10MHz CBW with 12dB based on shared antenna RF architecture with HTF. |
| R4-2010820 | Huawei, HiSilicon | Proposal 1: 0 dB ΔRIB,c values for DC_20_n38 can be reused for V2X_20_n38. Proposal 2: MSD values as well as UL configuration for V2X_20_n38 shall be defined similar to those for DC_20_n38. |

## Open issues summary

Before e-Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Issue 3-1: Delta Rib for V2X_20_n38

Candidate options:

- Option 1: 0.2dB Delta Rib values for V2X_20A_n38A

- Option 2: 0 dB ΔRIB,c values for DC_20_n38 can be reused for V2X_20_n38.

- Others are not excluded

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

### Issue 3-2: MSD for V2X_20_n38

Candidate options:

- Option 1 (R4-2009577)

| H3 MSD Result |  |  |  |
| --- | --- | --- | --- |
| CBW (MHz) | Thermal | w HTF | w/o HTF |
|  |  | MSD after MRC | MSD after MRC |
| 10 | -104 | 5.5 | 8.8 |
| 20 | -101 | 3.6 | 6.3 |
| 40 | -98 | 2.2 | 4.1 |

- Option 2 (R4-2010136)

| V2X inter-band con-current band combinations | Operating Bands / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| V2X_20_n38 | UL band | SL operation | 10 MHz (dB) | 20 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) |
|  | B20 | n38 | 12.0 | 9.0 | 7.1 | 6.0 |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) for which the 3rd transmitter harmonic is within the sidelink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 3: The MSD level applied to all supported SCSs in victim band. |  |  |  |  |  |  |

- Option 3 (R4-2010820)

Similar values as option 2, once consensus is reached for SNR and max SL allocated RB size numbers, update the MSD values accordingly

- Others are not excluded

Moderator’s recommendation:

- Recommended WF

  - TBA based on 1st round discussion

## Companies views’ collection for 1st round

### Open issues

| Issues | Company Comments |
| --- | --- |
| 3-1: Delta Rib for V2X_20_n38 | LGE: same as Tib, need to use HTF to reduce MSD level. So Rib also defined with 0.2dB - Option 1: 0.2dB Delta Rib values for V2X_20A_n38A CATT: slightly prefer Option 2. The value can be reused if the band combination has been previously specified. Xiaomi: prefer option 2. The 0 dB ΔRIB,c values for DC_20_n38 could be reused for V2X_20_n38. Huawei, HiSilicon: Option 2. Same value as that for V2X_20_n38 is preferred. Vodafone: Prefer option 2. To consider option 1, we would need to understand why the V2X Rib should differ from that already specified for DC_20_n38. Qualcomm: Option 2: 0 dB ΔRIB,c values for DC_20_n38 can be reused for V2X_20_n38. Should follow the guidelines specified in 37.716-11-11 for DC_20_n38 |
| 3-2: MSD for V2X_20_n38 | LGE: prefer option 2 Xiaomi’ MSD value is quite smaller than LGE’s MSD value even though the attenuation parameter are not well compare to LGE assumptions. Also they only consider different antenna architecture for V2X_20A_n38A. The worst RF architecture is shared antenna RF architecture, Huawei and LGE consider shared antenna RF architecture and performance difference is about 3dB for without HTF. CATT: Same view as Issue 3-1. Xiaomi: To LG, the reason we have quite different MSD values is that the assumption of the PA H3 level is quite different. Anyway, option 2 is also OK for us. Huawei, HiSilicon: Option 2 needs to be updated based on agreement on SNR value. Vodafone: Same view as Issue 3-1. Qualcomm: Option 4: Should follow guidelines specified in 37.716-11-11 for DC_20_n38 and once consensus is reached for SNR and allocated RB size numbers then update the MSD values accordingly |
| Others |  |

### CRs/TPs comments collection

Major close to finalize WIs and Rel-15 maintenance, comments collections can be arranged for TPs and CRs. For Rel-16 on-going WIs, suggest to focus on open issues discussion on 1st round.

| CR/TP number | Comments collection |
| --- | --- |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Topic #3 | Issue 3-1: delta Rib for V2X_20_n38 Candidate options: - Option 1: 0.2dB with HTF - Option 2: 0dB without HTF Recommendations for 2nd round: RAN4 need further discuss whether or not consider HTF. If RAN4 do not consider HTF, then specify the higher MSD level compare to consider HTF to reduced self-interference for V2X_20_n38. . Issue 2-2: MSD for V2X_20_n38 Candidate options: - Option 1 (R4-2010136) - Option 2, update MSD values based on agreements on SNR and allocated RB size numbers Recommendations for 2nd round: RAN4 need further discuss the required MSD level since the MSD level are dependent to whether or not use the HTF based on option1. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |

## Discussion on 2nd round (if applicable)

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |