3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2011599

# Electronic Meeting, 17th– 28th August, 2020

Source:		Verizon, Nokia, Samsung, Mediatek, Ericsson, Qualcomm

Title:			TP for TR 37.717-11-11 for DC_66_n77

Agenda item:		10.3.2

Document for:		Approval

# Introduction

In RAN#88-e meeting, the ”New WID on New WID on Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL)” [1] was approved. A similar study for the CA_n66-n77 item has been agreed in [2] and specified in 38.101-1. This contribution is a text proposal for TR 37.717-11-11 to include configurations for DC_66A_n77A band combination.

# Reference

[1] RP-201368: 	New WID on New WID on Dual Connectivity (DC) of 1 band LTE (1DL/1UL) and 1 NR band (1DL/1UL), CHTTL

[2] RP-200671: 	TR 38.716-02-00 v1.2.0 Rel-16 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), ZTE

# Text Proposal

<Start of Text Proposal>

## 6.1.x		DC_66_n77, DC_66-66_n77 and DC_66-66-66_n77

### 6.1.x.1	Configuration for DC

Table 6.1.x.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_66A_n77A | DC_66A_n77A | YES |
| DC_66A-66A_n77A | DC_66A_n77A | YES |
| DC_66A-66A-66A_n77A | DC_66A_n77A | YES |

### 6.1.x.2	Maximum output power for DC

Table 6.1.x.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_66A_n77A | 23 | +2/-3 |

### 6.1.x.3	Spurious emission band UE co-existence for DC

Table 6.1.x.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| EN-DC CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_66A_n77A | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 26, 29, 30, 41, 65, 66, 70, 71 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.1.x.4	MSD analysis for DC

For study of 2UL/2DL, Table 6.1.x4-1 lists up to 5th order harmonics and intermodulation products for DC_66A-n77A

Table 6.1.x.4-1: Harmonic and IMD analysis

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1780 | 3300 | 4200 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 3420 | 3560 | 6600 | 8400 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 5130 | 5340 | 9900 | 12600 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4*fy_low | 4*fy_high |
| 4th harmonics frequency limits (MHz) | 6840 | 7120 | 13200 | 16800 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 8550 | 8900 | 16500 | 21000 |
| Two tone 2nd order IMD products | fy_low – fx_high | fy_high – fx_low | fx_low + fy_low | fx_high + fy_high |
| IMD frequency limits (MHz) | 1520 | 2490 | 5010 | 5980 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | 2*fy_low – fx_high | 2*fy_high – fx_low |
| IMD frequency limits (MHz) | 780 | 260 | 4820 | 6690 |
| Two-tone 3rd order IMD products | 2*fx_low + fy_low | 2*fx_high + fy_high | 2*fy_low + fx_low | 2*fy_high + fx_high |
| IMD frequency limits (MHz) | 6720 | 7760 | 8310 | 10180 |
| Two-tone 4th order IMD products | ¦3*fx_low – fy_high¦ | ¦3*fx_high – fy_low¦ | 3*fy_low – fx_high | 3*fy_high – fx_low |
| IMD frequency limits (MHz) | 930 | 2040 | 8120 | 10890 |
| Two-tone 4th order IMD products | 3*fx_low + fy_low | 3*fx_high + fy_high | 3*fy_low + fx_low | 3*fy_high + fx_high |
| IMD frequency limits (MHz) | 8430 | 9540 | 11610 | 14380 |
| Two-tone 4th order IMD products | ¦2*fx_low – 2*fy_high¦ | ¦2*fx_high – 2*fy_low¦ | 2*fx_low + 2*fy_low | 2*fx_high + 2*fy_high |
| IMD frequency limits (MHz) | 4980 | 3040 | 10020 | 11960 |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 15090 | 11420 | 3820 | 2640 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 14910 | 18580 | 10140 | 11320 |
| Two-tone 5th order IMD products | ¦2*fx_low – 3*fy_high¦ | ¦2*fx_high – 3*fy_low¦ | ¦2*fy_low – 3*fx_high¦ | ¦2*fy_high – 3*fx_low¦ |
| IMD frequency limits (MHz) | 9180 | 6340 | 1260 | 3270 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 13320 | 16160 | 11730 | 13740 |

Table 6.1.x.4-2 lists up to 5th order harmonics mixing products for DC_66A_n77A

Table 6.1.x.4-2: Band 66 and Band n77 UL harmonic mixing products

|  |  |  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 4th Harmonic |  | 5th Harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge | DL Low Band Edge | DL High Band Edge |
| 66 | 1710 | 1780 | 2110 | 2200 | 4220 | 4400 | 6330 | 6600 | 8440 | 8800 | 10550 | 8900 |
| n77 | 3300 | 4200 | 3300 | 4200 | 6600 | 8400 | 9900 | 12600 | 13200 | 16800 | 16500 | 21000 |

In summary of study, it could be seen,

- The 2nd harmonic from band 66 UL may fall into band n77 DL frequency range (Note 1)

- No harmonic mixing products will fall into band n77 UL frequency range

- The 2nd and 5th order IMD products generated from dual uplinks of band 66 and n77 may fall into band 66 Rx frequency range

- The 4th and 5th order IMD products generated from dual uplink of band 66 and n77 may fail into band n77 frequency range, but there is no impact to TDD band.

The MSD should be considered to mitigate the impact of the interference for this combination.

Note-1: There is no a 2nd harmonic issue in the DC_66A_n77A band combination once the U.S. C-band (3700-3980MHz) is applied.

A study of sensitivity degradation for CA_n66-n77 has been captured in [2], and the same degradation is allowed for UL harmonic product to DL. The reference sensitivity is specified in Table 6.1.x.4-3 with uplink configuration specified in Table 6.1.x.4-4.

Table 6.1.x.4-3: MSD due to UL harmonic issue for DC_66_n77 (Note 1)

|  | NR Band / Channel bandwidth of the affected DL band / MSD |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 40 MHz (dB) | 50 MHz (dB) | 60 MHz (dB) | 70 MHz (dB) | 80 MHz (dB) | 90 MHz (dB) | 100 MHz (dB) |
| 66 | n772, 4 |  | 23.9 | 22.1 | 20.9 | 19.8 | 19.0 | 17.9 | 16.8 | 16.0 | 15.3 | 14.8 | 14.3 | 13.8 |
|  | n773 |  | 1.1 | 0.8 | 0.3 | 0.1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  | NOTE 2: The requirements should be verified for UL EARFCN or NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 4: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.1.x.4-4 Uplink configuration due to UL harmonic interference

|  |  | NR Band / Channel bandwidth of the affected DL band / UL RB allocation of the aggressor band |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL band | DL band | 5 MHz (LCRB) | 10 MHz (LCRB) | 15 MHz (LCRB) | 20 MHz (LCRB) | 25 MHz (LCRB) | 30 MHz (LCRB) | 40 MHz (LCRB) | 50 MHz (LCRB) | 60 MHz (LCRB) | 70 MHz (LCRB) | 80 MHz (LCRB) | 90 MHz (LCRB) | 100 MHz (LCRB) |
| 66 | n77 |  | 25 | 36 | 50 | 64 | 80 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |

Sensitivity degradation is allowed for the identified IMDs in the configuration of DC_66A_n77A. The study of reference sensitivity exceptions captured in [2] can be applied in Table 6.1.x.4-5 for the dual connectivity configuration.

Table 6.1.x.4-5: MSD test points for PCell due to uplink operation for EN-DC in NR FR1

| Operating Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EN CA CA Configuration | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| DC_66A_n77A DC_66-66_n77A DC_66-66-66_n77A | 66 | 1775 | 5 | 25 | 2175 | 31.0 | FDD | IMD2 |
|  | n77 | 3950 | 10 | 50 | 3950 | N/A | TDD | N/A |
|  | 66 | 1730 | 5 | 25 | 2130 | 5.0 | FDD | IMD5 |
|  | n77 | 3660 | 10 | 50 | 3660 | N/A | TDD | N/A |

### 6.1.x.5	∆TIB and ∆RIB values

For DC_66_n77, the ΔTIB,c and ΔRIB,c values are given in the tables below.

Table 6.1.x.5-1: ΔTIB,c

| Inter-band DA Configuration | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.6 |
|  | n77 | 0.8 |

Table 6.1.x.5-2: ΔRIB,c

| Inter-band DC Configuration | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_66_n77 DC_66-66_n77 DC_66-66-66_n77 | 66 | 0.2 |
|  | n77 | 0.5 |

### 6.1.x.6	Self-interference analysis

This section provides the formulation to determine whether the DL carriers would be impacted by UL harmonics, 2UL IMD products, or DL harmonic mixing with UL interferer based on UL and DL channel combinations. The potential self-interference mechanism has been identified in section 6.1.x.4 except UL harmonic order higher than 5 which are used to generate an interference mixing coefficient table for this DC combination, as shown in Table 6.1.x.6-1.

Table 6.1.x.6-1: Band 66 and Band n77 interference mixing coefficients

| EUTRA-NR DC Configuration | EUTRA/NR Band | UL Coefficient |  | DL Coefficient |  | Harmonic/IMD Order | Victim Band | Interference Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_66A_n77A | 66 | a | -1 | b | 1 | 2 | 66 | IMD |
|  | n77 | c | 1 | d | 0 |  |  |  |
|  | 66 | a | -3 | b | 1 | 5 | n77 | IMD |
|  | n77 | c | 2 | d |  |  |  |  |

Equations (1) and (2) below are used to calculate the interference centre frequency (fINT) and its effective bandwidth (BWINT) where coefficients a, b, c, and d are defined in Table 6.1.44.8-1 and CBW stands for channel bandwidth.

(1)

(2)

Formula (3-1) and (3-2) below are then used to indicate when the interference is overlapping with RX1 and RX2, respectively.

(3-1)

(3-2)

#### 6.x.1.x	OOB blocking exception requirements

For the band combination CA_66-n77, the blocking at 5010MHz in band n77 filter is identified, and the existing filter cannot reach to 30dB rejection. A further study is needed.

<Start of Text Proposal>