3GPP TSG-RAN4 WG4 Meeting # 96-e	R4-2011723

# Electronic meeting, August 17- 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 0470 | rev | 1 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for intra-band UL contiguous CA DC location |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-Core |  |  |  |  |  | Date: |  |  | 2020-07-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The applicable frequencies for LO leakage and IQ image is decided by the DC location indicating for NR. For intra-band contiguous CA, DC location may be variable with CCs activating/deactivating for UE with single PA architecture, then DC location cannot be identified by TE or gNB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For UE support additional DC location reporting, the frequnecy of LO leakage and Image depends on the additional signalling for intra-band UL CA. - For 3300/3301 is allowed for intra-band contiguous CA, and test procedure follow conclusion in Rel-15: the corresponding test points and procedure are waved. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | DC location for intra-band contiguous CA with single PA architecture can not indicated to TE or gNB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4A.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR’s revision history: |  |
| --- | --- |

<Start of change>

6.4A.2.4	Transmit modulation quality for intra-band contiguous CA

For intra-band contiguous carrier aggregation, the requirements in subclauses 6.4A.2.3.1, 6.4A.2.3.2 applies.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

In case the parameter 3300 or 3301 is reported from UE via txDirectCurrentLocation IE (as defined in TS 38.331 [13]), carrier leakage measurement requirement in subclause 6.4A.2.4.2 shall be waived, and the RF correction with regard to the carrier leakage and IQ image shall be omitted during the calculation of transmit modulation quality.

6.4A.2.4.1	Error Vector Magnitude

For the intra-band contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.4.2.1.

When a single component carrier is configured Table 6.4.2.1-1 apply.

The EVM requirements are according to Table 6.4A.2.4.1-1 if CA is configured in uplink with the parameters defined in Table 6.4.2.1-2.

Table 6.4A.2.4.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC |
| --- | --- | --- |
| Pi/2-BPSK | % | 30 |
| QPSK | % | 17.5 |
| 16 QAM | % | 12.5 |
| 64 QAM | % | 8 |
| 256 QAM | % | 3.5 |

6.4A.2.4.2	In-band emissions

For intra-band contiguous carrier aggregation, the requirements in Table 6.4A.2.4.2-1 and 6.4A.2.4.2-2 apply within the aggregated transmission bandwidth configuration with both component carrier (s) active and one single contiguous PRB allocation of bandwidth  at the edge of the aggregated transmission bandwidth configuration.

The inband emission is defined as the interference falling into the non allocated resource blocks for all component carriers. The measurement method for the inband emissions in the component carrier with PRB allocation is specified in annex F.3. For a non allocated component carrier a spectral measurement is specified.

Table 6.4A.2.4.2-1: Minimum requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Output power > 10 dBm | Image frequencies (NOTE 3) |
|  |  | -25 | 0≤ Output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTE 4,5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE indicated in active uplink carrier(s). For band combinations with supporting additional DC location reporting for intra-band CA, the applicable LO leakage frequency depend on the txDirectCurrentLocation indicated in the additional reporting IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. Otherwise, the applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE. For only one uplink carrier is activated, the applicable LO leakage frequency follow definition in subclause 6.4.2. NOTE 6: is the Transmission Bandwidth (see section 5.3) not exceeding . NOTE 7: is the Transmission Bandwidth Configuration (see section 5.3) of the component carrier with RBs allocated. NOTE 8: is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. |  |  |  |  |

Table 6.4A.2.4.2-2: Minimum requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB |  |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB | NOTE 2 |  |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -28 | Output power > 10 dBm |  |  |  |
|  |  |  | -25 | 0≤ Output power ≤ 10 dBm |  |  |  |
| Carrier leakage | dBc | BW of 1 RB | NOTE 3 |  |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -28 |  | Output power > 10 dBm |  |  |
|  |  |  | -25 |  | 0 dBm ≤ Output power ≤ 10 dBm |  |  |
|  |  |  | -20 |  | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 |  | -40 dBm Output power < -30 dBm |  |  |
| NOTE1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit is are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.4A.2.3.1-1 apply for Table 6.4A.2.3.2-2 as well. NOTE 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. |  |  |  |  |  |  |  |

6.4A.2.4.3	Carrier leakage

Carrier leakage is an additive sinusoid waveform that is confined within the aggrecated transmission bandwidth configuration. For intra-band contiguous CA, the carrier leakage requirement is defined with applicable frequencies dependent on parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE indicated in active uplink carrier(s). For band combinations with supporting additional DC location reporting for intra-band CA, the applicable LO leakage frequency depend on the txDirectCurrentLocation indicated in the additional reporting IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. Otherwise, the applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE. For only one uplink carrier is activated, the applicable LO leakage frequency follow definition in subclause 6.4.2.The measurement interval is one slot in the time domain.

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.4A.2.4.3-1.

Table 6.4A.2.4.3-1: Minimum requirements for Relative Carrier Leakage Power

| Parameters | Relative Limit (dBc) |
| --- | --- |
| Output power > 10 dBm | -28 |
| 0 dBm ≤ Output power ≤ 10 dBm | -25 |
| -30 dBm ≤ Output power < 0 dBm | -20 |
| -40 dBm ≤ Output power < -30 dBm | -10 |

<End of change>