3GPP TSG-RAN WG4 Meeting # 96-e                                                             R4-2011781

Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.307 | CR |  | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.307 on release independent update for the Rel.16 EN-DC and NR CA/DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CHTTL, ZTE Corporation, Dish, SGS Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CA_R16_intra-Core NR_CADC_R16_2BDL_xBUL-Core DC_R16_1BLTE_1BNR_2DL2UL-Core DC_R16_2BLTE_1BNR_3DL2UL-Core DC_R16_3BLTE_1BNR_4DL2UL-Core DC_R16_4BLTE_1BNR_5DL2UL-Core DC_R16_5BLTE_1BNR_6DL2UL-Core DC_R16_xBLTE_2BNR_yDL2UL-Core NR_SUL_combos_R16-Core NR_CA_R16_3BDL_1BUL-Core NR_CA_R16_4BDL_1BUL-Core NR_CADC_R16_3BDL_2BUL-Core DC_R16_LTE_NR_3DL3UL-Core |  |  |  |  |  |  | Date: |  | 2020-08-21 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | More Rel.16 EN-DC and NR CA/DC configurations have been introduced in latest TS 38.101-1, 38.101-2, 38.101-3, an update is needed for the release independent specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update release-independent information for Rel.16 EN-DC and NR CA/DC configurations to be release independent from Rel.15. Note that the NR-NR DC within FR1 is release independent from Rel.16. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Release-independent information for EN-DC, NR CA/DC configurations is not updated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.3, 5.4(new), 6.2, 7, 8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of changes >>

5	Release independent features for NR frequency range 1

5.1	Additional NR operating bands and UE power classes for NR frequency range 1

Requirements for a Rel-16 UE for additional NR operating bands and power classes compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.1-1: NR operating bands

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |

Table 5.1-2: NR UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 1 | FDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 2 | TDD | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Power Class 3 | FDD, TDD, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |

5.2	Additional NR CA configurations for NR frequency range 1

5.2.1	Intraband CA

Requirements for a Rel-16 UE for additional NR intraband CA configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.2.1-1: NR intraband contiguous CA within FR1

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR1 | DL | B, C, D, E, F, G, H, I, J, K, L | FDD,TDD | Rel-15 | Table B.4.2-1 |
|  | UL | A, B, C | FDD,TDD | Rel-15 |  |

Table 5.2.1-2: NR intraband non-contiguous CA within FR1

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations within FR1 | DL | 2 | 1 | FDD, TDD | Rel-15 | Table B.4.2-1 |
|  |  | 3 | 1 | TDD | Rel-15 |  |
|  |  | 4 | 1 | TDD | Rel-15 |  |

5.2.2	Interband CA

Requirements for a Rel-16 UE for additional NR interband CA configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.2.2-1: NR interband CA within FR1

| Feature | DL/UL | Maximum number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations within NR FR1 | DL | 4 | 5 | A, B, C | TDD, FDD, SDL and TDD, FDD and TDD | Rel-15 | Table B.4.2-1 |
|  | UL | 2 | 2 | A | TDD, FDD, FDD and TDD | Rel-15 |  |

5.3	Additional NR SUL configurations for NR frequency range 1

Requirements for a Rel-16 UE for additional NR SUL configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.3-1: NR SUL within FR1

| Feature | DL/UL | number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the SUL configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band SUL configurations within NR FR1 | DL | 1 | 2 | A | TDD | Rel-15 | Table B.4.3-1 |
|  | UL | 2 | 2 | A | TDD and SUL | Rel-15 |  |

5.4	Additional Inter-band NR-DC configurations for NR frequency range 1

Requirements for a Rel-16 UE for additional NR-DC configurations within FR1 compared to TS 38.101-1 of Rel-16 [2] are introduced via this clause.

Table 5.4.1-1: NR-DC within FR1

| Feature | DL/UL | Maximum number of bands | number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the DC configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR-DC configurations within NR FR1 | DL | 2 | 2 | A | FDD | Rel-16 |  |
|  | UL | 2 | 2 | A | FDD | Rel-16 |  |

6	Release independent features for NR frequency range 2

6.1	Additional NR operating bands and UE power classes for NR frequency range 2

Requirements for a Rel-16 UE for additional NR operating bands and power classes compared to TS 38.101-2 of Rel-16 [3] are introduced via this clause.

Table 6.1-1: NR operating bands

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Operating bands | TDD | Rel-15 | Table B.4.1-1 |

Table 6.1-2: NR UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Power Class 1, 2, 3, 4 | TDD | Rel-15 | Table B.4.1-1 |

6.2	Additional NR CA configurations for NR frequency range 2

6.2.1	Intraband CA

Requirements for a Rel-16 UE for additional NR intraband CA configurations within FR2 compared to TS 38.101-2 of Rel-16 [3] are introduced via this clause.

Table 6.2.1-1: NR intraband contiguous CA within FR2

| Feature | DL/UL | CA BW Class | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- |
| Intra-band contiguous CA configurations within FR2 | DL | B, C, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 | Table B.4.2-1 |
|  | UL | B, D, E, F, G, H, I, J, K, L, M, O, P, Q | TDD | Rel-15 |  |

Table 6.2.1-2: NR non-contiguous intraband CA within FR2

| Feature | DL/UL | number of sub-blocks | maximum number of CCs within a sub-block | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| Intra-band non-contiguous CA configurations within FR2 | DL | 2 | 6 | TDD | Rel-15 | Table B.4.2-1 |
|  |  | 3 | 3 | TDD | Rel-15 |  |
|  |  | 4 | 3 | TDD | Rel-15 |  |
|  |  | 5 | 2 | TDD | Rel-15 |  |
|  |  | 6 | 2 | TDD | Rel-15 |  |
|  |  | 7 | 2 | TDD | Rel-15 |  |
|  |  | 8 | 1 | TDD | Rel-15 |  |
|  |  | 9 | 1 | TDD | Rel-15 |  |
|  |  | 10 | 1 | TDD | Rel-15 |  |

7	Release independent features for NR interworking between NR frequency range 1 and NR frequency range 2

7.1	Additional NR interband CA configurations between frequency range 1 and frequency range 2

Requirements for a Rel-16 UE for additional NR interband CA configurations between FR1 and FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 7.1-1: NR interband CA between FR1 and FR2

| Feature | DL/UL | number of bands | maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band CA configurations for NR interworking between FR1 and FR2 | DL FR1 | 3 | 4 | A, C | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.4-1 |
|  | DL FR2 | 1 | 8 | A, D, E, F, G, H, I, J, K, L, M | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 1 | A | FDD, TDD | Rel-15 |  |
|  | UL FR2 | 1 | 8 | A, D, G, H, I, J, K, L, M | TDD | Rel-15 |  |

7.2	Additional Inter-band NR-DC configurations between frequency range 1 and frequency range 2

Requirements for a Rel-16 UE for additional Inter-band NR-DC configurations between FR1 and FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 7.2-1: Inter-band NR-DC between FR1 and FR2

| Feature | DL/UL | number of bands | maximum number of CCs | CA BW Classes | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Inter-band DC configurations for NR interworking between FR1 and FR2 | DL FR1 | 2 | 3 | A, C | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.5-1 |
|  | DL FR2 | 1 | 8 | A, D, E, F, G, H, I, J, K, L, M | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 1 | A | FDD, TDD | Rel-15 |  |
|  | UL FR2 | 1 | 8 | A, D, G, H, I, J, K, L, M | TDD | Rel-15 |  |

8	Release independent features for NR interworking between NR and E-UTRA

8.1	Additional EN-DC configurations

8.1.1	Intraband EN-DC

Requirements for a Rel-16 UE for additional EN-DC intraband configurations within FR1 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.1-0: EN-DC intraband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Intraband contiguous EN-DC power class 1.5 | TDD | Rel-15 | Table B.4.6-1 |
| Intraband contiguous EN-DC power class 2 | TDD | Rel-15 |  |
| Intraband contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 1.5 | TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 2 | TDD | Rel-15 |  |
| Intraband non-contiguous EN-DC power class 3 | FDD, TDD | Rel-15 |  |

Table 8.1.1-1: EN-DC contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- |
| intraband contiguous EN-DC | DL | 3 | 2 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | FDD, TDD | Rel-15 |  |

Table 8.1.1-2: EN-DC non-contiguous intraband configurations within FR1

| Feature | DL/UL | maximum number of sub-blocks | maximum number of E-UTRA CCs | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| intraband non-contiguous EN-DC | DL | 3 | 3 | 1 | FDD, TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 2 | 1 | 1 | FDD, TDD | Rel-15 |  |

8.1.2	Interband EN-DC

8.1.2.1	Interband EN-DC within frequency range 1

Requirements for a Rel-16 UE for additional EN-DC interband configurations within FR1 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.1-0: EN-DC interband UE power class

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| Interband EN-DC Power Class 2 | TDD | Rel-15 | Table B.4.6-1 |
| Interband EN-DC Power Class 3 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-1: EN-DC interband configurations without SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 6 | 6 | 2 | 3 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 2 | 1 | 2 | FDD, TDD, FDD and TDD | Rel-15 |  |

Table 8.1.2.1-2: EN-DC interband configurations with SUL within FR1

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 2 | 3 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 1 | 2 | 2 | FDD, TDD, FDD and TDD and SUL | Rel-15 |  |

8.1.2.2	Interband EN-DC including frequency range 2

Requirements for a Rel-16 UE for additional EN-DC interband configurations including FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.2-1: EN-DC interband configurations including FR2

| Feature | DL/UL | number of E-UTRA bands | maximum number of E-UTRA CCs | number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL | 4 | 6 | 1 | 10 | TDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | UL | 1 | 4 | 1 | 8 | TDD, FDD and TDD | Rel-15 |  |

8.1.2.3	Interband EN-DC including frequency range 1 and frequency range 2

Requirements for a Rel-16 UE for additional EN-DC interband configurations including FR1 and FR2 compared to TS 38.101-3 of Rel-16 [4] are introduced via this clause.

Table 8.1.2.3-1: EN-DC interband configurations including FR1 and FR2

| Feature | DL/UL | maximum number of E-UTRA bands | maximum number of E-UTRA CCs | maximum number of NR bands | maximum number of NR CCs | Duplex-mode | Release independent from | requirements to be fulfilled (see 38.307 of the REL in which the CA configuration was introduced) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interband EN-DC | DL FR1 | 4 | 6 | 1 | 2 | TDD, FDD, FDD and TDD | Rel-15 | Table B.4.6-1 |
|  | DL FR2 |  |  | 1 | 8 | TDD | Rel-15 |  |
|  | UL FR1 | 1 | 2 | 1 | 1 | FDD, TDD, FDD and TDD | Rel-15 |  |
|  | UL FR2 |  |  | 1 | 4 | TDD | Rel-15 |  |

<< End of changes >>