3GPP TSG-RAN WG4 Meeting # 96-e 	                                                          R4-2012032

Electronic Meeting, 17-28 Aug., 2020

Agenda item:			4.7

Source:	Moderator (Huawei, HiSilicon)

Title:	Email discussion summary for [96e][201] NR_NewRAT_RRM_Core

Document for:	Information

Introduction

This contribution provides the summary of Rel-15 NR RRM core maintenance in Agenda 4.7. It will be used to capture the comments in the 1st round and 2nd round. The tentative agreements will be provided based on the proposals and comments.

The topics include:

- UE measurement capability (4.7.1)

- Scell activation/de-activation (4.7.3)

- BWP switching (4.7.3)

- TCI switching (4.7.3)

- Others (4.7.4)

Topic #1: UE measurement capability

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2011092 | Huawei, HiSilicon | Discussion on reporting criteria in 38.133 Observation 1: The definition of NR reporting criteria and E-URA reporting criteria are not clear. Observation 2: Observed from the lasted agreed CR [1]: Ecat,EN-DC,NR is the reporting criteria on NR frequency configured by PSCell only. Ecat,EN-DC,E-UTRA is the reporting criteria configured by PCell on E-UTRA frequency and on NR-frequency except NR PSCell and NR SCell. Observation 3: Based on observation 2, for EN-DC, the current definition for NR/E-UTRA reporting criteria means reporting criteria configured by NR PSCell/E-UTRA PSCell. Observation 4: In NE-DC, E-UTRA reporting criteria is the reporting criteria measured on E-UTRA frequency. Observation 5: 10+9×n is only the reporting criteria configured by NR PSCell not the total number of reporting criteria configured by PSCell and E-UTRA PCell. Proposal 1: - For UE configured with EN-DC: Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA, where Ecat,EN-DC,NR=10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and n is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies, Ecat,EN-DC,E-UTRA is the total number of reporting criteria configured by E-UTRA PCell except PSCell and SCells carrier frequencies, as specified in TS 36.133 [15] for UE configured with EN-DC. The inter-RAT NR reporting criteria configured by E-UTRA PCell is not included in Ecat,EN-DC,NR. |
| R4-2011093 | Huawei, HiSilicon | CR on reporting criteria for EN-DC in 38.133 R15 Summary of change: Clarify that The inter-RAT NR reporting criteria configured by E-UTRA PCell is not included in Ecat,EN-DC,NR |
| R4-2011094 | Huawei, HiSilicon | CR on reporting criteria for EN-DC in 38.133 R16 (Cat-A) |
| R4-2009904 | Apple | CR on FR2 measurement capability for R15 Summary of change: Add NR-DC mode into current UE FR2 measurement capabilty |
| R4-2009905 | Apple | Cat A CR to R4-2009904 |
| R4-2010030 | Mediatek, Apple, Huawei | CR on Inter-RAT RSTD measurements (section 9.4.4) Summary of change: Add the additional condition for MIB decoding. |
| R4-2010031 | Mediatek, Apple, Huawei | Cat A CR to R4-2010030 |
| R4-2011130 | Huawei, HiSilicon | Correction on UE measurement capability in NR idle mode R15 Summary of change: In LTE before NR is introduced, the 8 frequency layers includes the serving layer. After NR carriers is introduced, whether the 10 effective carrier frequency layers supported by UE includes serving carrier is not clear. - 10 effective carrier frequency layers supported by UE shall include serving carrier in idle mode. |
| R4-2011131 | Huawei, HiSilicon | Cat A CR to R4-2011130 |
| R4-2011134 | Huawei, HiSilicon | CR to measurement capability for NE-DC in 36133 R15 Summary of change: For NE-DC, the inter-RAT E-UTRA measurement are configured by NR PCell instead of LTE PSCell, and inter-frequency E-UTRA measurement are configured by LTE PSCell instead of NR PCell. Also, E-UTRA RSTD measurement can only be configured via NR PCell via LPP which is inter-RAT measurement. Update the requirements for NE-DC measurement capability for above issues. Also add the missing Reference [57] for 37.355. |
| R4-2011135 | Huawei, HiSilicon | Cat A CR to R4-2011134 |

Open issues summary

Sub-topic 1-1

It is thought that the definition of NR reporting criteria and E-UTRA reporting criteria are unclear. Based on the analysis, the ambiguity of Ecat,EN-DC,NR in EN-DC is addressed.

Issue 1-1: Can we agree the proposed modification for 38.133 that E-UTRA inter-RAT reporting criteria is not included in Ecat,EN-DC,NR?

- Proposal: (Huawei, HiSilicon, R4-2011092, R4-2011093/4)

  - For UE configured with EN-DC: Ecat,EN-DC,NR+Ecat,EN-DC,E-UTRA, where

    - Ecat,EN-DC,NR=10+9×n is the total number of NR reporting criteria configured by PSCell and E-UTRA PCell applicable for UE configured with EN-DC according to Table 9.1.4.2-1, and n is the number of configured NR serving frequencies, including PSCell and SCells carrier frequencies,

    - Ecat,EN-DC,E-UTRA is the total number of reporting criteria configured by E-UTRA PCell except PSCell and SCells carrier frequencies, as specified in TS 36.133 [15] for UE configured with EN-DC.

    - The inter-RAT NR reporting criteria configured by E-UTRA PCell is not included in Ecat,EN-DC,NR.

- The proposed change in the CR:

- Recommended WF

  - TBA

Sub-topic 1-2

Please provide the comments on CRs directly in Section 1.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 1-1 : No, we disagree with the approach of defining what is not included. Instead, we should say where it is included (in E_(cat,EN-DC,E-UTRA), except for serving NR carriers) |
| Nokia | Our understanding is a bit different based on the earlier discussions. E-UTRAN inter-RAT measurements configured on serving NR carriers are not counted in E_(cat,EN-DC,E-UTRA). But measurements on non-serving NR carriers will be according to Note 5 |
| ZTE | The NR inter-RAT measurements configured by E-UTRA PCell is included in E_(cat,EN-DC,E-UTRA), so there is no need to say it is not included in NR. |
| Huawei | Issue 1-1: We are fine to revise the wording as following if it is acceptable to all. “The inter-RAT NR reporting criteria configured by E-UTRA PCell is included in E_(cat,EN-DC,E-UTRA), except for serving NR carriers.” |

CRs/TPs comments collection

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011093 R4-2011094 | Ericsson : Disagree with the change: "Inter-RAT reporting criteria" can be confusing (some may think it's any NR carrier configured by E-UTRA Pcell, while the reporting criteria for serving NR carriers configured by PCell are still counted in E_(cat,EN-DC,NR) according to 38.133. Also, rather than saying where it is not included, one should say where it is included (in E_(cat,EN-DC,E-UTRA), except for serving NR carriers) |
|  | Nokia: This is CR for discussion paper 1092. Based on our comment we have a different view on this and more discussion would be needed. |
|  | ZTE: The NR inter-RAT measurements configured by E-UTRA PCell is included in E_(cat,EN-DC,E-UTRA), so there is no need to say it is not included in NR. |
|  | Huawei: We are fine to revise the wording as following if it is acceptable to all. “The inter-RAT NR reporting criteria configured by E-UTRA PCell is included in Ecat,EN-DC,E-UTRA, except for serving NR carriers.” |
| R4-2009904 R4-2009905 | Ericsson : Ok, if such band combinations were agreed |
|  | MTK: It’s fine. |
|  | Nokia: Looks ok. Suggest to remove the additional space now when changing this section anyway. |
|  | Apple: To Ericsson, I think we have FR1+FR2 NR-DC where UE has FR2 PSCC. To Nokia, I think we may need a dedicated editorial CR for cleaning up all editorial things in spec. But if Nokia strongly prefer to revise this CR, we can also do that. |
|  | NEC: OK with the change |
|  | ZTE: Rel-15 CR is fine. But for Rel-16, the frequency layer should include PCC either for FR2+FR2 NR DC. Not sure how to handle this. |
| R4-2010030 R4-2010031 | Ericsson : OK |
| R4-2011130 R4-2011131 | Ericsson : OK |
|  | MTK: It’s fine. |
|  | Nokia: ok |
|  | Apple: fine |
|  | ZTE: OK |
| R4-2011134 R4-2011135 | Ericsson : OK |
|  | MTK: It’s fine. |
|  | Nokia: ok |
|  | Apple: fine |
|  | ZTE: OK |

Summary for 1st round

Open issues

|  | Status summary |
| --- | --- |
| Sub-topic#1-1 | Four companies provided the comments. Based on Ericsson comment, Huawei proposed to change as follows: “The inter-RAT NR reporting criteria configured by E-UTRA PCell is included in E_(cat,EN-DC,E-UTRA), except for serving NR carriers.” Tentative agreements: Candidate options: Recommendations for 2nd round: Check if the new proposed revision from Huawei is acceptable. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011093 | Return to. Need to check if companies are OK with revision suggested by Huawei. |
| R4-2011094 | Return to |
| R4-2009904 | Revised. Need check if Ericsson is OK. Need capture Nokia comment. |
| R4-2009905 | Return to. Need address ZTE comment. |
| R4-2010030 | Agreed. |
| R4-2010031 | Agreed. |
| R4-2011130 | Agreed. |
| R4-2011131 | Agreed. |
| R4-2011134 | Agreed. |
| R4-2011135 | Agreed. |

Discussion on 2nd round (if applicable)

In the second round the CRs R4-2011093/R4-2011094, and CRs R4-2009904/ R4-2009905 need further discussion.

- Please Huawei trigger the email discussion for R4-2011093/R4-2011094

- Please Apple trigger the email discussion for R4-2009904/ R4-2009905.

[Comments and responses will be captured by moderator here]

| Email | T-doc status summary |
| --- | --- |
| R4-2011093 R4-2011094 |  |
| R4-2009904 R4-2009905 |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2011093 |  |
| R4-2011094 |  |
| R4-2009904 |  |
| R4-2009905 |  |

Topic #2: Signaling characteristics: Scell activation/de-activation

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2011136 | Huawei, Hisilicon | Discussion on remaining issues in SCell activation and BWP switching Observation 1: It is technically reasonable to include the TCI indication (and L1-RSRP reporting if needed) in FR1 SCell activation process, when multiple Tx beams are used by the network. Observation 2: The specification efforts to include TCI indication in FR1 SCell activation process may be heavy, especially the impact on the existing products needs to be checked. Observation 3: Although not in optimal way, the current FR1 SCell activation requirements can still work. Proposal 1: RAN4 to not update FR1 SCell activation requirements for inclusion of TCI indication. Proposal 2: Update SCell deactivation requirements as in Table 1. Table 1: Suggested SCell deactivation requirements Delay; Start of interruption window. MAC CE based; n+THARQ+3ms; Between (n+1+ THARQ) and (n+1+ THARQ+3ms). Timer based; n+3ms; Between (n+1) and (n+1+3ms). Proposal 3: Clarify that Rel-15 DCI based BWP switching requirements are only applicable for self-scheduling scenario. |
| R4-2011137 | Huawei, Hisilicon | CR on SCell activation requirements R15 Summary of changes: Current SCell deactivation requirements have some issues - The delay and interruption is defined same for MAC CE based and timer based, but for timer based there is no HARQ-ACK feedback. - The way how interruption window is defined is not aligned with latest requirement for activation. - The interruption due to SCell deactivation cannot be X+SMTC duration for intra-band case, as AGC is not involved in deactivation. Update the SCell deactivation delay and interruption requirmenets for above issues. |
| R4-2011138 | Huawei, Hisilicon | Cat A CR to R4-2011137 |
| R4-2009902 | Apple | CR on SCell deactivation requirement for R15 Summary of change: The interruption duration may cause that the ending point of interruption exceed the interruption window range in SCell deactivation requirement, and therefore the interrutption window definition for SCell deactivation shall be revised. The interrutption window definition for SCell deactivation is revised. |
| R4-2009903 | Apple | Cat A CR to R4-2009902 |
| R4-2010116 | Qualcomm Incorporated | CR to T parameters in 8.3.2 of 38.133 (SCell activation) Summary of change: Clarified the definitions of TFirstSSB and TFirstSSB_MAX to explicitly include a specific SSB within an SSB burst duration. |
| R4-2010206 | MediaTek inc. | CR for SCell activation delay in FR2 in R15 Summary of change: Regarding the description of TFineTiming , removing semi-persistent CSI-RS for CQI reporting (when applicable) |
| R4-2010207 | MediaTek inc. | Cat A CR to R4-2010206 |
| R4-2009803 | CATT | CR for TS38.133 Rel-16, Correction for SCell activation delay requirement Summary of change: Adding “max” function in SCell Activation Delay Requirement for Deactivated SCell. |

Open issues summary

Sub-topic 2-1

Issue 2-1: TCI indication for FR2 SCell activation requirements

- Proposals (Huawei, HiSilicon, R4-2011136)

  - RAN4 to not update FR1 SCell activation requirements for inclusion of TCI indication.

- Proposals (Qualcomm, R4-2010116)

  - Clarified the definitions of TFirstSSB and TFirstSSB_MAX to explicitly include a specific SSB within an SSB burst duration.

- Recommended WF

  - TBA

Sub-topic 2-2

Issue 2-2: Whether and how RAN4 would specify the starting point and ending point of interruption range for SCell de-activation

- Proposal (Huawei, HiSilicon, R4-2011136, Apple R4-2009902)

  - Update SCell deactivation requirements as in Table 1.

Table 1: Suggested SCell deactivation requirements

|  | Delay | Start of interruption window |
| --- | --- | --- |
| MAC CE based | n+THARQ+3ms | Between (n+1+ THARQ) and (n+1+ THARQ+3ms) |
| Timer based | n+3ms | Between (n+1) and (n+1+3ms) |

- Recommended WF

  - TBA

Sub-topic 2-3

Issue 2-3:TCI/resource activation for semi-persistent CSI-RS or TCI/resource configuration for periodic CSI-RS

- Proposal (Mediatek, R4-2010206/7)

  - Regarding the description of  TFineTiming , removing semi-persistent CSI-RS for CQI reporting (when applicable)

- Recommended WF

  - TBA

Sub-topic 2-4

Please provide the additional comments on the CRs in Section 2.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Subtopic 2-1 : We agree with P1 to not update FR1 SCell activation requirements for inclusion of TCI indication. Subtopic 2-2 We agree that no ACK/NACK is needed for the case of timer-based deactivation, and hence the deactivation timelines can be different for the two cases. Subtopic 2-3: We are OK with the proposal |
| MTK | 2-1: We support HW’s observation in the tdoc that in FR1 if network does not use multiple beam operation, it would be only one SSB transmitted so that TCI indication is not needed. However, if the network uses multiple beams deployment, it will have the performance issue for SCell activation when network doesn’t know which one is the suitable beam to transmit CSI-RS resources. Thus, our proposal is: There is no requirement when ssb-PositionInBurst indicates multiple SSBs for unknown SCell activation in FR1. Otherwise, we think RAN4 shall consider to add the similar L1-RSRP feedback procedure for FR1 unknown cell similar as FR2. 2-2: We’re fine with this proposal. |
| Nokia | Issue 2-1: Related to the FR1 SCell activation requirements, it is our understanding that all requirements for FR1 are based on the assumption that UE is receiving in an omnidirectional manner. Concerning change of TFirstSSB: it is not clear to us what exactly this ‘corresponding to the active TCI state’ refers to. It is not clear that a deactivated SCell has an active TCI state – if that is the intention with the change? Issue 2-2: We can agree to this change there is no HARQ for timer-based deactivation. Issue 2-3: OK |
| Apple | Issue 2-1: Fine with proposal 1. Agree with Huawei that without including TCI indication it might have some issues to both network and UE, but it does not mean R15 cannot work without this TCI in FR1 activation scenario, e.g. network may configure multiple CSI-RS resources for CQI reporting (each CSI-RS is using the same Tx beam as the corresponding SSB), and as long as UE reports one valid CQI to network, network can also know which Tx beam is relatively good to this UE. It might not be a best solution but at least the system would not break. Issue 2-2: Fine with the proposal Issue 2-3: Fine with MTK proposal. |
| ZTE |  |
| Huawei | Issue 2-1: Currently TCI indication is not part of FR1 SCell activation, and we do see some potential problems, e.g. NW may not know the best Tx beam to use. However, to add TCI indication in FR1 SCell activation may cause big specification efforts and we are a bit concerned to do so at this stage for Rel-15, especially when the FR1 SCell activation can still work with current requirements. Regarding the proposal form QC, we understand that the change will make the specification inconsistent as there is currently no TCI indication for FR1 SCell activation, so it would be unclear what the “active TCI state” refers to. Issue 2-2: Support the proposal. Issue 2-3: We are fine with the change. |
| NEC | Issue 2-1: We support proposal from Huawei that RAN4 to not update FR1 SCell activation requirements for inclusion of TCI indication at this stage as it won’t break the system and major implication may be there may be some performance degradation. Regarding the change proposed by Qualcomm, is the intention is to have separate definition for TFirsrSSB and TFirstSSB_Max for known case/unknown case and FR1/FR2. If it is not that intention, then our understanding is existing definition is fine. Issue 2-2: We are OK with the proposal |
| Qualcomm | Issue 2-1: In our understanding of Huawei’s contribute R4-2011136, they acknowledge that the current FR1 requirement is developed based on an incorrect assumption which doesn’t comply with RAN1 spec. The reason why they propose to not update FR1 SCell activation requirements for inclusion of TCI indication is mainly because nontrivial specification effort is expected, which we tend to agree to. But we have a very strong concern about that since now we can see that there are already companies that have established non-spec compliant assumptions and incorrect understanding based on the way the current FR1 requirement is defined. If companies do believe TCI state indication is not necessary because of multiple reasons mentioned in companies’ comments, we propose to send an LS to RAN1 to ask to clarify it and consider RAN4 understanding/requirements/deployment assumption when they introduce new features that can be potentially impacted. Otherwise, we should at least leave a note in RRM spec about assumptions in terms of TCI/Beam/etc. |

CRs/TPs comments collection

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011137 R4-2011138 | Ericsson: We are fine with the contents of the CR. This is issue 2-2. |
|  | MTK: We’re OK. |
|  | Nokia: As commented in the discussion we can agree to updating that the current requirement applies to deactivation command. It is also fine to add the delay requirement for timer-based deactivation. However, as the network will not know when the UE timer expires, we’re wondering if there is a need for the detailed requirement related to when the interruption shall happen? |
|  | Apple: fine. |
|  | Huawei: To Nokia, we understand NW actually knows when the timer expires. This is because the timer expiry is based on the number of slots without scheduling, so NW and UE should have common understanding about the status of the timer. |
| R4-2009902 R4-2009903 | Ericsson: Would the radio reconfiguration window be longer than can be fit in the gap? |
|  | MTK: We’re fine with this CR, but it seems HW’s CR also captures the same things. |
|  | Nokia: 9902: ok 9903: This CR seems to change the current understanding of when the interrupt happens. Current understanding is that the interrupt happen within what is stated in the current specification. However, our understanding of the proposed text is, that it states that the start of the interrupt (which may be up 3ms) can happen within the time window and last 3ms. This means that from network point of the possible interruption window effectively increased with 3ms. Hence, this would need more discussion. |
|  | Huawei: CR is fine, and we proposed the same change in our CR 1137. Is it OK for Apple to merge? |
|  | NEC: 9902 is OK with us |
| R4-2010116 | Ericsson : We are fine with the CR. |
|  | MTK: I don’t think this CR can solve the issue raised from QC. Our proposal is capturing the wording like: There is no requirement when ssb-PositionInBurst indicates multiple SSBs for unknown SCell activation in FR1. |
|  | Nokia: This would need more discussion. It is not to clear to us what the phrase ‘corresponding to the active TCI state’ refers to. This would need to be clarified. |
|  | Apple: up to the conclusion of issue 2-1 |
|  | Huawei: same comment as for 2-1. |
|  | Qualcomm: we can work on the detailed wording if companies agree to a need for the CR in principle. |
| R4-2010206 R4-2010207 | Ericsson: we are fine with the CR. This is Issue 2-3. |
|  | Nokia: ok |
|  | Apple: fine with the CR |
|  | Huawei: OK. |
| R4-2009803 | Ericsson: we are fine with the CR. Not the only thing that has been incorrectly implemented in the Rel-16 version, though. See for instance R4-2010663. Maybe consider combining all CRs covering 8.3.2 corrections into one to make it easier for MCC? |
|  | MTK: We’re fine with this CR, but we think this is an editorial CR. |
|  | Nokia: OK with the change in this CR. We share the similar view as Ericsson. Should we treat the changes in one section into one CR? |
|  | Apple: fine with CR, as other companies commented it would be better to have a merged CR for editorial change or for one specific section. |
|  | Huawei: OK. |
|  | NEC: OK. |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#2-1 | 7 Companies participated in discussion. 3 companies supported Huawei proposal. 1 company provide the alternative solution. 1 company had question to Qualcomm proposal. Qualcomm expressed the concern and proposed to address the issue by sending LS or making the assumption clearer. Tentative agreements: Candidate options: Keep Huawei and Qualcomm proposals as option. In addition, consider Mediatek’s proposal. There are three proposals for 2nd round - Option 1 (Huawei, Ericsson, Apple, NEC): RAN4 to not update FR1 SCell activation requirements for inclusion of TCI indication. - Option 2 (Mediatek): There is no requirement when ssb-PositionInBurst indicates multiple SSBs for unknown SCell activation in FR1. - Option 3 (Qualcomm): Clarified the definitions of TFirstSSB and TFirstSSB_MAX to explicitly include a specific SSB within an SSB burst duration. - Send an LS to RAN1 to ask to clarify it, or - Leave a note in RRM spec about assumptions in terms of TCI/Beam etc. Recommendations for 2nd round: Further discussion is needed. |
| Sub-topic#2-2 | 6 Companies participated in discussion. The proposal is acceptable to companies. Tentative agreements: - No ACK/NACK is needed for the case of timer-based deactivation, and hence the deactivation timelines can be different for the two cases. - Update SCell deactivation requirements as in Table 1. Table 1: Suggested SCell deactivation requirements Delay; Start of interruption window. MAC CE based; n+THARQ+3ms; Between (n+1+ THARQ) and (n+1+ THARQ+3ms). Timer based; n+3ms; Between (n+1) and (n+1+3ms). Candidate options: Recommendations for 2nd round: N/A |
| Sub-topic#2-3 | 4 Companies participated in discussion. The proposal is acceptable to companies. Tentative agreements: - Regarding the description of TFineTiming , removing semi-persistent CSI-RS for CQI reporting (when applicable). The change is as follows: Candidate options: Recommendations for 2nd round: N/A |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011137 | Return to. Check if Nokia is OK with Huawei’s response. The other companies are OK with it. |
| R4-2011138 | Return to. |
| R4-2009902 | Merged into R4-2011137. Need reply to companies’ questions. |
| R4-2009903 | Merged into R4-2011138. Need reply to companies’ questions. |
| R4-2010116 | Return to. Need further discussion. The tentative agreements will be captured in WF before agreeing on CR. |
| R4-2010206 | Agreed. |
| R4-2010207 | Agreed |
| R4-2009803 | Agreed. |

Discussion on 2nd round (if applicable)

In the second round, the CR R4-2010116, and the CRs R4-2011137 and R4-2011138 need further discussion.

- Please Qualcomm trigger the email discussion on R4-2010116 for inclusion of TCI indication for FR1 SCell activation requirements.

- Please Huawei trigger the email discussion to check if R4-2011137 and R4-2011138 are agreeable to companies and if needed try to address the comments.

[Comments and responses will be collected by moderator here]

| Email | Status summary |
| --- | --- |
| R4-2010116 |  |
| R4-2011137 R4-2011138 |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2011137 |  |
| R4-2011138 |  |
| R4-2010116 |  |

Topic #3: Signaling characteristics: BWP switching

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009601 | Apple | CR on Active BWP switch and Active TCI State Switching requirements - Rel15 |
| R4-2009602 | Apple | Cat A CR to R4-2009601 |
| R4-2011136 | Huawei, Hisilicon | Proposal 3: Clarify that Rel-15 DCI based BWP switching requirements are only applicable for self-scheduling scenario. |
| R4-2011139 | Huawei, Hisilicon | CR on BWP switching delay requirements R15 Summary of changes: DCI based BWP switching can be triggered by cross-carrier scheduling. However, cross-carrier scheduling scenario was not considered when Rel-15 BWP switching requirements were specified. The existing Rel-15 requirements cannot be directly applied for the cross-carrier scheduling scenario due to signalling delay for UE internal cross-CC communication and receive time difference. Clarify that Rel-15 DCI based BWP switching requirements are only applicable for self-scheduling scenario. |
| R4-2009906 | Apple | Further discussion on R15 BWP switching delay requirement Proposal 1: Revise the UE behaviour in current RRC based BWP switching requirement as: UE is not required to transmit UL signals or receive DL signals from slot n+THARQ+1 until the end of switching delay when THARQ ≤ TRRCProcessingDelay. Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. When THARQ > TRRCProcessingDelay, the above BWP switch requirements are not applicable. Proposal 2: Revise the UE behaviour in current UE-specific CBW change delay requirement as: UE is not required to transmit UL signals or receive DL signals from slot n+THARQ+1 until the end of this UE-specific CBW change delay when THARQ ≤ TRRCProcessingDelay. Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. When THARQ > TRRCProcessingDelay, the above UE-specific CBW change requirements are not applicable. |
| R4-2010183 | Apple | CR on Active BWP switch and Active TCI State Switching requirements - Rel15 Summary of change: - For RRC based active BWP switch and active TCI state switch requirements apply only when THARQ ≤ TRRCProcessing . - For RRC based switch UE restriction on transmit and receive shall apply from slot n + THARQ + 1 until the end of switching period. |
| R4-2010184 | Apple | Cat A CR to R4-2010183 |
| R4-2010032 | MediaTek inc. | CR on active BWP switch Summary of change: Add some details to make the specification more clear. - For RRC based BWP switch, the requirement is applied when “one or more than one” BWP configuration(s) are configured for UE.\ - For DCI and timer based switch, the requirment is applied when “more than one” BWP configurations are configured for UE. |
| R4-2010033 | MediaTek inc. | Cat A CR to R4-2010032 |
| R4-2011306 | ZTE | CR to 38.133 correction to RRC based BWP switch delay requirements Summary of changes:  NR slot length is clarified that it is based on smaller SCS. |
| R4-2011307 | ZTE | Cat A CR to R4-2011306 |

Open issues summary

Sub-topic 3-1

Issue 3-1: Clarification related to self-scheduling or cross-carrier scheduling for BWP switching

- Proposal (Huawei, HiSilicon, R4-2011136)

  - Clarify that Rel-15 DCI based BWP switching requirements are only applicable for self-scheduling scenario.

- Recommended WF

  - TBA

Sub-topic 3-2

Issue 3-2: Clarification on BWP configuration(s) for active BWP switch

- Proposals (Mediatek, R4-2010032)

  - For RRC based BWP switch, the requirement is applied when “one or more than one” BWP configuration(s) are configured for UE.

  - For DCI and timer based switch, the requirment is applied when “more than one” BWP configurations are configured for UE.

- Recommended WF

  - TBA

Sub-topic 3-3

Issue 3-3: UE behaviour for transmission or reception for BWP switching

- Proposals (Apple R4-2009906, R4-2010183, R4-2010184)

  - Revise the UE behaviour in current RRC based BWP switching requirement as:

    - UE is not required to transmit UL signals or receive DL signals from slot n+THARQ+1 until the end of switching delay when THARQ ≤ TRRCProcessingDelay. Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3].

    - When THARQ > TRRCProcessingDelay, the above BWP switch requirements are not applicable.

- Recommended WF

  - TBA

Sub-topic 3-4

Issue 3-4: Clarification on NR slot length

- Proposals (ZTE, R4-2011306/7)

  - NR slot length is clarified that it is based on smaller SCS.

- Recommended WF

  - TBA

Sub-topic 3-5

Please provide the additional comments on the CRs in Section 3.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issue 3-1 : OK since we didn’t consider cross carrier scheduling Issue 3-2 : OK Issue 3-3 : This is making things unnecessary complicated. Exisiting requirement is OK i.e., restriction starting from slot n where RRC command is received. Too late for R15 since NW scheduling is already implemented Issue 3-4 : OK |
| MTK: | 3-1: We’re fine with this proposal. 3-3: We don’t think we need this update. Before UE finishing the RRC parsing, UE will definitely transmit the HARQ feedback to NW because UE didn’t know this RRC signalling is for which procedures. By the way, the wording here is ‘UE is not required to’ instead of ‘UE is not expected to’ in scheduling restriction. Thus, UE still can feedback HARQ in BWP switching. 3-4: We’re fine with this proposal. |
| Nokia | Issue 3-1: We agree that RAN4 should define when the requirements apply. Hence, we are fine to clarify this but CR wording needs discussion. Issue 3-2: Why is this change needed? A BWP switch can only be requested if UE has more than one BWP. Issue 3-3: We can agree on the principle of the discussion and change. However, the actual CR wording needs more discussion. It is not clear why 'when THARQ ≤ TRRCProcessingDelay' is needed. Issue 3-4: OK |
| Apple | Sub-topic 3-1 We need to further check if this should be restricted to self scheduling. Sub-topic 3-2 RRC based BWP switch should occur when only 1 active BWP is configured to the UE in and RRC re-configuration is needed to change the active BWP. Hence, propose to change to: - For RRC based BWP switch, the requirement is applied when “one” BWP configuration(s) are configured for UE. For DCI based more than 1 BWP needs to be configured to the UE For timer based it switches to defaultBWP when timer expires. Only 1 BWP might be configured in that case and more than 1 is not a required condition. Perhaps the definition of number of BWP configurations needs to be clarified, if it includes the defaultBWP, initalBWP. Sub-topic 3-4 We support the update to clarify. Suggestion for definition: NR Slot Length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves SCS change. |
| ZTE | Issue 3-1 : Fine. Issue 3-2 : The original requirements would be clear enough. The BWP switch should be among already configured BWPs, no matter what type of triggering. It may not be necessary to differentiate detail BWP types for different triggering method. Issue 3-3 : In principle we agree with the analysis. However the worst case would be the HARQ feedback is after RRC procedure delay. The BWP switch should start after RRC procedure delay. Then it is not sure if UE can feedback during BWP switch time. It would be up to UE implementation. If UE can finish RRC parsing in a short time, UE is allowed to transmit HARQ feedback. If this needs to be clarified, it is better to cover all UE implementation. Issue 3-4 : We considered the wording suggested by Apple because it was used for other requirements. But we think it would be concrete to use SCS of BWP since SCS is configured via BWP configuration. Anyway this is more like how company interpret the wording. The original wording needs a little bit revision, i.e. removing the highlighted word. NR Slot length is determined by the smaller SCS between the SCS of old BWP and the SCS of new BWP switch if the BWP switch involves changing of SCS. We can further check with Apple on the wording. |
| Huawei | Issue 3-1: Support, we think the clarification is needed. Issue 3-2: We are fine with the clarification, but we have different views as APPLE about the RRC-based BWP switch. The RRC configuration will p[impose a BWP switch when the firstActiveUplinkBWP-Id/firstActiveDownlinkBWP-Id is different from the active BWP, so we think it also applies when multiple BWP are configured. Issue 3-3: Similar views as Ericsson and MTK. This issue has been discussed from the last meeting. The wording does not forbid UE from HRAQ feedback. Before UE finishes parsing the RRC message, UE will definitely transmit the HARQ feedback as UE has no idea about RRC command. Issue 3-4: We understand the point here, but we think it is a bit different from the DCI-based BWP switch where the length of the delay is described as number of slots determined by SCS. The RRC-based BWP switching delay is described as an absolute time duration as: “…on the first DL or UL slot right after a time duration of (T_RRCprocessingDelay+T_BWPswitchDelayRRC)/(NR Slot length) slots…” So is it better to say “on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRC which begins from the beginning of DL slot n, where” |
| NEC | Issue 3-1: OK with clarification Issue 3-2: We do not see the need for this clarification. Issue 3-3: This change may cause unnecessary confusion. With existing wording, gNB may not schedule DL or UL once RRC based BWP switch command is transmitted as gNB may not know when the interruption can occur. With newly proposed wording it may lead to confusion that gNB can schedule before THARQ. We feel existing wording is fine. Issue 3-4: We prefer wording suggested by Huawei. |
| MTK | To Nokia, Apple, ZTE, NEC, Our intention is to add a missing scenario of the UE configured with one BWP configuration for RRC-based BWP switch. - For DCI and timer based BWP switch, UE shall be configured with more than one dedicated BWP configurations so that these two manners would switch an old dedicated BWP to a new one based on the existing BWP configurations, i.e., at least two dedicated BWP configurations are provided to UE through RRC message. - We agree with Apple’s observation. For timer-based BWP switch, UE may be configured with only one dedicated BWP because one of the BWP switch-to or switch-from could be initial BWP. However, this issue is a corner issue from current specification. The intention of our CR doesn’t plan to modify the spec. of DCI- and timer-based BWP switch. If RAN4 can make some consensus on this issue, we’re fine to update our CR based on the latest agreements. For the RRC based BWP switch, UE may be configured with one or more than one BWP configuration(s). Compared to the DCI based and timer based BWP switch, it is possible that only one dedicated BWP configuration is configured to UE through RRC message for BWP switching. We have to update current spec. because supporting only one BWP is the mandatory feature for UE and the RRC-based BWP switch test case is based on only one BWP. |
| Qualcomm | Sub-topic 3-1: To be more precise, if scheduling-cell’s numerology is the same as old or new BWP’s numerology of scheduled-cell, the current requirement can be applied. But, in principle, we agree to the proposal. Sub-topic 3-2: In principle, looks okay. But agree to Apple’s comment. Sub-topic 3-3: Though we see the motivation of the proposal, such update may create unexpected issues if THARQ is extraordinarily large. And since network decided to change UE BWP for whatever reason, T-put loss due to early stop using the old BWP should be marginal. Sub-topic 3-4: Don’t we have to consider a scenario, e.g. RRC from PCell and BWP switch on SCell, and PCell and SCell’s new/old BWP have different numerologies? |

CRs/TPs comments collection

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011139 | Ericsson: OK |
|  | MTK: We’re fine with this CR. |
|  | Nokia: Xcarrier scheduled BWP switch is not prohibited in Rel-15 (our understanding). It is defined and our understanding that it can be used when same numerology is used in both cells. It is not clear to us that the current BWP switch requirements are only for same cell scheduled BWP switch. Currently RAN4 does not have such restriction. Hence, this should be discussed first. |
|  | Qualcomm: Similar comment as Nokia. Unless 3 different numerologies are involved in cross-carrier scheduling based active BWP switching, it is not precluded. The detailed wording needs to be refined. |
| R4-2010183 R4-2010184 | Ericsson : Making things unnecessary complicated. Exisiting requirement is OK i.e., restriction starting from slot n where RRC command is received. Too late for R15 since NW scheduling is already implemented |
|  | MTK: The same view with Ericsson. |
|  | Nokia: It is not clear why 'when THARQ ≤ TRRCProcessingDelay' is needed. Our understanding is that Tharq is always smaller than TRRCProcessing delay and there is no need for such condition. |
|  | Huawei: Similar view as Ericsson and MTK |
|  | NEC: We do not see need for this modification. |
|  | Qualcomm: Similar view with Ericsson. |
| R4-2010032 R4-2010033 | Ericsson : OK |
|  | Nokia: We do not see a need for this CR. |
|  | NEC: We do not see need for this clarification |
|  | Qualcomm: Needs to consider Apple’s comment on Sub-topic 3-2 |
| R4-2011306 R4-2011307 | Ericsson : OK |
|  | MTK: We’re fine with this CR. |
|  | Nokia: ok |
|  | Huawei: Please find our comments in Issue 3-4. |
|  | Qualcomm: Please see our comment on Sub-topic 3-4: |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#3-1 | 8 companies participated in discussion. 7 of them can agree on the proposal. 1 company proposed further check whether we should restrict to self-scheduling. Tentative agreements: Candidate options: - Clarify that Rel-15 DCI based BWP switching requirements are only applicable for self-scheduling scenario. Recommendations for 2nd round: Encourage company to further check. |
| Sub-topic#3-2 | 7 companies provided the comments. 3 companies agree the change. 3 companies think no need for the changes. 2 companies suggested the changes. Tentative agreements: Candidate options: - Option 1 (Mediatek, Ericsson, Huawei/HiSilicon): - For RRC based BWP switch, the requirement is applied when “one or more than one” BWP configuration(s) are configured for UE. - For DCI and timer based switch, the requirment is applied when “more than one” BWP configurations are configured for UE. - Option 2 (Nokia, ZTE, NEC): No need for change. - Option 3 (Apple, Qulacomm): - For RRC based BWP switch, the requirement is applied when “one” BWP configuration(s) are configured for UE. - For DCI based more than 1 BWP needs to be configured to the UE - For timer based it switches to default BWP when timer expires. Only 1 BWP might be configured in that case and more than 1 is not a required condition. - The definition of number of BWP configurations needs to be clarified, if it includes the defaultBWP, initalBWP Recommendations for 2nd round: Further discussion is needed. |
| Sub-topic#3-3 | 7 companies provided the comments. The motivation is understandable for most companies. 1 companies supported it. 5 companies think it is unnecessary. Tentative agreements: Candidate options: - Option 1 (Apple, Nokia(who has additional comments on CR)): - Revise the UE behaviour in current RRC based BWP switching requirement as: - UE is not required to transmit UL signals or receive DL signals from slot n+THARQ+1 until the end of switching delay when THARQ ≤ TRRCProcessingDelay. Where THARQ is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - When THARQ > TRRCProcessingDelay, the above BWP switch requirements are not applicable. - Option 2 (Ericsson, Mediatek, Huawei, NEC, Qualcomm): No need for change. Recommendations for 2nd round: Further discussion is needed. Majority companies think there is no need for the change. |
| Sub-topic#3-4 | 8 companies provided the comments. In principle most of them can accept the changes. But the wording needs further discussion. Tentative agreements: Candidate options: - Option 1 (ZTE, Ericsson, Mediatek, Nokia): NR Slot length is determined by the smaller SCS between the SCS of old BWP and the SCS of new BWP switch if the BWP switch involves changing of SCS. - Option 2 (Ericsson, Mediatek, Huawei, NEC, Qualcomm): Suggestion for definition: NR Slot Length is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch if the BWP switch involves SCS change. Additional changes suggested by Huawei and supported by NEC - It better to say “on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRC which begins from the beginning of DL slot n, where” Questions from Qualcomm: - Don’t we have to consider a scenario, e.g. RRC from PCell and BWP switch on SCell, and PCell and SCell’s new/old BWP have different numerologies? Recommendations for 2nd round: Further discussion is needed. For the discussion, the above comments need be taken into account. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2011139 | Return to. Nokia and Qualcomm had comments. |
| R4-2010183 | Return to. Need convince most companies. |
| R4-2010184 | Return to. |
| R4-2010032 | Return to. Need convince Nokia and NEC. Qualcomm had comments. |
| R4-2010033 | Return to. |
| R4-2011306 | Revised. Capture comments from Huawei and Qualcomm. |
| R4-2011307 | Return to. |

Discussion on 2nd round (if applicable)

In the second round, the CRs R4-2011139, R4-2010183/ R4-2010184, R4-2010032/ R4-2010033, and R4-2011306/ R4-2011307 need further discussion. Please proponents trigger the email discussions:

- Huawei R4-2011139

- Apple R4-2010183/ R4-2010184

- Mediatek R4-2010032/ R4-2010033

- ZTE R4-2011306/ R4-2011307

[Comments and responses will be collected by moderator here]

| Email | Status summary |
| --- | --- |
| R4-2011139 |  |
| R4-2010183 R4-2010184 |  |
| R4-2010032 R4-2010033 |  |
| R4-2011306 R4-2011307 |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2011139 |  |
| R4-2010183 |  |
| R4-2010184 |  |
| R4-2010032 |  |
| R4-2010033 |  |
| R4-2011306 |  |
| R4-2011307 |  |

Topic #4: Signaling characteristics: TCI switching

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010034 | MediaTek inc. | Remaining issues on signalling characteristics (TCI state switching) In this paper, we provide our views on the issue of signalling characteristics. We have the following proposals. Proposal 1: RAN4 doesn’t need to clarify UE’s behavior from n+ THARQ + 3ms to UE finishing the active TCI state switch. Proposal 2: Before UE finishing the RRC reconfiguration parsing, UE will transmit the HARQ feedback to NW. RAN4 doesn’t need to update spec. to identify the UE’s behavior during RRC reconfiguration processing. Proposal 3: Delete TOk in active TCI list update requirement. |
| R4-2010208 | MediaTek inc. | CR on TCI state switch delay in R15 Summary of change: On 8.10.3, - Separate cases for T L1-RSRP and add T L1-RSRP = 0 in FR1. On 8.10.6, - Replace TOk by 1. |
| R4-2010209 | MediaTek inc. | Cat A CR to R4-2010208 |
| R4-2011304 | ZTE | CR to 38.133 correction to TCI state switch delay requirements Summary of changes: - TL1-RSRP for SSB in FR1 and for CSI-RS in FR1 is 0 - The structure of the requirements is re-organized |
| R4-2011305 | ZTE | Cat A CR to R4-2011304 |

Open issues summary

Sub-topic 4-1

Issue 4-1: MAC based TCI state switching

- Proposal (Mediatek, R4-2010034)

  - RAN4 doesn’t need to clarify UE’s behavior from n+ THARQ + 3ms to UE finishing the active TCI state switch.

- Recommended WF

  - TBA

Sub-topic 4-2

Issue 4-2: RRC based TCI state switching

- Proposal (Mediatek, R4-2010034)

  - Before UE finishing the RRC reconfiguration parsing, UE will transmit the HARQ feedback to NW. RAN4 doesn’t need to update spec. to identify the UE’s behavior during RRC reconfiguration processing.

- Recommended WF

  - TBA

Sub-topic 4-3

Issue 4-3: Active TCI list update

- Proposal (Mediatek, R4-2010034)

  - Delete TOk in active TCI list update requirement.

- Recommended WF

  - TBA

Sub-topic 4-4

Please provide the additional comments on the CRs in Section 4.3.2.

Companies views’ collection for 1st round

Open issues

| Company | Comments |
| --- | --- |
| Ericsson | Issues 4-1 thru 4-3 : Agree with proposals |
| Nokia | Issue 4-1: we should no longer change Rel-15. This can be addressed in later release. Issue 4-2: RAN4 can clarify that UE shall send the HARQ for RRC message. This proposal is less detailed than Apple proposal in 9906. Our understanding is that UE transmit the HARQ but as we have had some questions related to the LTE HO and detailed UE behavior it would be good to make the UE behavior clear such that network can rely on the HARQ being transmitted. Issue 4-3: needs more discussion if it relates to CR in 10208 [MTK response]: It relates to CR 10208. The new target TCI state should not be in the old active TCI state list. Otherwise, this update is unnecessary when the target TCI has already listed in the active TCI state list. |
| Apple | Sub-topic 4-2 The current wording is not clear and suggests that UE need not send ACK/NACKduring RRC based switch. It needs to be clarified especially when THARQ > TRRCProcessing. Sub-topic 4-3 We are fine with the change |
| ZTE | Issue 4-1: okay. Issue 4-2: Similar to RRC BWP switch delay, same solution should be used. If change is needed, then it should cover all the possible UE implemenations Issue 4-3: okay |
| Huawei | Issue 4-1: fine with the proposal, the UE behavior is left to UE implementation in R15. Issue 4-2: for the case that THARQ < TRRCProcessing, UE behavior is undefined during RRC processing tme. For the case that THARQ > TRRCProcessing, we think this is a corner case. So generally the proposal is fine. Issue 4-3: the proposal is fine. |
| NEC | In general network may not configure/schedule THARQ more than RRC processing delay. Hence in general we agree with proposals for issue 4-1 and 4-2. Issue 4-3: Agree with the change. |
| MTK | Issue 4-2: It can be discussed together with Issue 3-3(Apple 9906). From our understanding, the current spec. is ok. THARQ > TRRCProcessing only happens when SCS=15KHz with HARQ feedback scheduling larger than 10ms. We don’t think THARQ > TRRCProcessing is a practical scenario in real field. |
| Qualcomm | Sub-topic 4-1 through 4-3: Agree with proposals in principle. |

CRs/TPs comments collection

Please provide comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2010208 R4-2010209 | Nokia: Change 1 seems agreeable. Change 2: It is not clear what it means when removing TOk? Currently it means TOk=1 and Tfirst-SSB + TSSB-proc will always be added as delay. Hence, this change needs more discussion. [MTK response]: The new target TCI state should not be in the old active TCI state list which means fine timing tracking is always needed. In other words, this TCI list update is unnecessary once the target TCI has already listed in the active TCI state list. |
|  | ZTE: Change 1: the L1-RSRP measurement time in FR1 is 0 is included in our CR(R4-2011304). In addition there is another change - The structure of the requirements looks like TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS are re-specified for some use cases, which shouldn’t be. So this change is also necessary. Overall we think change in 11304 is to make the requirements more clear. Suggest to use our CR 11304 as baseline for this type of change. 10208 can be used to capture change 2. Change 2: we agree with the change |
|  | Huawei: the change on L1-RSRP in FR1 is colliding with R4-2011304. Two schemes are provided. We can discuss which solution is better. |
|  | Qualcomm: Similar view with Huawei |
| R4-2011304 R4-2011305 | Ericsson : Do not agree that no time for L1-RSRP measurement would be needed in FR1. In our view it would provide a single shot measurement for refining timing information. |
|  | MTK: We includes the similar changes for T_L1-RSRP in FR1. We think the spec.’s impact from our changes is smaller than this CR. Thus, we suggest to choose our CR(0208) to capture this update. |
|  | ZTE: @ Ericsson: The fine timing tracking is done during TOuk*(Tfirst-SSB+ TSSB-proc), TL1-RSRP is only for Rx beam selection, which is only applicable to FR2. @MTK: We don’t understand the spec’s impact you mentioned. Technically for the change for FR1 it is totally the same. Besides by taking additional necessary change in the CR, as commented above, it would be the best structure to specify the requirements. |
|  | Huawei: the change on L1-RSRP in FR1 is colliding with R4-2010208. Two schemes are provided. We can discuss which solution is better. |
|  | MTK: @ZTE, The intention of our CR is limited changes as small as possible to current spec. |
|  | Qualcomm: Similar view with Huawei |

Summary for 1st round

Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#4-1 | 6 companies provided comments. 5 of them agree with proposals. 1 companies have different view. Tentative agreements: Candidate options: Recommendations for 2nd round: Need further discussion. |
| Sub-topic#4-2 | 8 companies provided comments. It seems companies are generally OK with it. It was proposed to discuss together with 9906. Tentative agreements: Candidate options: Recommendations for 2nd round: Need further discussion. |
| Sub-topic#4-3 | 7 companies provided comments. 6 of them agree with proposals. 1 company had comments on CR. Tentative agreements: Candidate options: Recommendations for 2nd round: Need further discussion. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 |  |  |

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2010208 | Return to |
| R4-2010209 | Return to |
| R4-2011304 | Return to |
| R4-2011305 | Return to |

Discussion on 2nd round (if applicable)

In the second round the CRs R4-2010208/ R4-2010209 and R4-2011304/ R4-2011305 need more discussion. All the CRs should be discussed together.

Please Mediatek triggered the email discussions on all the related CRs.

[Comments and responses will be collected by moderator here]

| Email | Status summary |
| --- | --- |
| R4-2010208 R4-2010209 R4-2011304 R4-2011305 |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2010208 |  |
| R4-2010209 |  |
| R4-2011304 |  |
| R4-2011305 |  |

Topic #5: Others

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009804 | CATT | CR for TS38.133 Rel-15, Correction for RRM core requirements Summary of changes: 1. “with 1 in PCell” is corrected to “with 1UL in PCell”. 2. Deleting the “intra-frequency” in clause 6.1.1.3. 3. “PTAG” and “STAG” in clause 7.1.1 are corrected to “pTAG” and “sTAG”. 4. Some small corrections. 5. Correct the formulas in RLM/BFD/CBD/L1-RSRP measurement requirements. |
| R4-2009805 | CATT | Cat A CR to R4-2009804 |
| R4-2011109 | Huawei, Hisilicon | Correction to inter-RAT measurement on NR serving carrier Summary of change: Take inter-RAT measurement on serving carrier into account in the calculation of CSSFoutside_gap. |
| R4-2011110 | Huawei, Hisilicon | Cat A CR to R4-2011109 |
| R4-2011132 | Huawei, Hisilicon | CR on correction to CSSF within gap R15 Summary of change: Update the descriptions about which MOs are candidate for gap based measurement for caculation of CSSF within gap for above issues. |
| R4-2011133 | Huawei, Hisilicon | Cat A CR to R4-2011132 |
| R4-2011308 | ZTE | CR to 38.133 correction to interruption requirements for per-FR gap in FR2 Summary of changes: - Corrected reference time for per-FR gap in FR2 in NE-DC or NR-DC - Specified interruption requirements for per-FR gap in FR2 when time reference is FR1 serving cell |
| R4-2011309 | ZTE | Cat A CR to R4-2011308 |

Open issues summary

Sub-topic 5-1

Please provide the additional comments on the CRs in Section 5.3.1.

Companies views’ collection for 1st round

CRs/TPs comments collection

Please provide the comments in the table below.

| CR/TP number | Comments collection |
| --- | --- |
| R4-2009804 R4-2009805 | Ericsson : Given the wish of the chair to set a high bar for release 15 maintenance CRs we are not sure that these changes are sufficient to merit updates of the spec. although technically we agree these are errors rather than editorial corrections. However, the question should be whether the spec could be understood (with some interpretation) without these changes. |
|  | MTK: We’re fine with the CR, but we think this is an editorial CR. |
|  | Nokia: ok |
|  | Apple: fine |
|  | Huawei: OK. |
| R4-2011109 R4-2011110 | Ericsson : The phrase “"inter-RAT measurements on NR serving carriers " in the context of 38.series is not meaningful. We think the intention is to say that when 36.133 refers to this text in 38.133 it also needs to consider NR cells configured by the LTE eNB. The trouble is that this way fixes one problem, but creates others since I would be really confused in a 38.X spec if I read this how an NR serving carrier can be an interRAT carrier. |
|  | MTK: We don’t support this CR. We have one question to HW. When one intra-frequency has already considered in NR measurements, we don't think it needs to count again from inter-RAT measurements in the CSSF. Our suggestion is to add some clarifications on these inter-RAT measurements counting into NR measurements. |
|  | Nokia: Related to the statement in reason for change ‘However, It conflicts with the calculation of CSSFoutside_gap given in 38.133. cl. 9.1.5.1. One can observe that in RAN4’s understanding only intra-frequency meansurements are considered in CSSFoutside_gap in Rel-15. Then UE don’t know how to calculate CSSF for inter-RAT NR measurments on serving carriers’. We have a different view on this. NR Inter-RAT measurements on a serving carrier do not need additional measurement time in NR as the measurements are already performed as part of the NR serving carrier measurements. They should not count twice. Hence, this CR is not agreeable. |
|  | Apple: understand the motivation from Huawei but the revision is unclear, does the “number of inter-RAT measurement on serving carriers” have some overlapping with the NR PSCell configured carriers or not? The current wording of “configured FR1 SCell” might be clarified that it includes the target carriers configured from both LTE MN and NR SN. |
|  | ZTE: If inter-RAT measurement is on serving carriers, it should be covered by measurement on serving carriers already. There should be no need to be counted as another measurement. |
|  | Huawei: To Ericsson, We agree using the phrase “NR serving carriers” in 38.133 doesn’t comply with RAN4’s previous conclusion. We’re fine to change the wording. For example, maybe we can use the phrase “measurement on serving carriers configured by E-UTRA” instead? To MTK/Nokia/Apple/ZTE, We don’t agree that inter-RAT measurement is not needed counted again. Basically it is exactly the same story with MO merging (except MO merging are only considered between inter-RAT measurements and inter-frequency measurements). So principle used during discussion on MO merging still can provide some guidance. The principle is: two MO can be considered as one only if performing merged measurement won’t cause the increase of complexity. However, it is not always the case because: - TTI boundary of LTE PCell and NR PSCell may be not aligned (async EN-DC case). - SMTC configured by PCell and PSCell can be different. - SFN of PCell and PSCell may be not aligned. - Values of deriveSSB-IndexFromCell IE can be different in configuration of PCell and PSCell. - RSSI measurement configuration can be different For 1/2/3, a SSB covered by SMTC configured by PCell may be not covered by SMTC configured PSCell and vice versa. For 4/5, UE may need additional time and resource to perform measurement. Anyway, there is no guarantee that UE is always not need to count inter-RAT measurement again in spec. For MO merging case RAN4 has defined pretty harsh condition to limit the use case of MO merging (38.133 cl.9.1.3.2) in order to ensure 1/2/3/4/5 are all avoided. Of course we can also reuse MO merging condition in calculation of CSSF_outside_gap. But we really don’t see the necessity to further complicate CSSF_outside_gap calculation (which is already very complicate now). So we prefer to just treat inter-RAT measurement as measurement on SCC. [MTK]Thank you for Huawei’s further clarification. Considering this MO merging, our proposal is to capture the wording like ‘no requirement for this frequency if these two kind of MOs can’t be merged’. Then we don’t need to update this very complicated CSSF table. |
|  | Qualcomm: Do not support the CR, and share the similar concerns with other companies |
| R4-2011132 R4-2011133 | Ericsson : OK |
|  | MTK: We’re fine with this CR. |
|  | Nokia: ok |
|  | Apple: fine |
|  | ZTE: fine |
| R4-2011308 R4-2011309 | Ericsson : OK |
|  | MTK: We don’t support this CR. In RAN2, the signalling refServCellIndicator only indicates PCell or PSCell for reference. But in RAN4, there were some discussions and agreements on how to deduce MG’s timing based on multiple CCs. Due to MRTD between multiple CCs, the DL CC’s timing can’t align with each other. Thus, RAN4 agreed to define the timing following ‘the end of the latest NR subframe occurring immediately before the configured measurement gap’. At the same time, we agree to consider the update for RAN4 spec. to align with RAN2, but how to capture this signalling needs FFS. |
|  | Nokia: ok |
|  | Apple: fine with the CR, this indicator is used to decide MG pattern follow whose SFN and subframe timeline, but the starting boundary of MG shall still follow RAN4 assumption. |
|  | ZTE: Agree with Apple. To MTK: This will not change starting and end of MG pattern. It is just what reference is used for SFN and frame derivation. Looking forward your suggestion on how to capture the signaling. |
|  | Huawei: OK |
|  | MTK: Our suggestion is to capture the note in the spec. as follow. NOTE 1: The total number of interrupted slots is based on that SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR2 serving cell. NOTE 2: Slot occurs before or after the measurement gap may be interrupted additionally if SFN and subframe reference for per-FR gap in FR2 indicated by high layer parameter refServCellIndicator is an FR1 serving cell. |

Summary for 1st round

CRs/TPs

Moderator tries to summarize discussion status for 1st round and provides recommendation on CRs/TPs Status update

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| R4-2009804 | Agreed. According to moderator understanding, one editorial CR per spec per company is allowed. And the wording is incorrect in the equation. So we wonder if company has strong view on agreeing the CR. |
| R4-2009805 | Agreed. |
| R4-2011109 | Return to. |
| R4-2011110 | Return to. |
| R4-2011132 | Agreed. |
| R4-2011133 | Agreed. |
| R4-2011308 | Revised. Capture Mediatek comments. |
| R4-2011309 | Return to. |

Discussion on 2nd round (if applicable)

In the second round the CRs R4-2011109/ R4-2011110 and R4-2011308/ R4-2011309 need further discussion.

- Please Huawei trigger the discussion for R4-2011109/ R4-2011110.

- Please ZTE trigger the discussion for R4-2011308/ R4-2011309.

| Email | T-doc status summary |
| --- | --- |
| R4-2011109 R4-2011110 |  |
| R4-2011308 R4-2011309 |  |

Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2011109 |  |
| R4-2011110 |  |
| R4-2011308 |  |
| R4-2011309 |  |