3GPP TSG-RAN WG4 Meeting #96-e	R4-2012275

# Electronic Meeting, August 17 – 28, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1017 | rev | 2 | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR 38.133 SCell dormancy switching of multiple SCells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2020-08-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | As captured in the WI exception sheet, RP-200608, delay and interruption requirements for dormancy switching of multiple SCells shall be introduced in Rel-16 scope. Such requirements are currently missing in the specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing the following changes: - (8.2.1.2.15.1-3) Correcting the SCG acronym. Extending interruption requirement to multiple CCs. Adding requirement for interruptions due to CQI measurements. Adding requirements for interruptions due to SSB-based measurements. - (8.2.2.2.12.1-3) Correcting SCG to MCG as this is NR SA. Extending interruption requirement to multiple CCs. Adding requirement for interruptions due to CQI measurements. Adding requirements for interruptions due to SSB-based measurements. - (8.2.2.2.13.1-3) Correcting SCG to MCG as this is NE-DC. Extending interruption requirement to multiple CCs. Adding requirement for interruptions due to CQI measurements. Adding requirements for interruptions due to SSB-based measurements. - (8.2.4.2A.1-3) Correcting SCG to “MCG and SCG” as this is NR-DC. Extending interruption requirement to multiple CCs. Adding requirement for interruptions due to CQI measurements. Adding requirements for interruptions due to SSB-based measurements. - (8.6.2A.1) Adding clause that introduces dormancy switching delay requirement for switching of multiple SCells. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Performance for the following cannot be guaranteed: - Switching delay for transition between non-dormancy and dormancy BWPs for multiple SCells - Interruptions on other serving cells during switching of multiple SCells between dormancy and non-dormancy. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.2.15.1, 8.2.1.2.15.2, 8.2.1.2.15.3, 8.2.2.2.12.1, 8.2.2.2.12.2, 8.2.2.2.12.3, 8.2.3.2.13.1, 8.2.3.2.13.2, 8.2.3.2.13.3, 8.2.4.2A.1, 8.2.4.2A.2, 8.2.4.2A.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev.1: Changed scope to align with R4-2012120. Rev.2: Changed to ACK/NACK loss rate [0.5]% for interruptions. Removed interruption windows. |  |  |  |  |  |  |  |  |

Unchanged Sections Omitted

First Modification

8.2.1.2.15.1	Interruptions due to SCell dormancy switch

When one SCell in SCG is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on active serving cell in SCG as defined in clause 8.2.1.2.7, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

- 	The starting time of interruption shall be within the dormancy switching delay as defined in clause 8.6.2.

When multiple SCells in SCG are switched from dormancy to non-dormancy or vice versa when the UE is in DRX active time, the interruption requirement described above applies for each BWP switch.

8.2.1.2.15.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of CQI measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.1.2.15.3, on dormant SCells, shall not exceed [0.5]%.

8.2.1.2.15.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of RRM measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements, clause 8.2.1.2.15.2, and RRM measurements on dormant SCells, shall not exceed [0.5]%.

End of First Modification

Unchanged Sections Omitted

Second Modification

8.2.2.2.12.1	Interruptions due to SCell dormancy switch

When one SCell in MCG is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on active serving cell in MCG as defined in clause 8.2.2.2.5, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

- 	The starting time of interruption shall be within the dormancy switching delay as defined in clause 8.6.2.

When multiple SCells in MCG are switched from dormancy to non-dormancy or vice versa when the UE is in DRX active time, the interruption requirement described above applies for each BWP switch.

8.2.2.2.12.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of CQI measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.2.2.12.3, on dormant SCells, shall not exceed [0.5]%.

8.2.2.2.12.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of RRM measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements, clause 8.2.2.2.12.2, and RRM measurements on dormant SCells, shall not exceed [0.5]%.

End of SECOND Modification

Unchanged Sections Omitted

Third Modification

8.2.3.2.13.1	Interruptions due to SCell dormancy switch

When one SCell in MCG is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on active serving cell in MCG as defined in clause 8.2.3.2.7, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

- 	The starting time of interruption shall be within the dormancy switching delay as defined in clause 8.6.2.

When multiple SCells in MCG are switched from dormancy to non-dormancy or vice versa when the UE is in DRX active time, the interruption requirement described above applies for each BWP switch.

8.2.3.2.13.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of CQI measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.3.2.13.3, on dormant SCells, shall not exceed [0.5]%.

8.2.3.2.13.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of RRM measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements, clause 8.2.3.2.13.2, and RRM measurements on dormant SCells, shall not exceed [0.5]%.

End of Third Modification

Unchanged Sections Omitted

Fourth Modification

##### 8.2.4.2A.1	Interruptions due to SCell dormancy switch

When one SCell in MCG or SCG is switched from dormancy to non-dormancy or from non-dormancy to dormancy [7] when UE is in DRX active time,

-	the UE is allowed an interruption on active serving cell in MCG and SCG as defined in clause 8.2.4.2.5, except that the interruption is allowed regardless of which parameters change between the dormant BWP and the non-dormant BWP

- 	The starting time of interruption shall be within the dormancy switching delay as defined in clause 8.6.2.

When multiple SCells in MCG or SCG are switched from dormancy to non-dormancy or vice versa when the UE is in DRX active time, the interruption requirement described above applies for each BWP switch.

##### 8.2.4.2A.2	Interruptions due to CQI measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of CQI measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements and RRM measurements, clause 8.2.4.2A.3, on dormant SCells, shall not exceed [0.5]%.

##### 8.2.4.2A.3	Interruptions due to RRM measurements during SCell dormancy

When one or more SCells are in dormancy, the UE is for the purpose of RRM measurements on the dormant SCell(s) allowed to cause interruptions to non-dormant serving cell(s).

The total rate of ACK/NACK feedback loss on any non-dormant serving cell resulting from CQI measurements, clause 8.2.4.2A.2, and RRM measurements on dormant SCells, shall not exceed [0.5]%.

End of Fourth Modification

Unchanged Sections Omitted