3GPP TSG-RAN4 WG4 Meeting # 96-e	R4-2012693

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR: Introduction of NR PDSCH FR1 CA 2Rx performance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Revised Rel-16 NR performance requirements enhancement WI RP-200472 is approved in RAN#87-e meeting. NR CA PDSCH normal demodulation requirements for NR CA are agreed to be defined for the following CA configs: - FDD CA with 15kHz SCS - TDD CA - 30kHz SCS + 30kHz SCS - 15kHz SCS + 30kHz SCS - TDD FDD CA - FDD 15kHz SCS + TDD 15kHz SCS - FDD 15kHz SCS + TDD 30kHz SCS |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the test structure for the NR Rel-16 PDSCH normal demodulation requirements for 2Rx FR1 CA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of Changes >>

5.2	PDSCH demodulation requirements

The parameters specified in Table 5.2-1 are valid for all PDSCH tests unless otherwise stated.

<<Unchanged Sections Skipped>>

5.2A.2	2RX requirements

5.2A.2.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.2.1-4 based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.2.1-1 ~ Table 5.2A.2.1-3, with the parameters in Table 5.2A-1 ~ Table 5.2A-2 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-cluase do not apply for UE single carrier test.

Table 5.2A.2.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.8] |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.0] |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.8] |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.0] |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.4] |

Table 5.2A.2.1-2 Single carrier performance for TDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.8] |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.8] |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.9] |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.0] |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.9] |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.2] |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.5] |

Table 5.2A.2.1-3 Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 10 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 15 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.6] |
| 20 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.7] |
| 25 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.7] |
| 30 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.7] |
| 40 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [13.9] |
| 50 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.1] |
| 60 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.0] |
| 80 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.5] |
| 90 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.3] |
| 100 | TBD | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Low | 70 | [14.7] |

Table 5.2A.2.1-4: Minimum performance for multiple CA configurations

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Table 5.2A.2.1-1 |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1-3 |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1-1 and Table 5.2A.2.1-3 per CC |
| 4 | FDD 15 kHz + TDD 15 kHz | As defined in Table 5.2A.2.1-1 and Table 5.2A.2.1-2 per CC |
| 5 | TDD 15 kHz + TDD 30 kHz | As defined in Table 5.2A.2.1-2 and Table 5.2A.2.1-3 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.x. |  |  |

<< End of Changes >>