3GPP TSG-RAN4 WG4 Meeting #96-e	R4-2012697

# Electronic Meeting, 17-28 Aug., 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | - | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR: FR1 EN-DC power imbalance requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-Perf |  |  |  |  |  | Date: |  |  | 2020-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Revised Rel-16 NR performance requirements enhancement WI RP-200472 is approved in RAN#87-e meeting. FR1 CA and EN-DC power imbalance requirements are agreed to be defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of FR1 EN-DC power imbalance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WI will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5B (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<< Start of Changes >>

9.1	General

This clause covers the UE demodulation performance requirements for EN-DC, NE-DC, inter-band NR-DC between FR1 and FR2, and inter-band NR CA between FR1 and FR2.

<<Unchanged Sections Skipped>>

9.5B	Power imbalance requirements for DC

9.5B.1	EN-DC

9.5B.1.1	General

9.5B.1.1.1	Applicability of requirements

TBD

9.5B.1.2	Intra-band contiguous EN-DC within FR1

The requirements in this section verify the ability of intra-band contiguous EN-DC UE to demodulate the signal transmitted by the NR SCG in the presence of a stronger E-UTRA MCG. The parameters specified in Table 5.xx are valid for all intra-band contiguous EN-DC power imbalance tests unless otherwise stated. The test setup for E-UTRA MCG CC is specified in Clause 9.1.2. During the test, only the PDSCH performance on the NR SCG CC shall be verified.

9.5B.1.2.1	2Rx requirements

9.5B.1.2.1.1	FDD

The performance requirements are specified in Table 9.5B.1.2.1.1-1 and the downlink physical channel setup according to Annex C.3.1.

Table 9.5B.1.2.1.1-1: Minimum performance with 15 kHz SCS

| Test Number | Bandwidth (MHz) | Reference channel |  | Power at Antenna Port in dBm/Hz |  | Reference value Fraction of Maximum Throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCG CC | MCG CC | SCG CC | MCG CC | SCG CC | SCG CC |
| 1 | TBD | NA | TBD | TBD | TBD | 85 |

9.5B.1.2.1.2	TDD

The performance requirements are specified in Table 9.5B.1.2.1.2-1 [and Table 9.5B.1.2.1.2-2] and the downlink physical channel setup according to Annex C.3.1.

Table 9.5B.1.2.1.2-1: Minimum performance with 30 kHz SCS

| Test Number | Bandwidth (MHz) | Reference channel |  | Power at Antenna Port in dBm/Hz |  | Reference value Fraction of Maximum Throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCG CC | MCG CC | SCG CC | MCG CC | SCG CC | SCG CC |
| 1 | TBD | NA | TBD | TBD | TBD | 85 |

[Table 9.5B.1.2.1.2-2: Minimum performance with 15 kHz SCS]

| Test Number | Bandwidth (MHz) | Reference channel |  | Power at Antenna Port in dBm/Hz |  | Reference value Fraction of Maximum Throughput (%) |
| --- | --- | --- | --- | --- | --- | --- |
|  | SCG CC | MCG CC | SCG CC | MCG CC | SCG CC | SCG CC |
| 1 | TBD | NA | TBD | TBD | TBD | 85 |

9.5B.1.2.2	4Rx requirements

9.5B.1.2.2.1	FDD

9.5B.1.2.2.2	TDD

9.5B.1.3	Intra-band non-contiguous EN-DC within FR1

<< End of Changes >>