3GPP TSG-RAN WG4 Meeting # 96-e												R4-2012734

Electronic Meeting, 17 – 28 August., 2020

Agenda item:			7.8.1.1

Source:	Moderator (Ericsson)

Title:	Email discussion summary [94e Bis][318] NR_L1enh_URLLC_Demod_Part1

Document for:	Information

# Introduction

This thread continues the discussion on ultra-low BLER requirements and testing from previous meetings.

Two major outstanding issues are the need for FR2 requirements and the need/feasibility of ultra-low BLER CQI tests. Company views for/against are well known, so the e-mail discussion aims to elaborate and debate further the rationale.

For the BS, there are some open issues regarding testing.

The discussion should also cover whether there is a need for any further ultra-low BLER related requirements.

The ultra-low BLER requirement is based on assuming an early pass/fail test methodology and 99.999% confidence level. The description of the testing differs for the BS and the UE. For the UE, RAN5 will develop the test and so an LS to RAN5 should be discussed. For the BS, the test details should be captured in the RAN4 conformance specification.

For the CR development, it is suggested that the first-round focusses on how to write the test description for the BS specification (and how to write the LS for the UE testing). If there is sufficient convergence on this aspect, the BS CRs for the ultra-low BLER core requirement and conformance test can be reviewed in the second round. Other CR aspects (such as FRCs etc.) will be discussed in thread 319.

Simulation results will be collected in an Excel sheet provided on the server together with this summary. Further results and updating of results during the meeting is welcome.

List of candidate target of email discussion for 1st round and 2nd round

- 1st round: Capture argumentation for/against FR2 and CQI. Resolve open BS test issues. Converge on how to write LS to RAN5 for UE and BS test text.

- 2nd round: Further debate on FR2/CQI argumentation plus decision. Review CRs for ultra-low BLER sections.

# Topic #1: The need for FR2 requirements

The question of whether to define ultra-low BLER requirements for FR2 has been open for several meetings. Companies for/against positions are known from previous meetings and from the company contributions. To avoid simply repeating positions on having/not having FR2 requirements and use the opportunity for discussion in the meeting, it is requested that companies provide input on the following two topics in the first round:

- Is operation of 10^-5 BLER in FR2 realistic?

- Are there technical considerations for implementing 10^-5 in FR2 that would need discussion in 3GPP in order to set a requirement?

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009611 | Apple | Results: SNR @1e-5 BLER; 1x2; 1x4. FDD; -0.15; -2.95. TDD; 0; -2.7. Proposal #1: Do no define requirements in FR2 for ultra-low BLER for URLLC Proposal 2: Do not define CQI reporting test in AWGN channel with ultra-low BLER target. |
| R4-2009722 | Intel | Table 1. Summary of alignment simulation results 2 Rx; 4 Rx. FDD; 0.5; -1.7. TDD; 0.4; -1.8. Table 2. Summary of impairment simulation results 2 Rx; 4 Rx. FDD; 2.5; 0.3. TDD; 2.4; 0.2. Proposal 1: Define FR2 PDSCH requirements for ultra-low BLER. |
| R4-2010976 | Huawei | Proposal 1: No requirements are needed for URLLC FR2 with ultra-low BLER. Proposal 2: RAN4 needs further discuss this feature and decide whether the new performance requirements is needed for PDCCH. Proposal 3: Not to define PDSCH performance requirements for URLLC single-DCI multi-TRP transmission. |
| R4-2011443 | Qualcomm | Proposal 1: Do not define URLLC ultra-low BLER test cases for FR2. |
| R4-2009700 | NTT DoCoMo | FR2 requirement Observation 1: If no feasibility issue is identified, RAN4 can define the ultra-low BLER requirements for FR2. The feasibility should be discussed based on simulation results. Applicability rule Proposal 1: All supported SCS need to be tested for PUSCH ultra-low BLER requirements (Option 1). |
| R4-2009724 | Intel | Table 1. Summary of alignment simulation results 10 MHz, 15 kHz; 40 MHz, 30 kHz. PUSCH mapping Type A; -8.4; -8.7. PUSCH mapping Type B; -8.4; -8.8. Table 2. Summary of impairment simulation results 10 MHz, 15 kHz; 40 MHz, 30 kHz. PUSCH mapping Type A; -6.9; -7.2. PUSCH mapping Type B; -6.9; -7.3. Proposal 1: Define FR2 PUSCH requirements for ultra-low BLER. |
| R4-2009856 | Nokia | Introduction of FR2 requirements - FR2 data transmission is experiencing exaggerated (w.r.t. FR1) spatial and frequency selectivity, as well as, deep fading conditions brought about uncontrollable changes in the environment. - RAN4 to not test ultra-low BLER in FR2, as this is not a practically viable scenario for FR2. SCS test applicability The performance difference between 15 and 30kHz is negligible. RAN4 to only require testing of one SCS (per FR), if more than one is declared to be supported. Either 15kHz, or 30kHz, or per separate manufacturer declaration are acceptable. Test tolerance for AWNG requirements AWGN testing, outside of high reliability, uses 0.3dB as test tolerance. RAN4 to consider a TT of 0.3dB for high reliability testing with AWGN channel model. Simulation results: Tx/Rx; PUSCH mapping type; SCS and BW; MCS level; Propagation condition; DMRS; SNR (dB). Ideal; Impair-ment. 1T2R; type A; 15kHz, 5MHz; MCS5; AWGN; DMRS 1+1; -6.5; -4. 1T2R; type A; 15kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -7.3; -4.8. 1T2R; type A; 30kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -6.5; -4. 1T2R; type A; 30kHz, 40MHz; MCS5; AWGN; DMRS 1+1; -7.6; -5.1. 1T2R; type B; 15kHz, 5MHz; MCS5; AWGN; DMRS 1+1; -6.6; -4.1. 1T2R; type B; 15kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -7.3; -4.8. 1T2R; type B; 30kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -6.6; -4.1. 1T2R; type B; 30kHz, 40MHz; MCS5; AWGN; DMRS 1+1; -7.6; -5.1. |
| R4-2010840 | Ericsson | Proposal 1: Discuss and agree how to capture the test method and expected confidence level in the specification. Proposal 2: No FR2 ultra-low BLER test |
| R4-2010978 | Huawei | Proposal 1: No requirements are needed for URLLC FR2 with ultra-low BLER. Proposal 2: Only 15 kHz SCS or 30 kHz SCS needs to be tested. Proposal 3: PUSCH repetition type B should be further discussed in RAN4. Proposal 4: RAN4 needs to further study this feature that whether the performance requirements are needed. |
| R4-2010282 | Samsung | Proposal 1: Deprioritize FR2 URLLC requirement for ultra-lower BLER Proposal 2: Only 15KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities. (Moderator comment: The following proposals from this contribution are relevant to the ultra-low BLER thread) |

## Open issues summary

### Sub-topic 1-1: 10^-5 BLER scenario in FR2

Sub-topic description: In this sub-topic, companies should provide views on whether the 10^-5 scenario is realistic. Please avoid simply stating “realistic/unrealistic” and provide some reasoning for your view that can be discussed.

Responses to other companies’ arguments for/against the scenario being realistic are encouraged in order to move forward with the discussion.

Issue 1-1: Applicability of 10^-5 BLER in FR2 scenario

- Proposals

  - Option 1 (Nokia): Operating 10^-5 BLER in FR2 is realistic. The reasoning why we view it as realistic is ….

  - Option 2: Operating 10^-5 BLER in FR2 is not realistic. The reasoning why we view it as not realistic is ….

- Recommended WF

  - TBA

### Sub-topic 1-2: Issues to consider when setting an FR2 requirement

Sub-topic description: This sub-topic discusses whether there are issues that would require further work and investigation to set an FR2 requirement. Please do not simply state “There are issues” if you believe there are some; outline what they are. Responses to other companies’ argumentation on issues are encouraged in order to move forward with the discussion.

Issue 1-2: Any issues for setting an FR2 requirement

- Proposals

  - Option 1: We see no issues; just simulate and decide

  - Option 2 (Nokia): We see the following issues that would need to be considered: …

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 1-1: Applicability of 10^-5 BLER in FR2 scenario We do not see as operating ultra-low BLER systems in FR2 as realistic. FR2 data transmission is experiencing exaggerated (w.r.t. FR1) spatial and frequency selectivity, as well as, deep fading conditions brought about by uncontrollable changes in the environment. We would like to hear in which scenarios 1e-5 BLER systems are expected to be deployed in FR2. Issue 1-2: Any issues for setting an FR2 requirement We do see issues, i.e., option 2. The current simulation setting of using AWGN will produce a SNR result that can also be tested in a practical testing time. However, while the AWGN assumption is required for feasible testing, it does not hold in FR2. The tested SNR value will have no correlation with practical performance. At least a fading model would need to be simulated first, to decide if FR2 has a chance of achieving a 1e-5 BLER. Even then the requirement would not be in line with practical use cases. |
| Ericsson | Issue 1-1: Considering the need for beam management for FR2 we believe it is not obvious whether extreme BLER with no retransmissions is achievable. As indicated below, some more considerations are needed for the requirement, and before working no such considerations we should be sure that a requirement is useful. Issue 1-2: For FR2 there may be some factors that impact the requirement conditions and level such as the beamforming mechanism and stability, any calibration loops etc. in the RF. Also, the ability of the test equipment setup (including SG, PAs etc) to ensure complete fidelity with no error probability needs consideration (for FR1, we established this at least for conducted requirements). We should take care not to rush into a requirement without ensuring such considerations are considered. |
| Qualcomm | Issue 1-1: Given the higher pathloss in FR2, we are not sure if 1e-5 BLER can be achieved in practical FR2 deployment. So, we prefer not to define FR2 high reliability requirements. |
| Apple | Issue 1-1: Achieving 10-5 BLER in FR2 seems unrealistic given higher pathloss, need for beam management in FR2. We are not aware of practical deployments requiring ultra-low BLER in FR2. Hence, we propose not to introduce requirements for ultra-low BLER in FR2. |
| Samsung | Issue 1-1: Although both FR1 and FR2 are the targeting scenario for URLLC, as indicated in the specification 38.824, most of URLLC use cases (i.e. Rel-15 enabled use case, factory automation, transport industry and electrical power distribution), are evaluated under FR1, such as 700MHz. In that sense, FR1 should be the practical deployment for URLLC in current stage. For FR2, OTA test will be proceeded, we are not sure 10^-5 BLER in FR2 can apply for OTA test. Meanwhile, FR2 is operated in the TDD mode, the testing time should be considered due to the number of unavailable UL slots |
| Huawei | Issue 1-1 By considering the high pathloss and CQI feedback, we think this is unrealistic for FR2 to achieve 10^-5. We propose do not define the performance requirements at this stage. |
| Intel | Issue 1-1: We think that one of potential use case for FR2 URLLC is industrial area, for which reliability of transmission is rather important. Issue 1-2: Based on our understanding, OTA environment responsible for setup of required SNR level during the test. Taking into account that beam management/search will be applied before running of demodulation test and beam will be locked during the test, at current stage, we don’t see any issue with FR2 testing for Ultra-low BLER. |

### CRs/TPs comments collection

No CRs

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Realistic / not realistic: 6 companies (Nokia, Ericsson, Qualcomm, Apple, Samsung(?), Huawei) indicated that they do not believe or have doubts that operation of 10^-5 BLER in FR2 is realistic. Reasons expressed by these companies why the operation is not realistic include: - Spatial and frequency selectivity and deep fades due to uncontrollable environment changes - Beam management impacts - Higher pathloss and CQI feedback - Evaluations done for FR1 One company (Intel) indicated that they believe that FR2 operation is realistic in an industrial use case. Issues (or not) for setting requirement: 3 companies (Nokia, Ericsson, Huawei) indicated that they see some issues that would need to be resolved to set a FR2 requirement. - AWGN cannot be used for realistic testing in FR2 (as it will not correlate with actual performance) - Beamforming mechanism and stability need to be considered for the requirement - RF impacts for FR2 may impact BLER performance more - OTA Test equipment setup ability to achieve complete fidelity for FR2 - TDD slot pattern lengthening test time One company (Intel) argued that beamforming will not impact testing as the beam search will be performed before testing and the beam locked for the test. Recommendations for 2nd round: For the companies arguing FR2 requirement is not realistic: Explain further why you believe operation in an industrial scenario is not realistic (or alternatively indicate if you think it in fact is) For the company arguing operation is realistic: Provide comments to the reasons/concerns raised about the requirement being realistic. Recommended to be discussed in GTW to resolve whether to go ahead and define the requirements or not. |

Recommendations on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | WF on ultra-low BLER requirement (It is proposed that a single WF is created for this thread) | Moderator (Ericsson) |

### CRs/TPs

None

## Discussion on 2nd round (if applicable)

Issue 1-1: Applicability of 10^-5 BLER in FR2 scenario

Please provide further comments on the following reasons why it is suggested that the FR2 requirement is not realistic:

- Spatial and frequency selectivity and deep fades due to uncontrollable environment changes

- Beam management impacts

- Higher pathloss and CQI feedback

- Evaluations done for FR1

It is suggested that the requirement may be realistic in an industrial environment. Please provide further argument why/why not

Issue 1-2: Any issues for setting an FR2 requirement

Please provide further comments on the following issues which have been identified in respect to FR2 requirements:

- AWGN cannot be used for realistic testing in FR2 (as it will not correlate with actual performance)

- Beamforming mechanism and stability need to be considered for the requirement

- RF impacts for FR2 may impact BLER performance more

- OTA Test equipment setup ability to achieve complete fidelity for FR2

- TDD slot pattern lengthening test time

It is suggested that with locking of the beam direction (after beam setting before testing commences) will ensure FR2 testing works. Please comment further on this.

Issue 1-3: Create FR2 requirements

Based on the discussion of Issue 1-1 and 1-2, opinion on whether to create FR2 requirements:

- Option 1: Yes

- Option 2: No

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 1-1: Even in a relatively controlled industrial environment, the issue of blocking/shadowing cannot be excluded in FR2. It seems rather irresponsible for a factory floor operator to rely on URLLC to achieve 1e-5 BLER in all circumstances and contingencies need to be in place. We think that the 1% BLER discussed in 319 are much closer to realistic use cases for URLLC in FR2. The previous RAN1 Rel-15 baseline evaluations in TR 38.824 (for usefulness of Rel-15 features in achieving Rel-16 KPIs) have seemingly largely avoided FR2. Issue 1-2: The point of “• AWGN cannot be used for realistic testing in FR2” is a heavy simplification of our statement in the 1st round. We expect that in practical FR2 deployments, the KPI of 1e-5BLER cannot be sustained in all common environment configurations. Hence usage of AWGN to model the performance” in the cases where it actually works” is misleading. The beam locking testing is only relevant to UE demod and there it can only make FR2 1e-5 BLER work in testing. This will not guarantee performance anywhere close to a stable 1e-5 BLER performance in real applications and might raise false expectations. Issue 1-2: No. |
| Ericsson | Issue 1-1: Regarding the industrial environment, it is not clear that the impact of beam management (i.e. avoiding a situation where a glitch can ever occur) is mitigated by the environment being an industrial environment. There can still be blockages, changes in path etc. or some RF effects that cause momentary glitches. Issue 1-2: The beam can of course be pointed and locked for the testing, but more thought is needed as to whether there are low probability RF events (e.g. calibration, PA linearization impacts) which with low probability cause glitches and which the requirement would need to allow for. The close integration and complex beamforming challenges for FR2 mean that FR1 behaviour cannot simply be assumed (and anyhow beamforming may not be applied for FR1). Issue 1-3: In our view, there is no need for 10^-5 BLER requirements in this scenario. |
| Apple | Issue 1-1: Achieving a 10-5 BLER in FR2 in a industrial environment doesn’t seem practical to us. We have higher pathloss in FR2 and the UE needs to perform beam refinement and management in FR2 which might not be able to guarantee meeting such low BLER in reality. Issue 1-2: Although the methodology of beam lock might ensure that beam refinement is not required in the test, in reality or practical deployment we cannot guarantee that it will not introduce additional performance floor or degradation. Issue 1-3: Option 2: No requirements are introduced in FR2. |
| Intel | Issue 1-3: Create FR2 requirements Taking into account, that this topic is under discussion for several meeting and we are only company who propose this, we are fine not to introduce FR2 Ultra-low BLER requirements in the Rel-16 time frame. |
| Huawei | Issue 1-1: It is very difficult for CQI feedback in such low BLER for FR2. In addition, the pathloss is very high in FR2. With beaforming mechanism implemented in FR2, it is not realistic to achieve 10^-5 BLER. Issue 1-3. No performance requirements for FR2 with ultra-low BLER. |
| Samsung | Issue 1-2 For FR2, there is OTA test limitation up to 20 SNR dB, we are not sure whether there is test issue for FR2 with ultra-low BLER under OTA test environment Issue 1-3: No performance requirements for FR2 with ulra-low BLER, |

Moderator: Agreement in 2nd round GTW was as follows:

Agreement: Not introduce FR2 requirements in Rel-16 for URLLC with ultra-BLER.

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012646 | Way Forward on ultra-low BLER requirements: Propose to approve |

# Topic #2: CQI requirement for ultra-low BLER

It is well known from previous meetings that some companies think that a CQI requirement is feasible and necessary, whereas other companies take the view that there is no need and/or feasibility for a CQI requirement. To move the discussion forward, in the first round under this topic companies are encouraged to provide views on and discuss the following:

- If you prefer not to define a CQI requirement, is it because you see it as unfeasible or as unnecessary?

- Is the feasibility of the requirement basically related to test time or some other consideration?

- Can the early termination method be used for the CQI requirement?

- What is the expected test time for the CQI requirement?

- Should an applicability rule be defined between CQI and FMCS?

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009611 | Apple | Results: SNR @1e-5 BLER; 1x2; 1x4. FDD; -0.15; -2.95. TDD; 0; -2.7. Proposal #1: Do no define requirements in FR2 for ultra-low BLER for URLLC Proposal 2: Do not define CQI reporting test in AWGN channel with ultra-low BLER target. |
| R4-2009722 | Intel | Table 1. Summary of alignment simulation results 2 Rx; 4 Rx. FDD; 0.5; -1.7. TDD; 0.4; -1.8. Table 2. Summary of impairment simulation results FDD; 2.5; 0.3. TDD; 2.4; 0.2. Proposal 1: Define FR2 PDSCH requirements for ultra-low BLER. |
| R4-2009723 | Intel | Proposal 1: Do not define URLLC CQI requirements for ultra-low BLER. |
| R4-2010980 | Huawei | Proposal 1: We propose to define the CQI test for ultra-low BLER under AWGN channel and reuse the BLER criteria. Proposal 2: No rule; both of FMCS and CQI tested if CQI reporting test is agreed to be introduced in RAN4. |
| R4-2011444 | Qualcomm | Proposal 1: Define CQI reporting tests for testing 99.999% reliability under AWGN condition. Proposal 2: Define a lower bound for median reported CQI in the CQI reporting tests for 99.999% reliability. Observation 1: It is possible to have an applicability rule between CQI reporting test and fixed MCS test under AWGN. Observation 2: Only one long test needs to be run for testing CQI reporting under AWGN condition for 1e-5 BLER with 99.999% confidence level. Observation 3: SNR required to achieve 1e-5 BLER for different CQIs have enough difference that 0.5dB SNR difference to accommodate testability will not change reported CQI. Observation 4: Similar to fixed MCS test for ultra-low BLER, long test duration for CQI reporting test can be reduced by using the same X dB relaxation as in fixed MCS test. Proposal 3: Define CQI reporting test under AWGN condition with 99.999% confidence level. Proposal 4: Define an applicability rule between CQI reporting test and FMCS test under AWGN to reduce the number of tests as below:  If UE shows < 1e-5 BLER at the same SNR for an MCS greater than or equal to MCS in fixed MCS test, UE automatically passes the fixed MCS test.  If UE shows > 1e-5 BLER at the same SNR for an MCS less than or equal to MCS in fixed MCS test, UE automatically fails the fixed MCS test. Observation 5: RAN5 never used early pass/fail criteria for CQI reporting tests because the test duration for existing CQI reporting tests is very small. Observation 6: It is easy to apply early pass/fail criteria for CQI reporting tests when running the test for BLER criteria at median CQI and median+/-1 CQI similar to fixed MCS test cases. Observation 7: With CQI reporting test under fading, proponents had proposed to remove the BLER criteria. In that case, we cannot fulfil the purpose of CQI reporting tests. Observation 8: With CQI reporting under AWGN condition with higher BLER criteria, we will not be able to test the UE for 1e-5 BLER, which was the purpose of having CQI Table 3. |

## Open issues summary

### Sub-topic 2-1 Feasibility and necessity

Sub-topic description: The aim of this sub-topic is to understand whether the CQI requirement is or is not necessary and feasible. Companies are requested to clarify why they see the requirement as feasible/necessary (or not so). For feasibility, whether it relates to test time or some other factor.

Companies are encouraged to respond to other companies views during the discussion in order to move forward.

Open issues and candidate options before e-meeting:

Issue 2-1: Feasibility and necessity of CQI requirement

- Proposals

  - Option 1: We view a CQI requirement as both feasible and necessary. The reason it is necessary is:

  - Option 2: We view a CQI requirement as feasible but not necessary. The reason it is not necessary is:

  - Option 3: We view a CQI requirement as neither feasible nor necessary. The reasons it is not necessary and not feasible are:

  - Option 4: We view a CQI requirement as necessary but not feasible. The reason it is necessary is:

- Recommended WF

  - TBA

### Sub-topic 2-2 Early termination

Sub-topic description: This sub-topic aims to resolve the question of whether early termination is feasible during the CQI test. Please indicate why you do (not) think early termination is feasible.

Open issues and candidate options before e-meeting:

Issue 2-2: Early termination

- Proposals

  - Option 1: Early termination is feasible for CQI. Please state why

  - Option 2: Early termination is not feasible. Please state why.

- Recommended WF

  - TBA

### Sub-topic 2-3: Test time.

Sub-topic description: This sub-topic aims to resolve the expectation on test time.

Open issues and candidate options before e-meeting:

Issue 2-3: Test time

- Proposals

  - We expect the test time for CQI to be as follows …, based on the following calculation …

- Recommended WF

  - TBA

### Sub-topic 2-4: Applicability rule.

Sub-topic description: This sub-topic captures whether an applicability rule is feasuble

Open issues and candidate options before e-meeting:

Issue 2-4: Applicability rule

- Proposals

  - Option 1: Applicability rule between CQI and FMCS

  - Option 2: No applicability rule

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Ericsson | Issue 2-1: We believe that testing at 10^-5 and not some other BLER is needed for verification of the CQI. It may not be obvious that testing with 99.999% confidence is feasible. However, as long as FMCS is tested with 99.999% confidence we think that it would be acceptable to do the CQI test with a lower confidence level, which could reduce the test time. (For example, 99% confidence). We think a lower confidence level is acceptable, since the CQI is the link quality report, not the link itself and anyhow in real life the network cannot be 100% confident in the CQI report (since future interference and channel conditions are not 100% predictable from the CQI). So assuming some higher confidence level or simplification of the test, we believe that it can likely be realistic and feasible. Issue 2-2: It may be that the method can be applied even if the MCS switches. Another possibility is to consider that in reality, the network will need to allow some margin above the CQI report (considering that interference and channel are not perfectly predictable) and define the test such that the network schedules a fixed MCS with some margin above the average CQI. Then the early termination would apply. Alternatively, the test could apply (average CQI+1) fixed MCS and then (average CQI-1) MCS, but early termination may not happen as fast. Issue 2-3: With a lower confidence level, the test time can be quite short. In R4-1912163 we estimated 132 errors need to be observed worst case for 99% confidence level. This would be around 3.5 hours test time in the worst marginal case. However, assuming that the underlying BLER would be lower due to the BS adopting an MCS with some margin to the CQI, the test time would be much shorter in reality due to early pass; likely in the order of 10 times less. Issue 2-4: Assuming that a different confidence level and/or fixed MCS with margin would be used for keep CQI test time manageable, then no applicability rule should be applied. In case the confidence level is 99.999% then it could be considered, depending on how the test is defined. |
| --- | --- |
| Qualcomm | Issue 2-1: We prefer option 1 because CQI table 3 was designed for 1e-5 BLER. In our paper, we had showed how we can use the early pass/fail methodology and also consider the factor X to reduce the test time, similar to FMCS test. In our paper, we have shown simulation results for different CQI and it shows that SNR required for reaching 1e-5 BLER at each CQI have enough margin that factor X = 0.5dB won’t impact the result of the test. Issue 2-2: Option 1. In existing CQI reporting tests, RAN5 does not use early pass/fail method because the test duration itself is very small as shown in Intel’s paper. However, when we are targeting 1e-5 BLER, we can use early pass/fail criteria when running the BLER test at median CQI and median+/-1 CQI since in those tests, we are running a fixed MCS test with some expectation of BLER (1e-5 in this case). Issue 2-3: For the test time, test to find median CQI only depends on channel realizations. So, that can stay the same as existing CQI reporting definition tests (1000 ms). Then, for BLER test at median CQI, early pass/fail method will determine the test time. If UE BLER is 1e-5 or lower, test time will be same as FMCS test. If UE BLER is higher than 1e-5, test time will be a few mins. Based on this BLER, we will have to run the test for median +1 or median-1 CQI. If we had already run the long test (i.e., BLER was lower than 1e-5), this test will have higher BLER and test duration will be small and vice-versa. Therefore, test time will be time taken for FMCS long test + 1000ms + few mins depending on how high UE BLER is for higher BLER test. This test time seems feasible. Issue 2-4: Option 1. If UE is behaving as it is supposed to, applicability rule can reduce the number of tests. As shown in our paper, the SNR difference between two CQIs is large enough that if UE is behaving correctly, we should be able to apply the applicability rule proposed in our paper. If UE is not behaving correctly, conditions of proposed applicability rule may not be met. |
| Apple | Issue 2-1: Feasibility and necessity of CQI requirement Option 4: We view a CQI requirement as necessary but not feasible for 10-5 BLER. The reason it is necessary is to test the new CQI table with a different target BLER. In our opinion we can still test the new CQI table in fading channel without the necessity to test a very low BLER. With CQI test in AWGN channel, we cannot guarantee that at most only 1 long test would need to be run. Issue 2-2: Early termination Option 2: We are not convinced that early termination can be applied based on the same methodology as fixed MCS. When the Median CQI is determined, we need to determine the BLER by running long enough to determine whether BLER is lower or higher than 10-5. With early pass/fail, by observing some number of errors and running some number of samples, we can guarantee with some confidence that the BLER requirement is met or not met. Issue 2-4: Applicability rule Even if we finally decide on CQI reporting test in AWGN, an applicability rule might not be feasible. There is no guarantee that all companies report the same CQI for a given SNR. The FMCS test is defined with MCS14 with a SNR requirement based on average results from companies. Typically, when we compare CQI reporting results from companies, there is some distribution and not all companies report same median CQI at a given SNR. So, we don’t think an applicability rule can eventually be established. |
| Huawei | Issue 2-1: The CQI table 3 is designed for 10^-5. To verify the CQI table, the BLER target should be 10^-5. The only consideration is the long test time. The MCS14 is agreed for the FMCS test, if a test applicability rule is defined for CQI and FMCS, the corresponding CQI is CQI 8 which is not the new added CQI index comparing with the old CQI table. In CQI table 3, CQI 0- CQI 5 are the new added indices. From our understanding, it is better to test the CQI 0 – CQI 5. Issue 2-2: With proper confidence level, the early termination is feasible for CQI. But we need to discuss whether the extra margin of “X” is needed or not. Issue 2-4: As we state in issue 2-1. With applicability rule, CQI 8 will be tested which is not the new CQI index in table 3 comparing with the old CQI table. |
| Intel | Issue 2-1: Feasibility and necessity of CQI requirement We think that CQI requirements are necessary, but feasibility is rather questionable. The reason it is necessary is verification of correct CQI reporting for CQI table 3. The reason it is not feasible is higher testing time in comparison to FMCS test, because several SNR points are verified in the one CQI test (2 or 4 depending on DUT). Issue 2-2: Early termination We understand that early pass/fail methodology can be considered for CQI requirements. Same time, we think that methodology with SNR shift to trigger early pass cannot be considered for CQI test. This methodology is applicable to scenario with fixed MCS because information about expected BLER for tested SNR is available. Same time, information about BLER for {median CQI, median CQI+1/-1} is not available. If BLER for MCS corresponding to median CQI for SNR = Y is higher than 10^-5 then BLER for SNR = Y+X will be lower and can be rather close to 10^-5. Therefore, such methodology can increase CQI test time. Issue 2-3: Test time If we keep confidence level same as for FMCS then based on our calculations from R4-2000370 we expect the following test time for 15 kHz SCS: - In case UE can pass the test for first SNR point in each SNR set: ~6.66 hrs [(3.3 hrs in case BLER for Med CQI is around 10^-5 and 0.03 hrs in case BLER for Med CQI is around 0.0001)*2 SNR points] - In case UE can pass the test only for second SNR point in each SNR set and fail testing for first SNR point in each set: 7.74 [0.27*4 (under assumption that BLER for Med CQI and Med CQI+1 is lower than 10^-5 for first SNR point in each set) + 6.66 hrs] Same time, if high confidence level is considered for CQI requirements (based on suggestion from E///) then further investigation in testing time is needed. Issue 2-4: Applicability rule Option 2. If it will be agreed to define CQI requirements for Ultra-low BLER than we think that applicability rule between CQI and FMCS should not be considered because test setup can be different and it is rather hard to guaranty that MCS from FMCS test will be covered during CQI test (due to different UE implementation). |

### CRs/TPs comments collection

No CRs

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Issue 2-1: All companies view a CQI requirement at 10^-5 as necessary (but some companies that see the requirement as necessary do not see it as feasible). 3 companies (Ericsson, Qualcomm, Huawei) view the CQI requirement as feasible. - One approach to improve feasibility is to test at a lower confidence level than for the FMCS requirement - Another, possibly complimentary approach is to test with a fixed MCS based on the median CQI (median CQI+1/median CQI-1 or median CQI+margin) - One company (Qualcomm) believe that even if tested with 99.999% CL, early termination can be applied and test time is acceptable. - From the companies that view the test as feasible, estimates of test time are 0.5-3.5 hours (dependent on CL and test design) and 1000msec+few minutes 2 companies (Apple, Intel) had concerns on the feasibility. - Test time could be very long. Several SNR points may need to be verified. Early termination may not work properly - From the companies that view the test as not feasible, test time was estimated as being up to 6-8 hours. Regarding the ability to apply an applicability rule: 4 companies (Ericsson, Huawei, Apple, Intel) expressed the view that the applicability rule probably cannot be used: - Test setup, CL etc. may differ between CQI and FMCS - CQI operates with different MCS to FMCS 2 companies (Qualcomm, Ericsson) considered that the applicability rule may be usable if the same CL and BLER are applied and the Ue behaves as expected. Proposals for second round: - Discuss Confidence level: - Is it acceptable to operate the CQI test with a higher CL than the FMCS test ? What CL is proposed ? - Option 1: 99.999% - Option 2: 99% - Option 3: 95% - Other options… - Discuss possibilities for how to test: - Follow (CQI+1, CQI-1) - Test (median CQI+1), (median CQI-1) - Test (median CQI + margin) - Other… - For the companies with concerns on feasibility, do any of the combinations of CL + test approach resolve your concerns on test time ? If not, why not ? It is recommended that this topic is taken up in the GTW session. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | See section 1.4.1 |  |

### CRs/TPs

None

## Discussion on 2nd round (if applicable)

Issue 2-1: Is the CQI test feasible if a lower confidence level is used than for the BLER test ?

- Option 1: We are OK to define the requirement with 95% confidence

- Option 2: We are OK with 99% confidence

- Option 3: Yes, with 99.999% confidence

- Option 4: We do not see the requirement as feasible with any confidence level

- Please explain why you indicate the confidence level you do (or not feasible); what test time do you expect ?

Issue 2-2: How to define the requirement:

- Option 1: Median CQI, BLER for CQI+1/CQI-1

- Option 2: Median CQI, BLER for median CQI+1/medianCQI-1

- Option 3: Median CQI, BLER for median CQI+margin

Issue 2-3: Applicability rule

Moderator: This question is only applicable if Option 3 for 2-1 and Option 1 for 2-2 are agreed. Propose to come back to the discussion after resolving 2-1 and 2-2.

| Company | Comments |
| --- | --- |
| Ericsson | Issue 2-1: We are OK to define CL 95% or 99%. We believe that if we do this, in particular combined with a fixed CQI related MCS for the BLER test then test time will be reasonable (30 minutes or less). Issue 2-2: No strong view but we tend toward option 2 or option 3. |
| Apple | Issue 2-1: The only feasible approach seems to be to introduce CQI reporting with lower confidence level to avoid very long test in AWGN condition. But, would it be acceptable to introduce a CQI test with lower confidence level than used for FMCS BLER test? What does that imply from an overall performance requirement? We still prefer to introduce test in fading channel with modified metric as we don’t have to deal with long testing times and seems more practical. |
| Intel | Issue 2-1: Is the CQI test feasible if a lower confidence level is used than for the BLER test? Lower confidence level probably can be one of solution to resolve CQI testing time issue. We can analyze it and decide based on this analysis whether to define CQI requirements for AWGN conditions in the next RAN4 meeting. |
| Huawei | Issue 2-1: We prefer option 3. To align with the FMCS test. There is no particular reason to use other confidence level for 10^-5 BLER, which have been discussed a lot when define the FMCS test. Issue 2-2: We prefer option 2. |

Moderator: Agreement from GTW was as follows:

Work assumption: Introduce ultra-BLER static CQI test cases under the assumption that test time is similar as Fixed MCS test cases ; further discuss how to ensure reasonable test time with below possible options:

-Decrease confidence level (from 99.999% ~95%)

-Early pass/fail criteria

Note both options adopted not excluded.

Please use the following space to add further comments and discussion after the GTW session:

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 2-1: Prefer Option 3. To Intel: Regarding test time, it seems that the test time is getting long because you are considering two sets of tests. Can’t we just define single test at the same SNR (and SNR+1) as FMCS test instead of two SNR sets? In that case, the test time will be similar to FMCS test. Issue 2-2: Prefer Option 2. |
| Intel | Issue 2-1: Is the CQI test feasible if a lower confidence level is used than for the BLER test? Usually CQI requirements are defined to different SNR regions to ensure correct reporting under different conditions. We are open to further discuss whether definition of requirements for one SNR region can meet the test purpose. Same time, even if one SNR region will be used, we still observe that CQI testing time will be ~16 times longer than FMCS test (i.e. 3.33 hrs vs 17 min). Therefore, we prefer to analysis another CL values to estimate test time. Issue 2-2: How to define the requirement: We prefer to reuse typical CQI testing approach: Med CQI, BLER for Med CQI and Med CQI+1 or Med CQI-1 |

Moderator: Based on the above comments, Issue 2-2 is reformulated as follows in the WF. Please comment if this is acceptable. Also, Qualcomm please clarify whether you in fact mean “Median CQI, BLER for median CQI and median CQI+1 or median CQI – 1” as opposed to “Median CQI, BLER for median CQI+1/medianCQI-1”

Issue 2-2: How to define the requirement:

- Option 1: Median CQI, BLER for median CQI+1/medianCQI-1

- Option 2: Median CQI, BLER for median CQI and median CQI+1 or median CQI - 1

| Company | Comments |
| --- | --- |
| Qualcomm | Issue 2-1: To Intel: We are not sure why it will be 16 times the FMCS test time. Can you please explain more? In our opinion, we only need one long test when we are testing for 1e-5 BLER and other tests should not have significant test time. Issue 2-2: We meant Option 2, similar to existing AWGN tests. |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012646 | Way Forward on ultra-low BLER requirements: Propose to approve |

# Topic #3: BS open issues

This topic resolves the open issues for the BS test spec

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2009700 | NTT DoCoMo | FR2 requirement Observation 1: If no feasibility issue is identified, RAN4 can define the ultra-low BLER requirements for FR2. The feasibility should be discussed based on simulation results. Applicability rule Proposal 1: All supported SCS need to be tested for PUSCH ultra-low BLER requirements (Option 1). |
| R4-2009856 | Nokia | Introduction of FR2 requirements - FR2 data transmission is experiencing exaggerated (w.r.t. FR1) spatial and frequency selectivity, as well as, deep fading conditions brought about uncontrollable changes in the environment. - RAN4 to not test ultra-low BLER in FR2, as this is not a practically viable scenario for FR2. SCS test applicability The performance difference between 15 and 30kHz is negligible. RAN4 to only require testing of one SCS (per FR), if more than one is declared to be supported. Either 15kHz, or 30kHz, or per separate manufacturer declaration are acceptable. Test tolerance for AWNG requirements AWGN testing, outside of high reliability, uses 0.3dB as test tolerance. RAN4 to consider a TT of 0.3dB for high reliability testing with AWGN channel model. Simulation results: Tx/Rx; PUSCH mapping type; SCS and BW; MCS level; Propagation condition; DMRS; SNR (dB). Ideal; Impair-ment. 1T2R; type A; 15kHz, 5MHz; MCS5; AWGN; DMRS 1+1; -6.5; -4. 1T2R; type A; 15kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -7.3; -4.8. 1T2R; type A; 30kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -6.5; -4. 1T2R; type A; 30kHz, 40MHz; MCS5; AWGN; DMRS 1+1; -7.6; -5.1. 1T2R; type B; 15kHz, 5MHz; MCS5; AWGN; DMRS 1+1; -6.6; -4.1. 1T2R; type B; 15kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -7.3; -4.8. 1T2R; type B; 30kHz, 10MHz; MCS5; AWGN; DMRS 1+1; -6.6; -4.1. 1T2R; type B; 30kHz, 40MHz; MCS5; AWGN; DMRS 1+1; -7.6; -5.1. |
| R4-2010978 | Huawei | Proposal 1: No requirements are needed for URLLC FR2 with ultra-low BLER. Proposal 2: Only 15 kHz SCS or 30 kHz SCS needs to be tested. Proposal 3: PUSCH repetition type B should be further discussed in RAN4. Proposal 4: RAN4 needs to further study this feature that whether the performance requirements are needed. |
| R4-2010282 | Samsung | Proposal 1: Deprioritize FR2 URLLC requirement for ultra-lower BLER Proposal 2: Only 15KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities. (Moderator comment: The following proposals from this contribution are relevant to the ultra-low BLER thread) |

## Open issues summary

Two issues are identified, both of which relate to BS testing

### Sub-topic 3-1 SCS test applicability for FR1

Sub-topic description: Test applicability when both SCS supported in FR1

Open issues and candidate options before e-meeting:

Issue 3-1: SCS test applicability for FR1

- Proposals

  - Option 1: If both 15kHz and 30kHz SCS are supported, both are tested. The following is the reason why testing of both is needed and it is insufficient test coverage if only one is tested:

  - Option 2: If both 15kHz and 30kHz SCS are supported, 15kHz SCS is tested. The following is the reason why testing of only 15kHz is needed and provides sufficient test coverage to assure performance with 30kHz:

  - Option 3: If both 15kHz and 30kHz SCS are supported, 30kHz SCS is tested. The following is the reason why testing of only 30kHz is needed and provides sufficient test coverage to assure performance with 15kHz:

  - Option 4 (Nokia): If both 15kHz and 30kHz SCS are supported, the manufacturer can declare/decide which one is tested. The following is the reason why testing of only one of either 15kHz or 30kHz is needed and provides sufficient test coverage to assure performance with the non-tested SCS:

- Recommended WF

  - TBA

### Sub-topic 3-2 Test tolerance

It is proposed to adopt a TT of 0.3dB for this requirement, which is defined in AWGN

Open issues and candidate options before e-meeting:

Issue 3-2: Test tolerance

- Proposals

  - Option 1 (Nokia): Adopt 0.3dB

  - Option 2: Different proposal; please provide and explain

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 3-1: SCS test applicability for FR1 Given that the performance difference between 15 and 30kHz is negligible, the ultra-low BLER testing is exorbitantly expensive, and that implementation of all declared supported SCSs is already tested in Rel-15, we see it justified to ease testing load via giving the manufacturer the choice which one to test. I.e., Option 4. Issue 3-2: Test tolerance AWGN testing, outside of high reliability, uses 0.3dB as test tolerance. It should be left up to TE vendor input, if this should be changed for URLLC. |
| Ericsson | Issue 3-1: We think option 3 is sufficient. Receiver performance with different SCS is tested with the existing requirements. There is no reason to believe that additional mechanisms that would impact ultra-low BLER decoding would arise due to the SCS. Issue 3-2: We are Ok with option 1; due to no fading channel simulator the TT reduces to 0.3dB. |
| NTT DOCOMO | Issue 3-1: Originally, we proposed option 1 to follow Rel-15 applicability rule. If we need to reduce the number of tests, we prefer option 4. The SCS to be tested should be chosen and declared since the typical SCS depends on the operator and the operating band. Issue 3-2: We agree with option 1 which is based on the same principle as other AWGN tests. |
| Sasmung | Issue 3-1: SCS test applicability for FR1 We are fine with option4, to reduce the number of tests. Since the current test is based on AWGN, the performance is not expected to show the difference with different SCS. Issue 3-2: Test tolerance We are fine with option 1, which is similar with AWGN tests in Rel-15 |
| Huawei | Issue 3-1: We prefer option 4. By considering the long test time, only 1 long test is enough. Besides, Rel-15 has already covered all the SCS. Issue 3-2: We are ok with option 1. |
| Intel | Issue 3-1: SCS test applicability for FR1 We support Option 3, because it reduces final testing time, which is one of controversial topic for this item. Same time, Option 4 also can be considered in case it will be supported by many companies. Issue 3-2: Test tolerance We are fine with Option 1. |

### CRs/TPs comments collection

No CRs or TPs

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Tentative agreements: Test tolerance: Adopt 0.3dB Candidate options: Number of SCS to test: 5 companies support to test one SCS, arguing that test time is saved and performance in AWGN would not be expected to differ. 1 company argues that 2 SCS should be tested because this is the release 15 approach. Recommendations for 2nd round: Number of SCS: Check whether it is possible to compromise to testing single SCS for ultra-low BLER. If not possible, candidate for discussion in GTW. Check whether if a single SCS is tested option 4 is agreeable or the companies mentioning option 3 have some concern. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | See section 1.4.1 |  |

### CRs/TPs

None

## Discussion on 2nd round (if applicable)

Issue 3-1: SCS test applicability for FR1

- Proposals

  - Option 1: If both 15kHz and 30kHz SCS are supported, both are tested.

  - Option 3: If both 15kHz and 30kHz SCS are supported, 30kHz SCS is tested.

  - Option 4: If both 15kHz and 30kHz SCS are supported, the manufacturer can declare/decide which one is tested.

- Recommended WF

  - Moderator proposal: Considering views expressed so far, it is proposed to compromise to Option 4 because it is not expected that in AWGN the performance will differ between SCS or that sources of BLER would differ (unlike the case for fading channel tests). Please indicate if you object to this.

[Nokia]: No objection. Option 4 is fine.

[Intel]: Option 4 is fine for us.

[Huawei]: Agree with the recommended WF.

[Samsung]: we are ok for option 4

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012646 | Way Forward on ultra-low BLER requirements: Propose to approve |

# Topic #4: Other

This topic provides places for comments on the workplan, discussion on whether further ultra-low BLER requirements are needed and discussion on the need/contents of an LS to RAN5

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2010994 | Huawei | Workplan |
| R4-2010976 | Huawei | Proposal 1: No requirements are needed for URLLC FR2 with ultra-low BLER. Proposal 2: RAN4 needs further discuss this feature and decide whether the new performance requirements is needed for PDCCH. Proposal 3: Not to define PDSCH performance requirements for URLLC single-DCI multi-TRP transmission. |
| R4-2011435 | Qualcomm | LS on Test Methodology for UE URLLC Ultra Low BLER Tests |
| R4-2010978 | Huawei | Proposal 1: No requirements are needed for URLLC FR2 with ultra-low BLER. Proposal 2: Only 15 kHz SCS or 30 kHz SCS needs to be tested. Proposal 3: PUSCH repetition type B should be further discussed in RAN4. Proposal 4: RAN4 needs to further study this feature that whether the performance requirements are needed. |
| R4-2010282 | Samsung | Proposal 1: Deprioritize FR2 URLLC requirement for ultra-lower BLER Proposal 2: Only 15KHz SCS tested for ultra-lower BLER requirement if BS supports both 15 KHz SCS and 30 KHz SCS Proposal 12: FFS on the test methodology for test metric of 99.9999% and low latency requirements (0.5ms~1ms), FFS on the performance requirements based on Rel-16 URLLC functionalities. (Moderator comment: The following proposals from this contribution are relevant to the ultra-low BLER thread) |

## Open issues summary

### Sub-topic 4-1 Workplan

Sub-topic description: Please provide any comments on ultra-low BLER aspects of the workplan

Open issues and candidate options before e-meeting:

Issue 4-1: Workplan

### Sub-topic 4-2 LS to RAN5 on test methodology

Sub-topic description: This sub-topic should discuss whether to send an LS to RAN5 and the contents

Open issues and candidate options before e-meeting:

Issue 4-2: LS to RAN5

- Proposals

  - Option 1: Agree to send the LS in R4-2011435

  - Option 2: Agree to send an LS, but have some comments to the text in R4-2011435 (please clarify)

  - Option 3: Disagree to send an LS for the following reason:

- Recommended WF

  - TBA

### Sub-topic 4-3 Additional requirements/tests for rel-16 RAN1 features

Sub-topic description: Please comment on whether an ultra-low BLER test is needed for the mentioned features (Other higher BLER tests for these features can be discussed in thread 319)

Open issues and candidate options before e-meeting:

Issue 4-3: Additional requirements/tests for rel-16 RAN1 features

- Proposals

  - PDCCH enhancements

  - Multi-TRP for URLLC

  - PUSCH repetition type B

  - UCI enhancements

  - 10^-6 BLER

- Recommended WF

  - TBA

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 4-1: Workplan The workplan will be heavily influence by the decisions concerning issue 4-3-3. But assuming the work should be finished in the next meeting, there only remains the CR delivery for next meeting. Issue 4-2: LS to RAN5 We are not sure how to judge the submission of the LS proposal [R4-2011435] before the start of this meeting, due to the rather clear chair guidance on this matter. However, we agree that information about how RAN4 arrived at the requirements, needs to be forwarded to RAN5. If this is done via an LS or informally, is of lesser concern to us. If the LS is used, we would like for it to become open to direct commenting and modification in the draft folder. We have some corrections concerning the representation of Nokia’s earlier contributions within it; it would be quickest to just include them by ourselves. Issue 4-3: Additional requirements/tests for rel-16 RAN1 features Given the time it took for RAN4 demod to come to an agreement concerning Rel-15 features, we see it infeasible to add further features to this WI at this point. In particular, 1e-6 BLER seems unlikely to conclude and multi-TRP might not be a good fit for this WI. It seems promising to add this task to the RAN plenary discussion concerning the content of Rel-17 further enhancement demod WI (moderator: CTC). |
| Ericsson | Issue 4-1: It may take another meeting cycle to agree the final simulation results Issue 4-2: We are in general OK to send an LS, because the decision to use 99.999 CL should be communicated. We think some more description is needed on the motivation for the requirement and description of what X is. We also think that it should be clarified that the Annex is advisory; RAN5 can decide the decision step risk but they should ensure 99.999% CL. Update 2020-08-19: We are OK to work on an LS, but wonder if it is really needed to send it this meeting ? The LS relates to requriements that have not formally been agreed and introduced to the RAN4 specifications yet. Maybe the right time to send the LS is once we agree the CRs and requriements ? Issue 4-3: We view all issues except 10^-6 BLER as considerations for high BLER requirement, not ultra-low BLER. Regarding 10^-6 BLER, to set a requirement would need further consideration on the associated confidence level, test time, X etc. We do not see this as so necessary in the current WI timescale. |
| Qualcomm | Issue 4-2: We prefer to send an LS because we need to inform RAN5 about the parameter X and confidence level. We have uploaded the LS to draft folder (draft_R4-201xxxx_LS_URLLC.docx ) and companies can edit it based on their comments. Issue 4-3: We have very little time left in this WI to be able to conclude on 1e-6 BLER at this time. |
| Apple | Issue 4-1: We don’t have a conclusion on CQI reporting testcases yet and results alignment will take another meeting, once we conclude on the testcase definition. The work plan needs to be adjusted for that for Rel-15 requirements. If we have additional testcases for Rel-16, we would need at least 2 meetings to finalize requirements. Issue 4-2: We are fine with sending LS to RAN5 Issue 4-3: Should we discuss all the listed topics in this thread or split them with the other thread? For ultra-ultra-low BLER of 10-6, we would need a study all over again to determine the confidence interval, testing time, feasibility. We are not sure if we have time in this WI for that. |
| Samsung | Issue 4-1: Workplan Based on the WID of Rel-16 URLLC, the performance requirements, including UE/BS demodulation requirements, will be specified. Considering there are still some open issues existed based on Rel-15 URLLC feature, we prefer to deprioritize the requirement specified for Rel-16 as least in Q3. Considering the time line, we are not sure whether we can finalize Rel-16 feature requirement within 2 meeting cyclic. Issue 4-3: Additional requirements/tests for rel-16 RAN1 features Compared with Rel-15, the high reliability requirement and lower latency requirement are further tighter in Rel-16, e.g., higher reliability (up to 10^-6) and short latency in the order of 0.5 to 1ms. We wonder if we consider 10^-6, whether the existed methodology for 99.999% can be applied, from test methodology perspective As for multi-TRP URLLC has already discussed under the Emmo WI, we prefer to focus on other feature related with single TRP operation discussion. |
| Huawei | Issue 4-1: According to the URLLC WI, Rel-15 and Rel-16 features are all included. We need to finish Rel-16 in this WI. Issue 4-2: We are ok to send the LS to RAN5. Issue 4-3: We prefer to have the general discussion about the Rel16 features in thread 319, and only concentrate 10^-6 in this thread if any. |
| Intel | Issue 4-2: LS to RAN5 We support sending LS to RAN5, but plan to provide some comments. Issue 4-3: Additional requirements/tests for rel-16 RAN1 features Requirements for URLLC schemes for Multi-TRP scenarios is already under discussion on Rel-16 Emimo work item. We suggest to keep this discussion there. In this AI, we can continue discussion on whether to define requirements for other Rel-16 URLCC features. |

### CRs/TPs comments collection

No CRs or TPs

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Workplan: One company indicated that more time may be needed to agree CRs, one company indicated that there is a dependency on decisions on rel-16, one company that more time may be needed for the CQI, one company proposed to deprioritize rel-16 and one company that Rel-16 should be finished in this WI. LS to RAN5: A concern was expressed about the submission of the LS considering the RAN4 meeting guidelines (LS submitted as separate document to discussion document). Most companies support sending an LS to RAN5 (and none opposed), but several indicated they would like to discuss the wording. One company questioned whether it is premature to send an LS from this meeting. Rel-16 requirements: No company suggested that an ultra-low BLER requirement should be created for any of the Rel-16 features other than 10^-6 BLER. For the 10^-6 BLER requirement, 4 companies indicated concerns that it can be done within the WI timescale, and a further company indicated that a discussion on the test method and time would be needed (without indicating a view on whether it should be done within the Rel-16 WI) Tentative agreements: No ultra-low BLER requirements for the following features within this WI (high BLER requirements for these features to be discussed separately in thread 319): - PDCCH enhancements - Multi-TRP for URLLC - PUSCH repetition type B - UCI enhancements Candidate options: Recommendations for 2nd round: Workplan: Rapporteur encouraged to take comments into account. No further discussion/action in the meeting. LS to RAN5: Chair to confirm if LS can be treated and allocate a document for a revision if so. Interested companies to comment on whether the LS should be sent from this meeting or later. Assuming the LS should be sent, interested companies to discuss wording using the drafts folder in the second round. 10^-6 BLER: Indicate views on the following: - Option 1: 10^-6 BLER requirements in rel-16 - Option 2: No 10^-6 BLER requirements in rel-16 If there are differing views, companies are encouraged to give a list of what tasks they think are needed to create 10^-6 BLER requirements. If there are differing views, it is recommended to take up the topic in GTW. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | LS on Test Methodology for UE URLLC Ultra Low BLER Tests (LS to RAN5) | Qualcomm |

### CRs/TPs

None

## Discussion on 2nd round (if applicable)

Issue 4-1: LS to RAN5

- Option 1: Send an LS to RAN5 this meeting

  - Please provide proposed revision to Qualcomm LS in drafts folder

- Option 2: Postpone sending an LS to RAN5 until requirement and final CRs are ready

Issue 4-2: 10^-6 BLER

- Option 1: Create a requirement for 10^-6 BLER in release 16

- Option 2: Do not create a requirement for 10^-6 BLER in release 16

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 4-1: We are fine to send an LS and have uploaded an annotated version of the draft into the drafts folder. Issue 4-2: Given the remaining time and issues generally expected with 1e-6 BLER test feasibility, it does not seem feasible to create a requirement for 10^-6 BLER in release 16. |
| Ericsson | Issue 4-1: Although we support to send an LS, we think it is probably better not to send an LS from this meeting, and to send it when the requirement and CR are agreed. We have uploaded a proposed update to the draft CR. I have included the proposed Nokia changes in the uploaded version and marked them (the change marks are all mine because of the way I did the editing) Issue 4-2: We prefer option 2 because significant work may be needed to re-evaluate test time, feasibility, X etc. to develop such a requirement, and we do not see it as needed within the rel-16 timescale. |
| Apple | Issue 4-1: Option 1. We are fine to send LS. We have one minor correction over Ericsson’s version annotated and uploaded Issue 4-2: Option 2. We prefer not to introduce requirement for 10-6 BLER as it would take significant time and effort to evaluate test time, test parameters and feasibility for 10-6 BLER. |
| Intel | Issue 4-1: LS to RAN5 The latest version of LS (corrected by Apple) is fine for us. Issue 4-2: 10^-6 BLER We are open to further discuss definition of requirements for 10^-6 BLER. At least, we can check whether feasible testing time can be reached using methodology from 10^-5 BLER analysis. |
| Huawei | Issue 4-1: We are fine to send an LS to RAN5 and the latest version of LS is fine for us. Issue 4-2: Option 2 By considering the test time when we defined the 10^-5 BLER performance requirements, we prefer do not create 10^-6 BLER test. |
| Samsung | Issue 4-1: We are fine to send LS to RAN5 Issue 4-2: Option 2 Based on timeline of WI, it is not realistic to define 10^-6 BLER within two meeting, since the target will impact on the confidence level, test time as well as additional margin. |
| Qualcomm | Issue 4-1: We prefer sending the LS to RAN5. We have uploaded some wording corrections on ftp. Issue 4-2: Given the limited time left for R16, we prefer Option 2. |

Moderator:

Issue 4-1: All companies seem OK to send the LS this meeting apart from Ericsson. Moderator proposal is to send the LS this meeting; please clarify if any objection.

Issue 4-2: Many companies prefer not to define 10^-6 BLER in rel-16, however Intel and Samsung views are not quite clear. Can we agree to not define 10^-6 BLER requirements ? Please clarify if you would object to this conclusion.

| Company | Comments |
| --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012646 | Way Forward on ultra-low BLER requirements: Propose to approve |
| R4-2012647 | LS on Test Methodology for UE URLLC Ultra Low BLER Tests: Propose to approve |

# Topic #4: CR development

It is proposed to discuss CR development as follows:

Step 1: Thread 319 confirms the overall specification structure for URLLC requirements. This thread discusses how to express the early pass/fail test methodology in the BS specification.

Step 2: Once Thread 319 has confirmed on the overall specification structure and step 1 the test description for BS, in this thread CRs on ultra-low BLER core requirement, test methodology and requirement, test setup annex and test tolerance annex are discussed.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2011340 | Nokia | draftCR for 38.104: High reliability and low latency BS demod requirements Moderator note: See moderator proposal for how to handle CRs. This CR may be revised accordingly. |
| R4-2010986 | Huawei | CR to TS 38.101-4: Applicability rules for URLLC UE demodulation requirements Moderator proposal: Discuss this CR in thread 319 |
| R4-2010836 | Ericsson | Draft CR to 38.141-2: Introduction of URLLC 0.001% BLER requirement |
| R4-2010837 | Ericsson | Draft CR to 38.141-1 on test methodology and FRCs for URLLC and test requirements for 0.001% BLER |
| R4-2010840 | Ericsson | Proposal 1: Discuss and agree how to capture the test method and expected confidence level in the specification. Proposal 2: No FR2 ultra-low BLER test |
| R4-2010990 | Huawei | CR to TS38.141-1 Test applicability for URLLC BS demodulation requirements Moderator proposal: Discuss this CR in thread 319 |

## Open issues summary

Step 1 will be handled in the first round.

### Sub-topic 5-1 Description of early pass/fail methodology

Sub-topic description: Description of the early pass/fail methodology for the BS spec

Open issues and candidate options before e-meeting:

Issue 5-1: Early pass/fail methodology

- Proposals

  - The following text has been provided for the BS spec. Please discuss and comment the text. Copy/paste the text to the comments section and update it (or propose new text in the comments section) if needed. If a large amount of space is needed to propose new text, consult with moderator on whether to create a folder and provide the proposal in a separate document.

For each of the reference channels in table 8.2.x.5-1 to 8.2.x.5-8 applicable for the base station, measure the BLER. BLER is re-evaluated every time a new block error is encountered. The test may be stopped when either an early pass criterion is met, an early fail criterion is met, or a maximum test time is reached.

The definition of an early pass or early fail is as follows. No decision is allowed before the minimum test time is elapsed. The minimum test time is TBD. After the minimum test time, after every slot the number of errors (ne) and total number of slots (ns) should be counted.

The BLER is calculated as (ne)/(ns). Each time (ne) increases, an early pass or early fail decision can be attempted.

Early fail: bler/10^-5 ≥ blerlimfail

blerlimfail(D,ne)=2*neqchisq(D,2*ne)	(1)

Early pass: bler/10^-5  ≤blerlimbadpass

blerlimbadpass(D,ne)=2*ne*Mqchisq(1-D,2*ne)	(2)

With

D:	wrong decision probability for a test step. The value of D is [TBD] This is numerically evaluated and is designed such that the probability of an incorrect decision at the end of the test is 0.001%.

M:	bad DUT factor. [TBD]

qchisq:	inverse-cumulative-function of the chi-squared-distribution

If the number of slots (ns) is greater than or equal to TBD and (ne) is equal to zero, then early pass can be evaluated assuming (ne=1). If the early pass criterion is met assuming (ne=1) in these circumstances then the device may be passed.

If the maximum test time is reached and the early pass criterion has not been met then the test is failed.

### Sub-topic 5-2 Description of X factor

Sub-topic description : Discussion on how to capture the additional X factor in the BS specification

Open issues and candidate options before e-meeting:

Issue 5-2: How to capture X

- Proposals

  - It is proposed to capture the following on X in the test tolerance table. Please provide your views.

Formula: SNR + TT + 1dB

1dB is added to the test requirement to facilitate early test pass. The BLER delivered by the device during the test will be lower than the test requirement, which enables compliance to the requirement to be demonstrated with a number of observed block errors lower than a certain threshold.

## Companies views’ collection for 1st round

### Open issues

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 5-1: Early pass/fail methodology The discussion on this subject will need to go into deep technical detail and is unlikely to conclude in this meeting. In our general opinion, the methodology description should be included as an appendix in 38.141-1/2 and only describe the higher reliability testing statistical background/process (FFS to limit description to only 1e-5). The need to describe the methodology in 38.104 is not yet clear to us, but the CR split from the last WF is assumed to still be in force. The descriptions should be based on the LTE early termination frame work, using asymmetric per step risks (for false early fail and false early pass), and calculations based on inverse binomial distributions. A table of pre-calculated decision coordinates should be included, which can also serve to force a minimum, as well as maximum, testing time (in samples). The whole LTE description of shared risk principle and test design is not required. Issue 5-2: How to capture X We think that the note does not need to be that explicit. The following note should suffice: “Formula: SNR + TT + 1dB 1dB is added to the test requirement to allow test termination within a feasible time frame” |
| Intel | Issue 5-2: How to capture X Note from proposal looks fine for us. It provides comprehensive description of reason to add 1 dB. @ Nokia: Based on our understanding, wording “feasible time frame” is rather contradictory for spec drafting. |

### CRs/TPs comments collection

| CR/TP number | Comments collection |
| --- | --- |
| R4-2011340 | Moderator: CR should be revised and discussed in the 2nd round. 1st round focusses on how to describe the test. |
|  | [Nokia]: Our main intention with this CR was threefold: 1) Introduce a subsection in “PUSCH” called “for lower latency”, which hold all future features/requirements pertaining to low latency, no matter what the latency will be. 2) Introduce a subsection in “PUSCH” called “for higher reliability”, which hold all future features/requirements pertaining to higher reliably, no matter what the BLER will be (as long as it is lower than 10%). 3) Capture both 1e-5 and 1% BLER in “for higher reliability”. The minimum requirement tables are then titled with the exact feature tested (e.g., “2 symbols” or “PUSCH repetition type A”). |
| R4-2010836 | Moderator: CR should be revised and discussed in the 2nd round.1st round focusses on how to describe the test. |
|  | Apple: The requirements applicability doesn’t capture CQI table 3 (cqi-TableAlt) as agreed in last meeting for ultra-low BLER tests. |
| R4-2010837 | Moderator: CR should be revised and discussed in the 2nd round.1st round focusses on how to describe the test. |
|  | [Nokia]: Could Ericsson please read and comment our rationale behind R4-2011340 above? We would prefer to not have separate subsection for each BLER level of higher reliability. Furthermore, we prefer to have the statistical testing explanations moved to an appendix (as in LTE). [Ericsson] The main thing is to make sure that the low latency, 1% slot aggregation/high reliability and 10^-5 requirements are treated separately; a BS may support a subset of them. Also in case a lower BLER requirement would be introduced the BS should be able to support 10^-5 only if so declared. We have a (not strong) preference to make this clear through sectioning. We suggest to discuss how to split the sections in thread 319 though. |

## Summary for 1st round

### Open issues

Moderator tries to summarize discussion status for 1st round, list all the identified open issues and tentative agreements or candidate options and suggestion for 2nd round i.e. WF assignment.

|  | Status summary |
| --- | --- |
| Sub-topic#1 | Recommendations for 2nd round: Issue 5-1: Prior to working on text drafting and based on the feedback, companies are encouraged to express views on the following. The group should aim to get agreements on these issues by the latest end of the meeting in order that drafting can be done for next meeting: - Should the methodology be captured in an Annex or in the text on test methodology ? - If in an Annex should the Annex capture only 10^-5 BLER and 99.999% CL or also other BLER ? - Should asymmetric per-step risks be assumed ? - Should a table of decision co-ordinates be included or simply a formula for assessing early pass/fail ? Issue 5-2: Two options have been presented for the note on how to capture X: - Option 1: 1dB is added to the test requirement to facilitate early test pass. The BLER delivered by the device during the test will be lower than the test requirement, which enables compliance to the requirement to be demonstrated with a number of observed block errors lower than a certain threshold. - Option 2: 1dB is added to the test requirement to allow test termination within a feasible time frame Views on these options or any other wording should be discussed in the second round. CR handling: The CR section numbering and division should be agreed in thread 319. Unless rapid progress is made on thread 319 and issue 5-1 above so that CRs can be drafted next week, it is suggested that CRs are noted and companies revise for next meeting. |

Suggestion on WF/LS assignment

|  | WF/LS t-doc Title | Assigned Company, WF or LS lead |
| --- | --- | --- |
| #1 | See section 1.4.1 |  |

### CRs/TPs

Moderator tries to summarize discussion status for 1st round and provided recommendation on CRs/TPs Status update suggestion

| CR/TP number | CRs/TPs Status update recommendation |
| --- | --- |
| XXX | Based on 1st round of comments collection, moderator can recommend the next steps such as “agreeable”, “to be revised” |

## Discussion on 2nd round (if applicable)

How to capture the BLER early pass/fail methodology in the BS specification:

Issue 5-1: Should the methodology be captured in an Annex or in the text on test methodology ?

  - Option 1: Annex

  - Option 2: Within test methodology text for the 10^-5 BLER conformance test

Issue 5-2: If in an Annex should the Annex capture only 10^-5 BLER and 99.999% CL or also other BLER

  - Option 1: Only 10^-5 BLER requirement

  - Option 2: Applicable for any BLER requirement and any CL

  - Option 3: Applicable for any requirement and any CL

Issue 5-3: Should asymmetric per-step risks be assumed ?

  - Option 1: Symmetric per step risks (i.e. early pass/early fail the same)

  - Option 2: Asymmetric risks

Issue 5-4: Should a table of decision co-ordinates be included or simply a formula for assessing early pass/fail ?

  - Option 1: Include table of decision coordinates

  - Option 2: No table; express by formulae only

Issue 5-5: How should the 1dB (X) parameter be described in the Annex note ?

- Option 1: 1dB is added to the test requirement to facilitate early test pass. The BLER delivered by the device during the test will be lower than the test requirement, which enables compliance to the requirement to be demonstrated with a number of observed block errors lower than a certain threshold.

- Option 2: 1dB is added to the test requirement to allow test termination within a feasible time frame

| Company | Comments |
| --- | --- |
| Nokia, Nokia Shanghai Bell | Issue 5-1: A suitable explication of the test methodology will be too long to be included in the “procedure” section. Hence an annex is preferred. Issue 5-2: We think that the case for 1e-5 BLER is sufficient but are open to include some additional text to cover other BLER requirements as well. Please note that the derivation of per step risks is time consuming for every BLER level. Issue 5-3: Option 2. Asymmetric per step risks should be used, as in RAN5 LTE (TS 36.521-1, G.7.10) and RAN5 NR (TS 38.521-4 G.2.10). Going against RAN5 would be difficult to motivate. Issue 5-4: In the test feasibility studies, the full coordinate tables were an invaluable tool to understand the statistical methodology under discussion. Furthermore, these tables help to align testing (and thus comparison) between vendors, while allowing to easily set minimum testing times. Hence we are in favor of option 1, but we will not block progress for it. Issue 5-5: Both options are viable, but option 2 is slightly preferred to not force unnecessary (as in “not required”) information on the normal reader of the specification. Other - CR handling: We agree with the recommendations for 2nd round. [319] should decide the structure and draftCRs can be “postponed” (since noting is no longer welcome from MCC pov). |
| Ericsson | Issue 5-1: We have a slight preference to include the description in the text, not in an Annex since we do not believe the text needs to be so descriptive as in other examples (e.g. 3G) but are also OK for an Annex. Issue 5-2: Option 1: The description should be limited to describing 10^-5 BLER testing at 99.999% CL. Issue 5-4: It is a good suggestion to add a table of decision co-ordinates. Issue 5-5: (Option 1 was E/// proposal) |
| Huawei | Issue 5-1: We prefer option 1. Introduce the methodology in Annex. In the text, we prefer to only introduce parameters and performance requirements, etc. The introduction of the test methodology seems too long and unexpected. Issue 5-2: If CQI test is agreed to be defined with same CL. We propose to introduce the Annex for only 10^-5 and 99.999% CL. Otherwise, other CL may need to be included. Issue 5-3: We prefer option 2. Following the RAN5 existing definition. Issue 5-4: We are fine with option 1 and option 2. |

Moderator:

Issue 5-1: Ericsson seems to not have a strong preference and Huawei/Nokia to prefer Annex. Moderator proposal is to assume Annex. Please indicate if anyone has an objection.

Issue 5-2: Companies that commented so far propose to keep scope for 10^-5 and 99.999%. To Huawei: Could you clarify your comment; it is not clear how the CQI test impacts the BS specs.

Issue 5-5: It seems Intel and Ericsson propose option 1, Nokia propose option 2 but think both could work. Is it possible to agree this meeting ?

Also please comment below on the wording for the open issue description in the WF.

| Company | Comments |
| --- | --- |

## Summary on 2nd round (if applicable)

Moderator tries to summarize discussion status for 2nd round and provided recommendation on CRs/TPs/WFs/LSs Status update suggestion

| CR/TP/LS/WF number | T-doc Status update recommendation |
| --- | --- |
| R4-2012646 | Way Forward on ultra-low BLER requirements: Propose to approve |
| R4-2012660 | Draft CR: Propose to Note |