3GPP TSG-RAN WG4 Meeting #96-e                        	           R4-2011787

# Online, 17th Aug. – 28th Aug. 2020

Title:	LS on UE capability for PC2 inter-band EN-DC (LTE FDD+NR TDD)

Response to:

Release:	Rel-16

Work Item:	ENDC_UE_PC2_FDD_TDD

Source:	RAN WG4

To:	RAN WG2

Cc:

Contact Person:

#### Name:	Basaier Jialade

E-mail Address:	basejld@chinaunicom.cn

1. Overall Description:

RAN4 has discussed the SAR issues for PC2 inter-band EN-DC (Case1: LTE FDD PC3 + NR TDD PC3, Case2: LTE FDD PC3 + NR TDD PC2). It is agreed to introduce 2 UE capabilities of maxUplinkDutyCycle-FDD&TDD-EN-DC1 and maxUplinkDutyCycle-FDD&TDD-EN-DC2 which indicate the maxUplinkDutyCycle capability of NR band corresponding to different LTE reference configurations i.e. LTE band’s maxUplinkDutyCycle of 70% and 40%.

The maxUplinkDutyCycle-FDD&TDD-EN-DC1 and maxUplinkDutyCycle-FDD&TDD-EN-DC2 capabilities for PC2 inter-band EN-DC (FDD+TDD) are optional, when the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than its capabilities (The exact evaluation period is no less than one radio frame), the requirements for PC2 inter-band EN-DC (FDD+TDD) are not applicable, and the corresponding requirements for a PC3 inter-band EN-DC (FDD+TDD) UE shall apply.

The capabilities of maxUplinkDutyCycle-FDD&TDD-EN-DC1 and maxUplinkDutyCycle-FDD&TDD-EN-DC2 are reported by UE as a per band combination capability. The granularity is 10%. The value range is as below:

- maxUplinkDutyCycle-FDD&TDD-EN-DC1, maxUplinkDutyCycle-FDD&TDD-EN-DC2 ∈ {30%, 40%, 50%, 60%, 70%, 80%, 90%, 100%}

2. Actions:

To RAN 2:

RAN4 kindly asks RAN2 to take the above agreement into consideration and design the capability signalling.

3. Date of Next RAN4 Meetings:

TSG RAN WG4 Meeting #96-Bis-e	                           October 2020, Online

TSG RAN WG4 Meeting #97				 March 2021, Online