3GPP TSG-RAN4 Meeting #96-e	R4-2009594

# Online, , 17th Aug 2020 - 28th Aug 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0214 | rev | - | Current version: | 15.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EESS protection related requirements for FR2 bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia Japan |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduction of EESS protection based on WRC-19 decision and European regulatory body. In addition, this CR resolves smooth transmision of the requirements whose EESS protection limist changes before or after the changeover date to avoid unnecesasry A-MPR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | All the necessary NS signals and relevant additional spurious emission requirements are introduced based on R4-2009593/R4-2009141. Applicability conditions are added to some NS values as well. With respect to only n257, 1dBm/200MHz is reflected into Spurious emission band UE co-existence. That measn NS_200 is used. A-MPR are also introduced if necessary for the emission requirements based on R4-2000216. Note that we adopted 0.5dB glanurality to derive A-MPR from R4-2000216 so that in some case, the required A-MPR has 0.5dB difference. Regarding A-MPR for NS_206 and CA_NS_206, the emisison limit is a combination of NS_202(CA_NS_204) and NS204(CA_NS_204). Thus, the A-MPR is derived considering A-MPR values for NS_202(CA_NS_204) and NS204(CA_NS_204). Finally, as was approved in R4-2009141, modifiedMPR is used for UEs to signal newly introduced NS values. Note that this only applies to WRC19 resolution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WRC-19 decision cannot be implemented into UEs in a timely manner. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3, 6.2A.3, 6.5.3.1, 6.5.3.2, 6.5A.3.1, 6.5A.3.2, Annex H |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | [Isolated impact] UEs are allowed to explictly signal newly supported requirements via modifiedMPR until a certain time and network can differentiate UEs impelmenting the CR and the legacy UEs via modifiedMPR signalings. The network can appropriately treat these UEs based on the signalings. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of changes>

### 6.2.3	UE maximum output power with additional requirements

#### 6.2.3.1	General

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band (the IE field freqBandIndicatorNR) and an associated value of additionalSpectrumEmission in the relevant RRC information elements

To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2.1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

Table 6.2.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. The mapping of NR frequency band numbers and values of and the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2.2.

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_200 |  |  |  |  | N/A |
| NS_2011 | 6.5.3.2.2 | n258 |  |  | 6.2.3.2 |
| NS_2022 | 6.5.3.2.3 | n257, n258 |  |  | 6.2.3.3 |
| NS_2033 | 6.5.3.2.4 | n258 |  |  | 6.2.3.4 |
| NS_2044 | 6.5.3.2.5 | n257, n258 |  |  | 6.2.3.5 |
| NS_205 | 6.5.3.2.6 | n260 |  |  | 6.2.3.6 |
| NS_2065 | 6.5.3.2.7 | n257, n258 |  |  | 6.2.3.7 |
| NOTE 1: NS_201 is not applicable. NOTE 2: This limit applies to UE brought into use on or before 1 January 2024. NOTE 3: This limit applies to UE brought into use on or before 1 September 2027. NOTE 4: This limit applies to UE brought into use after 1 September 2027. NOTE 5: This limit applies to UE brought into use after 1 January 2024. |  |  |  |  |  |

Table 6.2.3.1-2: Mapping of Network Signaling label

| NR Band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | NS_200 | NS_2023 | NS_2045 | NS_2066 |  |  |  |  |
| n258 | NS_200 | NS_201 | NS_2023 | NS_2034 | NS_2045 | NS_2066 |  |  |
| n260 | NS_200 | NS_205 |  |  |  |  |  |  |
| n261 | NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in sub-clause 6.3.2 of TS 38.331 [13]. NOTE 2: NS_201 is not applicable. NOTE 3: This limit applies to UE brought into use on or before 1 January 2024. NOTE 4: This limit applies to UE brought into use on or before 1 September 2027. NOTE 5: This limit applies to UE brought into use after 1 September 2027. NOTE 6: This limit applies to UE brought into use after 1 January 2024. |  |  |  |  |  |  |  |  |

#### 6.2.3.2	A-MPR for NS_201

##### 6.2.3.2.1	A-MPR for NS_201 for power class 1

For power class 1, A-MPR for NS_201 shall be 9 dB.

Table 6.2.3.2.1-1: (Void)

##### 6.2.3.2.2	A-MPR for NS_201 for power class 2

For power class 2, A-MPR specified in clause 6.2.3.2.3 applies

Table 6.2.3.2.2-1: (Void)

##### 6.2.3.2.3	A-MPR for NS_201 for power class 3

Table 6.2.3.2.3-1: AMPR for NS_201 for power class 3

| Offset Frequency | Channel Bandwidth, MHz |
| --- | --- |
|  | 400 |
|  | Outer RB allocations |
| 0 MHz, ≤ 100 MHz | ≤ 1.5 |
| > 100 MHz, ≤ 300 MHz | 0 |
| > 300 MHz | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from 24.25 GHz to the lower channel edge. NOTE 2: The allowable back off is max(MPR, AMPR), where the MPR is defined in Table 6.2.2.3-1 NOTE 3: Any undefined region, MPR applies |  |

##### 6.2.3.2.4	A-MPR for NS_201 for power class 4

For power class 4, A-MPR for NS_201 specified in clause 6.2.3.2.3 applies.

#### 6.2.3.3	A-MPR for NS_202

##### 6.2.3.3.1	A-MPR for NS_202 for power class 1

For power class 1, A-MPR for NS_202 shall be 11.0 dB.

##### 6.2.3.3.2	A-MPR for NS_202 for power class 2

For power class 2, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

##### 6.2.3.3.3	A-MPR for NS_202 for power class 3

For power class 3, A-MPR for NS_202 shall be 1.0 dB.

##### 6.2.3.3.4	A-MPR for NS_202 for power class 4

For power class 4, A-MPR for NS_202 specified in clause 6.2.3.3.3 applies.

#### 6.2.3.4	A-MPR for NS_203

##### 6.2.3.4.1	A-MPR for NS_203 for power class 1

For power class 1, A-MPR for NS_203 shall be 2.5 dB if Offset frequency < BWchannel, 0.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC

##### 6.2.3.4.2	A-MPR for NS_203 for power class 2

For power class 2, AMPR for NS_203 specified in subclause 6.2.3.4.3 applies

##### 6.2.3.4.3	A-MPR for NS_203 for power class 3

For power class 3, AMPR for NS_203 shall be 0 dB.

##### 6.2.3.4.4	A-MPR for NS_203 for power class 4

For power class 4, AMPR for NS_203 specified in subclause 6.2.3.4.3 applies.

#### 6.2.3.5	A-MPR for NS_204

##### 6.2.3.5.1	A-MPR for NS_204 for power class 1

For power class 1, A-MPR for NS_204 shall be 6.5 dB if Offset frequency < BWchannel, 6.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC

##### 6.2.3.5.2	A-MPR for NS_204 for power class 2

For power class 2, AMPR for NS_204 specified in subclause 6.2.3.5.3 applies

##### 6.2.3.5.3	A-MPR for NS_204 for power class 3

For power class 3, A-MPR for NS_204 shall be 1.0 dB if Offset frequency < BWchannel, 0.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC

##### 6.2.3.5.4	A-MPR for NS_204 for power class 4

For power class 4, AMPR for NS_204 specified in subclause 6.2.3.5.3 applies.

#### 6.2.3.6	A-MPR for NS_205

##### 6.2.3.6.1	A-MPR for NS_205 for power class 1

For power class 1, AMPR for NS_205 shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart  LCRB/2

Offset frequency < 10% of BWchannel

Where:

The Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the lowest CC

The AMPR for low-edge allocations shall be per table 6.2.3.6.1-1.

Table 6.2.3.6.1-1: AMPR for low-edge allocations for NS_205 for power class 1

|  | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 16.0 | 14.0 | 7.0 | 6.0 |

##### 6.2.3.6.2	A-MPR for NS_205 for power class 2

For power class 2, AMPR for NS_205 specified in subclause 6.2.3.6.3 applies

##### 6.2.3.6.3	A-MPR for NS_205 for power class 3

For power class 3, AMPR for NS_205 shall be 0.0 dB unless the allocation meets the following conditions to qualify as a low-edge allocation:

RBstart < BWchannel/10 MHz

RBstart  LCRB/2

Offset frequency < 10% of BWchannel

Where:

The Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the lowest CC

The AMPR for low-edge allocations shall be per table 6.2.3.6.3-1.

Table 6.2.3.6.3-1: AMPR for low-edge allocations for NS_205 for power class 3

|  | BWchan (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 50 | 100 | 200 | 400 |
| A-MPR(dB) | 12.0 | 10.0 | 3.0 | 2.0 |

##### 6.2.3.6.4	A-MPR for NS_205 for power class 4

For power class 4, AMPR for NS_205 specified in subclause 6.2.3.6.3 applies.

#### 6.2.3.7	A-MPR for NS_206

##### 6.2.3.7.1	A-MPR for NS_206 for power class 1

For power class 1, A-MPR for NS_206 shall be 11.0 dB.

##### 6.2.3.7.2	A-MPR for NS_206 for power class 2

For power class 2, A-MPR for NS_206 specified in clause 6.2.3.7.3 applies.

##### 6.2.3.7.3	A-MPR for NS_206 for power class 3

For power class 3, A-MPR for NS_206 shall be 1.0 dB.

##### 6.2.3.7.4	A-MPR for NS_206 for power class 4

For power class 4, A-MPR for NS_206 specified in clause 6.2.3.7.3 applies.

< Unnecessary sections are omitted>

### 6.2A.3	UE maximum output power with additional requirements for CA

#### 6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_2011 | 6.5.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_2022 | 6.5.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_2033 | 6.5.3.2.4 | n258 |  |  | 6.2A.3.4 |
| CA_NS_2044 | 6.5.3.2.5 | n257, n258 |  |  | 6.2A.3.5 |
| CA_NS_205 | 6.5.3.2.6 | n260 |  |  | 6.2A.3.6 |
| CA_NS_2065 | 6.5.3.2.7 | n257, n258 |  |  | 6.2A.3.7 |
| NOTE 1: CA_NS_201 is not applicable. NOTE 2: This limit applies to UE brought into use on or before 1 January. NOTE 3: This limit applies to UE brought into use on or before 1 September 2027. NOTE 4: This limit applies to UE brought into use after 1 September 2027. NOTE 5: This limit applies to UE brought into use after 1 January 2024. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_2023 | CA_NS_2045 | CA_NS_2066 |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_2012 | CA_NS_202 | CA_NS_2034 | CA_NS_2045 | CA_NS_2066 |  |  |
| n260 | CA_NS_200 | CA_NS_205 |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. NOTE 2: CA_NS_201 is not applicable. NOTE 3: This limit applies to UE brought into use on or before 1 January. NOTE 4: This limit applies to UE brought into use on or before 1 September 2027. NOTE 5: This limit applies to UE brought into use after 1 September 2027. NOTE 6: This limit applies to UE brought into use after 1 January 2024. |  |  |  |  |  |  |  |  |

#### 6.2A.3.2	A-MPR for CA_NS_201

##### 6.2A.3.2.1	A-MPR for CA_NS_201 for power class 1

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.1-1: (Void)

For power class 1 CA non-contiguous RB allocations, the following rule for AMPR (dB) applies:

AMPR = max(AMPRC_CA, -10*A +12.0)

Where AMPRC_CA is 9.0

##### 6.2A.3.2.2	A-MPR for CA_NS_201 for power class 2

For intra-band contiguous CA, A-MPR specified in sub-clause 6.2A.3.2.3 applies.

Table 6.2A.3.2.2-1: (Void)

##### 6.2A.3.2.3	A-MPR for CA_NS_201 for power class 3

For intra-band contiguous CA, AMPR is specified as follows.

Table 6.2A.3.2.3-1: Contiguous Allocations, AMPRC_CA for CA_NS_201 for power class 3

| Offset Frequency | Cumulative Aggregated Bandwidth, MHz |  |
| --- | --- | --- |
|  | < 400 | ≥ 400, ≤ 800 |
| 0 MHz, ≤ 100 MHz | ≤ 1.5 | ≤ 3.0 |
| > 100 MHz, ≤ 300 MHz | 0 | 0 |
| > 300 MHz | 0 | 0 |
| NOTE 1: The Offset frequency is defined as the frequency from 24.25 GHz to the lower channel edge. NOTE 2: The allowable back off is max(MPR, AMPR), where the MPR is defined in Table 6.2A.2.4-1. NOTE 3: Any undefined region, MPR applies. |  |  |

For power class 3 CA non-contiguous RB allocations, the following rule for AMPR applies:

AMPR = max(AMPRC_CA, - 10*A + 5.0) , Offset Frequency ≤ 550 MHz

##### 6.2A.3.2.4	A-MPR for CA_NS_201 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_201 specified in sub-clause 6.2A.3.2.3 applies.

#### 6.2A.3.3	A-MPR for CA_NS_202

##### 6.2A.3.3.1	A-MPR for CA_NS_202 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 11.0 dB.

##### 6.2A.3.3.2	A-MPR for CA_NS_202 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

##### 6.2A.3.3.3	A-MPR for CA_NS_202 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 2.0 dB.

##### 6.2A.3.3.4	A-MPR for CA_NS_202 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.3.3 applies.

#### 6.2A.3.4	A-MPR for CA_NS_203

##### 6.2A.3.4.1	A-MPR for CA_NS_203 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 6.5 dB, if Offset frequency < frequency separation or BWChannel_CA of the UL CA configuration, 0.0 dB, otherwise
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC.

##### 6.2A.3.4.2	A-MPR for CA_NS_203 for power class 2

For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

##### 6.2A.3.4.3	A-MPR for CA_NS_203 for power class 3

For intra-band contiguous CA, A-MPR for CA_NS_203 shall be 2.5 dB, if Offset frequency < frequency separation or BWChannel_CA of the UL CA configuration, 0.0 dB otherwise. 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC.

##### 6.2A.3.4.4	A-MPR for CA_NS_203 for power class 4

For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.4.3 applies.

#### 6.2A.3.5	A-MPR for CA_NS_204

##### 6.2A.3.5.1	A-MPR for CA_NS_204 for power class 1

For intra-band contiguous CA,  A-MPR for CA_NS_204 shall be 9.0 dB if Offset frequency < frequency separation or BWChannel_CA of the  UL CA configuration, 6.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC.

##### 6.2A.3.5.2	A-MPR for CA_NS_204 for power class 2

For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.5.3 applies.

##### 6.2A.3.5.3	A-MPR for CA_NS_204 for power class 3

For intra-band contiguous CA,  A-MPR for CA_NS_204 shall be 5.0 dB if Offset frequency < frequency separation or BWChannel_CA of the  UL CA configuration, 0.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC.

##### 6.2A.3.5.2	A-MPR for CA_NS_204 for power class 4

For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.5.3 applies.

#### 6.2A.3.6	A-MPR for CA_NS_205

##### 6.2A.3.6.1	A-MPR for CA_NS_205 for power class 1

For intra-band contiguous CA, the following rule for AMPR (dB) applies:

AMPR = max(AMPRnonEdge, AMPREdge,CA)

Where

AMPREdge,CA= AMPR as determined per sub-section 6.2.3.6.1, for the lowest CC and the contiguous RB allocation closest to lower edge of that CC

AMPRnonEdge is 5.0 dB if Offset frequency < frequency separation or BWChannel_CA of the  UL CA configuration, 0.0 dB otherwise.

The Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the lowest CC

##### 6.2A.3.6.2	A-MPR for CA_NS_205 for power class 2

For intra-band contiguous CA, AMPR specified in sub-clause 6.2A.3.6.3 applies.

##### 6.2A.3.6.3	A-MPR for CA_NS_205 for power class 3

For intra-band contiguous CA, the following rule for AMPR (dB) applies:

AMPR = max(AMPRnonEdge, AMPREdge,CA)

Where

AMPREdge,CA= AMPR as determined per sub-section 6.2.3.6.3, for the lowest CC and the contiguous RB allocation closest to lower edge of that CC

AMPRnonEdge is 1.0 dB if Offset frequency < frequency separation or BWChannel_CA of the  UL CA configuration, 0.0 dB otherwise

The Offset frequency is defined as the frequency from 37.0 GHz to the lower edge of the lowest CC

##### 6.2A.3.6.4	A-MPR for CA_NS_205 for power class 4

For intra-band contiguous CA, AMPR for CA_NS_205 specified in sub-clause 6.2A.3.6.3 applies.

#### 6.2A.3.7	A-MPR for CA_NS_206

##### 6.2A.3.7.1	A-MPR for CA_NS_206 for power class 1

For intra-band contiguous CA, A-MPR for CA_NS_202 shall be 11.0 dB.

##### 6.2A.3.7.2	A-MPR for CA_NS_206 for power class 2

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.7.3 applies.

##### 6.2A.3.7.3	A-MPR for CA_NS_206 for power class 3

For intra-band contiguous CA,  A-MPR for CA_NS_204 shall be 5.0 dB if Offset frequency < frequency separation or BWChannel_CA of the  UL CA configuration, 2.0 dB otherwise 
The Offset frequency is defined as the frequency from 24.0 GHz to the lower edge of the lowest CC.

##### 6.2A.3.7.4	A-MPR for CA_NS_206 for power class 4

For intra-band contiguous CA, A-MPR for CA_NS_202 specified in sub-clause 6.2A.3.7.3 applies.

< Unnecessary sections are omitted>

#### 6.5.3.1	Spurious emission band UE co-existence

This clause specifies the requirements for the specified NR band, for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.5.3.1-1: Requirements

| NR Band | Spurious emission |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Protected band/frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) |
| n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |
| n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |
| n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |
| n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void |  |  |  |  |  |  |

#### 6.5.3.2	Additional spurious emissions

##### 6.5.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious

emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for

a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5.3.2.2	Additional spurious emission requirements for NS_201

When "NS_201" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.2-1: Additional requirements (NS_201)

| Frequency band (GHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |  |  |
| 23.6 f 24 | -8 | -8 | -8 | -8 | 200 MHz | 1 |
| NOTE 1: The protection of frequency range 23600 - 24000 MHz is meant for protection of satellite passive services. |  |  |  |  |  |  |

##### 6.5.3.2.3	Additional spurious emission requirements for NS_202

When "NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

Table 6.5.3.2.3-1: Additional requirements (NS_202)

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz f 24.0 GHz | +1 dBm | 200 MHz |

##### 6.5.3.2.4	Additional spurious emission requirements for NS_203

When "NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.4-1: Additional requirements (NS_203)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | +1 | 200 MHz |

##### 6.5.3.2.5	Additional spurious emission requirements for NS_204

When "NS_204" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.5-1: Additional requirements (NS_204)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 23.6 f 24.0 | -5 | 200 MHz |

##### 6.5.3.2.6	Additional spurious emission requirements for NS_205

When "NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.6-1: Additional requirements (NS_205)

| Frequency band (GHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 36.0 f 37.0 | +7 | 1000 MHz |
|  | -13 | 1 MHz |

##### 6.5.3.2.7	Additional spurious emission requirements for NS_206

When "NS_206" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3-1 from the edge of the channel bandwidth.

Table 6.5.3.2.7-1: Additional requirements (NS_206)

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 7.25 GHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band | -10 dBm | 100 MHz |
| 23.6 GHz f 24.0 GHz | -5 dBm | 200 MHz |

< Unnecessary sections are omitted>

#### 6.5A.3.1	Spurious emission band UE co-existence for CA

This clause specifies the requirements for the specified carrier aggregation configurations for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For intra-band contiguous carrier aggregation, the requirements in Table 6.5A.3-1 apply.

Table 6.5A.3-1: Requirements for CA

| UL CA for any CA bandwidth class | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band / frequency range | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_n257 | NR Band n260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 23600 | - | 24000 | 1 | 200 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n258 | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n260 | NR Band 257 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | NR Band 261 | FDL_low | - | FDL_high | -5 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| CA_n261 | NR Band 260 | FDL_low | - | FDL_high | -2 | 100 |  |
|  | Frequency range | 57000 | - | 66000 | 2 | 100 |  |
| NOTE 1: FDL_low and FDL_high refer to each NR frequency band specified in Table 5.2-1 NOTE 2: Void |  |  |  |  |  |  |  |

#### 6.5A.3.2	Additional spurious emissions

##### 6.5A.3.2.1	General

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

##### 6.5A.3.2.2	Additional spurious emission requirements for CA_NS_201

When "CA_NS_201" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in clause 6.5A.3.

##### 6.5A.3.2.3	Additional spurious emission requirements for CA_NS_202

When "CA_NS_202" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.3-1.

##### 6.5A.3.2.4	Additional spurious emission requirements for CA_NS_203

When "CA_NS_203" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.5	Additional spurious emission requirements for CA_NS_204

When "CA_NS_204" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.6	Additional spurious emission requirements for CA_NS_205

When "CA_NS_205" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) as defined in section 6.5A.3.

##### 6.5A.3.2.7	Additional spurious emission requirements for CA_NS_206

When "CA_NS_206" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.2.7-1.

< Unnecessary sections are omitted>

Annex H (normative): Modified MPR behavior

# H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [14] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination. Moreover, the bits in the field can explicitly indicate NS value(s) supported by a UE.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [14] by an 8-bit bitmap per supported NR band.

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n257 | 0 (leftmost bit) | FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | -This bit may be set to 1 by a UE supporting n257 |
|  | 1 | NS_204 as defined in clause 6.5.3.2.5 of 38.101-2 v15.10.0 | This bit shall be set to 1 by a UE supporting n257 and brought up into use after 1 September 2027. Otherwise this bit may be set to 1. |
|  | 2 | CA_NS_204 as defined in clause 6.5A.3.2.5 of 38.101-2 v15.10.0 |  |
|  | 3 | NS_206 as defined in clause 6.5.3.2.7 of 38.101-2 v15.10.0 | This bit shall be set to 1 by a UE supporting n257 and brought up into use after 1 January 2024. Otherwise this bit may be set to 1. |
|  | 4 | CA_NS_206 as defined in clause 6.5A.3.2.7 of 38.101-2 v15.10.0 |  |
| n258 | 0 (leftmost bit) | FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | This bit may be set to 1 by a UE supporting n258 |
|  | 1 | AMPR for NS_201 as defined in clausue 6.2.3.2 of 38.101-2 v15.7.0 | This bit may be set to 1 by a UE supporting n258 |
|  | 2 | NS_203 as defined in clause 6.5.3.2.4 of 38.101-2 v15.10.0 | This bit shall be set to 1 by a UE supporting n258 and brought up into use on or before 1 September 2027. Otherwise this bit may be set to 1. |
|  | 3 | CA_NS_203 as defined in clause 6.5A.3.2.4 of 38.101-2 v15.10.0 |  |
|  | 4 | NS_204 as defined in clause 6.5.3.2.5 of 38.101-2 v15.10.0 | This bit shall be set to 1 by a UE supporting n258 and brought up into use after 1 September 2027. Otherwise this bit may be set to 1. |
|  | 5 | CA_NS_204 as defined in clause 6.5A.3.2.5 of 38.101-2 v15.10.0 |  |
|  | 6 | NS_206 as defined in clause 6.5.3.2.7 of 38.101-2 v15.10.0 | This bit shall be set to 1 by a UE supporting n258 and brought up into use after 1 January 2024. Otherwise this bit may be set to 1. |
|  | 7 | CA_NS_206 as defined in clause 6.5A.3.2.7 of 38.101-2 v15.10.0 |  |
| n260 | 0 (leftmost bit) | FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | This bit may be set to 1 by a UE supporting n260 |
| n261 | 0 (leftmost bit) | FR2 power class 3 MPR as defined in clause 6.2.2.3 of 38.101-2 v16.2.0 | This bit may be set to 1 by a UE supporting n261 |

< The end of changes>