3GPP TSG-RAN WG4 Meeting # 96-e 	R4-2009610

# Electronic Meeting, 17 – 28 Aug, 2020

Agenda item:	7.16.1.2

Source:	Apple Inc.

Title:	On PMI Reporting Requirements with Larger Number of TX Ports

Release:	Rel-16

Document for:	Discussion

# 1. Introduction

In RAN4#95e PMI reporting requirements with Single panel Type I codebook and Type II codebook were discussed and way forward [1] was approved. In this contribution we address the open issues related to PMI reporting requirements with larger than 8 TX ports with Type II codebook and provide simulation results for PMI reporting with Type I codebook.

# 2. Discussion

## Requirements with Type II codebook

In [1] the open issues related to PMI reporting requirements with Type II codebook are:

Test setup - Option 1: Only use SU-MIMO test setup, i.e., one tested UE - Option 2: MU-MIMO based test setup, i.e., one tested UE + one co-scheduled UE (generated by TE) Test Parameters for SU-MIMO test setup Npsk (phaseAlphabetSize) - Option 1: 4 - Option 2: 8 SubbandAmplitude - Option 1: False - Option 2: True PMI-FormatIndicator - Option 1: Wideband - Option 2: Subband MIMO Correlation - Option 1: XP High - Option 2: XP Medium

Test setup

In RAN4#95e, there was an extensive discussion on the test setup for test cases with Type II codebook. Some of the agreements regarding the setup were:

- The baseline receiver assumption is UE without interference cancellation capability with/without co-scheduled UE.

- Under the baseline UE receiver assumption, the PMI calculation processing will not change with and without co-scheduled UE.

The purpose of introducing PMI reporting requirements with Type II codebook is to verify PMI reporting by the UE with Type II codebook. Type II codebook was designed to be used in MU-MIMO scenario. But the gains from Type II codebook in a MU-MIMO setup would only be realized from a system level when we have multiple UEs and the overall system throughput is improved. For a link level assessment, we don’t believe that MU-MIMO test setup would have significant improvement in performance over SU-MIMO setup.

Observation #1: For link level assessment, no performance improvement would be observed with MU-MIMO compared to SU-MIMO test setup.

Also, the agreement has been that the UE has no knowledge of co-scheduled user and the receiver assumption is MMSE-IRC. With this assumption, there is no impact on PMI reporting from the UE.

Observation #2: There is no impact on UE PMI reporting based on no knowledge of co-scheduled UE and baseline receiver as MMSE-IRC with MU-MIMO setup compared to SU-MIMO.

MU-MIMO test setup is more complicated compared to SU-MIMO, involving specifying details and aligning the scheduling mode on the gNB side with co-scheduled UE. And moreover, we have no input from TE vendors on this aspect.

Observation #3: MU-MIMO setup is more complicated compared to SU-MIMO, involving aligning scheduling mode with co-scheduled UE.

Based on the above observations, we don’t see the necessity to specify requirements for Type II codebook with MU-MIMO setup and hence propose to introduce requirements for PMI reporting with Type II codebook with SU-MIMO setup.

Proposal #1: Use SU-MIMO test setup for requirements for PMI reporting with Type II codebook.

Test Parameters

For the baseline simulation assumptions agreed in [1], we evaluate performance in SU-MIMO test setup for Type II codebook.

The key simulation parameters:

MCS: 20; Rank: 2

Duplex Mode: FDD

Channel Model: TDLA30-5Hz

Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

L (Number of beams): 2

Npsk (phaseAlphabetSize): 4; 8

SubbandAmplitude: False; True

PMI-FormatIndicator: Subband; Wideband

Antenna Correlation: XP-Medium, XP-High

Table 1: Simulation Results for 16x2 with SB PMI Reporting

| Antenna Correlation | Subband Amplitude | Npsk | SNR @ 90%Max | TP Gain |
| --- | --- | --- | --- | --- |
| Medium | FALSE | 8 | 12.23 | 4.94 |
| Medium | FALSE | 4 | 12.78 | 4.50 |
| Medium | TRUE | 8 | 11.99 | 5.20 |
| Medium | TRUE | 4 | 12.46 | 4.74 |
| High | FALSE | 8 | 13.86 | 4.98 |
| High | FALSE | 4 | 13.99 | 4.74 |
| High | TRUE | 8 | 13.87 | 4.99 |
| High | TRUE | 4 | 13.91 | 4.77 |

With SB PMI reporting, better performance is observed with Medium antenna correlation compared to high correlation. With medium antenna correlation, we observe better performance with subband amplitude set to TRUE and with Npsk = 8.

Table 2: Simulation Results for 16x2 with WB PMI Reporting

| Antenna Correlation | Subband Amplitude | Npsk | SNR @ 90%Max | TP Gain |
| --- | --- | --- | --- | --- |
| Medium | FALSE | 8 | 13.15 | 4.39 |
| Medium | FALSE | 4 | 13.46 | 3.95 |
| Medium | TRUE | 8 | 13.15 | 4.38 |
| Medium | TRUE | 4 | 13.46 | 3.94 |
| High | FALSE | 8 | 13.99 | 4.84 |
| High | FALSE | 4 | 14.09 | 4.56 |
| High | TRUE | 8 | 13.97 | 4.85 |
| High | TRUE | 4 | 14.08 | 4.62 |

With WB PMI reporting, we have similar observation as with SB PMI reporting – better performance is observed with medium correlation, subband amplitude set to TRUE and Npsk = 8.

Observation #4: With both SB and WB PMI reporting, better performance is observed with medium correlation, subband amplitude set to TRUE and Npsk = 8.

Between SB and WB PMI, SB PMI has slightly better performance is observed with SB PMI. Also SB PMI reporting might be more widely used in actual deployment, hence we propose using SB PMI reporting for Type II test cases.

Observation #5: SB PMI reporting has better performance compared to WB PMI

Proposal #2: For Type II codebook, introduce requirements with SU-MIMO test setup with the following assumptions:

Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

Channel Model: TDLA30-5Hz

Antenna Correlation: XP-Medium

PMI format Indicator: Subband

Subband Amplitude: TRUE

Npsk: 8

## Simulation Results for Type I

The simulation assumptions for requirements with single panel Type I codebook for 16 and 32 TX ports were approved in [2]. We provide results for the agreed simulation assumptions.

Table 3: Simulation Results for Type I

| Duplex Mode | Antenna Config | SNR @ 90% Max TP | TP Gain |
| --- | --- | --- | --- |
| FDD | 16x2 | 13.12 | 3.2 |
|  | 16x4 | 8.36 | 3.4 |
| TDD | 16x2 | 13.71 | 3.2 |
|  | 16x4 | 8.79 | 3.6 |

# 3. Conclusion

In this paper, we provide our views on the remaining open issues related to PMI reporting requirements with larger than 8 TX ports with Type II codebook. Our observations and proposals are captured below:

Test Setup

Observation #1: For link level assessment, no performance improvement would be observed with MU-MIMO compared to SU-MIMO test setup.

Observation #2: There is no impact on UE PMI reporting based on no knowledge of co-scheduled UE and baseline receiver as MMSE-IRC with MU-MIMO setup compared to SU-MIMO.

Observation #3: MU-MIMO setup is more complicated compared to SU-MIMO, involving aligning scheduling mode with co-scheduled UE.

Proposal #1: Use SU-MIMO test setup for requirements for PMI reporting with Type II codebook.

Test Parameters

Observation #4: With both SB and WB PMI reporting, better performance is observed with medium correlation, subband amplitude set to TRUE and Npsk = 8.

Observation #5: SB PMI reporting has better performance compared to WB PMI

Proposal #2: For Type II codebook, introduce requirements with SU-MIMO test setup with the following assumptions:

Number of CSI-RS ports: 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

Channel Model: TDLA30-5Hz

Antenna Correlation: XP-Medium

PMI format Indicator: Subband

Subband Amplitude: TRUE

Npsk: 8

# Reference

- R4-2008846, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson, Samsung.

- R4-2008847, “Simulation assumptions for NR PMI reporting requirements for more than 8 Tx ports”, Ericsson.