3GPP TSG-RAN WG4 Meeting #96-e	     	          										 R4-2009615

Electronic Meeting, August 17th – 28th, 2020

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS38.101-1 | CR | 0409 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for n26 AMPR for 256QAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_n26-Core |  |  |  |  |  |  | Date: |  | 2020-07-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Missing backoff for 256QAM AMPR Technically endorsed draft CR R4-2005183 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase back-off for 256QAM to the same as LTE total backoff (MPR+AMPR) for low LCRB allocation. The extra back-off is for the IMD product of carrier leakage and TX signal for 256QAM, where transceiver LO suppression in implementation gets worse for higher order modulations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE cannot meet emission requirement for 256QAM for low LCRB allocations |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.21, 6.2.3.22, 6.2.3.23, 6.2.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  | 38.521-1 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 6.2.3.21	A-MPR for NS_12

Table 6.2.3.21-1: A-MPR regions for NS_12

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 5MHz | ≤1.8 | >0 | A1 |
| 10MHz | ≤3.6 | >0 | A1 |

Table 6.2.3.21-2: A-MPR for NS_12

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 |
| DFT-s-OFDM QPSK | ≤ 5 |
| DFT-s-OFDM 16 QAM | ≤ 5.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 |
| DFT-s-OFDM 256 QAM | ≤ 9.5 |
| CP-OFDM QPSK | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 9.5 |

< end of changes >

< start of changes >

#### 6.2.3.22	A-MPR for NS_13

Table 6.2.3.22-1: A-MPR regions for NS_13

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 819.5 ≤ Fc < 821.5 | ≤1.44 | <1.08 | A1 |
|  |  | ≤1.44 | ≥1.08 | A2 |
| 5MHz | Fc ≥ 821.5 | ≤0.54 | <1.08 | A1 |
|  |  |  | ≥3.24 | A3 |

Table 6.2.3.22-2: A-MPR for NS_13

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3.5 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 4.5 | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 256 QAM | ≤ 8 | ≤ 6 |  |
| CP-OFDM QPSK | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 6.5 | ≤ 4.5 |
| CP-OFDM 256 QAM | ≤ 8 | ≤ 8 |  |

< end of changes >

< start of changes >

#### 6.2.3.23	A-MPR for NS_14

Table 6.2.3.23-1: A-MPR regions for NS_14

| Channel BW | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- |
| 10MHz | ≤0.18 | <1.08 | A1 |
|  | >0 | ≥9 | A2 |
| 15MHz | ≤1.8 | <1.8 | A1 |
|  | >0 | ≥9 | A2 |
| 20MHz | ≤3.42 | <1.8 | A3 |
|  | >0 | ≥9 | A2 |

Table 6.2.3.23-2: A-MPR for NS_14

| Modulation/Waveform | A1 | A2 | A3 |
| --- | --- | --- | --- |
|  | Outer/Inner | Outer | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 3 | ≤ 2 | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3 |  | ≤ 3 |
| DFT-s-OFDM 256 QAM |  |  | ≤ 8 |
| CP-OFDM QPSK | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 16 QAM | ≤ 5 | ≤ 4 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 6 |  | ≤ 6 |
| CP-OFDM 256 QAM | ≤ 8 |  | ≤ 8 |

< end of changes >

< start of changes >

#### 6.2.3.24	A-MPR for NS_15

Table 6.2.3.24-1: A-MPR regions for NS_15

| Channel BW | Carrier Frequency, Fc, MHz | RBend*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz | 840.5 < Fc ≤ 846.5 | ≥3.24 | >0 | A1 |
|  |  | <3.24, ≥2.52 | ≥1.44 | A2 |
|  |  | <0.9 | ≤0.36 | A3 |
| 10MHz | 840 < Fc ≤ 844 | ≥5.76 | >1.08 | A1 |
|  |  | ≥5.76 | ≤1.08 | A4 |
|  |  | <5.76, ≥4.14 | ≥2.7 | A2 |
|  |  | <2.52 | ≤0.36 | A3 |
|  | 835 < Fc ≤ 840 | ≥7.2 | >0 | A1 |
|  |  | <7.2, ≥5.22 | ≥4.32 | A2 |
|  |  | <1.08 | ≤0.36 | A3 |
| 15MHz | 837.5 < Fc ≤ 841.5 | ≥9.36 | >1.08 | A1 |
|  |  | ≥9.36 | ≤1.08 | A4 |
|  |  | <9.36, ≥4.68 | ≥3.6 | A2 |
|  |  | <3.96 | ≤0.36 | A3 |
|  | 831.5 < Fc ≤ 837.5 | ≥10.8 | >1.08 | A1 |
|  |  | ≥10.8 | ≤1.08 | A4 |
|  |  | <10.8, ≥6.48 | ≥3.6 | A2 |
|  |  | <2.7 | ≤0.36 | A3 |
|  | Fc ≤ 831.5 | ≥13.14 | >0 | A1 |
|  |  | <13.14, ≥7.92 | ≥3.6 | A2 |
|  |  | <0.72 | ≤0.36 | A3 |
| 20MHz | 835 < Fc ≤ 839 | ≥12.24 | >1.08 | A1 |
|  |  | ≥12.24 | ≤1.08 | A4 |
|  |  | <12.24, ≥8.46 | ≥5.4 | A2 |
|  |  | <5.58 | ≤0.36 | A3 |
|  | Fc ≤ 835 | ≥13.68 | >1.08 | A1 |
|  |  | ≥13.68 | ≤1.08 | A4 |
|  |  | <13.68, ≥8.46 | ≥5.4 | A2 |
|  |  | <4.32 | ≤0.36 | A3 |

Table 6.2.3.24-2: A-MPR for NS_15

| Modulation/Waveform | A1 | A2 | A3 | A4 |
| --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM QPSK | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 16 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 64 QAM | ≤ 9 | ≤ 5 | ≤ 4 | ≤ 9 |
| DFT-s-OFDM 256 QAM | ≤ 9 | ≤ 5 | ≤ 9 | ≤ 13.5 |
| CP-OFDM QPSK | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 16 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 64 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 4 | ≤ 10.5 |
| CP-OFDM 256 QAM | ≤ 10.5 | ≤ 6.5 | ≤ 9 | ≤ 13.5 |

< end of changes >