3GPP TSG-RAN4 Meeting #96-e 		R4-2009839

Electronic Meeting, 17-28 Aug., 2020

Title:	Discussion on CSI-RS measurement bandwidth

Source:	CATT

Agenda Item:	7.14.1.1

Document for:	Discussion

- Introduction

CSI-RS requirement and the applicable configurations have been extensively discussed in previous RAN4 meetings. The following consensus was captured in the Chairman’s Note [1] and the agreed WF [2] in RAN4#94e-bis meeting:

- Define requirements for CSI-RS configuration with {D=3 with PRBs ≥ 48}. FFS whether to define requirements for {D=1 with PRBs ≥ 96}.

- Not define additional CSI-RS based measurement requirement

In RAN4 #95e meeting, RAN4 further discussed the CSI-RS configuration of {D=1 with PRBs ≥ 96}. However no further progress was achieved. In this contribution, we further discuss this open issue and give our proposal.

- Discussion

CSI-RS configuration for measurement requirement

In RAN4 #94e-bis meeting, it was agreed that one common set of measurement requirement is defined for CSI-RS L3 measurement. And the CSI-RS configuration of {D=3 with PRBs ≥ 48} is applied to the measurement requirement. FFS on whether the defined requirements applies to CSI-RS configuration of {D=1 with PRBs ≥ 96}.

Until now no further progress is made regarding CSI-RS configuration of {D=1 with PRBs ≥ 96}. We fully agree that it will bring configuration flexibility for network and potential benefit from system operation point of view. This is why we also support to specify requirement for this configuration from the beginning. However, we cannot ignore the discussions and situation we are facing, E.g.

- It is not justified whether {D=1 with PRBs ≥ 96} can use the same set of requirements as {D=3 with PRBs ≥ 48}.

- More time are needed for trade-off between network configuration flexibility and UE complexity, which cannot be completed in one meeting cycle.

- Little time is left for Rel-16 completion.

Furthermore we think no requirement for {D=1 with PRBs ≥ 96} dost not mean it cannot be configured in the network.

Given the current situation and the discussion in RAN#88e meeting, we propose to leave the discussion for this configuration to Rel-17.

Proposal 1: Leave the discussion on {D=1 with PRBs ≥ 96} to Rel-17.

- Conclusion

In this contribution, we further discussed the remaining issues on CSI-RS measurement configuration and provided our proposal as following:

Proposal 1: Leave the discussion on {D=1 with PRBs ≥ 96} to Rel-17.

- Reference

[1] RAN4_94ebis_RRM_chairman_report_10_Apr_30_EOM

[2] R4-2005355 WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement, CATT

[3] R4-2009256 WF on CSI-RS configuration and intra/inter-frequency measurements definition for CSI-RS based L3 measurement, CATT