3GPP TSG-RAN4 Meeting #96-e 		R4-2009843

Electronic Meeting, 17-28 Aug., 2020

Title:	UE feature on support of RRM requirements for CSI-RS based L3 measurement

Source:	CATT

Agenda Item:	7.14.1

Document for:	Discussion

- Introduction

In RAN4 #94ebis meeting, initial discussion on the Rel-16 NR and LTE UE feature list took place. Companies’ views on the NR UE feature list are collected since then. This contribution provides our views on the feature on support of RRM requirements for CSI-RS based L3 measurement.

- Discussion

The RRM requirements for CSI-RS based L3 measurements will be introduced. And we note that there are no Rel-15 requirements defined and UE behavior is not known to the gNB. To allow gNB to effectively differentiate the implementations which support newly defined requirements, we propose to introduce a new Rel-16 feature “Support of RRM requirements for CSI-RS based L3 measurement”.

Proposal 1: Introduce a new Rel-16 feature “Support of RRM requirements for CSI-RS based L3 measurement” as provided in Table 1 for NR CSI-RS based L3 measurements WI.

Table 1. NR RRM requirements for CSI-RS based L3 measurement feature list

| Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Support of RRM requirements for CSI-RS based L3 measurement | Support of RRM requirements for CSI-RS based L3 measurement as defined in TS 38.133 | TBA | Yes |  | UE is not guaranteed to satisfy the RRM requirements for CSI-RS based L3 measurement as defined in TS 38.133 | Per UE | No | TBD |  |  | Optional with capability signalling |

- Conclusion

In this contribution, we give our views on the feature on support of RRM requirements for CSI-RS based L3 measurement and provide following proposal:

Proposal 1: Introduce a new Rel-16 feature “Support of RRM requirements for CSI-RS based L3 measurement” as provided in Table 1 for NR CSI-RS based L3 measurements WI.

- Reference

[1]	R4-2005193, RAN4 UE features list for Rel-16”, NTT DoCoMo, RAN4 #94ebis, April 2020.