3GPP TSG-RAN4 Meeting #96-e	R4-2009908

# Online, 17-28 Aug., 2020

Agenda item:	7.1.5.11

Source:	Apple

Title:		On MO merging for NR-U

Document for:	Discussion

# Introduction

In last RAN4 meeting, the draft CR of R4-2006183 has been technically endorsed, but there are still some remaining issues in the UE measurement capability requirement, as duplicated below,

#### 9.1A.3.2 EN-DC: Maximum allowed layers for multiple monitoring with CCA If a UE is configured with EN-DC operation when CCA is used on PSCell, the UE shall be capable of monitoring at least: - Depending on UE capability, 7 NR inter-frequency carriers configured by PScell, and - Depending on UE capability, 7 NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell [15], and - Depending on UE capability, 6 E-UTRA TDD inter-frequency carriers configured by E-UTRA PCell [15], and - Depending on UE capability, 6 E-UTRA FDD inter-frequency carriers configured by E-UTRA PCell [15], and - Depending on UE capability, 3 FDD UTRA carriers, and - Depending on UE capability, 3 TDD UTRA carriers, and - Depending on UE capability, 32 GSM carriers (one GSM layer corresponds to 32 carriers), and In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM (one GSM layer corresponds to 32 carriers) layers. The UE shall be capable of monitoring a total of at least 7 effective NR carrier frequency layers excluding NR serving carrier(s), comprising of any above defined combination of NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell and NR inter-frequency carriers configured by PSCell. Editor note: the MO merging requirement is FFS

Besides this scenario B, we also need to revisit the MO merging requirement for other licensed DC scenarios since the NR-U measurement could be also configured to the UE who only has the licensed serving cell(s), like licensed EN-DC and licensed NR-DC. In this contribution we discuss the MO merging requirement for licensed EN-DC, licensed NR-DC and NR-U scenario B.

# MO merging for NR-U

In the current TS38.133, the UE behaviour for MO merging is defined as,

#### 9.1.3.2 EN-DC: Maximum allowed layers for multiple monitoring <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> When the E-UTRA PCell and PSCell configures the same NR carrier frequency layer to be monitored by the UE in synchronous intra-band EN-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFN-s and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have - different RSSI measurement resources or - different deriveSSB-IndexFromCell indications or - different SMTC configurations. Note 1: The E-UTRA-NR dual connectivity capable UE configured with PSCell shall fulfil the requirements defined in only one of clause 9.1.3.2 and clause 8.1.2.1.1b.1 of TS 36.133 [15]. <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> #### 9.1.3.2c NR-DC: Maximum allowed layers for multiple monitoring <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD and UTRA FDD layers. The UE shall be capable of monitoring a total of at least 7 effective NR carrier frequency layers excluding NR serving carrier(s), which are configured by PCell and PSCell. When PCell and PSCell configures the same NR carrier frequency layer to be monitored by the UE in NR-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFN-s and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have - different RSSI measurement resources or - different deriveSSB-IndexFromCell indications or - different SMTC configurations.

From the previous discussion/agreement on the MO merging criteria, the principle to decide if the MOs could be merged or not depends on whether those MOs would need different measurement efforts at UE, e.g. maintaining different timeline, or perform measurement on different measurement occasion. Here, we also believe the same principle shall be used for NR-U MO merging requirement.

Proposal 1: Same as licensed MO merging requirement on same NR carrier frequency layer, the principle to merge MOs on same NR-U carrier frequency layer is that those MOs would not need different measurement efforts from UE.

However, compared with licensed MO configuration, there are some new parameters in the NR-U MO configuration and therefore it’s necessary to consider those new parameters in the MO merging requirement.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need R absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, -- Need R nrofSS-BlocksToAverage INTEGER (2..maxNrofSS-BlocksToAverage) OPTIONAL, -- Need R nrofCSI-RS-ResourcesToAverage INTEGER (2..maxNrofCSI-RS-ResourcesToAverage) OPTIONAL, -- Need R quantityConfigIndex INTEGER (1..maxNrofQuantityConfig), offsetMO Q-OffsetRangeList, cellsToRemoveList PCI-List OPTIONAL, -- Need N cellsToAddModList CellsToAddModList OPTIONAL, -- Need N blackCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N blackCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N whiteCellsToRemoveList PCI-RangeIndexList OPTIONAL, -- Need N whiteCellsToAddModList SEQUENCE (SIZE (1..maxNrofPCI-Ranges)) OF PCI-RangeElement OPTIONAL, -- Need N ..., [[ freqBandIndicatorNR FreqBandIndicatorNR OPTIONAL, -- Need R measCycleSCell ENUMERATED {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280} OPTIONAL -- Need R ]], [[ smtc3list-r16 SSB-MTC3List-r16 OPTIONAL, -- Need R rmtc-Config-r16 SetupRelease {RMTC-Config-r16} OPTIONAL, -- Need M t312-r16 SetupRelease { T312-r16 } OPTIONAL -- Need M ]] } ReferenceSignalConfig::= SEQUENCE { ssb-ConfigMobility SSB-ConfigMobility OPTIONAL, -- Need M csi-rs-ResourceConfigMobility SetupRelease { CSI-RS-ResourceConfigMobility } OPTIONAL -- Need M } SSB-ConfigMobility::= SEQUENCE { ssb-ToMeasure SetupRelease { SSB-ToMeasure } OPTIONAL, -- Need M deriveSSB-IndexFromCell BOOLEAN, ss-RSSI-Measurement SS-RSSI-Measurement OPTIONAL, -- Need M ..., [[ ssb-PositionQCL-Common-r16 SSB-PositionQCL-Relation-r16 OPTIONAL, -- Cond SharedSpectrum ssb-PositionQCL-CellsToAddModList-r16 SSB-PositionQCL-CellsToAddModList-r16 OPTIONAL, -- Need N ssb-PositionQCL-CellsToRemoveList-r16 PCI-List OPTIONAL -- Need N ]] }

From the above MO configuration IE in TS38.331, there are two main parts for NR-U MO configuration: (1) RMTC configuration, (2) SSB-PositionQCL-Relation (the Q value in RAN1 design).

RMTC configuration contains all the timing and SCS/CP information for RSSI measurement of NR-U carrier. Thus, as long as rmtc-Config-r16 is differently configured from PCell and PSCell, the UE cannot merge those MOs.

SSB-PositionQCL-Relation determines the SSB floating relation within all candidate SSBs, and all the cell detection and measurement functions in NR-U would reply on this float information (Q value). Moreover, this Q value could also be indicated for specific cells as defined in the SSB-PositionQCL-CellsToAddModList-r16, and therefore UE needs to perform the measurement individually if this list in MOs is configured differently from PCell and PSCell. Thus, as long as ssb-PositionQCL-Common-r16 or cell list of ssb-PositionQCL is differently configured from PCell and PSCell, UE cannot merge those MOs.

Since in LTE spec TS36.331 we also have similar configuration IEs for RMTC and ssb-PositionQCL, the above two additional rules shall apply to licensed EN-DC, licensed NR-DC and NR-U scenario B.

Proposal 2: UE won’t merge NR-U MOs on the same frequency layer from PCell and PSCell if any of the following conditions is met,

-	different RSSI measurement resources or

-	different deriveSSB-IndexFromCell indications or

-	different SMTC configurations or,

- 	different ssb-PositionQCL-Common-r16 indications or cell list of ssb-PositionQCL or,

-	different rmtc-Config-r16 indication.

# Proposed revision to the R16 TS38.133

Based on the analysis in section 2, we propose the following revision to the current R16 TS38.133 spec.

#### 9.1.3.2 EN-DC: Maximum allowed layers for multiple monitoring <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> When the E-UTRA PCell and PSCell configures the same NR carrier frequency layer to be monitored by the UE in synchronous intra-band EN-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFN-s and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have - different RSSI measurement resources or - different deriveSSB-IndexFromCell indications or - different SMTC configurations or - different ssb-PositionQCL-Common-r16 indications or cell list of ssb-PositionQCL on NR carrier frequency layer with CCA or - different rmtc-Config-r16 indication on NR carrier frequency layer with CCA. Note 1: The E-UTRA-NR dual connectivity capable UE configured with PSCell shall fulfil the requirements defined in only one of clause 9.1.3.2 and clause 8.1.2.1.1b.1 of TS 36.133 [15]. <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> #### 9.1.3.2c NR-DC: Maximum allowed layers for multiple monitoring <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> When PCell and PSCell configures the same NR carrier frequency layer to be monitored by the UE in NR-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFN-s and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have - different RSSI measurement resources or - different deriveSSB-IndexFromCell indications or - different SMTC configurations or - different ssb-PositionQCL-Common-r16 indications or cell list of ssb-PositionQCL on NR carrier frequency layer with CCA or - different rmtc-Config-r16 indication on NR carrier frequency layer with CCA. <<<<<<<<<<<<<<<<<<<<<<<<<<Unrelated part omitted>>>>>>>>>>>>>>>>>>>> #### 9.1A.3.2 EN-DC: Maximum allowed layers for multiple monitoring with CCA If a UE is configured with EN-DC operation when CCA is used on PSCell, the UE shall be capable of monitoring at least: - Depending on UE capability, 7 NR inter-frequency carriers configured by PScell, and - Depending on UE capability, 7 NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell [15], and - Depending on UE capability, 6 E-UTRA TDD inter-frequency carriers configured by E-UTRA PCell [15], and - Depending on UE capability, 6 E-UTRA FDD inter-frequency carriers configured by E-UTRA PCell [15], and - Depending on UE capability, 3 FDD UTRA carriers, and - Depending on UE capability, 3 TDD UTRA carriers, and - Depending on UE capability, 32 GSM carriers (one GSM layer corresponds to 32 carriers), and In addition to the requirements defined above, the UE shall be capable of monitoring a total of at least 13 effective carrier frequency layers comprising of any above defined combination of NR, E-UTRA FDD, E-UTRA TDD, UTRA FDD, UTRA TDD and GSM (one GSM layer corresponds to 32 carriers) layers. The UE shall be capable of monitoring a total of at least 7 effective NR carrier frequency layers excluding NR serving carrier(s), comprising of any above defined combination of NR inter-RAT carriers excluding NR serving carrier(s) configured by E-UTRA PCell and NR inter-frequency carriers configured by PSCell. When the E-UTRA PCell and PSCell configures the same NR carrier frequency layer to be monitored by the UE in synchronous intra-band EN-DC, this layer shall be counted only once to the total number of effective carrier frequency layers provided that the SFN-s and slot boundaries are aligned, unless the configured NR carrier frequency layers to be monitored have - different RSSI measurement resources or - different deriveSSB-IndexFromCell indications or - different SMTC configurations or - different ssb-PositionQCL-Common-r16 indications or cell list of ssb-PositionQCL on NR carrier frequency layer with CCA or - different rmtc-Config-r16 indication on NR carrier frequency layer with CCA.

# Conclusion

In this contribution we discuss the MO merging requirement for licensed EN-DC, licensed NR-DC and NR-U scenario B.

Proposal 1: Same as licensed MO merging requirement on same NR carrier frequency layer, the principle to merge MOs on same NR-U carrier frequency layer is that those MOs would not need different measurement efforts from UE.

Proposal 2: UE won’t merge NR-U MOs on the same frequency layer from PCell and PSCell if any of the following conditions is met,

-	different RSSI measurement resources or

-	different deriveSSB-IndexFromCell indications or

-	different SMTC configurations or,

- 	different ssb-PositionQCL-Common-r16 indications or cell list of ssb-PositionQCL or,

-	different rmtc-Config-r16 indication.

# References