3GPP TSG-RAN WG4 Meeting #96-e                                                          R4-2009928

Electronic Meeting, 17 - 28 August 2020

Source:	Huawei, HiSilicon

Title:	TP to TR 37.717.21-11: Addition of DC configuration for DC_5-7_n66

Agenda item:	10.4.2

Document for:	Approval

# Background

This contribution provides text proposal on the EN-DC configuration DC_5-7_n66 as defined in WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL) [1].

# Text Proposal

##### ---Start of changes---

## 5.x	DC_5-7_n66

### 5.x.1	Configurations for DC

Table 5.x.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_5A-7A_n66A DC_5A-7C_n66A | DC_5A_n66A DC_7A_n66A |

### 5.x.2	Co-existence studies

Co-existence studies on DC_7_n66 and DC_5_n66 show own Rx impact on the 3rd band due to dual uplink as follows:

- IMD3 of dual uplink of Band 7 and n66 may fall into Band 5 DL,

- IMD2 and IMD3 of dual uplink of Band 5 and n66 may fall into Band 7 DL.

### 5.x.3	∆TIB and ∆RIB values

For DC_5-7_n66, the TIB,c and RIB,c values are reused from the LTE combination CA_5-7-66, and are given in the tables below.

Table 5.x.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-n66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.x.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-n66 | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.x.4	REFSENS requirements

The MSD values for own Rx impact of the 3rd band stated in 5.x.2 are shown in the following table.

Table 5.x.4-1: Reference sensitivity exceptions for Scell due to dual uplink operation for EN-DC in NR FR1 (three bands)

| EN-DC Configuration | EUTRA/NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode | IMD order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_5A-7A_n66A DC_5A-7C_n66A | 5 | 835 | 5 | 25 | 880 | 18.0 | FDD | IMD3 |
|  | 7 | 2560 | 5 | 25 | 2680 | N/A | FDD | N/A |
|  | 66 | 1720 | 5 | 25 | 2120 | N/A | FDD | N/A |
|  | 5 | 846.5 | 5 | 25 | 891.5 | N/A | FDD | N/A |
|  | 7 | 2504 | 5 | 25 | 2624 | 29.0 | FDD | IMD21 |
|  | 66 | 1777.5 | 5 | 25 | 2177.5 | N/A | FDD | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. |  |  |  |  |  |  |  |  |

##### ---End of changes---

# Reference

[1]	RP-201369, “New WID on Dual Connectivity of 2 band LTE (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei, RAN#88-e