3GPP TSG RAN4 Meeting #96e	R4-2010151

# Electronic Meeting, August 17 - 28, 2020

Title: 	Discussion on the new SUL band for 1880MHz - 1920MHz

Source: 	Huawei, HiSilicon

Agenda item:	10.22

Document for:	Discussion

# Introduction

In the last plenary meeting, a new WI to introduce a new SUL band for 1880 – 1920MHz is approved in [1]. The new proposed SUL band overlaps with the exact frequencies of TDD band n39. With the previous experiences in introducing the SUL band n95 which overlaps with the TDD band n34, it is straightforward that RAN4 introduces the corresponding RF requirements to facilitate complete introduction of the new SUL band n96.

For UE RF requirements, introducing SUL band n96 involves the items listed in the below box which are all specified in TS 38.101-1:

UE RF requirements - Operating bands - UE channel bandwidth - Channel raster - UE MOP and MPR/AMPR - Spurious emissions

For the BS requirements, introducing SUL band n97 involves the BS coexistence requirements: Additional spurious emission and co-location emission requirements. The related specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n96. Companion draftCRs are in [2] – [12].

# UE RF Requirements

Operating band number

In success to n95, the new SUL band to be introduced for 1880 – 1920MHz can be named with n96. And after that n97 can be used for 2300 – 2400MHz SUL band, of which the WI is also approved in June 2020.

Table 5.2-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n96 | 1880 MHz – 1920 MHz | N/A | SUL |

Proposal 1:  Introduce the new SUL band for 1880 – 1920MHz as band n96.

UE channel bandwidth

It is natural that for the new SUL band n96, the UE supporting this band should support all the channel bandwidths specified for band n39 as listed below, since n39 and n96 have the exact same frequencies for UL.

Table 5.3.5-1 Channel bandwidths for each NR band

| NR band / SCS / UE Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n96 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |

Channel raster

The channel raster for n96 should follow exactly the one for band n39 UL.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n96 | 100 | 376000 – <20> – 384000 | N/A |

MOP and A-MPR

The MOP requirements of PC3 for SUL band n96 should follow the ones for band n39 and a few others.

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n96 |  |  |  |  | 23 | ±2 |

For MPR requirements, the generic ones apply to n96.

For A-MPR requirements, it is reasonable that band n96 should follow band n39 at least with a default NS_01. Regarding NS_50, since it is to protect the 1805 – 1880MHz frequencies, we also propose to add NS_50 for band n96 since the protected frequencies are widely used globally.

Table 6.2.3.1-1A: Mapping of network signaling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n96 | NS_01 | NS_50 |  |  |  |  |  |  |

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_50 | 6.5.3.3.16 | n39, n96 | 25, 30, 40 |  | Clause 6.2.3.19 |

Spurious emissions

Regarding spurious emissions, band n96 should have the same requirements as those for band n39 as below.

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band |  | Spurious emission for UE co-existence |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Protected band |  | Frequency range (MHz) |  |  |  | Maximum Level (dBm) |  | MBW (MHz) |  | NOTE |  |
| n39, n96 |  | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 74, NR Band n79 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  |  |  |
|  |  | NR Band n77, n78 |  | FDL_low | - | FDL_high |  | -50 |  | 1 |  | 2 |  |
|  |  | Frequency range |  | 1805 | - | 1855 |  | -40 |  | 1 |  | 33 |  |
|  |  | Frequency range |  | 1855 | - | 1880 |  | -15.5 |  | 5 |  | 15, 26, 33 |  |

Proposal 2:  Specify UE RF requirements for the new SUL band for 1880 – 1920MHz following band n39.

# BS RF Requirements

Several BS specs need to be considered to introduce a new band. Taking TS 38.104 as an example, the corresponding parts shall be introduced for spurious emissions as below and in every other spec.

Table 6.6.5.2.3-1: BS spurious emissions basic limits for BS for co-existence with systems operating in other frequency bands

| System type for NR to co-exist with | Frequency range for co-existence requirement | Basic limits | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- |
| NR Band n96 | 1880 – 1920 MHz | -52 dBm | 1 MHz | This requirement does not apply to BS operating in band n39 since it is already covered in clause 6.6.5.2.2. |

Table 6.6.5.2.4-1: BS spurious emissions basic limits for BS co-located with another BS

| Type of co-located BS | Frequency range for co-location requirement | Basic limits |  |  | Measurement bandwidth | Note |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | WA BS | MR BS | LA BS |  |  |
| NR band n96 | 1880 – 1920MHz | -96 dBm | -91 dBm | -88 dBm | 100 kHz |  |

The related BS specs are: TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2.

Proposal 3:  Specify BS spurious emissions requirements for the new SUL band for 1880 – 1920MHz in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

# Conclusions

In this paper, we discuss the necessary RF requirements to be specified in order to accomplish introduction of the new SUL band n96. Companion draftCRs are in [2] – [12]. We propose:

Proposal 1:  Introduce the new SUL band for 1880 – 1920MHz as band n96.

Proposal 2:  Specify UE RF requirements for the new SUL band for 1880 – 1920MHz following band n39.

Proposal 3:  Specify BS spurious emissions requirements for the new SUL band for 1880 – 1920MHz in all the BS specs such as TS 38.104, 38.141-1/-2, 36.104, 36.141, 37.104, 37.141, 37.105, 37.145-1/-2, whenever necessary.

# References

[1] RP-201363, New WID on introduction of 1880-1920MHz SUL (supplemental uplink) band for NR, CMCC

[2] R4-2010152, draftCR to 38101-1 on introducing new SUL band n96, Huawei, HiSilicon

[3] R4-2010153, draftCR to 38104 on introducing new SUL band n96, Huawei, HiSilicon

[4] R4-1010156, draftCR to 36104 on introducing new SUL band n96, Huawei, HiSilicon

[5] R4-2010154, draftCR to 38141-1 on introducing new SUL band n96, Huawei, HiSilicon

[6] R4-2010155, draftCR to 38141-2 on introducing new SUL band n96, Huawei, HiSilicon

[7] R4-2010157, draftCR to 36141 on introducing new SUL band n96, Huawei, HiSilicon

[8] R4-2010158, draftCR to 37104 on introducing new SUL band n96, Huawei, HiSilicon

[9] R4-2010159, draftCR to 37141 on introducing new SUL band n96, Huawei, HiSilicon

[10] R4-2010160, draftCR to 37105 on introducing new SUL band n96, Huawei, HiSilicon

[11] R4-2010161, draftCR to 38145-1 on introducing new SUL band n96, Huawei, HiSilicon

[12] R4-2010162, draftCR to 38145-2 on introducing new SUL band n96, Huawei, HiSilicon