TSG-RAN Working Group 4 (Radio) meeting #96-E	R4-2010176

Electronic Meeting,  17th – 28th August 2020

Source:	Ericsson

Title:	On 52.6 to 71 GHz phase noise characteristics and draft LS to RAN1

Agenda item:	13.2.2

Document for:	Approval

# Introduction

In RAN#86, a rel-17 SI covering support for NR in 52.6 – 71 GHz was approved [1]. The SI and the consecutive WI aims to maximize the leverage of FR2 based implementations and minimize the specification burden, where possible extension of FR2 operation up to 71GHz would imply the adoption of one or more new numerologies.

Thus, one of the main objectives of the SI is to study applicable numerology including subcarrier spacing and maximum channel BW. This is a joint responsibility of RAN1 and RAN4 where RAN4 will start to discuss this SI from August meeting as agreed in RAN plenary.

To further investigate the applicable numerology and sub-carrier spacing, it is essential to have a realistic up to date model of phase noise. A realistic phase noise model is also important to decide the supported modulation order and corresponding requirements. This issue has been discussed in RAN1 e-meeting in May and a LS requesting the needed studies around impairments and phase noise modelling was sent to RAN4 [2]. The expected action from RAN4 is as following:

RAN WG1 respectfully requests timely feedback from RAN WG4 on the applicability of the above modelling to NR in the 52.6 to 71 GHz frequency range. In the meantime, RAN WG1 will continue the study on the objectives with the existing models.

Due to expected larger sub-carrier spacing for this SI compared to FR2, the different regions of phase noise characteristics will affect the overall performance in different proportions (compared to FR2 with maximum bandwidth significantly smaller than the maximum bandwidths foreseen for 52.6 – 71 GHz). We further believe in the need of a better phase noise modeling that is more representative of integrated RF circuit solutions more suited for unlicensed and licensed band operations in this frequency range. As for NR FR2 discussion, proxy frequencies of 30, 45 and 70 GHz was used and thus in this contribution, the model uses the 70 GHz proxy frequency.

In this contribution, we present a new phase noise model based on recently published data on both state-of-the-art PLL and crystal oscillators that lead to an improved model representing the current technology envelope. It should of course be understood that it will be more challenging to realize a higher frequency PLL, and for the same performance it will consume more power. In addition, comparison between existing models, new proposed models and state-of-the-art high performance PLL published data is done.

Furthermore, an implementation design margin is introduced to account for inevitable variations in semi-conductor process manufacturing, voltage, temperature, and frequency of operation. The implementation design margin ensures that the model represents realistic performance for numerology evaluation studies.

At the end on this contribution a draft LS to RAN1 is attached.

# Discussion

There are many different high performance PLL architectures, each with several design parameters, that can be used to generate local oscillator signals in 7-100GHz transceivers.

The scaling of semiconductor technology has resulted in increased speed and reduced power consumption of digital circuits, leading to competitive digital PLL architectures. Analog PLL architectures have also seen advancements with technology, e.g. sub-sampling PLLs. Today both analog and digital PLLs are viable choices to achieve state-of-the-art performance. Regardless of architecture the PLL phase noise characteristic will have a similar profile, which facilitates the modelling.

  - State-of-the-art PLL published characteristics

To be able to derive an up-to-date phase noise model for 70 GHz many references published in leading IEEE conferences, journals and other conference papers from 2015 to 2020 have been investigated.

Figure 2.1-1 depicts all the published state-of-the-art phase noise characteristics scaled to 70 GHz where references covered a large variety of frequencies. The scaling is performed by adding a term 20*log(70GHz/f0) dB to the phase published phase noise data, where f0 is the frequency of operation where the phase noise was measured.

Figure 2.1-1: Phase noise characteristics for all references.

All outlier references with poor performance in any offset frequency range were removed as shown in Figure 2.1-2 to provide a better picture of achievable state-of-the-art.

Figure 2.1-2: Phase noise characteristics for all references excluding the low performing outliers.

Furthermore, as references cover a large variety of PLL output frequencies, Figure 2.1-3 represents heat coloring with respect to PLL output frequencies. Note that the intention with Figure 2 and Figure 3 is not only to present the input data to derive a relevant phase noise model but also to later demonstrate relevant published high performing state-of-the-art PLL characteristics in comparison with existing models as well as a new up-to-date model. The approach for derivation of the new proposed phase noise model is described in section 2.2.

As shown in Figure 2.1-3, the published PLL data operating at 50-100 GHz indicate lower performance compared to PLL data operating at lower frequencies, which is expected due to reduced circuit performance at higher frequency.

Figure 2.1-3: Heat coloring with respect to PLL output frequencies

  - Derivation approach of new up-to-data phase noise model

At low offset frequencies the phase noise will be dominated by that of the reference, as the PLL multiplies its frequency and tracks slow phase variations. The phase noise in that region will equal that of the reference plus 20*log(N) dB, where N is the frequency multiplication factor of the PLL. For a certain RF frequency, N will be inversely proportional to the reference frequency (N=RF/Fref). A high reference frequency is thus beneficial in reducing N and the reference phase noise contribution. At higher offset frequencies the PLL will dominate the phase noise. The region below the PLL bandwidth, which can be chosen differently but is often of the order of 1MHz in high performance PLLs for this frequency range, is called the in-band noise. The in-band noise is due to noise of different building blocks in the PLL, and typically has a -10dB/decade slope at lower offset frequencies due to flicker noise, and a zero slope at higher offsets due to thermal noise. Above the PLL bandwidth, the out-of-band noise is dominated by oscillator phase noise, typically thermal noise with a slope of -20dB/decade. At very high offset frequencies the PLL phase noise reaches a floor with zero slope, but typically buffers or frequency dividers outside the PLL itself will dominate this floor in a transceiver. Figure 2.2-1 shows the PLL phase noise profile and its regions.

Phase Noise (dBc/Hz)

Log frequency

Reference noise

In-band flicker noise

In-band thermal noise

Oscillator noise

Noise floor

PLL BW

Figure 2.2-1: PLL phase noise profile and regions

With a bandwidth typically of the order of about 1MHz, at 10MHz offset the oscillator phase noise will dominate in most PLLs. Since the oscillator phase noise is well studied and there are fundamental relations derived this is a good starting point.  At 10MHz offset the oscillator will likely be dominated by thermal noise, modelled in the classic paper by Leeson in 1966 [3]:

L(Δf)=10∙log2FkTPS∙f02Q∙∆f2	(Eq. 2.2-1)

Where L is the phase noise, Δf the offset frequency, F the noise factor, equal to 1 in an ideal oscillator, k is Boltzmanns constant equal to 1.38065∙10-23 J/K, T is the temperature equal to 293 K in room temperature, PS is the power dissipation in the resonator, f0 the oscillation frequency, and Q is the resonator quality factor.

Let’s assume that we have an oscillator at 70GHz that consumes 10mW, and that the oscillator has an efficiency of 20%, so that PS is equal to 2mW. Further assume a noise factor equal to 5, and a quality factor of the resonator equal to 10, which is realistic for high performance on-chip LC resonators at this frequency. We can then calculate the phase noise at 10MHz offset using Eq. 2.2-1:

L10M=10∙log2∙5∙1.38065∙10-23∙2932∙10-3∙70∙1092∙10∙10∙1062= -116 dBc/Hz	(Eq. 2.2-2)

This is a level that should be reachable in a stand-alone high-performance oscillator integrated in a CMOS process suitable for high integration level, and with a power consumption compatible with the power budget of many published PLLs.

Assuming a 20dB/decade slope would then give -96dBc/Hz at 1MHz offset, and if we assume a bandwidth of 1MHz and a flat phase noise below, we get the in-band phase noise level of -96 dBc/Hz. The onset of the flicker noise region is difficult to estimate as it depends on the process and architecture.

To gain a better understanding of what is achievable, especially in-band, a study of state-of-the-art publications is needed. The phase noise level at offset frequencies corresponding to out-of-band, in-band thermal, and in-band 1/f should be investigated for state-of-the-art PLLs with different operating frequencies. The phase noise of state-of-the-art crystal oscillators at relevant reference frequencies used by the PLLs should also be investigated. From this it should then be possible to derive a model for the phase noise profile and how this is related to operating frequency. It should be noted there is a clear trend where the power consumption increases with PLL output frequency as shown in Figure 2.2-2.

Figure 2.2-2: Power consumption versus upper operating frequency

    - PLL Phase noise at different frequency offsets

Based on the input phase noise data from references discussed in previous sections, Figures 2.2.1-1 to Figure 2.2.1-3 present phase noise at 100kHz, 1MHz and 10MHz offset, respectively, versus output frequency. In the figures are also curves shown with a slope of 20dB per decade, corresponding to equal phase noise performance. The phase noise performance is considered equal in a sense that comparing two signals, one at a first frequency f1 with a phase noise L1 dBc/Hz, and one at an N times higher frequency N*f1 and a phase noise L1+20*log(N) dBc/Hz, frequency division by N of the second signal will provide a signal with frequency f1 and phase noise L1 dBc/Hz. Two signals on the curve can thus achieve the same phase noise performance when used in a transceiver receiving or transmitting a signal at a certain frequency. As can be seen the curves represent the frequency trend in the data-points satisfactory, and from this we draw the conclusion that the phase noise performance is close to frequency independent, which is achieved at the expense of a clear power consumption increase versus frequency.

Figure 2.2.1-1: Phase noise at 100kHz offset versus upper operating frequency

Figure 2.2.1-2: Phase noise at 1MHz offset versus upper operating frequency

Figure 2.2.1-3: Phase noise at 10MHz offset versus upper operating frequency

    - State-of-the-art crystal oscillators

The PLLs in this contribution use reference frequencies of 40 MHz and above. Since the problem of reference noise increases at lower reference frequencies, due to an increased effective frequency multiplication factor, to be conservative low power crystal oscillators around 40MHz were investigated. Considering limited publications on crystal oscillator circuits compared to PLLs, three references  [21]-[23] from recent years in IEEE Journal of Solid-State Circuits were used to represent state-of-the-art in CMOS low power crystal oscillators at about 40MHz. To span the design space, also the 491.25 MHz crystal oscillator of [4] is included in Table 2.2.2-1, summarizing the crystal oscillator performance.

Table 2.2.2-1: Crystal Oscillator performance

|  | [4] | [21] | [22] | [23] |
| --- | --- | --- | --- | --- |
| Frequency (MHz) | 491.25 | 39.25 | 48 | 39.25 |
| Core power (uW) | 840 | 181 | 1500 | 19 |
| Process (nm) | 28 | 180 | 28 | 65 |
| PN @ 1kHz (dBc/Hz) | -107 | -147 | -114 | -139 |
| PN @ 10 kHz (dBc/Hz) | -130 | -161 | -148 | -153 |
| PN @100 kHz (dBc/Hz) | -140 | -172 | -156 | -153 |
| PN @ 1MHz (dBc/Hz) | -148 | -175 | -158 | -153 |
| 70GHz PN @ 1kHz (dBc/Hz) | -64 | -82 | -51 | -74 |
| 70GHz PN @ 10 kHz (dBc/Hz) | -87 | -96 | -85 | -88 |
| 70GHz PN @100 kHz (dBc/Hz) | -97 | -107 | -93 | -88 |
| 70GHz PN @ 1MHz (dBc/Hz) | -105 | -110 | -95 | -88 |
| FoM (dBc/Hz) | -227 | -246 | -220 | -248 |

To translate the phase noise into equivalent phase noise at a PLL output, an amount equal to 20∙log⁡(fPLLfref) dB was added. For instance, for a 70 GHz PLL with a 39.25 MHz reference, the amount to add is thus 65 dB, and with 48 MHz reference it is 63 dB, and with 491MHz it is 43 dB. This has been done in the fields starting with 70GHz in the table, where the phase noise of the different crystal oscillators can be compared directly in terms of their impact on a 70 GHz PLL output signal.

Based on the above, it can be assumed that the crystal oscillator can be made non-dominant above a few kHz offset frequency, and that the PLL output phase noise will have a slope of -30dB/decade below that frequency. This is achievable with reference frequencies ranging from 40MHz to 491MHz, and power consumptions below 1mW, using CMOS technologies as different as 28nm and 180nm technologies. Thus, considering the foreseen numerologies for 52.6-71 GHz with sub-carrier spacings larger than for FR2, the impact of crystal oscillators is likely to be limited.

    - Noise floor

When it comes to the noise floor it is difficult to find published works that include measurements in this region. Circuit simulations have therefore been performed to find realistic levels of the noise floor. The noise floor is typically set not by the oscillator of the PLL, but by buffers and LO distribution. For this reason, the phase noise of an LO distribution circuitry operating at 48GHz transporting the LO signal 2mm was simulated. The circuit featured 4 differential tuned buffer stages. The simulated phase noise is given in Figure 2.2.3-1. At 100MHz offset the thermal noise floor is at -158dBc/Hz.

Figure 2.2.3-1: Phase noise of simulated differential LO buffers for a 48GHz signal

Distributing a signal at higher frequency, maintaining the same load impedance of the buffer stages, and using the same inductor quality factor (and hence relative bandwidth), would require the load capacitance to be scaled inversely with frequency. The number of buffer stages per distance would then be proportional to the LO frequency, and since each buffer would operate with the same load impedance, the power in each buffer would be the same, and the power consumption per distance would also be proportional to the LO frequency. The phase noise would also increase with LO frequency, due to the increasing number of cascaded buffers. The resulting scaling would then be that both phase noise and power would increase by 10dB per decade, as LO frequency is increased, unless an architecture with shorter LO distribution is used. Since the frequency ratio of 10 times between 100GHz and 10GHz equals the Q, the buffer load impedance would get the same magnitude, and the same buffer bias and sizing could be used without the inductors. If inductors were used, however, the power consumption could be reduced. At frequencies in between, different architectures are possible, tuned, non-tuned, and partially tuned. The exact phase noise floor achievable at different frequencies is dependent on architecture and technology. A conservative estimate is to put a flat line between the non-tuned 10GHz buffer and the fully tuned 100GHz resulting in noise floor of -155dBc/Hz.

    - Proposed phase noise model

Using the trend curves in Figure 2.2.4-1 to 2.2.4-3 as a starting point, defining state-of-the-art performance, the levels for these curves at 70GHz are -88dBc/Hz, -91dBc/Hz, and -111dBc/Hz, at 100kHz, 1MHz, and 10MHz, respectively.

From this we create a phase noise profile valid at other offset frequencies. We then also include crystal oscillator phase noise and the noise floor. Below 1.6kHz offset the crystal oscillator phase noise is dominant, with a slope of -30dB/decade, and above 1.6GHz the noise is at the -155dBc/Hz floor.

Figure 2.2.4-1: State-of-the-art phase noise vs. offset from a 70GHz carrier

Figure 10 shows the phase noise versus offset from a 70GHz carrier. When operating at another frequency fc an offset of 20*log(fc/70GHz) dB is to be added.

Published measurements are often performed under favorable conditions, i.e. the measurements are often taken in room temperature, the supply voltage, bias current settings etc. may be fine-tuned, only a few samples are measured (not covering the expected variation in integrated circuit manufacturing), some realistic conditions may not be investigated, etc. To obtain phase noise requirements that can be applied to PLLs in practical applications, we therefore propose to add a ~5 dB-design margin to the state-of-the-art phase noise profile also presented in Figure 2.2.4-2.

Figure 2.2.4-2: PLL phase noise model for 70GHz with and without 5 dB design margins.

Using this model the phase noise requirements are in a sense PLL frequency independent, so that it will not matter if a sliding IF using a PLL with 2/3 of the carrier signal frequency is used, or a homodyne with the PLL at the carrier, or a homodyne using a PLL at 2 times the carrier frequency. With this phase noise model the PLL requirements will be equivalent from a system performance point of view. It should of course be understood that it will be more challenging to realize a higher frequency PLL, and for the same performance it will consume more power.

An essential benefit of this model is that different parts of the 7 to 100GHz frequency ranges can be addressed by a single PLL or a set of PLLs with different frequency divider arrangements, and the PLL phase noise requirements will with this model be the same for all the different operating modes, drastically simplifying analysis for a large frequency range covering 7-24 GHz as well as higher frequencies than 71 GHz in the future.

To better adapt the model to calculations, an equation was fitted to the above curves. Like in [25] an equation of the form below was used, using poles and zeros;

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m	(Eq. 2.2.4-1)

Where S is the single sideband phase noise power spectral density, fo the offset frequency, fz,1 .. fz,N the zeros, fp,1 .. fp,M the poles, αz,1 .. αz,N the order of the zeros, and αp,1 .. αp,M the orders of the poles. To fit Eq. 2.2.4-1 to the curves in Figure 2.2.4-1 the following parameters were used:

Table 2.2.4-1: Parameters of phase noise model equation with design margin

| Parameter | Value/expression | Parameter | Value |
| --- | --- | --- | --- |
| PSD0 | fc3.55e92∙10DM/10 |  |  |
| fz,1 | 1.6e3 | αz,1 | 2 |
| fz,2 | 200e3 | αz,2 | 1 |
| fz,3 | fc44.8 | αz,3 | 2 |
| fp,1 | 1 | αp,1 | 3 |
| fp,2 | 1e6 | αp,2 | 2 |

Here, fc and DM represent the PLL output frequency (Hz) and design margin (dB), respectively.

Note that for simplicity the orders of all poles and zeros are integers in this model. This is possible in an expression for power spectral density. In an expression for voltage spectral density on the other hand, fractional orders would have been needed to represent the slopes in regions where 1/f noise dominates.

As can be seen the parameters PSD0 and fz,3 depend on the carrier frequency, whereas the other parameters are carrier frequency independent. At fc =70GHz and DM=5dB, PSD0 will be equal to 1230, or 31 dB. To compare the result of equation (3) to figure 11, S must be converted to a logarithmic scale:

Lfo=10∙log⁡(Sfo) dBc/Hz					(Eq. 2.2.4-2)

The comparison can be seen in Figure 2.2.4-3, and as can be seen there is a good fit, and Eq. 2.2.4-2 with the parameters in Table 2.2.4-1 can be used to represent the phase noise.

Figure 2.2.4-3: PLL phase noise model for 70GHz with and without 5 dB design margin, with asymptotic curves of Figure 2.2.4-2 in grey for comparison.

Since the proposed model is based on state-of-the-art high performance PLL design with low power consumptions, the proposed model can be used both for BS and UE.

  - Comparison of recent published data, proposed new phase noise model and existing models

The new proposed phase noise model matches the published state-of-the-art PLL performance very well where both the new parameterized model as well as an average characteristics of input data is presented in Figure 2.3-1.

Figure 2.3-1: Proposed phase noise model for 70GHz with relating to input data

As the LS from RAN1 pose a question on existing models, a comparison between state-of-the art PLL data, proposed new models (with and without 5 dB design margins) and existing models is given in Figure 2.3-2.

Figure 2.3-2: Comparison of proposed new model, existing models and state-of-the-art PLL input data and characteristics.

As shown in Figure 2.3-2, the existing models (denoted as Ex1, Ex2 BS and Ex2 UE) seem to be either over-optimistic or over-pessimistic at certain frequency offsets relevant for foreseen SCS for 52.6-71 GHz) while the proposed new model very good matches the available high performing state-of-the art PLL performance. Thus, we believe that a revised phase noise model is needed to properly capture the recent research on PLL performance for RAN1 and RAN4 evaluations and future work.

We thus propose the following:

Proposal 1:

Consider the following phase noise model for evaluating the designs of NR operation in the 52.6 – 71 GHz frequency range:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

where

| Parameter | Value/expression | Parameter | Value |
| --- | --- | --- | --- |
| PSD0 | fc3.55e92∙10DM/10 |  |  |
| fz,1 | 1600 | αz,1 | 2 |
| fz,2 | 200e3 | αz,2 | 1 |
| fz,3 | fc44.8 | αz,3 | 2 |
| fp,1 | 1 | αp,1 | 3 |
| fp,2 | 1e6 | αp,2 | 2 |

Proposal 2:

Consider 5 dB design margin (DM) used in the proposed new phase noise model to cater for variation in manufacturing process, frequency and temperature.

Proposal 3:

Send a RAN4 LS response to RAN1 about the new proposed model and usage of proposed model for numerology studies in RAN1.

# Conclusion

In the previous sections, we discussed the need of a more up-to-date phase noise modeling that is more representative of integrated RF circuit solutions suited for 52.6-71 GHz range, low cost unlicensed band operations as well as licensed operation. A realistic phase noise model is also important to decide the supported modulation order and corresponding requirements.

To find state-of-the-art PLL performance, published papers in leading IEEE conferences and journals since 2015 up to 2020 have been investigated. To support high level of integration circuits in CMOS technologies were selected, with a power consumption below about 50mW making the models applicable both for UE and BS.

Based on the literature survey, we develop and present a new phase noise model better representing the current technology envelope. Using the proposed model, the phase noise requirements are in a sense PLL frequency independent. In addition, with proposed phase noise model the PLL requirements will be equivalent from a system performance point of view. It should of course be understood that it will be more challenging to realize a higher frequency PLL, and for the same performance it will consume more power.

Furthermore, an implementation design margin is introduced to account for inevitable variations in semi-conductor process manufacturing, voltage, temperature, and frequency of operation. The implementation design margin ensures that the model represents realistic performance for numerology evaluation studies.

In addition, a comparison between existing models, new proposed models and input data were made to demonstrate that the existing models are either over-optimistic or over-pessimistic implying the need for new up-to-date phase noise models presenting the current technology envelope.

Based on the above, we propose;

Proposal 1:

Consider the following phase noise model for evaluating the designs of NR operation in the 52.6 – 71 GHz frequency range:

Sfo=PSD0n=1N1+(fofz,n)αz,nm=1M1+(fofp,m)αp,m

where

| Parameter | Value/expression | Parameter | Value |
| --- | --- | --- | --- |
| PSD0 | fc3.55e92∙10DM/10 |  |  |
| fz,1 | 1.6e3 | αz,1 | 2 |
| fz,2 | 200e3 | αz,2 | 1 |
| fz,3 | fc44.8 | αz,3 | 2 |
| fp,1 | 1 | αp,1 | 3 |
| fp,2 | 1e6 | αp,2 | 2 |

Proposal 2:

Consider 5 dB design margin (DM) used in the proposed new phase noise model to cater for variation in manufacturing process, frequency and temperature.

Proposal 3:

Send a RAN4 LS response to RAN1 about the new proposed model and usage of proposed model for numerology studies in RAN1.

# References

RP-193952, “Study on supporting NR from 52.6 GHz to 71 GHz”

R1-2005196,”LS to RAN4 on Phase noise and other RF Impairment modelling”, RAN1

D. B. Leeson, “A simple model of feedback oscillator noise spectrum”, Proceeding of the IEEE, vol. 54, no. 2, pp. 329-330, 1966.

S. Ek et. al. “A 28-nm FD-SOI 115-fs Jitter PLL-Based LO System for 24–30-GHz Sliding-IF 5G Transceivers”, IEEE Journal of Solid-State Circuits, vol. 53, no. 7, pp. 1988-2000, July 2018.

K. Raczkowski et. al. “A 9.2–12.7 GHz Wideband Fractional-N Subsampling PLL in 28 nm CMOS with 280 fs RMS Jitter”, IEEE Journal of Solid-State Circuits, vol. 50, no. 5, pp. 1203-1213, May 2015.

Hussein, S. Vasadi, J. Paramesh, “A 50–66-GHz Phase-Domain Digital Frequency Synthesizer with Low Phase Noise and Low Fractional Spurs”, IEEE Journal of Solid-State Circuits, vol. 52, no. 12, pp. 3329-3347, Dec. 2017.

Z. Zong, P. Chen, R. Staszewski, “A Low-Noise Fractional-N Digital Frequency Synthesizer with Implicit Frequency Tripling for mm-Wave Applications”, IEEE Journal of Solid-State Circuits, vol. 54, no. 3, pp. 755-767.

Ximenes, G. Vlachogiannakis, R. Staszewski, “An Ultracompact 9.4–14.8-GHz Transformer-Based Fractional-N All-Digital PLL in 40-nm CMOS”, IEEE Transactions on Microwave Theory and Techniques, vol. 65, no. 11, pp. 4241-4254, Nov. 2017.

M. Ferriss et. al. “A 13.1-to-28GHz Fractional-N PLL in 32nm SOI CMOS with a ΔΣ Noise-Cancellation Scheme”, IEEE International Solid-State Circuits Conference 2015, pp. 192-193

N. Markulic et. al. “A Self-Calibrated 10Mb/s Phase Modulator with -37.4dB EVM Based on a 10.1-to-12.4GHz, -246.6dB-FOM, Fractional-N Subsampling PLL”, IEEE International Solid-State Circuits Conference 2016, pp. 176-177.

L. Grimaldi et. al. “30GHz Digital Sub-Sampling Fractional-N PLL with 198fsrms Jitter in 65nm LP CMOS”, IEEE International Solid-State Circuits Conference 2019, pp. 268-269

W. El-Halwagy et. al. “A 28GHz Quadrature Fractional-N Synthesizer for 5G Mobile Communication with Less than 100fs jitter in 65nm CMOS”, IEEE Radio Frequency Integrated Circuits Symposium 2016, pp. 118-121.

W. Wu et. al. “A 5.5-7.3 GHz Analog Fractional-N Sampling PLL in 28-nm CMOS with 75 fsrms Jitter and -249.7 dB FoM”, IEEE Radio Frequency Integrated Circuits Symposium 2018, pp. 52-55

M. Dayanik, N. Collins, M. Flynn, “A 28.5 – 33.5GHz Fractional-N PLL Using a 3rd Order Noise Shaping Time-to-Digital Converter with 176fs Resolution”, IEEE European Solid-State Circuits Conference 2015, pp. 376-379.

Y. Chao, H. C. Luong, Z. Hong “Analysis and Design of a 14.1-mW 50/100-GHz Transformer-Based PLL With Embedded Phase Shifter in 65-nm CMOS”, IEEE Transactions on Microwave Theory and Techniques, vol. 63, no. 4, pp.1193-1201, Apr. 2015.

Z. Huang et. al. “A 70.5-to-85.5GHz 65nm Phase-Locked Loop with Passive Scaling of Loop Filter”, IEEE International Solid-State Circuits Conference 2015, pp. 448-449.

Z. Huang, H. C. Luong, ”An 82–107.6-GHz Integer-N ADPLL Employing a DCO With Split Transformer and Dual-Path Switched-Capacitor Ladder and a Clock-Skew-Sampling Delta–Sigma TDC”, IEEE Journal of Solid-State Circuits, Vol. 54, No. 2, pp. 358-367, Feb. 2019.

X. Liu, H. C. Luong, “A Fully Integrated 0.27-THz Injection-Locked Frequency Synthesizer with Frequency-Tracking Loop in 65-nm CMOS”, IEEE Journal of Solid-State Circuits, 2019

V. Szortyka et. al. “A 42 mW 200 fs-Jitter 60 GHz Sub-Sampling PLL in 40 nm CMOS”, IEEE Journal of solid-State Circuits, Vol. 50, No. 9, pp.2025-2036, Sept. 2015.

X. Yi et. al. “A 93.4–104.8-GHz 57-mW Fractional-N Cascaded PLL With True In-Phase Injection-Coupled QVCO in 65-nm CMOS Technology”, IEEE Transactions on Microwave Theory and Techniques, Vol. 67, No. 6, pp. 2370-2381, June 2019.

S. Iguchi et. al. “Variation-Tolerant Quick-Start-Up CMOS Crystal Oscillator with Chirp Injection and Negative Resistance Booster”, IEEE Journal of Solid-State Circuits, vol. 51, no. 2, pp. 496-508, Feb. 2016.

Y. Rajavi et. al. “A 48-MHz Differential Crystal Oscillator With 168-fs Jitter in 28-nm CMOS”, IEEE Journal of Solid-State Circuits, vol. 52, no. 10, pp. 2735-2745, Oct. 2017.

S. Igushi, T. Sakurai, M. Takamiya, “A Low-Power CMOS Crystal Oscillator Using a Stacked-Amplifier Architecture”, IEEE Journal of Solid-State Circuits, vol. 52, no. 11, pp. 3006-3017, Nov. 2017.

A. Santiccioli et al., "17.2 A 66fsrms Jitter 12.8-to-15.2GHz Fractional-N Bang-Bang PLL with Digital Frequency-Error Recovery for Fast Locking," 2020 IEEE International Solid- State Circuits Conference - (ISSCC), San Francisco, CA, USA, 2020, pp. 268-270.

- [25]	Ericsson, “Further elaboration on mm-wave phase noise modelling”, R4-1703087, 3GPP TSG-RAN WG4 #82bis

DRAFT LS:

3GPP TSG RAN WG4 Meeting #96-e		TDoc R4-xxyyzzz

Electronic Meeting, 17 – 28 August, 2020

Title:	LS on Phase noise and other RF Impairment modelling

Response to:	RAN WG1: R1-2005196, LS to RAN4 on Phase noise and other RF Impairment modelling

Release:	Rel-17

Work Item:	FS_NR_52_to_71GHz

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact person:	Torbjörn Elfström

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

[1]		R4-2010176, “On 52.6 to 71 GHz phase noise characteristics and draft LS to RAN1”, Ericsson

# 1	Overall description

RAN WG4 received the incoming LS from RAN WG1on Phase noise and other RF Impairment modelling considering RAN WG1 studies on Evaluation Methodology for the Study on supporting NR from 52.6 GHz to 71 GHz. In the LS, WG RAN1 states that Phase noise (PN) modelling is necessary in the RAN1 evaluation of applicable numerology including subcarrier spacing and channel BW and requests timely feedback from RAN WG4 on the applicability of the two PN models provided in TR 38.803 (Ex-1 and Ex-2 developed during rel 15 NR study item) to NR in the 52.6 to 71 GHz frequency range.

RAN WG4 has studied the latest representative published state-of-the-art PLL phase noise performance and has developed an up-to-date Phase noise model including reasonable implementation margin. In addition, a comparison between PLL input data, new proposed model and Ex-1 and Ex-2 were investigated [1].

RAN WG4 concluded that the existing models are either over-optimistic or over-pessimistic compared to state-of-the- art PLL and recommend RAN WG1 to use the new up to date PN model representing the technology envelope described in [1].

Modelling of other impairments e.g. power amplifier (PA), either directly or approximately via EVM injection, and other RF impairments, such as I/Q imbalance and frequency offset which will be optionally considered in the RAN1 evaluation was not considered since RAN WG4. Developing such models will take excessive amount of time and effort and it is not clear to RAN WG4 how such impairment models affect the numerology evaluations.

# 2	Actions

To RAN WG1

ACTION: 	RAN WG4 kindly request RAN WG1 to use the proposed new PN model for numerology evaluation studies on supporting NR from 52.6 GHz to 71 GHz.

# 3	Dates of next TSG RAN WG 4 meetings

3GPP RAN4#97	2020-11-16 - 2020-11-20	US

3GPP RAN4#98	2021-03-01 - 2021-03-05	EU, Athens