3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010265

# Electronic Meeting, 17-28 Aug., 2020

Source: 	Skyworks Solutions, Inc.

Title: 	Spectrum Utilization for 35MHz and 45MHz Channels in FR1 and their Support

Agenda Item:	10.18.2	Spectrum utilization	[NR_FR1_35MHz_45MHz_BW-Core]

Document for:	Approval

# Introduction

In RAN#88 e-meeting, a new WI was agreed [1] to introduce 35MHz channel BW for bands n3, n7, n8, n25, n66, n71 and 45MHz channel BW for bands n3, n25, n66. In this contribution, we provide our input on spectrum utilization and support of these new channel bandwidths.

# Discussion

## Spectrum Utilization Calculations

We previously discussed how to derive spectrum utilization for new channel bandwidths in [2] using a linear fit of the SU defined for the channel bandwidths supported in Rel16. In this case we are discussing “regular” channel bandwidths (multiple of 5MHz). The linear fit coefficients proposed in [2] can be found in Table 1.

Table 1: Linear fit coefficients for SU

| SCS [kHz] | 15 | 30 | 60 |
| --- | --- | --- | --- |
| A1 | 5.4509 | 2.7619 | 1.3788 |
| A0 | -2.7399 | -3.9468 | -3.3747 |

The SU calculated with this linear fit can be found in Table 2 and compared with SU with twice the channel BW and twice the SCS.

Table 2: SU calculations and resulting guard band

| Channel BW | SCS | 15kHz |  | 30kHz |  | 60kHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Approach | SU | GB [MHz] | SU | GB [MHz] | SU | GB [MHz] |
| 35MHz | Linear fit | 188 | 0.5725 | 93 | 0.745 | 45 | 1.27 |
|  | 70 MHz SU | 189 | 0.4825 | 93 | 0.745 |  |  |
| 45MHz | Linear fit | 243 | 0.6225 | 120 | 0.885 | 59 | 1.23 |
|  | 90 MHz SU | 245 | 0.4425 | 121 | 0.705 |  |  |

Observations:

- For 35 MHz, if the 30 kHz SU for 70 MHz is used at 15 kHz then the guard band is smaller than the 30 and 40 MHz guard bands at 15 kHz => should use 188

- With SU of 93, the 35 MHz guard band at 30 kHz SCS is smaller than the corresponding 30 MHz guard band => should use 92

- With SU of 45, the 35 MHz guard band at 60 kHz SCS is slightly smaller than the corresponding 30 MHz guard band => 45 can be kept

- For 45 MHz, if the 30 kHz SU for 90 MHz is used at 15 kHz then the guard band is smaller than the corresponding the 40 and 50 MHz guard bands => should use 243

- For 45 MHz, if the 60 kHz SU for 90 MHz is used at 30 kHz then the guard band is smaller than the corresponding 40 and 50 MHz guard bands => should use 120

- With SU of 59, the 45 MHz guard band at 60 kHz SCS is s smaller than the corresponding 30, 40 and 50 MHz guard band => should use 58.

With the above corrected values, Table 3 provides a comparison of the 35 and 45 MHz parameters with 30, 40 and 50MHz values, and shows that these values are appropriate in terms of resulting guard-bands when compared with their immediate neighboring existing channel bandwidths.

Table 3: 35 and 45 MHz parameters vs existing BW

|  |  | Channel BW [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter | SCS [kHz] | 30 | 35 | 40 | 45 | 50 |
|  |  | NRB | NRB | NRB | NRB | NRB |
| SU_CP-OFDM [#RB] | 15 | 160 | 188 | 216 | 243 | 270 |
|  | 30 | 78 | 92 | 106 | 120 | 133 |
|  | 60 | 38 | 45 | 51 | 58 | 65 |
| SU_DFT-s-OFDM [max#RB full allocation] | 15 | 160 | 180 | 216 | 243 | 270 |
|  | 30 | 75 | 90 | 100 | 120 | 128 |
|  | 60 | 36 | 45 | 50 | 54 | 64 |
| SU_CP-OFDM [max%] | 15 | 96.0% | 96.7% | 97.2% | 97.2% | 97.2% |
|  | 30 | 93.6% | 94.6% | 95.4% | 96.0% | 95.8% |
|  | 60 | 91.2% | 92.6% | 91.8% | 92.8% | 93.6% |
| SU_DFT-s-OFDM [max%] | 15 | 96.0% | 92.6% | 97.2% | 97.2% | 97% |
|  | 30 | 90.0% | 92.6% | 90.0% | 96.0% | 92% |
|  | 60 | 86.4% | 92.6% | 90.0% | 86.4% | 92% |
| ACLR Meas BW [MHz] | lowest | 28.815 | 33.855 | 38.895 | 43.755 | 48.615 |
| Min guard band [MHz] | 15 | 0.5925 | 0.5725 | 0.5525 | 0.6225 | 0.6925 |
|  | 30 | 0.945 | 0.925 | 0.905 | 0.885 | 1.045 |
|  | 60 | 1.290 | 1.270 | 1.610 | 1.590 | 1.570 |

Proposals 1 for 35 and 45 MHz NRB, guard band and ACLR measurement BW provided in Table 4:

Table 4: proposed SU for 35 and 45 MHz BW and associated parameters

|  | Channel BW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 35 MHz |  |  | 45 MHz |  |  |
| SCS [kHz] | 15 | 30 | 60 | 15 | 30 | 60 |
| NRB | 188 | 92 | 45 | 243 | 120 | 58 |
| Min Guard-band [MHz] | 0.5725 | 0.925 | 1.27 | 0.6225 | 0.885 | 1.59 |
| ACLR measurement BW [MHz] | 33.855 |  |  | 43.755 |  |  |

## Support of New Channel Bandwidths

These new channel bandwidths are currently requested only for given bands and we should not include them as generic bandwidths in the specification as of yet. Furthermore, we should understand which potential limitations there are in terms of release independence.

As an example, 35MHz in n71 cannot work for a phone that implements this band with dual duplexers. Also, legacy ET PA may not support such bandwidth and, as of today does not even support 25 and 30 MHz. This issue may be generic for low bands and is a further problem when there are short duplex distance like for n71 as the ET BW must be carefully controlled to avoid noise being injected in the receive band.

In general, if the new channel bandwidth is smaller than the largest one already supported it can be introduced more easily and at least we know that the ET BW is sufficient. Table 5 shows how the new bandwidths compare to the current maximum bandwidth supported in each band.

Table 5: Current supported maximum BW per band vs new bandwidths

| Band | New BW | Max Rel 16 BW | comment |
| --- | --- | --- | --- |
| n3 | 35, 45 MHz | 40 MHz | 35 MHz BW covered, 45MHz extends BW |
| n7 | 35 MHz | 50 MHz | 35 MHz BW covered |
| n8 | 35 MHz | 20 MHz | 25 and 30MHz not even yet agreed, significant extension |
| n25 | 35, 45 MHz | 40 MHz | 35 MHz BW covered, 45MHz extends BW |
| n66 | 35, 45 MHz | 40 MHz | 35 MHz BW covered, 45MHz extends BW |
| n71 | 35 MHz | 20 MHz | 25 and 30MHz not even yet agreed, significant extension, dual duplexer cases |

Observations:

- n7 already cover 35 MHz

- n3, n25, n66 already cover 35 MHz but not 45 MHz

- n8 and n71 see significant BW extension vs the current 20 MHz and have not yet been extended to 25 and 30 MHz

- The largest BW supported in LB is 30MHz for band n28.

With this it is clear that new bandwidth introduction cannot be done in a generic way and release independence should be discussed and a number of new requirements should be defined.

Proposal 2:

- New 35 and 45MHz channel bandwidths should not be introduced as generic channel bandwidths in release 16

- Release independence shall be discussed cases by case per band and bandwidths.

# Conclusions

In this contribution, we studied the possible ways to derive spectrum utilization for 35 and 45 MHz channel bandwidths and looked at the resulting guard-bands to provide the following proposal.

Proposals 1 for 35 and 45 MHz NRB, guard band and ACLR measurement BW provided in Table 4:

Table 4: proposed SU for 35 and 45 MHz BW and associated parameters

|  | Channel BW |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 35 MHz |  |  | 45 MHz |  |  |
| SCS [kHz] | 15 | 30 | 60 | 15 | 30 | 60 |
| NRB | 188 | 92 | 45 | 243 | 120 | 58 |
| Min Guard-band [MHz] | 0.5725 | 0.925 | 1.27 | 0.6225 | 0.885 | 1.59 |
| ACLR measurement BW [MHz] | 33.855 |  |  | 43.755 |  |  |

Looking at the current BW support in the candidate bands, the following proposal is made.

Proposal 2:

- New 35 and 45MHz channel bandwidths should not be introduced as generic channel bandwidths in release 16

- Release independence shall be discussed cases by case per band and bandwidths.

# References

[1]	RP-201321 New WID proposal: introduction of channel bandwidths 35MHz and 45MHz for NR, Huawei, HiSilicon, Etisalat, RAN#88-e

[2]	R4-1913811 Introduction of New FR1 Channel Bandwidths and Related SU, Skyworks Solutions Inc., RAN4#93