3GPP TSG-RAN WG4 Meeting #96-e              	R4-2010278

Electronic meeting, 17th Aug– 28th Aug, 2020

Agenda item:	7.15.3.1.2

Source:	Samsung

Title:	Simulation results for NR HST UE demodulation

Document for:	Discussion

Introduction

During the last RAN4 meeting, the WF for NR Rel-16 UE demodulation was approved.  In this contributions, we summarize the simulation results for HST single tap, HST SFN, and fading channel based on the agreed simulation assumption [1]

Common test setup

The simulation assumption can be indicated with table 1 to table 3, respectively

Table1: Simulation assumption for HST-single tap channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 1×2; 1×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 17 based on 64QAM table |  |
| Propagation condition | HST-single tap |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=300m, Dmin=2m |  |
| Rank | Rank =1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 972Hz | 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Table2: Simulation assumption for HST-SFN channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13 |  |
| Propagation condition | HST-SFN |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Ds and Dmin | Ds=700m, Dmin=150m |  |
| Rank | Rank = 2 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 870Hz | 1667Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Table3: Simulation assumption for multi-path fading channel

| Parameter | Value |  |
| --- | --- | --- |
|  | FDD 15KHz SCS | TDD 30KHz SCS |
| Antenna configuration | 2×2; 2×4 |  |
| DMRS type | type 1 |  |
| Number of DMRS symbols | DMRS 1+1+1 |  |
| TDD pattern | 7D1S2U, S: 6D 4G 4U |  |
| MCS | MCS 13 based on 64QAM table |  |
| Propagation condition | TDL-C 300ns |  |
| TRS periodicity | 10ms, 2slot pattern |  |
| PDSCH mapping | Type A, Start symbol 2, Duration 12 |  |
| Rank | Rank = 1 |  |
| BW | 10MHz | 40MHz |
| Maximum Doppler shift | 600Hz | 1200Hz |
| Testing metric | SNR @70% of maximum throughput |  |

Simulation results

The ideal and impairment simulation results can be indicated with table 4 to table 6, respectively

Table 4: Ideal and impairment simulation results for NR PDSCH under HST- single tap channel

| CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR（dB）@70% Max TP (ideal) | SNR（dB）@70% Max TP (impairment) |
| --- | --- | --- | --- | --- | --- |
| 10MHz/15kHz | 972 | 1Tx 2Rx ULA Low | 17 | 7.5 | 10 |
|  |  | 1Tx 4Rx ULA Low | 17 | 4.4 | 6.9 |
| 40MHz/30kHz | 1667 | 1Tx 2Rx ULA Low | 17 | 6.6 | 9.1 |
|  |  | 1Tx 4Rx ULA Low | 17 | 3.7 | 6.2 |

Table 5: Ideal Simulation results for NR PDSCH under HST-SFN channel

| CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | MCS | SNR@70% Max TP (ideal) | SNR（dB）@70% Max TP (impairment) |
| --- | --- | --- | --- | --- | --- |
| 10MHz/15kHz | 712 | 2Tx 2Rx ULA Low | 13 | 10.1 | 12.6 |
|  |  | 2Tx 4Rx ULA Low | 13 | 7.5 | 10 |
| 40MHz/30kHz | 1667 | 2Tx 2Rx ULA Low | 13 | 12.3 | 14.8 |
|  |  | 2Tx 4Rx ULA Low | 13 | 9.5 | 12 |

Table 6: Ideal Simulation results for NR PDSCH under multi-path fading channel

| CHBW/ SCS | Max Doppler shift fd (Hz) | MIMO | Rank | MCS | SNR@70% Max TP (ideal) | SNR（dB）@70% Max TP (impairment) |
| --- | --- | --- | --- | --- | --- | --- |
| 10MHz/15kHz | 600 | 2Tx 2Rx ULA Low | 1 | 13 | 7.1 | 9.6 |
|  |  | 2Tx 4Rx ULA Low | 1 | 13 | 3.2 | 5.7 |
| 40MHz/30kHz | 1200 | 2Tx 2Rx ULA Low | 1 | 13 | 7.2 | 9.7 |
|  |  | 2Tx 4Rx ULA Low | 1 | 13 | 4.1 | 6.6 |

Conclusion

In this contribution, the ideal and impairment simulation results are provided to facilitate the requirement finalization.

Reference

- R4-2008820, WF on UE demodulation for NR HST, RAN4#95-e, CMCC