3GPP TSG-RAN WG4 Meeting #96-e	R4-2010336

Electronic Meeting, 17 Aug. – 28 Aug., 2020

Source: 	vivo

Title: 	Discussion on RLM/BFD relaxation in R17 Power saving

Agenda Item:	7.6.1

Document for:	Discussion

Introduction

In R17, Power saving WI include the following scope: [1]

- Study and specify, if agreed, enhancements on power saving techniques for connected-mode UE, subject to minimized system performance impact [RAN1, RAN4]

  - Study and specify, if agreed, extension(s) to Rel-16 DCI-based power saving adaptation during DRX Active Time for an active BWP, including PDCCH monitoring reduction when C-DRX is configured [RAN1]

    - NOTE: Rel-15 and Rel-16 available power saving solutions should be supported by the UE and included in the evaluation. RAN1 will ask the confirmation from RAN2 that Rel-15 and Rel-16 available power saving solutions are properly utilized.

  - Study the feasibility and performance impact of relaxing UE measurements for RLM and/or BFD, particularly for low mobility UE with short DRX periodicity/cycle, and specify, if agreed, relaxation in the corresponding requirements [RAN4]

    - NOTE: Supplementary RAN2 work, if needed, can be triggered by RAN4 LS

This contribution provides our initial thoughts on this issue.

Discussion on the power saving benefit of R17 RLM/BFD relaxation

R17 power saving will be discussed based on completed R16 power saving. For connected mode UE, the following was specified for R16.

- DCP(DCI with CRC scrambled by P-RNTI) to indicate whether UE need to wake-up before next DRX on-duration.

- Cross-slot scheduling

- Per-BWP MIMO layer adaptation

Based on above power saving techniques in companion with DRX, UE can achieve quite significant power saving gain if traffic data is not active for quite a period, e.g. around 1s to 10s, and network does not release UE from connected state. However, for some traffic type in which data packet arrives with interval of around 100ms to 200ms, relatively short DRX cycle can be configured to save UE power, e.g. 40ms. In these cases, as shown in our companion paper [2], when DCP is configured, RLM/BFD measurement takes a great portion of the total power consumption. A straight forward illustration is provided in Table 1. Since the RLM/BFD offset is not necessarily aligned with DRX, additional 15%~27% power saving gain can be achieved if DCP is configured and RLM/BFD is relaxed from 1x to 5x.

Table 1 Illustration of power comsuption for RLM relaxation

Observation 1: To optimise the case where data packet arrives with interval of around 100ms to 200ms, relaxation of RLM/BFD may further achieve power saving gain on top of R16 power saving techniques. If PDCCH WUS is configured and relaxing RLM-RS measurement from 1x to 5x,15 ~ 27% additional gain can be achieved

Regarding to the RSs for RLM/BFD and RRM in the connected state, we have the following observation:

- As stated in TS 38.213, for RLM/BFD, the RSs used for RLM and/or BFD, should be the RSs providing QCL information for active TCI states of PDCCH, by default. Moreover, as stated in TS 38.214, for TCI-states of dedicated CORESETs, the QCL source configured should be CSI-RS configured in a NZP-CSI-RS-ResourceSet.

- As stated in TS 38.331, serving cell measurement is indicated in the servingcellMO.

- As stated in TS 38.133, for at least SSB based RRM, which is mandatory for UE, serving cell measurement is done in the procedure of intra-frequency measurement, and measurement period requirement is also specified.

It is clear that in most cases RLM-RSs or BFD-RSs are different from the RSs used for RRM. That means for each DRX cycle that needs measurement, UE need to measure different RSs with different frequency/time location, and if necessary, perform filtering on these results. This would lead to unnecessary power consumption, which can be optimized.

Observation 2: By default, RLM/BFD RSs are different from the RSs that used for RRM, and UE is required to perform unnecessary measurement and filtering on these RSs for RLM/BFD.

Based on above observation, we see the necessity and motivation for R17 RLM/BFD relaxation, and as stated in the WID, RAN4 need to perform analysis on the feasibility and performance impact on these issue.

Discussion on mobility performance impact

In R15, there was one agreed system level simulation assumption [3], which was used to justify the number of cells/beams to be measured by the UE. Regarding to RLM/BFD requirement, most discussion was on how much samples are needed, where link level simulation was used. The basic assumption is that UE can perform one measurement for each DRX cycle or each pre-defined period, and there was no justification on how much relaxation can be done according to different mobility state.

In R16 power saving, due to lack of time, RAN4 did not perform system level evaluation before specifying idle state RRM relaxation. There was no agreement on the evaluation assumption in RAN4 and the discussion was based on evaluation results in RAN1 TR.

In R17 power saving, in our view, RAN4 should start discussion of evaluation assumption early. In our understanding, the evaluation assumption listed in Table II can be a starting point for this discussion. Some of the evaluation assumptions are reused from [3].

| Parameters | Urban macro | Dense urban |
| --- | --- | --- |
| Metric | CDF for SS-RSRP/SS-SINR variation |  |
| Mobility | 3km/h; 30km/h |  |
| Multipath fading | No |  |
| Layout | Single layer: Macro layer: Hex. Grid |  |
| Inter-BS distance | 500m | Macro layer: 200m |
| Carrier frequency and SCS | 4GHz and 30kHz SCS, |  |
| Simulation bandwidth | 20MHz, SSB with 20 PRB and no multiplexing with data |  |
| LoS probability | According to Table 7.4.2-1 in TR 38.901 |  |
| Pathloss and shadow fading std. | Refer to UMa model in TR38.901 | Refer to UMi model in TR38.901 |
| Shadow fading correlation distance | 37 for LOS and 50 for NLOS | 10 for LOS and 13 for NLOS |
| BS Tx power | Below 6GHz: 44 dBm |  |
| UE receiver noise figure | Below 6GHz: 9dB |  |
| UE distribution | 100% outdoor |  |
| BS antenna height | 25m | 10m |
| UE antenna height | 1.5m |  |
| BS antenna element gain + connector loss | Follow 38.802 |  |
| UE antenna gain | Follow 38.802 |  |
| BS antenna down-tilt | 15 degree |  |
| BS array configuration | (M,N,P,Mg,NgB) =(2,4,2,1,1) dH,dV) = (0.5, 0.5)λ +45°, -45° polarization |  |
| BS beam set | For direction of analog beam steering (in LCS): Azimuth angle φi = [-45, -15, 15, 45] degree Zenith angle θj = [65 90] degree |  |

Table II Evaluation assumption for RLM/BFD relaxation in FR1

Some key points on above assumption:

- On performance metric: It is proposed to use CDF of SS-RSRP/SS-SINR variation in a given period as performance metric. If under a given mobility state, the variation is below a threshold, then RLM/BFD relaxation can be considered as feasible.

- On small scale fading: It is proposed to agree no small scale fading as baseline for simplicity. In our understanding, small scale fading may only provide random variation, and such variation can also be modelled by shadow fading.

- On UE mobility: 17-sector hexagonal layout is used, and UE randomly move within the defined layout. If UE move across the border of the layout, wrap around function is considered.

- On interference assumption: UE is assume to be interfered by one specific beam from each neighbour cell and the signal strength of this interference is generated based on large scale fading and array gain.

Based on above evaluation assumptions, some immediate results are available as below.

Figure 1 shows the CDF of SS-RSRP variation and SS-SINR variation within the measurement interval(MI). To align with analysis with [2], 40ms DRX cycle length is assumed, so that the baseline for measurement interval is 1.5 x 10 x 40ms = 600ms. Relaxation of 4 times and 10 times, i.e. 2400ms and 6000ms are shown in the figure. Additionally we provide 20ms and 200ms as reference. It can be observed that the variation of either RSRP or SINR is not significant, even if the MI = 6000ms. That means there should be room for such RLM/BFD relaxation.

Figure 1 For UMa and UE speed = 3km/h, CDF of SS-RSRP variation and SS-SINR variation within the measurement interval(MI)

To check more details on the feasibility of the relaxation, we suggest to check the probability of variation smaller than 10-2 within a given MI. As we know the PDCCH BLER is targeting as 10-2, so if a threshold is set so that within the MI the occurrence of channel variation below the threshold would be lower than the PDCCH BLER, the impact from variation of SINR can be comparable to the absolute SINR. From Figure 2 and Figure 3, i.e. if UE movement is 3km/h, in each case we can figure out a proper threshold for MI = 2400ms, where within this MI, the probability of RSRP variation larger than that threshold is less 10-2. By setting such threshold in UE RLM/BFD relaxation, UE may relax at least 4 times to achieve power saving gain. However, if UE movement is 30km/h, as in Figure 4, there is no room for RLM relaxation.

Figure 2 For UMa and UE speed = 3km/h, probability of variation above a predefined threshold. Within 2400ms measurement period, probability of SS-RSRP and SS-SINR variation being more than 8dB is below 10-2

Figure 3 For UMi and UE speed = 3km/h, probability of variation above a predefined threshold. Within 2400ms measurement period, probability of SS-RSRP and SS-SINR variation being more than 12dB is below 10-2

Figure 4 For UMa scenario and UE speed = 30km/h, probability of variation above a predefined threshold. Within 600ms measurement period, probability of SS-RSRP and SS-SINR variation being more than 10dB is above 10-2

Observation 3: If UE movement is less than 3km/h, immediate results based on the given evaluation assumption show that there is room for RLM relaxation.

Proposal 1: In R17 UE power saving WI, RAN4 need to discuss and agree on the evaluation assumption in the first meeting so that companies may provide results in the next meetings. The motivation of such evaluation is to identify whether and how much RLM/BFD relaxation can be done and what is the condition for such relaxation. Table II can be a start point for such discussion.

Discussion on the potential schemes for RLM/BFD relaxation

For idle mode UE, RRM relaxation was extensively discussed and RAN4 identified 3 scenarios for RRM relaxation. One scenario is that UE is stationary and not at cell-edge, in which UE may stop neighbour cell measurement if serving cell RSRP and/or RSRQ meets a specific criterion defined in 38.304. Such feature may also provide good reference for R17 power saving discussion. As clearly stated in the WID, R17 RLM/BFD relaxation will focus on the low mobility scenario only.

Observation 4: R17 RLM/BFD relaxation will focus on the low mobility scenario only and R16 criterion for low mobility can be considered as a reference.

Conclusion

Based on above analysis, we have following observations and proposals.

Observation 1: To optimise the case where data packet arrives with interval of around 100ms to 200ms, relaxation of RLM/BFD may further achieve power saving gain on top of R16 power saving techniques. If PDCCH WUS is configured and relaxing RLM-RS measurement from 1x to 5x,15 ~ 27% additional gain can be achieved.

Observation 2: By default, RLM/BFD RSs are different from the RSs that used for RRM, and UE is required to perform unnecessary measurement and filtering on these RSs for RLM/BFD.

Observation 3: If UE movement is less than 3km/h, immediate results based on the given evaluation assumptions show that there is room for RLM relaxation.

Proposal 1: In R17 UE power saving WI, RAN4 need to discuss and agree on the evaluation assumption in the first meeting so that companies may provide results in the next meetings. The motivation of such evaluation is to identify whether and how much RLM/BFD relaxation can be done and what is the condition for such relaxation. Table II can be a start point for such discussion.

Observation 4: R17 RLM/BFD relaxation will focus on the low mobility scenario only and R16 criterion for low mobility can be considered as a reference.

- References

[1]  RP-193239	New WI: UE Power Saving Enhancements, MediaTek.  RAN #86e

[2]  R1-2005392  Discussion on RLM/BFD relaxation, vivo.  RAN1 #102e

[3]  R4-1709903  System level simulation assumptions in NR RRM, Ericsson.  RAN4 #NR-AH-03