3GPP TSG-RAN WG4 Meeting #96-e	R4-2010441

# Electronic meeting, August  17th–28th 2020

Source:		Ericsson

Title:			Bands n41 and n48 – Add 70 MHz Channel BW

Agenda item:		10.17.2

Document for:	Approval

# Introduction

A new basket WI [1] has been approved in last RAN#88 meeting. As part of this WI, it was already agreed to add support for 70 MHz channel BW in band n41 and n48.

This contribution is describing the identified impacts in TS 38.104 and TS 38.101-1

# Discussion

## Adding 70 MHz CBW in band n41

### TS 38.104

70 MHz channel BW was already supported in TS 38.104, it’s already listed as supported in table 5.3.5-1 (that was actually not consistent with TS 38.101-1).

No update is then expected.

### TS 38.101-1

70 MHz channel BW shall be added:

- As supported CBW for n41 in table 5.3.5-1.

- To the list of channel BWs for which NS_04 shall be considered in table 6.2.3.1-1.

- To NS_04 tables 6.5.2.3.2-1, 6.5.2.3.2-2 and 6.5.2.3.2-3 for the additional spectrum emissions requirements.

We propose to update REFSENS table 7.3.2-1 and specify n41 requirements for 70 MHz CBW by scaling channel BW. The related RB allocations table 7.3.2-3 shall also be updated for 70MHz / band n41.

## Adding 70 MHz CBW in band n48

Note that 70 MHz CBW was only request for DL.

### TS 38.104

70 MHz channel BW shall be added to table 5.3.5-1 (a note mentioning this is only for DL should also be added).

### TS 38.101-1

70 MHz channel BW shall be added as supported CBW for n48 in table 5.3.5-1.

No A-MPR update should be considered as this channel BW is only for DL.

We propose to update REFSENS table 7.3.2-1 and specify n48 requirements for 70 MHz CBW by scaling channel BW.

# Conclusion

In this contribution, we analyzed the impacts of adding 70 MHz channel BW support in bands n41 and n48. The companion draft CRs [2], [3] and [4] are capturing those changes.

# References

- RP-201294, New Basket WID on adding channel bandwidth support to existing NR bands, Ericsson

- R4-2010443, Draft CR to 38.101-1 Band n48 - Additional 70MHz Channel BW, Ericsson

- R4-2010442, Draft CR to 38.101-1 Band n41 - Additional 70MHz Channel BW, Ericsson

- R4-2010444, Draft CR to 38.104 Band n48 - Additional 70MHz Channel BW, Ericsson