3GPP TSG-RAN WG4 Meeting #96-e	R4-2010478

Electronic Meeting, 17 – 28 August 2020

Agenda Item:	7.6.4

Source:	Ericsson

Title:	Evaluation of WUS-PDCCH decoding performance

Document for:	Discussion

# 1	Introduction

RAN4#95-e agreed with the way forward on WUS-PDCCH decoding requirements [1].

Whether to introduce joint test for PDCCH-WUS during DRX OFF and PDCCH during DRX ON for power saving UE? - Keep it open whether to introduce test cases or not - RAN4 will further discuss detailed test set-up (simulation assumption) considering test feasibility and also need to be checked from RRM aspect. - Make decision on whether introducing test cases in Q3 2020.

We show our simulation results based on the simulation assumption discussed in RAN4#95-e [2].

# 2	Evaluation results

## 2.1	Simulation parameters

Table 1 shows simulation parameters for WUS-PDCCH decoding performance and Table 2 shows FRC of PDCCH and WUS-PDCCH.

Table 1	Simulation parameters for WUS-PDCCH decoding performance

| Parameter | Value |
| --- | --- |
| Bandwidth | 10MHz |
| Channel model | TDLA30-10 |
| Subcarrier spacing | 15kHz |
| Number of BS antennas | 1Tx |
| Number of UE antennas | 2Rx |
| DM-RS channel estimation | Realistic |

Table 2	FRC for PDCCH/WUS-PDCCH

| Parameters | Values |  |
| --- | --- | --- |
|  | PDCCH | WUS-PDCCH |
| DCI format | 1_0 | 2_6 |
| DCI length (excluding 24bits CRC) | 39 bits | 12 bits, 36 bits |
| Aggregation level | 16 | 8, 16 |
| CORESET symbol | 2 | 2 |
| CORESET bandwidth | 48RB | 48RB |
| Mapping type | Interleaved | Interleaved |

We used the following metrics for evaluation:

- BLERPDCCH: BLER of PDCCH for the case that only PDCCH transmission

- BLERPDCCH-WUS: BLER of PDCCH-WUS for the case that only PDCCH-WUS transmission

- BLERPDCCH-JOINT: BLER of PDCCH for the case that joint transmission of PDCCH-WUS and PDCCH (UE does not wake up when missing PDCCH-WUS in DRX-OFF period), that is,

  - BLERPDCCH-JOINT = BLERPDCCH-WUS + (1 – BLERPDCCH-WUS) * BLERPDCCH

## 2.2	Simulation results

Figure 1 shows our ideal simulation for PDCCH/WUS-PDCCH according to the simulation setup in Table 1 and Table 2. Table 3 summarizes SNRs to achieve BLERPDCCH-WUS=0.1% and BLERPDCCH-JOINT=0.1%.

Figure 1	Simulation results of PDCCH/WUS-PDCCH decoding performance.

Table 3	SNR to achieve BLERPDCCH-WUS=0.1%

| Payload (excluding 24 bits CRC) | 12 | 12 | 36 | 36 |
| --- | --- | --- | --- | --- |
| Aggregation level | 8 | 16 | 8 | 16 |
| SNR to achieve BLERPDCCH_WUS=0.1% | 2.4 | 0.7 | 4.5 | 2.2 |
| SNR to achieve BLERPDCCH_JOINT=0.1% | 3.3 | 2.9 | 5.1 | 4.0 |

## 2.3	Discussion

As it is discussed in RAN4#95-e, DCI 2_6 in PDCCH-WUS only triggers UE to receive PDCCH for DL scheduling. This means it is not possible to measure the WUS-PDCCH decoding performance itself because UE does not transmit any feedback on WUS-PDCCH. Therefore the PDCCH-WUS reception performance should be measured with the number of HARQ-ACK from PDSCH scheduled by PDCCH, triggered by PDCCH-WUS.

If RAN4 set the requirements of PDCCH-WUS to achieve BLERPDCCH-WUS=0.1%, the BLERPDCCH should be much less than 0.1% at the SNR achieving BLERPDCCH-WUS=0.1% to avoid the impact of PDCCH decoding error. According to our simulation results, however, it is observed that BLERPDCCH for DL scheduling is not negligible at the SNR where WUS-PDCCH achieves BLER=0.1%. For example, if we assume WUS-WDCCH is configured with DCI=12bits/AL16, SNR=0.7dB for BLERPDCCH-WUS=0.1%. At this SNR point, BLERPDCCH is about 0.2%.

Observation: PDCCH BLER is not negligible at the SNR where PDCCH-WUS achieves BLER=0.1%.

Alternatively we could define PDCCH/WUS-PDCCH decoding requirements for SNR achieving BLERPDCCH-JOINT=1%, for which it is interpreted PDCCH decoding performance is degraded by PDCCH-WUS decoding error. Table 4 shows the SNR to achieve BLERPDCCH-JOINT=1%. Note SNR at BLERDPCCH=1% is -2.7dB. From the simulation results, for example, SNR for BLERPDCCH=1% is degraded by 0.8dB duel to WUS-PDCCH decoding error, when we set WUS-PDCCH parameter as DCI=12bits/AL16.

Table 4	SNR to achieve BLERPDCCH-JOINT=1%

| Payload (excluding 24 bits CRC) | 12 | 12 | 36 | 36 |
| --- | --- | --- | --- | --- |
| Aggregation level | 8 | 16 | 8 | 16 |
| SNR to achieve BLERPDCCH-JOINT=1% | -1.2 | -2.0 | 0.0 | -1.3 |

RAN4 is still discussing whether to introduce WUS-PDCCH/PDCCH decoding requirements, but if RAN4 will introduce the requirements, we prefer to set the requirements based on the existing PDCCH decoding requirements, i.e., if we set the PDCCH decoding requirements in TS38.101-4 5.3.2.1.1 Test 5 as baseline (see below), the PDCCH+WUS-PDCCH requirements is set by -2.1 + Δ dB, with BLERPDCCH-JOINT=1%. For example Δ=1.0 in the case WUS-PDCCH is configured as DCI=12bits/AL16.

Table 5.3.2.1.1-1: Minimum performance for PDCCH with 15 kHz SCS (TS 38.101-4)

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

# 3	Summary

Observation: PDCCH BLER is not negligible at the SNR where PDCCH-WUS achieves BLER=0.1%.

# 4	References

- R4-2008802, “WF on power saving demodulation​”, CATT.

- R4-2008803, “Simulation assumption for PDCCH-WUS test”, MediaTek.