3GPP TSG-RAN WG4 Meeting # 96-e	R4-2010485

# Electronic Meeting, 17-28 Aug., 2020

Agenda item:	11.1.2

Source: 	Huawei, HiSilicon

Title: 	TP to TR 38.921: BS IMT technology related parameters

Document for:	Approval

- Introduction

The WF on on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz was approved in [1].

This paper provides a TP for TR 38.921 to capture the above agreements for BS and some proposals on remaining requirements.

References

[1] R4-2009151, WF on BS and UE IMT parameters for the SI on 6.425-7.125GHz and 10.0-10.5GHz, CATT

[2] R4-2010370, TR 38.921

TP to TR 38.921

< START OF CHANGE>

# 6	BS parameters

## 6.1	Transmitter characteristics

### 6.1.1	Power dynamic range

There is no power control in downlink and fixed power per resource block is assumed in the co-existence simulation. Hence 0 dB power dynamic range was agreed for the LS reply.

### 6.1.2	Spectral mask

[Co-existence simulations are carried out to derive the limits]

### 6.1.3	ACLR

[Co-existence simulations are carried out to derive the limits]

### 6.1.4	Spurious emissions

The general spurious emissions defined TS 38.104 clause 6.6.5.2.1 and 9.7.5.2 can apply to both frequency ranges, 6.425-7.125GHz and 10.0-10.5GHz.

From the Rel-15 discussion, the offset ΔfOBUE for the boundary between OBUE and spurious emissions is the most demanding parameters for OTA or Hybrid limits AAS BS implementation. ΔfOBUE is deferred to the future WI phase.

### 6.1.5	Maximum output power

The maximum output power will be provided in the antenna parameter table. It was agreed to be aligned with antenna characteristics.

### 6.1.6	Average output power

It was agreed the average output power won’t be mentioned in the reply LS.

## 6.2	Receiver characteristics

### 6.2.1	Noise figure

For 10.0-10.5 GHz it was approved 7 dB to be adopted for BS from the SI of 7 - 24 GHz.

Table 5.5.1.1-1 of TR 38.820: Typical noise figure for 7 – 24 GHz example frequencies

| Example frequency (GHz) | Typical NF values for NR BS (dB) | Typical NF values for NR UE (dB) |
| --- | --- | --- |
| 10 | 7 | 9 |
| 15 | 8 | 10 |
| 20 | 9 | 10 (Note) |

For 6.425-7.125 GHz, it was agreed to adopt 6 dB which is the average value of noise figures for below 5 GHz and 10 GHz.

### 6.2.2.	Sensitivity

As it is not clear if it will have a conducted sensitivity requirement for both frequency ranges, however the OTA sensitivity requirement will be needed either way and will be based on the NF and the antenna gain:

EISREFSENS=PkT+10*log10BW+NF+IM+SNR-G    (dBm)

Where: BW is the noise BW of the FRC, NF is the noise figure, IM is implantation margin not related to antenna array (2.5dB), SNR is the required SNR for demodulation and G is the antenna gain and RF losses.

The sensitivity would not be used directly in the sharing studies hence it was agreed the sensitivity level won’t be mentioned in the reply LS.

### 7.2.3	Blocking response

[Co-existence simulations are carried out to derive the limits]

### 7.2.4	ACS

[Co-existence simulations are carried out to derive the limits]

< END OF CHANGE>