3GPP TSG-RAN WG4 #96-e	R4-2010547

Online, 17th – 28th August, 2020

Source:	Huawei, HiSilicon, Etisalat, CMCC

Title:	TP to TR 37.717-00-00 SUL_n41A-n83A

Agenda item:			10.8.2

Document for:	Approval

# 1. Introduction

In this contribution, we propose a text proposal for TR 37.717-00-00 for SUL band combinations in Rel-17 to introduce SUL_n41A-n83A.

# 2. Text Proposal

----- Unchanged sections omitted -----

## 5.x	SUL_ n41A-n83A

### 5.x.1	Operating bands

Table 5.x.1-1: SUL band combination

| NR Band combination for SUL | NR Band (Table 5.2-1) |
| --- | --- |
| SUL_n41-n832 | n41, n83 |
| NOTE 1: If a UE is configured with both NR UL and NR SUL carriers in a cell, the switching time between NR UL carrier and NR SUL carrier is 0 us. NOTE 2: For UE supporting SUL band combination simultaneous Rx/Tx capability is mandatory. |  |

### 5.x.2	Channel bandwidths per operating band

Table 5.x.2-1: Supported bandwidths per SUL band combination

| SUL configuration | NR Band | Subcarrier spacing (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SUL_n41A-n83A | n41 | 15 |  | Yes | Yes | Yes |  | Yes | Yes | Yes |  |  |  |  | 0 |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | 60 |  | Yes | Yes | Yes |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  | n83 | 15 | Yes | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |
|  |  | 30 |  | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |

### 5.x.3	Maximum output power

There is only single UL in uplink so this requirement is not applicable.

### 5.x.4	Spurious emission band UE co-existence

There is only single UL in uplink so this requirement is not applicable.

### 5.x.5	MSD

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands shall be met for a supplementary uplink transmission bandwidth less than or equal to that in Table 5.x.5-1.

Table 5.x.5-1: Supplementary Uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| n41 | n83 | 15 |  | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |
| n41 | n83 | 30 |  | 25 | 25 | 25 |  | 25 | 25 | 25 | 25 | 25 | 25 | 25 |

For SUL_n41-n83, there is no harmonic or harmonic mixing product generated by Band n83 that may fall into the RX band of Band n41. Therefore, MSD due to harmonic or harmonic mixing is not needed.

### 5.x.6	∆TIB and ∆RIB values

For SUL_n41-n83, the ∆TIB,c and ∆RIB values are given in the tables below.

# Table 5.x.6-1: ΔTIB,c

| SUL Band combination | NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n83 | n41 | 0.3 |
|  | n83 | 0.3 |

# Table 5.x.6-2: ΔRIB,c

| SUL Band combination | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n41-n83 | n41 | 0 |
|  | n83 | 0 |

----- Unchanged sections omitted -----

# Reference