3GPP TSG-RAN WG4 Meeting #96-e	R4-2010609

Electronic meeting, 17 – 28 August, 2020

Source:	Ericsson

Title:	Discussion on HST UL TA remain issues.

Agenda item:	7.15.3.2.3

Document for:	Approval

# Introduction

In the last meeting, RAN4 got some agreements on organization, test metric and configuration, but there are still some issues can’t get agreement in WF [1].

Issue 1: Additional scenario “X”

- Option 1: Specify requirements for scenario X.

- Option 2: Do not specify scenario “X”.

No consensus continue discussion. Companies are encouraged to bring more analysis the necessity and un-necessity of introducing this test cases and make decisions in Q3 2020.

Issue 2: Scenario “X” implicit test passing

- Discuss after additional scenario “X” introduction is decided.

Issue 3: Applicability for 120kph HST UL TA

- Discuss after additional scenario “X” introduction is decided.

Issue 4: UL TA supported speed declaration for 120kph/Scenario X

- Option 1: No declaration for scenario X is needed; testing scenario X is always required.

- Option 2: No declaration for scenario X is needed; no requirements for scenario X.

- Option 3: No declaration for scenario X is needed; testing scenario X is only required, if 350 or 500kph UL TA is not declared to be supported (“overwritten”).

- Option 4: Postpone to after additional scenario “X” introduction is decided.

- Option 5: No declaration for scenario X is needed.

Proposed WF: Discuss after additional scenario “X” introduction is decided.

Issue 5: Organization of HST requirements for UL TA 500kph in specifications

- Option 1: Requirements for different scenarios captured in same table.

- Option 2: Requirements for different scenarios captured in separate tables.

- Option 3: Capture the 500kph UL TA scenario in the same table as the 350kph UL TA scenario.

This contribution will keep on discussing remain issues.

# Discussion

Issue 1: Additional scenario “X”

Issue 2: Scenario “X” implicit test passing.

Issue 3: Applicability for 120kph HST UL TA

Scenario X is defined at 120km/h which UE is in ETU200 propagation channel model. In LTE specification, ETU models are used in only non-HST requirements. Thus, we don’t think scenario X is suitable to be included in HST requirement which velocity is defined at 350km/h and 500km/h.

We suggest including it in Rel-17 normal NR enhancement discussion.

Proposal 1: Include scenario X requirement discussion in Rel-17 normal NR enhancement discussion.

If it is not possible to include it in Rel-17 discussion and must be discussed in Rel-16, then we think the requirements should be added in non-HST section to avoid misleading again in the future.

Proposal 2: If scenario X requirements have to be discussed together with Rel-16 HST requirements, adding it in non-HST sections/tables to avoid misleading.

As defined in LTE specification, HST requirements are optional including scenario 2 for UL TA. In that case, the normal multi-path fading channel requirements are not optional for BS performance, thus scenario X should also be mandatory requirements. It won’t have any implicit test passing relationship with scenario Y and Z, and it won’t need special applicability rule neither.

Proposal 3: No implicit test passing for scenario X.

Proposal 4: No applicability rule for scenario X.

Issue 4: UL TA supported speed declaration for 120kph/Scenario X

As discussed above, scenario X requirements should be mandatory and don’t need declaration for it. BS should always test these requirements.

Proposal 5: Agree with Option 1 that no declaration is needed for scenario X and the test of scenario X is always required.

Issue 5: Organization of HST requirements for UL TA 500kph in specifications

No matter scenario X will be introduced or not, capturing scenario Y and Z in the same table could be clearer. The phrase “for high speed train” should be added in the table titles.

Proposal 6:  Agree with Option 3 that capture the 500kph UL TA scenario in the same table as the 350kph UL TA scenario. Adding phrase “for high speed train” in the table title.

# Conclusion

Issue 1: Additional scenario “X”

Issue 2: Scenario “X” implicit test passing.

Issue 3: Applicability for 120kph HST UL TA

Proposal 1: Include scenario X requirement discussion in Rel-17 normal NR enhancement discussion.

Proposal 2: If scenario X requirements have to be discussed together with Rel-16 HST requirements, adding it in non-HST sections/tables to avoid misleading in the future.

Proposal 3: No implicit test passing for scenario X.

Proposal 4: No applicability rule for scenario X.

Issue 4: UL TA supported speed declaration for 120kph/Scenario X

Proposal 5: Agree with Option 1 that no declaration is needed for scenario X and the test of scenario X is always required.

Issue 5: Organization of HST requirements for UL TA 500kph in specifications

Proposal 6:  Agree with Option 3 that capture the 500kph UL TA scenario in the same table as the 350kph UL TA scenario. Adding phrase “for high speed train” in the table title.

# References

[1]	R4-2008821, “WF on Rel-16 NR HST PUSCH BS demodulation requirements”, Nokia, Nokia Shanghai Bell, …

[2]   R4-2009042, “Email discussion summary for [95e][322] NR_HST_Demod_BS”, Nokia, Nokia Shanghai Bell, …