3GPP TSG-RAN WG4 Meeting #96-e                                          R4-2010818

Electronic Meeting, 17-28 Aug., 2020

Source: 	Huawei, Hisilicon

Title: 	On NR V2X SL allocated RB size numbers for different CBWs

Agenda Item:	7.3.2

Document for:	Approval

# Introduction

Transmission bandwidth configuration for NR V2X so far refers the NRB values for NR Uu in the specification [1]. Based on the latest RAN1 agreement, the sub channel size for NR V2X which indicates the minimum granularity in frequency domain for the sensing for PSSCH resource selection in the unit of PRB has been reflected in the RAN2 specification [2].

This contribution provides suggested NRB values for NR V2X based on these updated information.

# Discussion

The sidelink allocated RB (LCRB) size will be used for both Tx and Rx requirements. In the table cited below we can see the NOTE which mentions the numbers could be updated based on RAN2 specification.

TS 38.101-1 Table 7.3E.2-2: Sidelink TX configuration for reference sensitivity of NR V2X Bands (PC5)

| NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | 52 | 106 |  | 216 | HD |
|  | 30 | 24 | 51 |  | 106 | HD |
|  | 60 | 11 | 24 |  | 51 | HD |
| n47 | 15 | 52 | 106 | 160 | 216 | HD |
|  | 30 | 24 | 51 | 78 | 106 | HD |
|  | 60 | 11 | 24 | 38 | 51 | HD |
| NOTE 1: The sidelink allocated RB (LCRB) size could be adjusted according to resource pool configuration in [7]. |  |  |  |  |  |  |

At the time RAN4 discussed the issue, RAN1 agreement was not reflected in the RAN2 spec yet. But now the corresponding SL-ResourcePool parameters are available as following.

SL-ResourcePool information element

-- ASN1START

-- TAG-SL-RESOURCEPOOL-START

SL-ResourcePool-r16 ::=            SEQUENCE {

sl-PSCCH-Config-r16                SetupRelease { SL-PSCCH-Config-r16 }                                  OPTIONAL,   -- Need M

sl-PSSCH-Config-r16                SetupRelease { SL-PSSCH-Config-r16 }                                  OPTIONAL,   -- Need M

sl-PSFCH-Config-r16                SetupRelease { SL-PSFCH-Config-r16 }                                  OPTIONAL,   -- Need M

sl-SyncAllowed-r16                 SL-SyncAllowed-r16                                                    OPTIONAL,   -- Need M

sl-SubchannelSize-r16              ENUMERATED {n10, n12, n15, n20, n25, n50, n75, n100}                  OPTIONAL,   -- Need M

……

The valid sidelink allocated RB size is determined by number of sub-channels and sub-channel size, but the NRB values to be used in the spec for RF requirements should be as large as possible as long as it does not exceeds the NRB values of Uu for supported CBWs.

Based on this principle, we calculated the corresponding sidelink allocated RB size in the table below.

Table 1: Max available sidelink allocated RB size

| 10 MHz | NsubCH*LsubCH | 20 MHz | NsubCHNsubCH*LsubCH | 30 MHz | NsubCH*LsubCH LsubCH | 40 MHz | NsubCH*LsubCH |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NRB |  | NRB |  | NRB |  | NRB |  |
| 52 | 10*5 | 106 | 15*7 | 160 | 10*16 | 216 | 12*18 |
| 24 | 12*2 | 51 | 10*5 | 78 | 15*5 | 106 | 15*7 |
| 11 | 10*1 | 24 | 12*2 | 38 | 12*3 | 51 | 10*5 |

With the numbers calculated in Table 1, we can update the SL Tx configuration as below:

Table 2: Updated Sidelink TX configuration

| NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | 50 | 105 |  | 216 | HD |
|  | 30 | 24 | 50 |  | 105 | HD |
|  | 60 | 10 | 24 |  | 50 | HD |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 10 | 24 | 36 | 50 | HD |

# Conclusion

Updated sidelink allocated RB size based on RAN1 and RAN2 agreements was discussed in this contribution. It is proposed to update the NRB values for NR V2X accordingly.

Proposal: It is proposed to update the max sidelink allocated RB numbers as in the table below.

| NR Band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| NR V2X Band | SCS kHz | 10 MHz | 20 MHz | 30 MHz | 40 MHz | Duplex Mode |
| n38 | 15 | 50 | 105 |  | 216 | HD |
|  | 30 | 24 | 50 |  | 105 | HD |
|  | 60 | 10 | 24 |  | 50 | HD |
| n47 | 15 | 50 | 105 | 160 | 216 | HD |
|  | 30 | 24 | 50 | 75 | 105 | HD |
|  | 60 | 10 | 24 | 36 | 50 | HD |

# References

[1] 38.101-01 v16.4.0

[2] 38.331 v16.1.0