3GPP TSG-RAN WG4 Meeting #96-e 											         R4-2010822

E-meeting, August 17 – 28, 2020

Source: 	Huawei, HiSilicon

Title: 	On NR V2X reference measurement channels

Agenda Item:	7.3.1

Document for:	Approval

# Introduction

This contribution provides V2X reference measurement channels for receiver requirements over PC5 based on RAN1 specifications.

# Discussion

Within a PSSCH transmitting, some necessary overhead, i.e., PSCCH, PSSCH DMRS, AGC symbol and GAP symbol should be considered in a slot. Based on RAN1 specification [1], PSCCH is TDM with its associated PSSCH in a SL slot, and there are multiple possible configurations for PSCCH, but only one set of PSCCH parameter is configured in a resource pool [4]. Here the code-rate of LTE-V2X PSCCH (2PRBs, 13 sidelink symbols, 4 PSSCH DRMS symbols without FDM with PSSCH DATA, QPSK modulation order, payload with 24 bits CRC [2]) is introduced as a reference point. Thus 10RBs and 3 symbols are configured to NR-V2X PSCCH transmission with the maximum 48 bits payload and 24 bits CRC [3], where PSCCH DMRS density is 1/3 in every PSCCH symbol, PSCCH modulation order is fixed to QPSK [1]. Meanwhile 2 symbols are assumed as PSSCH DRMS overhead, because at least 2 PSSCH DMRS symbols are mapped in a slot [1]. According TS 38.211, the first sidelink symbol in a slot is duplicated in the immediately preceding OFDM symbol, which means that the first sidelink symbol is working as AGC overhead symbol. Similarly, the last sidelink symbol used for PSSCH in a slot serves as a guard symbol (GAP symbol), which is used as transition period.

In short, the following aspects are considered in defining the FRC for NR V2X.

- 12 symbols are allocated to PSSCH transmission in a SL slot

- 10RBs and 3 symbols is considered as PSCCH transmission overhead.

- 2 symbols are allocated to PSSCH DMRS, which is FDMed between DMRS and PSSCH is supported, and have non-overlap with PSCCH in time domain.

- 24 bits CRC for all CB/TB length

- If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block

Based on above overhead assumption, fixed reference channel is designed for different bandwidth and modulation orders.

## FRC for receiver requirements for QPSK

For V2X transmission over PC5, Table 1, Table 2 and Table 3 are applicable for measurements on the Receiver Characteristics with the exception of Maximum input level.

Table 1 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 3904 | 8192 | 12552 | 16896 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 2 | 3 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 13008 | 27264 | 41520 | 56304 |
| Max. Throughput averaged over 1 frame | Mbps | 3.514 | 7.373 | 11.297 | 15.206 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 2 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 1736 | 3840 | 6016 | 8192 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 5616 | 12744 | 19872 | 27264 |
| Max. Throughput averaged over 1 frame | Mbps | 1.5624 | 3.456 | 5.414 | 7.373 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 3 Fixed reference channel for V2X receiver requirements (SCS 60 kHz, QPSK)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 4 | 4 | 4 | 4 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 672 | 1736 | 2792 | 3840 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 2 | 2 | 2 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 2184 | 5616 | 9312 | 12744 |
| Max. Throughput averaged over 1 frame | Mbps | 0.605 | 1.5624 | 2.513 | 3.456 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

## FRC for maximum input level for 64QAM

For V2X transmission over PC5, Table 4, Table 5 and Table 6 are applicable for Maximum input level when the maximum modulation order is 64QAM.

Table 4 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 29192 | 61480 | 94248 | 127080 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 4 | 8 | 12 | 16 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 39024 | 81792 | 124560 | 168912 |
| Max. Throughput averaged over 1 frame | Mbps | 26.273 | 55.332 | 84.823 | 114.37 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 5 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 12808 | 28680 | 45096 | 61480 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 2 | 4 | 6 | 8 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 16848 | 38232 | 59616 | 81792 |
| Max. Throughput averaged over 1 frame | Mbps | 11.527 | 25.812 | 40.586 | 55.332 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM PSSCH between DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 6 Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 64QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 24 | 24 | 24 | 24 |
| MCS Table for TBS determination | 64QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 4864 | 12808 | 21000 | 28680 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 2 | 3 | 4 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 6552 | 16848 | 27936 | 38232 |
| Max. Throughput averaged over 1 frame | Mbps | 4.3776 | 11.527 | 18.9 | 25.812 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

## FRC for maximum input level for 256QAM

For V2X transmission over PC5, Table 7, Table 8 and Table 9 are applicable for Maximum input level when the 256QAM is supported.

Table 7 Fixed reference channel for V2X receiver requirements (SCS 15 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 15 | 15 | 15 | 15 |
| Subcarrier spacing configuration |  | 0 | 0 | 0 | 0 |
| Allocated resource blocks |  | 52 | 106 | 160 | 216 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 9 | 9 | 9 | 9 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 1,2,3,4,5,6,7,8,9 | Bits | 40976 | 83976 | 129128 | 176208 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 5 | 10 | 16 | 21 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 52032 | 109056 | 166080 | 225216 |
| Max. Throughput averaged over 1 frame | Mbps | 36.878 | 75.578 | 116.22 | 158.59 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 8 Fixed reference channel for V2X receiver requirements (SCS 30 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 30 | 30 | 30 | 30 |
| Subcarrier spacing configuration |  | 1 | 1 | 1 | 1 |
| Allocated resource blocks |  | 24 | 51 | 78 | 106 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 17424 | 39936 | 61480 | 83976 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 3 | 5 | 8 | 10 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 22464 | 50976 | 79488 | 109056 |
| Max. Throughput averaged over 1 frame | Mbps | 15.682 | 35.942 | 55.332 | 75.578 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

Table 9 Fixed reference channel for V2X receiver requirements (SCS 60 kHz, 256QAM)

| Parameter | Unit | Value |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 10 | 20 | 30 | 40 |
| Subcarrier spacing | kHz | 60 | 60 | 60 | 60 |
| Subcarrier spacing configuration |  | 2 | 2 | 2 | 2 |
| Allocated resource blocks |  | 11 | 24 | 38 | 51 |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 |
| Allocated slots per Frame |  | 10 | 10 | 10 | 10 |
| MCS Index |  | 23 | 23 | 23 | 23 |
| MCS Table for TBS determination | 256QAM |  |  |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK |
| Maximum number of HARQ transmissions |  | 0 | 0 | 0 | 0 |
| Information Bit Payload per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 6784 | 17424 | 29192 | 39936 |
| Transport block CRC | Bits | 24 | 24 | 24 | 24 |
| LDPC base graph |  | 1 | 1 | 1 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |  |
| For Slots 0 | CBs | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | CBs | 1 | 3 | 4 | 5 |
| Binary Channel Bits per Slot |  |  |  |  |  |
| For Slots 0 | Bits | N/A | N/A | N/A | N/A |
| For Slots 0,1,2,3,4,5,6,7,8,9 | Bits | 8736 | 22464 | 37248 | 50976 |
| Max. Throughput averaged over 1 frame | Mbps | 6.1056 | 15.682 | 26.273 | 35.942 |
| NOTE 1: There are 14 symbols in a SL slot. Considering AGC and GAP symbol, 12 symbols are allocated to PSSCH transmission. NOTE 2: 10RBs and 3 symbols allocated to PSCCH transmission NOTE 3: 2symbols allocated to PSSCH DMRS. FDM between PSSCH DMRS and PSSCH is supported NOTE 4: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 5: Slot i is slot index per frame |  |  |  |  |  |

# Conclusion

This contribution provides FRC for V2V sidelink over PC5.

Proposal: It is proposed that the FRC in above tables is adopted.

# References

- 3GPP TS 38.211: "NR; Physical channels and modulation"

- 3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation"

- 3GPP TS 38.212: "NR; Multiplexing and channel coding"

- 3GPP TS 38.213: "NR; Physical layer procedures for control "