3GPP TSG-RAN WG4 Meeting#96-e                                 R4-2010942

E-meeting,17th Aug – 28th Aug, 2020

Agenda item:	11.1.3.3

Source: 	ZTE Corporation

Title: 	UL simulation results for 6425-7125MHz and 10-10.5GHz

Document for:	Discussion

Introduction

In the RAN4#95-e-Bis meeting, WF on simulation assumption [1] and WF for BS antenna parameters [2-3] were approved for coexistence evaluation work. Therefore in this contribution, we share some updated simulation results for further discussion.

Discussion

In the following sections, simulation results for both coordinated case and uncoordinated case in urban macro scenarios are provided based on the approved simulation assumption [1]. In addition, baseline ACIR for demonstration of legacy FR1 NR system is assuming that using ACLR=30dBc at UE and ACS=46dBc at BS) at 7GHz and 10GHz, relative offset to baseline ACIR is used for further comparison to check whether relative relaxed requirements could be achieved.

## 2.1. Uplink

In the WF [1], it was agreed to have 3 user scheduled in the uplink and however corresponding uplink ACIR model was not explicitly described yet which is also quite critical for evaluation work in uplink coexistence simulation study. Similar as what has been agreed for uplink ACIR model for LTE coexistence study captured in TR 36.942, some minor modifications is made as following considering channel bandwidth 100MHz and 30KHz SCS for both aggressive system and victim system.

Figure 2.2-1 Uplink ACIR model

Table 2.2-1  ACIR value

| Frequency offset between aggressor (91RBs) and victim (91RBs) | ACIR value |
| --- | --- |
| 0-90 RBs | 30 + X |
| 91-181RBs | 43 + X |
| >181RBs | 43+ X |

Table 2: Uplink throughput loss of victim BS at 7GHz and 10GHz

| Simulation scenarios | Throughput loss | Relative ACIR offset |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | -1 | -2 | -3 | -4 | -5 |
| Coordinated case | Average throughput loss (7GHz) | 0.59 | 0.68 | 0.80 | 0.92 | 1.06 | 1.22 |
|  | 5%-tile throughput loss (7GHz) | 1.32 | 1.86 | 2.27 | 2.93 | 3.66 | 4.33 |
| Uncoordinated case | Average throughput loss (7GHz) | 0.74 | 0.85 | 0.98 | 1.13 | 1.29 | 1.49 |
|  | 5%-tile throughput loss (7GHz) | 1.82 | 2.10 | 2.62 | 3.31 | 4.06 | 4.91 |
| Coordinated case | Average throughput loss (10GHz) | 0.59 | 0.68 | 0.79 | 0.92 | 1.08 | 1.25 |
|  | 5%-tile throughput loss (10GHz) | 1.69 | 2.05 | 2.57 | 2.99 | 3.37 | 3.86 |
| Uncoordinated case | Average throughput loss (10GHz) | 0.75 | 0.86 | 1.01 | 1.17 | 1.36 | 1.54 |
|  | 5%-tile throughput loss (10GHz) | 2.33 | 2.73 | 3.21 | 4.21 | 5.01 | 6.18 |

Observation 1: for 7GHz, the uplink throughput loss of the victim BS in the urban macro scenario can still be limited to 5% with uplink ACIR offsets of -5dB,;

Observation 2: for 10GHz, the uplink throughput loss of the victim BS in the urban macro scenario can still be limited to 5% with uplink ACIR offsets of -4 to -5dB;

Conclusions

In this contribution, we shared initial simulation results for 6.425-7.125GHz and 10.0-10.5GHz for further discussion.

Observation 1: for 7GHz, the uplink throughput loss of the victim BS in the urban macro scenario can still be limited to 5% with uplink ACIR offsets of -5dB,;

Observation 2: for 10GHz, the uplink throughput loss of the victim BS in the urban macro scenario can still be limited to 5% with uplink ACIR offsets of -4 to -5dB;

References

- R4-2008927, WF on Simulation Assumptions for the SI on 6.425-7.125GHz and 10.0-10.5GHz, approved.

- R4-xxx•WF R4-2008925 in BS parameters

- WF R4-2008926 in BS antenna parameters

- TR 36.942

- R4-2006108,Urban Macro Downlink Coexistence Simulation Results for Frequency Ranges 6.425-7.125GHz and 10.0-10.5GHz, Noted