3GPP TSG-RAN WG4 Meeting#96-e                                 R4-2010943

E-meeting,25th Aug – 5th Aug, 2020

Agenda item:	6.4.2

Source: 	ZTE Corporation

Title: 	Impacts on BS RF requirement of new introduced numerology

Document for:	Approval

Introduction

This contribution focused on the impacts on BS RF requirement due to new introduced numerology in R16 WID.

Similar as discussion in R14 FeMBMS, EVM window length in TS 36.104 Annex E should be updated considering

the introduction of the new 2.5kHz and 0.37kHz. According to WF [1] that FFT size should have factors of 2 and/or 3 based on RAN1 agreement, therefore some updates for FFT side for 0.37KHz SCS are necessary.

Discussion

## 2.1. Unit of Measurement and observation periods for EVM measurement

For 15kHz SCS, EVM measurement is calculated based on 1 subframe in time domain as measurement unit and

then average over all subframes in a measurement period at least 10ms for EVM smoothing due to some

ON-OFF/OFF-ON transition period in case. Basically speaking, 1ms observation period is used for timing/freq

estimation and channel estimation.

For 15kHz SCS, one EVM measurement is calculated based on 168 REs and for evaluation of sample timing offset

in case of FFT window length of 2048 samples per OFDM symbol, 140 FFTs (14 per frame) covering 286720 out

the acquired number of samples (307200 samples in 10 subframes) are considered [1].

For 2.5kHz SCS as shown in Figure 1, one EVM measurement is calculated based on 144 REs where 180kHz over

1 subframe includes two slots and one symbol per slot and for evaluation of sample timing offset in case of FFT

window length of 12288 samples per OFDM symbol, 20 FFTs (2 per subframe) covering 245760 out the acquired

number of samples (307200 samples in 10 subframes) are considered.

Figure 1. SCS 2.5kHz with CP 100us for mobility up to 250km/h

Taking into account that the number of REs associated to one EVM value and the FFT samples impacting estimation of the FFT window timing for 15 kHz and 2.5 kHz subcarrier spacing are of the same order of magnitude, it is proposed not to change the observation period and measurement intervals for EVM measurement for 2.5kHz SCS.

For 0.37kHz SCS as shown in Figure 2, one EVM measurement is calculated based on 486 REs where 180kHz per OFDM symbol 3ms is considered and for evaluation of sample timing offset in case of FFT window length of 81920 samples per OFDM symbol, 3 FFTs per frame covering 245760 out the acquired number of samples (303104 samples in 10 subframes) are considered.

Taking into account that OFDM symbol duration of 0.37kHz SCS and the FFT samples impacting estimation of the FFT window timing for 15 kHz and 0.37 kHz subcarrier spacing are of the same order of magnitude, it is proposed not to change measurement intervals for EVM measurement for 0.37kHz SCS and change the observation period as 3ms.

Figure 2. 0.37kHz SCS with CP 300us for rooftop reception

Based on the above considerations, proposals are made as following:

Proposal 1: for basic unit of measurement, propose to adopt option 1: 1 slot per 38.104 definition

Proposal 2: for measurement interval, propose to adopt option 1 10ms;

## 2.2. EVM window length

During the last meeting, it has been agreed that EVM window ratio should reuse the value of 15KHz with extended CP in WF [1]; Based on that agreement on EVM window length, detailed calculation for PMCH/PDSCH FFT and CP FFT and EVM window are listed in the following table.

Table 1. R16 LTE based 5G terrestrial broadcast SCS 2.5kHz

| BW [MHz] | SCS [kHz] | FFT2 | FFT_update | Ratio1 | CP [us] | CP FFT | CP FFT update | EVM window3 | EVM window ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 2.5 | 560 | 768 | 0.729166667 | 100 | 192 | 192 | 168 | 87.5 |
| 3 | 2.5 | 1200 | 1536 | 0.78125 | 100 | 384 | 384 | 348 | 90.6 |
| 5 | 2.5 | 2000 | 3072 | 0.651041667 | 100 | 768 | 768 | 746 | 96.9 |
| 10 | 2.5 | 4000 | 6144 | 0.651041667 | 100 | 1536 | 1536 | 1500 | 97.7 |
| 15 | 2.5 | 6000 | 9216 | 0.651041667 | 100 | 2304 | 2304 | 2268 | 98.4 |
| 20 | 2.5 | 8000 | 12288 | 0.651041667 | 100 | 3072 | 3072 | 3024 | 98.4 |
| Note 1: Ratio is FFT/FFT_selected to avoid too high FFT occupancy resulting in side truncation effect. Note 2 : FFT in the 3rd column is determined by BW/SCS. Note 3: EVM window length should be even number, therefore some rounding adjustment is added. |  |  |  |  |  |  |  |  |  |

Table 2. R16 LTE based 5G terrestrial broadcast SCS 0.37kHz

| BW [MHz] | SCS [kHz] | FFT2 | FFT_update | Ratio1 | CP [us] | CP FFT | CP FFT update | EVM window3 | EVM window ratio |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 0.37 | 3783.7838 | 6144 | 0.6159 | 300 | 681.98 | 682 | 596 | 87.5 |
| 3 | 0.37 | 8108.1081 | 12288 | 0.6598 | 300 | 1363.97 | 1364 | 1236 | 90.6 |
| 5 | 0.37 | 13513.5135 | 18432 | 0.7332 | 300 | 2045.95 | 2046 | 1982 | 96.9 |
| 10 | 0.37 | 27027.0270 | 36864 | 0.7332 | 300 | 4091.90 | 4092 | 3998 | 97.7 |
| 15 | 0.37 | 40540.5405 | 65536 | 0.6186 | 300 | 7274.50 | 7276 | 7160 | 98.4 |
| 20 | 0.37 | 54054.0541 | 73728 | 0.7332 | 300 | 8183.81 | 8184 | 8054 | 98.4 |
| Note 1: Ratio is FFT/FFT_selected to avoid too high FFT occupancy resulting in side truncation effect. Note 2 : FFT in the 3rd column is determined by BW/SCS. Note 3: EVM window length should be even number, therefore some rounding adjustment is added. |  |  |  |  |  |  |  |  |  |

Proposal 3: to adopt the EVM window requirement in Table 1 and Table 2.

## 2.3. Estimation window/ Observation period

For 2.5kHz SCS for support of mobility up to 250km/h, RS pattern is shown in the Figure 3.The timing error estimation could be conducted in the frequency domain via single symbol, therefore 1 symbol could be used to enable that function. For the frequency error estimation, it could be conducted via {1st symbol and 2rd symbol}, therefore based on the following RS illustration, maybe 2 slots should be enabled for frequency error estimation which should be corresponding to 1 subframes (1ms).

Figure 3. RS pattern of Df=2 and Dt=2 for the support of mobility up to 250km/h

For 0.37kHz SCS for support of rooftop reception, there are two types of MBSFN reference which is up to network configure and its applicability rule is still under discussion in RAN4.

For type with Df=3 and Dt=4, similar as mentioned for 2.5kHz SCS, timing error estimation could be conducted in frequency domain via single symbol, therefore 1 symbol could be used to enable that function. For the frequency error estimation, it could be conducted via {1st symbol and 2rd symbol}, therefore based on the following RS pattern illustration in Figure 4, then 2 slots/symbols should be enabled for frequency error estimation which is corresponding to 6ms.

Figure 4. RS patter for Df=3 and Dt=4 for rooftop reception

For type with Df=3 and Dt=2, for the frequency error estimation, it could be conducted via {1st symbol and 2nd symbol}, therefore based on the following RS pattern illustration in Figure 5, then 2 slots/symbols should be enabled for frequency error estimation which is corresponding to 6ms.

Figure 5. RS patter for Df=3 and Dt=2 for rooftop reception

Based on the above considerations, it’s proposed that

Proposal 4:

For 2.5kHz, observation period for frequency error and timing error as 1ms;

For 0.37kHz with MBSFN type 1/2, observation period for frequency error and timing error as 6ms;

Conclusions

In this contribution, we shared some further inputs on  definition and proposals are made as following:

Proposal 1: for 2.5kHz SCS, not to change the observation period and measurement intervals for EVM

measurement.

Proposal 2: for 0.37kHz SCS, not to change measurement intervals for EVM measurement and change observation period from 1ms to 3ms.

Proposal 3: to adopt the EVM window requirement in Table 1 and Table 2.

Proposal 4:

For 2.5kHz, observation period for frequency error and timing error as 1ms;

For 0.37kHz with MBSFN type 1/2, observation period for frequency error and timing error as 6ms;

References

- R4-2008869,WF on the measurement interval and observation time for frequency/time correction for 2.5kHz and 0.37kHz, ZTE, approved.

- R4-2007396,Impacts on BS RF requirement of new introduced numerology, Noted