3GPP TSG-RAN WG4 Meeting#96-e                                 R4-2010954

E-meeting,17th Aug – 28th Aug, 2020

Agenda item:	7.4.2.2.1

Source: 	ZTE Corporation

Title: 	IAB-MT REFSENS and FRC design

Document for:	Approval

Introduction

In the RAN4#95-e meeting, there were some further discussion how to implement IAB-MT FRC and WF was approved as following, therefore in this contribution, we want to share some further inputs on that.

Sub topic 1-4: FRC to be used by IAB-MT

All companies agree option 2 is ok so:

Agreement: Selected UE FRC can be for IAB-MT based the same criteria as BS

Companies are encouraged to bring FRC proposals in next RAN4 meeting.

Discussion

  - . FR1

- As it was agreed that selected UE FRC can be for IAB-MT based the same criteria as BS. For FRC design for FR1 BS, it is different from UE FRC spanning over the whole frequency range, therefore we need to review how FR1 BS FRC was designed firstly in Rel-15 and then how to select UE FRC for IAB-MT. As shown in the following Table 1-1, for 5,10,15MHz with 15KHz, the corresponding FRC G-FR1-A1-1 is based 5MHz with 15KHz and so on.

Table 1-1: FR1 NR BS reference sensitivity levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | corresponding channel for FRC |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | G-FR1-A1-1 [25] | 5MHz |
| 10, 15 | 30 | G- FR1-A1-2 [11] | 5MHz |
| 10, 15 | 60 | G- FR1-A1-3 [11] | 10MHz |
| 20, 25, 30, 40, 50 | 15 | G- FR1-A1-4 [106] | 20MHz |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | G- FR1-A1-5 [51] | 20MHz |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | G- FR1-A1-6 [24] | 20MHz |

Based on the above approach, selected UE FRC for FR1 IAB-MT are listed as following

Table 1-2: FR1 IAB-MT reference sensitivity levels

| IAB-MT channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | corresponding channel for FRC |
| --- | --- | --- | --- |
| 5, 10, 15 | 15 | I-FR1-A1-1 [25] | 5MHz |
| 10, 15 | 30 | I- FR1-A1-2 [11] | 5MHz |
| 10, 15 | 60 | I- FR1-A1-3 [11] | 10MHz |
| 20, 25, 30, 40, 50 | 15 | I- FR1-A1-4 [106] | 20MHz |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 30 | I- FR1-A1-5 [51] | 20MHz |
| 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | 60 | I- FR1-A1-6 [24] | 20MHz |

Table 1-3 Fixed Reference Channel for FR1 IAB-MT Receiver Requirements

| Parameter | Unit | Value |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | I-FR1-A1-1 | I-FR1-A1-2 | I-FR1-A1-3 | I-FR1-A1-4 | I-FR1-A1-5 | I-FR1-A1-6 |  |  |
| [Channel bandwidth] | [MHz] | 5 | 20 | 5 | 20 | 10 | 20 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 30 | 30 | 60 | 60 |  |  |
| Subcarrier spacing configuration |  | 0 | 0 | 1 | 1 | 2 | 2 |  |  |
| Allocated resource blocks |  | 25 | 106 | 11 | 51 | 11 | 24 |  |  |
| Subcarriers per resource block |  | 12 | 12 | 12 | 12 | 12 | 12 |  |  |
| Allocated slots per Frame |  | 5 | 5 | 13 | 13 | 26 | 26 |  |  |
| MCS Index |  | 4 | 4 | 4 | 4 | 4 | 4 |  |  |
| Modulation |  | QPSK | QPSK | QPSK | QPSK | QPSK | QPSK |  |  |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 | 1/3 |  |  |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |  |  |
| For Slots 2,5,6,7 | Bits | 1672 | 6912 | 736 | 3368 | 736 | 1608 |  |  |
| Transport block CRC | Bits | 16 | 24 | 16 | 16 | 16 | 16 |  |  |
| LDPC base graph |  | 2 | 1 | 2 | 2 | 2 | 2 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | CBs | N/A | N/A | N/A | N/A | N/A | N/A |  |  |
| For Slots 2,5,6,7 | CBs | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| Binary Channel Bits per Slot |  |  |  |  |  |  |  |  |  |
| For Slots 0,1,3,4,8,9 | Bits | N/A | N/A | N/A | N/A | N/A | N/A |  |  |
| For Slots 2,5,6,7 | Bits | 5400 | 22896 | 2376 | 11016 | 2376 | 5184 |  |  |
| Max. Throughput averaged over 1 frame | Mbps | 0.669 | 2.765 | 0.810 | 3.705 | 1.766 | 3.859 |  |  |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |  |  |  |  |  |

Additional test parameters for TDD. e.g.DL-UL pattern configuration and HARQ number for FR1 IAB-MT could be reused from Table A.3.1-1 in TS 38.101-1 and test parameters of CORESET configuration, PDSCH, DMRS, PT-TS configuration for FR1 IAB-MT could also be reused from Table A.3.3.1-1 in TS 38.101-1.

  - . FR2

- As it was agreed that selected UE FRC can be for IAB-MT based the same criteria as BS. For FRC design for FR2 BS, it is also different from UE FRC spanning over the whole frequency range.

For example for 50,100,200MHz with 60KHz SCS, then BS FRC G-FR2-A1-1 are based on 50MHz with 60KHz SCS,66PRB, therefore 100MHz with 60KHz could be tested by two G-FR2-A1-1 FRCs and 200MHz with 60KHz could be tested by four G-FR2-A1-1 FRCs.

For 50,100,200,400MHz with 120KHz SCS, as PRB number of 100MHz, 200MHz, 400MHz is not multiples of that of 50MHz while PRB number of 200MHz, 400MHz is multiple of that of 100MHz, therefore one specific FRC G-FR2-A1-2 is designed for 50MHz and one FRC G-FR2-A1-3 is designed for 100MHz, 200MHz, 400MHz to avoid overlapping PRB test.

To adopt the above criteria to select UE FRC for IAB-MT, then IAB-MT REFSENS and FRC could be defined in the following table:

Table 2-1: FR2 IAB-MT OTA Reference sensitivity requirement

| IAB-MT channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | FRC | Corresponding Channel for FRC |
| --- | --- | --- | --- |
| 50, 100, 200 | 60 | I-FR2-A1-1 [66] | 50MHz |
| 50 | 120 | I-FR2-A1-2 [32] | 50MHz |
| 100, 200, 400 | 120 | I-FR2-A1-3 [66] | 100MHz |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Table 2-2 Fixed Reference Channel for IAB-MT FR2 Receiver Requirements

| Parameter | Unit | Value |  |  |
| --- | --- | --- | --- | --- |
| Reference channel |  | I-FR2-A2-1 | I-FR2-A2-2 | I-FR2-A2-3 |
| [Channel bandwidth] | [MHz] | [50] | [50] | [100] |
| Subcarrier spacing configuration |  | 2 | 3 | 3 |
| Allocated resource blocks |  | 66 | 32 | 66 |
| Subcarriers per resource block |  | 12 | 12 | 12 |
| Allocated slots per Frame |  | 23 | 47 | 47 |
| MCS index |  | 4 | 4 | 4 |
| Modulation |  | QPSK | QPSK | QPSK |
| Target Coding Rate |  | 1/3 | 1/3 | 1/3 |
| Maximum number of HARQ transmissions |  | 1 | 1 | 1 |
| Information Bit Payload per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,39} | Bits | 4224 | 2088 | 4224 |
| Transport block CRC | Bits | 24 | 16 | 24 |
| LDPC base graph |  | 1 | 2 | 1 |
| Number of Code Blocks per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} | CBs | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,39} | CBs | 1 | 1 | 1 |
| Binary Channel Bits Per Slot |  |  |  |  |
| For Slots 0 and Slot i, if mod(i, 5) = {3,4} for i from {0,…,39} | Bits | N/A | N/A | N/A |
| For Slot i, if mod(i, 10) = {0,1,2} for i from {1,…,39} | Bits | 14256 | 6912 | 14256 |
| Max. Throughput averaged over 1 frame | Mbps | 9.715 | 9.814 | 19.853 |
| NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.3.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot 0 of each frame NOTE 4: Slot i is slot index per frame |  |  |  |  |

Additional test parameters for TDD. e.g.DL-UL pattern configuration and HARQ number for FR1 IAB-MT could be reused from Table A.3.3.1-1 in TS 38.101-2 and test parameters of CORESET configuration, PDSCH, DMRS, PT-TS configuration for FR1 IAB-MT could also be reused from Table A.3.1-1 in TS 38.101-2.

Conclusions

In this contribution, we shared more further inputs on IAB-MT FRC design and proposals are made as following:

Proposal : adopt the FRC in this contribution for IAB-MT.

References

- R4-2008785,WF on IAB-MT reference sensitivity, Huawei, Hisilicon