3GPP TSG-RAN WG4 Meeting #96-e	R4-2011160

Electronic Meeting, 17 – 28 August, 2020

Title: 	Simulation results for UE Rx-Tx time difference

Source: 	Huawei, HiSilicon

Agenda item:	7.7.2.1.3

Document for:	Discussion

Introduction

In RAN4#95-e, the simulation assumption for UE Rx-Tx time difference has been agreed in [1].

In this paper we will provide our simulation results for UE Rx-Tx time difference measurement.

Discussion

  - Simulation assumption

A subset of cases have been simulated from the simulation assumptions in [1].

- Only Synchronous case with time shifts <0, 0, 3 us> is simulated

- PRS periodicity is 160ms

- 100% RE utilization in non-PRS symbols

- All listed SCS and BW are simulated in all listed channels

- All comb sizes are simulated

- Repetition factor 1,2,4 are simulated with PRS-ResourceTimeGap = 1

Simulation results

Table 1-3 show the simulation results for Rx-Tx time difference accuracy with different SCS, which is 90%-tile value in the CDF of Rx-Tx time difference estimation error in Tc.

Table 1: Rx-Tx time difference accuracy for 15kHz

| RB num | Comb | Rep | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 52 | 2 | 1 | 57.1 | 57.9 | 58.5 | 61.5 | 130.4 | 111.7 | 136.7 | 176.1 | 177.9 | 392.6 | 411.9 | 404.6 |
|  |  | 2 | 57.6 | 57.5 | 57.1 | 59.7 | 61.9 | 62.5 | 122.4 | 137.0 | 148.5 | 386.9 | 384.5 | 384.4 |
|  |  | 4 | 57.0 | 58.0 | 57.8 | 59.3 | 59.7 | 61.2 | 117.9 | 155.1 | 139.2 | 381.3 | 382.3 | 388.8 |
|  | 4 | 1 | 56.9 | 57.4 | 57.4 | 60.7 | 106.9 | 109.4 | 135.6 | 171.0 | 166.4 | 377.7 | 423.3 | 403.9 |
|  |  | 2 | 57.6 | 57.6 | 58.2 | 60.2 | 61.3 | 62.2 | 135.9 | 153.3 | 141.0 | 391.7 | 386.5 | 383.0 |
|  |  | 4 | 58.0 | 57.5 | 57.6 | 60.5 | 62.3 | 60.4 | 126.1 | 136.0 | 138.2 | 379.3 | 379.0 | 398.5 |
|  | 6 | 1 | 58.5 | 57.3 | 57.0 | 60.7 | 122.2 | 116.6 | 126.4 | 168.7 | 184.7 | 379.9 | 420.1 | 416.0 |
|  |  | 2 | 58.0 | 57.3 | 57.7 | 61.2 | 62.5 | 62.8 | 133.6 | 139.7 | 151.1 | 390.4 | 393.4 | 391.5 |
|  |  | 4 | 58.2 | 57.8 | 57.6 | 60.2 | 61.2 | 61.7 | 129.8 | 137.3 | 153.1 | 383.1 | 393.1 | 392.7 |
|  | 12 | 1 | 57.7 | 57.5 | 57.4 | 59.8 | 116.5 | 116.1 | 129.0 | 176.5 | 163.4 | 365.9 | 417.4 | 420.5 |
|  |  | 2 | 57.8 | 57.9 | 57.8 | 60.3 | 62.4 | 62.8 | 132.6 | 135.6 | 134.7 | 385.2 | 393.1 | 377.9 |
|  |  | 4 | 57.5 | 58.3 | 56.2 | 61.1 | 60.7 | 61.5 | 117.7 | 138.3 | 133.9 | 401.0 | 386.2 | 378.1 |
| 104 | 2 | 1 | 28.9 | 29.0 | 28.7 | 48.8 | 57.5 | 48.1 | 83.4 | 83.7 | 85.5 | 390.4 | 396.7 | 385.6 |
|  |  | 2 | 28.7 | 28.8 | 29.0 | 41.7 | 56.6 | 47.9 | 85.0 | 87.4 | 83.4 | 393.7 | 390.5 | 390.9 |
|  |  | 4 | 29.1 | 28.6 | 29.1 | 36.1 | 41.7 | 42.5 | 83.2 | 84.8 | 83.8 | 390.5 | 391.7 | 393.0 |
|  | 4 | 1 | 28.9 | 28.6 | 28.6 | 44.3 | 56.9 | 59.2 | 81.4 | 84.4 | 84.8 | 391.2 | 388.3 | 391.2 |
|  |  | 2 | 28.6 | 28.7 | 28.5 | 43.8 | 52.4 | 40.5 | 83.4 | 83.7 | 81.7 | 388.4 | 390.5 | 390.0 |
|  |  | 4 | 28.9 | 28.4 | 28.6 | 45.6 | 40.5 | 44.9 | 81.1 | 83.6 | 82.7 | 391.3 | 384.6 | 391.2 |
|  | 6 | 1 | 29.3 | 28.4 | 28.1 | 38.8 | 51.3 | 63.5 | 80.0 | 86.2 | 86.5 | 389.2 | 391.1 | 391.0 |
|  |  | 2 | 28.7 | 28.0 | 29.3 | 46.8 | 53.2 | 44.4 | 83.2 | 81.9 | 82.6 | 389.2 | 392.8 | 392.0 |
|  |  | 4 | 28.9 | 29.5 | 28.7 | 34.5 | 47.0 | 38.2 | 81.1 | 83.6 | 84.0 | 386.7 | 393.1 | 391.0 |
|  | 12 | 1 | 29.1 | 28.5 | 29.5 | 40.4 | 52.2 | 56.1 | 83.9 | 84.9 | 84.6 | 392.9 | 391.5 | 396.5 |
|  |  | 2 | 29.2 | 29.1 | 29.2 | 35.1 | 34.5 | 41.4 | 82.0 | 85.2 | 83.6 | 390.1 | 388.3 | 393.5 |
|  |  | 4 | 28.6 | 29.0 | 28.3 | 33.3 | 42.3 | 47.4 | 78.6 | 84.9 | 83.9 | 392.6 | 391.7 | 389.9 |
| 268 | 2 | 1 | 14.5 | 14.1 | 14.6 | 44.8 | 45.0 | 45.4 | 67.0 | 67.3 | 63.7 | 385.7 | 387.9 | 387.9 |
|  |  | 2 | 14.3 | 14.7 | 14.4 | 44.9 | 45.5 | 45.1 | 65.4 | 66.4 | 65.0 | 387.6 | 388.8 | 387.2 |
|  |  | 4 | 14.4 | 14.5 | 14.4 | 44.8 | 45.4 | 45.5 | 65.8 | 67.4 | 66.9 | 385.5 | 388.7 | 387.0 |
|  | 4 | 1 | 14.6 | 14.3 | 14.5 | 44.8 | 45.4 | 44.6 | 66.7 | 66.3 | 67.0 | 389.7 | 386.9 | 387.5 |
|  |  | 2 | 14.3 | 14.7 | 14.5 | 44.8 | 44.4 | 45.2 | 65.8 | 68.5 | 63.2 | 387.7 | 386.8 | 387.6 |
|  |  | 4 | 14.3 | 14.2 | 14.4 | 45.1 | 44.8 | 44.8 | 67.7 | 60.6 | 64.7 | 383.6 | 386.8 | 388.7 |
|  | 6 | 1 | 14.4 | 14.3 | 14.6 | 44.8 | 45.1 | 44.5 | 64.4 | 65.6 | 65.2 | 385.9 | 388.3 | 389.8 |
|  |  | 2 | 14.5 | 14.6 | 14.2 | 45.4 | 45.0 | 45.3 | 64.8 | 60.6 | 66.0 | 389.8 | 386.8 | 387.9 |
|  |  | 4 | 14.4 | 14.6 | 14.5 | 44.5 | 44.7 | 44.8 | 66.8 | 67.1 | 64.6 | 389.6 | 386.2 | 389.3 |
|  | 12 | 1 | 14.4 | 14.6 | 14.6 | 45.0 | 44.6 | 45.1 | 65.1 | 65.4 | 65.4 | 389.3 | 388.2 | 387.1 |
|  |  | 2 | 14.5 | 14.2 | 14.4 | 44.8 | 44.8 | 45.1 | 64.1 | 66.8 | 65.2 | 386.6 | 387.4 | 386.5 |
|  |  | 4 | 14.1 | 14.4 | 14.5 | 44.7 | 45.2 | 44.8 | 67.3 | 67.0 | 64.3 | 384.6 | 386.4 | 387.5 |

Table 2: Rx-Tx time difference accuracy for 30kHz

| RB num | Comb | Rep | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN |  |  | TDL-A |  |  | TDL-B |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 48 | 2 | 1 | 28.5 | 29.3 | 29.9 | 65.5 | 85.3 | 80.4 | 79.7 | 95.9 | 117.9 | 388.3 | 408.0 | 406.2 |
|  |  | 2 | 28.6 | 28.8 | 28.8 | 57.2 | 63.0 | 63.9 | 81.5 | 83.1 | 83.4 | 388.7 | 388.8 | 390.5 |
|  |  | 4 | 29.1 | 28.7 | 28.5 | 54.4 | 57.9 | 58.6 | 81.6 | 85.8 | 79.9 | 388.1 | 389.4 | 390.7 |
|  | 4 | 1 | 29.1 | 29.2 | 29.6 | 60.1 | 78.9 | 81.7 | 82.6 | 95.4 | 267.9 | 391.3 | 406.8 | 406.2 |
|  |  | 2 | 29.5 | 29.0 | 28.7 | 60.4 | 63.9 | 68.3 | 83.5 | 86.0 | 86.5 | 386.1 | 389.4 | 387.2 |
|  |  | 4 | 28.9 | 28.9 | 28.9 | 55.4 | 58.3 | 55.0 | 81.3 | 82.8 | 83.0 | 388.2 | 389.9 | 387.6 |
|  | 6 | 1 | 29.1 | 29.6 | 29.5 | 58.0 | 86.4 | 72.0 | 85.8 | 119.0 | 93.3 | 393.8 | 409.8 | 412.2 |
|  |  | 2 | 29.5 | 29.4 | 28.7 | 49.3 | 59.6 | 65.6 | 83.6 | 86.2 | 86.4 | 392.6 | 387.8 | 389.9 |
|  |  | 4 | 28.8 | 28.6 | 28.7 | 53.0 | 59.3 | 64.7 | 79.6 | 78.9 | 80.7 | 390.6 | 388.7 | 388.6 |
|  | 12 | 1 | 28.6 | 30.1 | 29.4 | 49.4 | 80.7 | 76.2 | 81.2 | 112.5 | 111.7 | 394.0 | 405.2 | 406.0 |
|  |  | 2 | 29.0 | 29.1 | 28.6 | 55.3 | 65.0 | 67.6 | 79.4 | 84.5 | 83.7 | 388.6 | 387.0 | 394.4 |
|  |  | 4 | 28.7 | 28.4 | 29.1 | 54.1 | 57.9 | 56.1 | 85.7 | 79.2 | 82.7 | 391.1 | 392.7 | 393.5 |
| 132 | 2 | 1 | 14.3 | 14.2 | 14.4 | 45.0 | 45.6 | 45.4 | 66.6 | 66.8 | 63.4 | 386.2 | 387.4 | 387.1 |
|  |  | 2 | 14.3 | 14.5 | 14.4 | 45.0 | 45.1 | 44.6 | 65.6 | 63.6 | 65.5 | 388.0 | 386.8 | 389.2 |
|  |  | 4 | 14.2 | 14.5 | 14.3 | 45.0 | 44.5 | 45.5 | 66.4 | 63.9 | 64.5 | 387.5 | 387.8 | 386.9 |
|  | 4 | 1 | 14.3 | 14.4 | 14.7 | 45.2 | 44.5 | 45.3 | 64.0 | 67.1 | 65.8 | 388.6 | 387.9 | 388.5 |
|  |  | 2 | 14.6 | 14.2 | 14.7 | 45.3 | 44.6 | 45.0 | 67.9 | 64.5 | 65.8 | 389.5 | 387.5 | 386.6 |
|  |  | 4 | 14.3 | 14.6 | 14.6 | 45.2 | 44.6 | 44.6 | 65.3 | 66.2 | 64.2 | 387.4 | 388.3 | 387.8 |
|  | 6 | 1 | 14.1 | 14.5 | 14.4 | 44.1 | 44.9 | 45.4 | 64.0 | 66.0 | 63.2 | 386.6 | 387.5 | 388.3 |
|  |  | 2 | 14.3 | 14.4 | 14.4 | 44.8 | 44.9 | 45.1 | 67.1 | 65.3 | 67.2 | 388.7 | 387.4 | 389.8 |
|  |  | 4 | 14.3 | 14.4 | 14.6 | 44.6 | 44.9 | 44.9 | 68.4 | 65.1 | 64.7 | 388.9 | 387.0 | 387.6 |
|  | 12 | 1 | 14.6 | 14.3 | 14.4 | 44.8 | 44.9 | 45.3 | 65.0 | 66.8 | 67.1 | 386.9 | 389.4 | 387.6 |
|  |  | 2 | 14.3 | 14.3 | 14.3 | 45.0 | 45.1 | 44.7 | 64.9 | 67.0 | 64.9 | 385.9 | 387.6 | 387.7 |
|  |  | 4 | 14.3 | 14.3 | 14.4 | 44.7 | 45.1 | 44.7 | 65.1 | 64.6 | 64.5 | 386.6 | 388.8 | 388.8 |
| 272 | 2 | 1 | 7.3 | 7.1 | 7.1 | 27.9 | 26.5 | 26.4 | 71.4 | 67.2 | 68.1 | 372.3 | 373.1 | 371.7 |
|  |  | 2 | 7.3 | 7.1 | 7.2 | 29.7 | 29.0 | 25.9 | 69.6 | 66.5 | 69.7 | 370.6 | 372.1 | 369.8 |
|  |  | 4 | 7.2 | 7.2 | 7.2 | 25.6 | 27.9 | 27.7 | 66.9 | 64.1 | 70.7 | 372.9 | 373.8 | 370.4 |
|  | 4 | 1 | 7.3 | 7.2 | 7.0 | 28.8 | 28.3 | 28.4 | 66.9 | 69.5 | 69.5 | 368.0 | 369.4 | 370.7 |
|  |  | 2 | 7.1 | 7.2 | 7.2 | 25.2 | 25.7 | 28.1 | 68.3 | 65.5 | 68.4 | 371.0 | 370.5 | 371.1 |
|  |  | 4 | 7.1 | 7.2 | 7.3 | 26.3 | 26.1 | 25.6 | 66.3 | 66.8 | 69.6 | 370.9 | 371.6 | 372.9 |
|  | 6 | 1 | 7.1 | 7.4 | 7.3 | 27.4 | 26.9 | 30.2 | 68.5 | 67.9 | 68.4 | 371.0 | 372.5 | 372.9 |
|  |  | 2 | 7.2 | 7.4 | 7.2 | 27.5 | 26.0 | 27.5 | 69.8 | 67.8 | 64.9 | 369.8 | 371.5 | 368.5 |
|  |  | 4 | 7.3 | 7.2 | 7.2 | 27.1 | 28.5 | 25.9 | 66.7 | 64.9 | 65.5 | 372.7 | 372.2 | 369.8 |
|  | 12 | 1 | 7.3 | 7.3 | 7.2 | 26.5 | 28.2 | 27.4 | 66.3 | 68.8 | 68.4 | 370.5 | 369.3 | 370.3 |
|  |  | 2 | 7.3 | 7.2 | 7.3 | 26.9 | 26.8 | 27.1 | 68.3 | 67.9 | 63.9 | 371.1 | 372.2 | 370.5 |
|  |  | 4 | 7.1 | 7.2 | 7.2 | 30.5 | 26.9 | 27.5 | 66.0 | 68.7 | 66.4 | 372.7 | 369.7 | 371.4 |

Table 3: Rx-Tx time difference accuracy for 120kHz

| RB num | Comb | Rep | Channel |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | SNR=(-6,-13) |  |  |  |  |  | SNR=(-3,-10) |  |  |  |  |  |
|  |  |  | AWGN |  |  | TDL-C |  |  | AWGN |  |  | TDL-C |  |  |
|  |  |  | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 | Cell1 | Cell2 | Cell3 |
| 48 | 2 | 1 | 14.4 | 14.3 | 14.3 | 66.3 | 79.7 | 77.5 | 14.2 | 14.6 | 14.7 | 67.7 | 69.8 | 68.0 |
|  |  | 2 | 14.4 | 14.5 | 14.4 | 65.2 | 67.8 | 66.9 | 14.2 | 14.2 | 14.4 | 65.2 | 66.4 | 65.8 |
|  |  | 4 | 14.4 | 14.3 | 14.1 | 67.9 | 65.1 | 67.4 | 14.4 | 14.3 | 14.3 | 67.0 | 66.6 | 66.5 |
|  | 4 | 1 | 14.4 | 14.5 | 14.4 | 64.3 | 79.1 | 78.8 | 14.1 | 14.4 | 14.4 | 63.9 | 67.8 | 66.8 |
|  |  | 2 | 14.3 | 14.3 | 14.3 | 67.2 | 68.5 | 67.8 | 14.4 | 14.5 | 14.5 | 66.9 | 66.9 | 64.9 |
|  |  | 4 | 14.3 | 14.4 | 14.5 | 63.7 | 64.5 | 65.0 | 14.3 | 14.7 | 14.3 | 66.4 | 67.7 | 65.3 |
|  | 6 | 1 | 14.4 | 14.1 | 14.5 | 65.6 | 78.6 | 77.9 | 14.3 | 14.3 | 14.3 | 65.8 | 65.7 | 68.2 |
|  |  | 2 | 14.4 | 14.8 | 14.5 | 64.9 | 68.1 | 66.5 | 14.2 | 14.1 | 14.2 | 67.8 | 67.7 | 67.5 |
|  |  | 4 | 13.9 | 14.2 | 14.2 | 64.1 | 63.9 | 66.3 | 14.4 | 14.4 | 14.5 | 64.7 | 64.5 | 64.7 |
|  | 12 | 1 | 14.3 | 14.5 | 14.6 | 64.2 | 85.9 | 78.3 | 14.5 | 14.5 | 14.3 | 66.3 | 66.1 | 67.2 |
|  |  | 2 | 14.5 | 14.3 | 14.7 | 67.0 | 67.2 | 66.8 | 14.4 | 14.3 | 14.4 | 65.1 | 65.9 | 62.7 |
|  |  | 4 | 14.3 | 14.3 | 14.7 | 65.5 | 63.8 | 64.1 | 14.5 | 14.5 | 14.5 | 65.9 | 66.9 | 63.2 |
| 132 | 2 | 1 | 7.2 | 7.2 | 7.1 | 82.0 | 83.5 | 82.3 | 7.1 | 7.2 | 7.2 | 81.7 | 82.5 | 81.1 |
|  |  | 2 | 7.3 | 7.2 | 7.2 | 81.8 | 81.1 | 80.8 | 7.3 | 7.3 | 7.1 | 81.5 | 82.7 | 81.9 |
|  |  | 4 | 7.2 | 7.2 | 7.3 | 81.9 | 82.4 | 82.0 | 7.2 | 7.2 | 7.2 | 81.1 | 81.8 | 83.0 |
|  | 4 | 1 | 7.2 | 7.2 | 7.2 | 82.6 | 83.1 | 83.0 | 7.2 | 7.2 | 7.1 | 82.7 | 82.2 | 83.2 |
|  |  | 2 | 7.3 | 7.3 | 7.2 | 81.2 | 82.2 | 81.8 | 7.2 | 7.4 | 7.2 | 82.0 | 82.2 | 82.1 |
|  |  | 4 | 7.1 | 7.2 | 7.2 | 82.0 | 81.9 | 80.7 | 7.2 | 7.2 | 7.1 | 81.2 | 80.7 | 81.0 |
|  | 6 | 1 | 7.3 | 7.3 | 7.2 | 81.7 | 82.3 | 82.9 | 7.2 | 7.2 | 7.3 | 82.0 | 81.7 | 81.3 |
|  |  | 2 | 7.1 | 7.3 | 7.2 | 81.5 | 81.6 | 82.2 | 7.0 | 7.3 | 7.1 | 81.3 | 83.4 | 81.4 |
|  |  | 4 | 7.2 | 7.2 | 7.2 | 81.9 | 81.4 | 81.5 | 7.1 | 7.2 | 7.3 | 81.9 | 81.4 | 81.6 |
|  | 12 | 1 | 7.3 | 7.2 | 7.1 | 81.2 | 81.5 | 83.5 | 7.4 | 7.1 | 7.0 | 82.0 | 81.1 | 81.8 |
|  |  | 2 | 7.4 | 7.1 | 7.2 | 81.0 | 82.2 | 82.5 | 7.2 | 7.2 | 7.0 | 81.5 | 81.7 | 82.4 |
|  |  | 4 | 7.3 | 7.2 | 7.3 | 82.4 | 82.7 | 80.4 | 7.2 | 7.1 | 7.2 | 82.1 | 82.7 | 80.8 |
| 272 | 2 | 1 | 3.6 | 3.6 | 3.6 | 72.7 | 71.8 | 73.4 | 3.6 | 3.6 | 3.6 | 72.1 | 71.7 | 71.4 |
|  |  | 2 | 3.6 | 3.6 | 3.6 | 72.0 | 72.1 | 72.4 | 3.6 | 3.7 | 3.6 | 72.2 | 72.5 | 72.9 |
|  |  | 4 | 3.6 | 3.6 | 3.5 | 72.5 | 73.5 | 73.3 | 3.6 | 3.5 | 3.6 | 72.0 | 73.0 | 73.7 |
|  | 4 | 1 | 3.7 | 3.5 | 3.6 | 72.4 | 72.2 | 72.9 | 3.5 | 3.6 | 3.5 | 73.8 | 72.4 | 73.0 |
|  |  | 2 | 3.6 | 3.6 | 3.6 | 71.6 | 73.2 | 72.5 | 3.6 | 3.6 | 3.6 | 73.2 | 72.3 | 73.2 |
|  |  | 4 | 3.5 | 3.6 | 3.6 | 72.0 | 72.8 | 72.3 | 3.6 | 3.6 | 3.6 | 72.3 | 72.7 | 70.5 |
|  | 6 | 1 | 3.6 | 3.6 | 3.6 | 72.0 | 72.0 | 73.2 | 3.6 | 3.5 | 3.6 | 71.1 | 73.5 | 73.1 |
|  |  | 2 | 3.7 | 3.5 | 3.6 | 72.6 | 73.0 | 71.4 | 3.6 | 3.6 | 3.6 | 72.3 | 71.8 | 71.5 |
|  |  | 4 | 3.6 | 3.6 | 3.5 | 72.7 | 71.7 | 73.2 | 3.6 | 3.6 | 3.6 | 72.4 | 71.0 | 73.1 |
|  | 12 | 1 | 3.6 | 3.6 | 3.6 | 73.0 | 73.5 | 71.2 | 3.7 | 3.6 | 3.6 | 72.6 | 73.3 | 72.5 |
|  |  | 2 | 3.6 | 3.6 | 3.6 | 72.9 | 72.8 | 73.5 | 3.6 | 3.6 | 3.6 | 71.6 | 72.5 | 72.3 |
|  |  | 4 | 3.6 | 3.6 | 3.6 | 71.8 | 73.5 | 73.2 | 3.6 | 3.5 | 3.6 | 72.4 | 71.5 | 71.2 |

As RSTD and UE Rx-Tx time different are both timing measurements, the observations from the performance simulations are quite similar.

Observation 1: The performance for all comb size are similar.

Observation 2: Repetition up to 4 improves the performance for all RB number in all channels, while the gain is more significant for small RB number at low SNR.

Observation 3: The performance are dependent on the channel profile, but the difference is not much.

Observation 4: The accuracy improves in proportion with BW in RB due to more REs for estimation.

Observation 5: The performance is quite dependent on SNR condition.

Conclusions

In this paper we provided our simulation results for UE Rx-Tx time difference measurement.

Observation 1: The performance for all comb size are similar.

Observation 2: Repetition up to 4 improves the performance for all RB number in all channels, while the gain is more significant for small RB number at low SNR.

Observation 3: The performance are dependent on the channel profile, but the difference is not much.

Observation 4: The accuracy improves in proportion with BW in RB due to more REs for estimation.

Observation 5: The performance is quite dependent on SNR condition.

Reference

- R4-2009127, Link-level simulation assumptions for UE Rx-Tx time difference measurements, Qualcomm Incorporated